# Trigger Board External Interface

Martin Postranecky and Matthew Warren. HEP Group, UCL

## 1. Introduction

The Trigger Board will be a flexible and re-configurable unit.

Although the design allows the inputs and outputs to be re-defined through updated PLD firmware, a definition drawn up from the most likely requirement is used here.

To facilitate more than one unit in a system, each input has a corrosponding output (with a PLD in the middle) to allow daisy-chaining or a master-slave configuration.

In addition, a second trigger input is provided, and a delayed version of the trigger can be output.

A subset of the signals is also available for distribution to other stand-alone components using 40 LVDS outputs.

## 2. Signals

| Name | Type | No. | Pins | Description |
| --- | --- | --- | --- | --- |
| ClkIn | NIM | 1 | 1 | Read-out system master clock (12.5MHz)* |
| ClkOut | NIM | 1 | 1 |  |
| TrigIn | NIM | 1 | 1 | Trigger to start read-out cycle* |
| TrigOut | NIM | 1 | 1 |  |
| Trig2In | NIM | 1 | 1 | Additional trigger input |
| DTrigOut | NIM | 1 | 1 | Delayed trigger (delay programmable) |
| ActIn | NIM | 1 | 1 | Activity - OR of raw trigger sources |
| ActOut | NIM | 1 | 1 |  |
| BeamOnIn | NIM | 1 | 1 | Beam spill in progress (from machine) |
| BeamOnOut | NIM | 1 | 1 |  |
| VetoIn | NIM | 1 | 1 | Inhibits triggering, high when busy |
| VetoOut | NIM | 1 | 1 |  |
| AbortIn | NIM | 1 | 1 | Cancel trigger-veto - resets read-out cycle* |
| AbortOut | NIM | 1 | 1 |  |
| CalIn | NIM | 1 | 1 | Calibration mode enable |
| CalOut | NIM | 1 | 1 |  |
| Aux1In | NIM | 1 | 1 | Spare 1 |
| Aux1Out | NIM | 1 | 1 |  |
| Aux2In | NIM | 1 | 1 | Spare 2 |
| Aux2Out | NIM | 1 | 1 |  |
| LTrigOut | LVDS | 32 | 64 | Fan-out of triggers to external devices ** |
| LVetoOut | LVDS | 4 | 8 | Veto fan-out ** |
| LClkOut | LVDS | 4 | 8 | Clock fan-out ** |

*   These signals can be connected directly to the back-plane, bypassing PLD control.

**  10x4 fan-out - they need not be composed 8-1-1 as shown here.

## 3. Connectors and Cables

All NIM signals will connect via 20 LEMO 00 connectors and on the front panel.

The LVDS fan-out will be via 4 IDC 20-way connectors. The fan-out will effectively be 4 way with all pin 1s connected etc.

It is expected that a twist-and-flat type IDC cable will be used for distribution of these signals.