# Trigger Board VME Interface

Martin Postranecky and Matthew Warren. HEP Group, UCL

## 1. Introduction

With the VME interface we have tried to keep the system as simple, and as generic, as possible for use in the largest variety of crates.

User definable pins on J2 VME connector are used to bus broadcast signals to ReadOut boards on a partially customised backplane.

## 2. Backplane (non-VME) Signals

LVDS will be used to bus signals from the Trigger Board to the ReadOut Boards

| Signal | Spec | No. | Pins | Description |
| --- | --- | --- | --- | --- |
| BClkOut | LVDS | 1 | 2 | Master clock distribution |
| BTrigOut | LVDS | 1 | 2 | Trigger distribution |
| BAbortOut | LVDS | 1 | 2 | Abort distribution |
| BAuxOut | LVDS | 1 | 2 | Spare |
| GAIn(4:0) | TTL | 5 | 5 | Geographical address input* |

## 3. VME Configuration

A32 and A24 simple VME slave only.

D32 and D16 data transfers supported, but only D(15:0) will contain valid data.

Base-address will be specified by geographical-address pins on the back-plane* or via hex-switches on the PCB for non-customised crates.

A single interrupt will be implemented, but DMA is not needed.

The interface will be located on a PLD, using firmware already written by UCL.

## 4. Register Map

| Offset | Name | Description |
| --- | --- | --- |
| 0x00 | Input Enables | Enables the each front-panel signal |
| 0x02 | Output Enables | Enables each front-panel and backplane signal |
| 0x04 | Commands | Generates internal equivalents of external signals |
| 0c06 | Trigger Modes | Enables special trigger modes (as yet undefined) |
| 0x08 | Trigger Delay | Set the delay for the DTrig signal |
| 0x0A | Trigger Window | Configures the enables and window delay-units |
| 0x0C | Status Register | General information |
| 0x0E | Output Test Register | Traps signals on the output path |

* See the ReadOut Board VME interface documentation for more information.