PLDocument:  pld4a.doc

TITLE PAGE   Fri Nov 16 15:28:58 2001

A MACHXL6.2.1.11 - (c) Copyright MINC Incorporated 1987-1998

==============================================================================

TITLE    : ATLAS SCT	TIM, PLD4a:	L1ID Registers

FILE     : pld4a

DATE     : Mon Jun 01 04:54:48 1970

ENGINEER : Martin	Postranecky / Dominic Hayes / John Lane / Matthew Warren

COMPANY  : University College	London

==============================================================================

MODULES  :

Document Generator     	  3.70

File Handler           	  6.2

Language Compiler      	  3.83

Architectural Optimizer	  3.110

Device Lib Scan        	  3.2

Device Library         	  3.17

Device Partition       	  3.21

Device Fusemap         	  3.4

SWITCH VALUES :

(Value in parenthesis represents batch mode switch value)

PLCOMP PRODUCT TERM LIMIT : 128

PLOPT  PRODUCT TERM LIMIT : 128

PLOPT REDUCTION  :   Espresso (1)

NODE GENERATION  :   Procedure Instantiation

Arithmetic and Relational Operators (1)

PLDocument:  pld4a.doc

EQUATIONS    Fri Nov 16 15:28:58 2001

EQUATIONS FOR SYSTEM

INPUT SIGNALS (64) :

nOA_rst

bReadb

bWriteb

Reg10

Reg12

nLEb2

BCntIn[11..0]

RunMode

idTRIG

enL1ID

clk4

L1CntLowStrobe

L1CntHighStrobe

idECR

ENSAECR2

sgbB[7..0]

sgbC[7..0]

sgbD[5..0]

s2_4a[7..0]

s4a_4b[7..0]

OUTPUT SIGNALS (50) :

db[15..0]

sgbA[2..3]

L1_strobe

SAECR2

sgbA[0]

sgbA[1]

L1IDL[11..0]

L1IDH[11..0]

sgbA[7..4]

PHYSICAL NODE SIGNALS (63) :

dboe

qidTRIG

qLow

qHigh

q2L1CntHighStrobe

StrobeRst

L1IDcounter[23..0]

L1IDcounterClk

MaxCount

add_carry-4

add_carry-7

add_carry-10

add_carry-13

add_carry-16

add_carry-19

add_carry-22

RegisterL1IDL[11..0]

RegisterL1IDH[11..0]

REDUCED EQUATIONS:

db[15].EQN = 0 ;  "(1 term, 0 symbols)

db[15].OE = dboe ;  "(1 term, 1 symbol)

db[14].EQN = 0 ;  "(1 term, 0 symbols)

db[14].OE = dboe ;  "(1 term, 1 symbol)

db[13].EQN = 0 ;  "(1 term, 0 symbols)

db[13].OE = dboe ;  "(1 term, 1 symbol)

db[12].EQN = 0 ;  "(1 term, 0 symbols)

db[12].OE = dboe ;  "(1 term, 1 symbol)

db[11].EQN = Reg10*RegisterL1IDL[11]

+ /Reg10*RegisterL1IDH[11] ;  "(2 terms, 3 symbols)

db[11].OE = dboe ;  "(1 term, 1 symbol)

db[10].EQN = Reg10*RegisterL1IDL[10]

+ /Reg10*RegisterL1IDH[10] ;  "(2 terms, 3 symbols)

db[10].OE = dboe ;  "(1 term, 1 symbol)

db[9].EQN = Reg10*RegisterL1IDL[9]

+ /Reg10*RegisterL1IDH[9] ;  "(2 terms, 3 symbols)

db[9].OE = dboe ;  "(1 term, 1 symbol)

db[8].EQN = Reg10*RegisterL1IDL[8]

+ /Reg10*RegisterL1IDH[8] ;  "(2 terms, 3 symbols)

db[8].OE = dboe ;  "(1 term, 1 symbol)

db[7].EQN = Reg10*RegisterL1IDL[7]

+ /Reg10*RegisterL1IDH[7] ;  "(2 terms, 3 symbols)

db[7].OE = dboe ;  "(1 term, 1 symbol)

db[6].EQN = Reg10*RegisterL1IDL[6]

+ /Reg10*RegisterL1IDH[6] ;  "(2 terms, 3 symbols)

db[6].OE = dboe ;  "(1 term, 1 symbol)

db[5].EQN = Reg10*RegisterL1IDL[5]

+ /Reg10*RegisterL1IDH[5] ;  "(2 terms, 3 symbols)

db[5].OE = dboe ;  "(1 term, 1 symbol)

db[4].EQN = Reg10*RegisterL1IDL[4]

+ /Reg10*RegisterL1IDH[4] ;  "(2 terms, 3 symbols)

db[4].OE = dboe ;  "(1 term, 1 symbol)

db[3].EQN = Reg10*RegisterL1IDL[3]

+ /Reg10*RegisterL1IDH[3] ;  "(2 terms, 3 symbols)

db[3].OE = dboe ;  "(1 term, 1 symbol)

db[2].EQN = Reg10*RegisterL1IDL[2]

+ /Reg10*RegisterL1IDH[2] ;  "(2 terms, 3 symbols)

db[2].OE = dboe ;  "(1 term, 1 symbol)

db[1].EQN = Reg10*RegisterL1IDL[1]

+ /Reg10*RegisterL1IDH[1] ;  "(2 terms, 3 symbols)

db[1].OE = dboe ;  "(1 term, 1 symbol)

db[0].EQN = Reg10*RegisterL1IDL[0]

+ /Reg10*RegisterL1IDH[0] ;  "(2 terms, 3 symbols)

db[0].OE = dboe ;  "(1 term, 1 symbol)

dboe.EQN = Reg10*bReadb

+ Reg12*bReadb ;  "(2 terms, 3 symbols)

qidTRIG.D = idTRIG ;  "(1 term, 1 symbol)

qidTRIG.CLK = clk4 ;  "(1 term, 1 symbol)

qidTRIG.RESET = /nOA_rst ;  "(1 term, 1 symbol)

sgbA[2].D = L1CntLowStrobe ;  "(1 term, 1 symbol)

sgbA[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

sgbA[2].CLK(~) = clk4 ;  "(1 term, 1 symbol)

sgbA[3].D = L1CntHighStrobe ;  "(1 term, 1 symbol)

sgbA[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

sgbA[3].CLK(~) = clk4 ;  "(1 term, 1 symbol)

qLow.D = 1 ;  "(1 term, 0 symbols)

qLow.CLK = sgbA[2] ;  "(1 term, 1 symbol)

qLow.RESET = StrobeRst

+ /nOA_rst ;  "(2 terms, 2 symbols)

qHigh.D = qLow ;  "(1 term, 1 symbol)

qHigh.CLK = sgbA[3] ;  "(1 term, 1 symbol)

qHigh.RESET = StrobeRst

+ /nOA_rst ;  "(2 terms, 2 symbols)

q2L1CntHighStrobe.D = sgbA[3] ;  "(1 term, 1 symbol)

q2L1CntHighStrobe.CLK = clk4 ;  "(1 term, 1 symbol)

q2L1CntHighStrobe.RESET = StrobeRst

+ /nOA_rst ;  "(2 terms, 2 symbols)

StrobeRst.D = q2L1CntHighStrobe ;  "(1 term, 1 symbol)

StrobeRst.CLK = clk4 ;  "(1 term, 1 symbol)

StrobeRst.RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1_strobe.D = RunMode*enL1ID*qHigh

+ /RunMode*enL1ID*qidTRIG ;  "(2 terms, 4 symbols)

L1_strobe.CLK = clk4 ;  "(1 term, 1 symbol)

L1_strobe.RESET = StrobeRst

+ /nOA_rst ;  "(2 terms, 2 symbols)

SAECR2.EQN = ENSAECR2*MaxCount ;  "(1 term, 2 symbols)

L1IDcounter[23].T = L1IDcounter[22]*/MaxCount*/Reg10*

/Reg12*add_carry-22

+ L1IDcounter[22]*/MaxCount*RunMode*

add_carry-22

+ L1IDcounter[22]*/MaxCount*

add_carry-22*/bWriteb

+ L1IDcounter[23]*MaxCount

+ L1IDcounter[23]*/Reg10*Reg12*

/RunMode*bWriteb*/db[11]

+ /L1IDcounter[23]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[11] ;  "(6 terms, 9 symbols)

L1IDcounter[23].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[23].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[22].T = L1IDcounter[22]*MaxCount

+ L1IDcounter[22]*/Reg10*Reg12*

/RunMode*bWriteb*/db[10]

+ /L1IDcounter[22]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[10]

+ /MaxCount*/Reg10*/Reg12*

add_carry-22

+ /MaxCount*RunMode*add_carry-22

+ /MaxCount*add_carry-22*/bWriteb ;  "(6 terms, 8 symbols)

L1IDcounter[22].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[22].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[21].T = L1IDcounter[19]*L1IDcounter[20]*

/MaxCount*/Reg10*/Reg12*add_carry-19

+ L1IDcounter[19]*L1IDcounter[20]*

/MaxCount*RunMode*add_carry-19

+ L1IDcounter[19]*L1IDcounter[20]*

/MaxCount*add_carry-19*/bWriteb

+ L1IDcounter[21]*MaxCount

+ L1IDcounter[21]*/Reg10*Reg12*

/RunMode*bWriteb*/db[9]

+ /L1IDcounter[21]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[9] ;  "(6 terms, 10 symbols)

L1IDcounter[21].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[21].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[20].T = L1IDcounter[19]*/MaxCount*/Reg10*

/Reg12*add_carry-19

+ L1IDcounter[19]*/MaxCount*RunMode*

add_carry-19

+ L1IDcounter[19]*/MaxCount*

add_carry-19*/bWriteb

+ L1IDcounter[20]*MaxCount

+ L1IDcounter[20]*/Reg10*Reg12*

/RunMode*bWriteb*/db[8]

+ /L1IDcounter[20]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[8] ;  "(6 terms, 9 symbols)

L1IDcounter[20].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[20].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[19].T = L1IDcounter[19]*MaxCount

+ L1IDcounter[19]*/Reg10*Reg12*

/RunMode*bWriteb*/db[7]

+ /L1IDcounter[19]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[7]

+ /MaxCount*/Reg10*/Reg12*

add_carry-19

+ /MaxCount*RunMode*add_carry-19

+ /MaxCount*add_carry-19*/bWriteb ;  "(6 terms, 8 symbols)

L1IDcounter[19].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[19].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[18].T = L1IDcounter[16]*L1IDcounter[17]*

/MaxCount*/Reg10*/Reg12*add_carry-16

+ L1IDcounter[16]*L1IDcounter[17]*

/MaxCount*RunMode*add_carry-16

+ L1IDcounter[16]*L1IDcounter[17]*

/MaxCount*add_carry-16*/bWriteb

+ L1IDcounter[18]*MaxCount

+ L1IDcounter[18]*/Reg10*Reg12*

/RunMode*bWriteb*/db[6]

+ /L1IDcounter[18]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[6] ;  "(6 terms, 10 symbols)

L1IDcounter[18].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[18].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[17].T = L1IDcounter[16]*/MaxCount*/Reg10*

/Reg12*add_carry-16

+ L1IDcounter[16]*/MaxCount*RunMode*

add_carry-16

+ L1IDcounter[16]*/MaxCount*

add_carry-16*/bWriteb

+ L1IDcounter[17]*MaxCount

+ L1IDcounter[17]*/Reg10*Reg12*

/RunMode*bWriteb*/db[5]

+ /L1IDcounter[17]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[5] ;  "(6 terms, 9 symbols)

L1IDcounter[17].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[17].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[16].T = L1IDcounter[16]*MaxCount

+ L1IDcounter[16]*/Reg10*Reg12*

/RunMode*bWriteb*/db[4]

+ /L1IDcounter[16]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[4]

+ /MaxCount*/Reg10*/Reg12*

add_carry-16

+ /MaxCount*RunMode*add_carry-16

+ /MaxCount*add_carry-16*/bWriteb ;  "(6 terms, 8 symbols)

L1IDcounter[16].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[16].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[15].T = L1IDcounter[13]*L1IDcounter[14]*

/MaxCount*/Reg10*/Reg12*add_carry-13

+ L1IDcounter[13]*L1IDcounter[14]*

/MaxCount*RunMode*add_carry-13

+ L1IDcounter[13]*L1IDcounter[14]*

/MaxCount*add_carry-13*/bWriteb

+ L1IDcounter[15]*MaxCount

+ L1IDcounter[15]*/Reg10*Reg12*

/RunMode*bWriteb*/db[3]

+ /L1IDcounter[15]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[3] ;  "(6 terms, 10 symbols)

L1IDcounter[15].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[15].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[14].T = L1IDcounter[13]*/MaxCount*/Reg10*

/Reg12*add_carry-13

+ L1IDcounter[13]*/MaxCount*RunMode*

add_carry-13

+ L1IDcounter[13]*/MaxCount*

add_carry-13*/bWriteb

+ L1IDcounter[14]*MaxCount

+ L1IDcounter[14]*/Reg10*Reg12*

/RunMode*bWriteb*/db[2]

+ /L1IDcounter[14]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[2] ;  "(6 terms, 9 symbols)

L1IDcounter[14].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[14].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[13].T = L1IDcounter[13]*MaxCount

+ L1IDcounter[13]*/Reg10*Reg12*

/RunMode*bWriteb*/db[1]

+ /L1IDcounter[13]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[1]

+ /MaxCount*/Reg10*/Reg12*

add_carry-13

+ /MaxCount*RunMode*add_carry-13

+ /MaxCount*add_carry-13*/bWriteb ;  "(6 terms, 8 symbols)

L1IDcounter[13].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[13].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[12].T = L1IDcounter[10]*L1IDcounter[11]*

/MaxCount*/Reg10*/Reg12*add_carry-10

+ L1IDcounter[10]*L1IDcounter[11]*

/MaxCount*RunMode*add_carry-10

+ L1IDcounter[10]*L1IDcounter[11]*

/MaxCount*add_carry-10*/bWriteb

+ L1IDcounter[12]*MaxCount

+ L1IDcounter[12]*/Reg10*Reg12*

/RunMode*bWriteb*/db[0]

+ /L1IDcounter[12]*/MaxCount*/Reg10*

Reg12*/RunMode*bWriteb*db[0] ;  "(6 terms, 10 symbols)

L1IDcounter[12].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[12].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[11].T = L1IDcounter[10]*/MaxCount*/Reg10*

/Reg12*add_carry-10

+ L1IDcounter[10]*/MaxCount*RunMode*

add_carry-10

+ L1IDcounter[10]*/MaxCount*

add_carry-10*/bWriteb

+ L1IDcounter[11]*MaxCount

+ L1IDcounter[11]*Reg10*/RunMode*

bWriteb*/db[11]

+ /L1IDcounter[11]*/MaxCount*Reg10*

/RunMode*bWriteb*db[11] ;  "(6 terms, 9 symbols)

L1IDcounter[11].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[11].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[10].T = L1IDcounter[10]*MaxCount

+ L1IDcounter[10]*Reg10*/RunMode*

bWriteb*/db[10]

+ /L1IDcounter[10]*/MaxCount*Reg10*

/RunMode*bWriteb*db[10]

+ /MaxCount*/Reg10*/Reg12*

add_carry-10

+ /MaxCount*RunMode*add_carry-10

+ /MaxCount*add_carry-10*/bWriteb ;  "(6 terms, 8 symbols)

L1IDcounter[10].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[10].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[9].T = L1IDcounter[7]*L1IDcounter[8]*

/MaxCount*/Reg10*/Reg12*add_carry-7

+ L1IDcounter[7]*L1IDcounter[8]*

/MaxCount*RunMode*add_carry-7

+ L1IDcounter[7]*L1IDcounter[8]*

/MaxCount*add_carry-7*/bWriteb

+ L1IDcounter[9]*MaxCount

+ L1IDcounter[9]*Reg10*/RunMode*

bWriteb*/db[9]

+ /L1IDcounter[9]*/MaxCount*Reg10*

/RunMode*bWriteb*db[9] ;  "(6 terms, 10 symbols)

L1IDcounter[9].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[9].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[8].T = L1IDcounter[7]*/MaxCount*/Reg10*/Reg12

*add_carry-7

+ L1IDcounter[7]*/MaxCount*RunMode*

add_carry-7

+ L1IDcounter[7]*/MaxCount*

add_carry-7*/bWriteb

+ L1IDcounter[8]*MaxCount

+ L1IDcounter[8]*Reg10*/RunMode*

bWriteb*/db[8]

+ /L1IDcounter[8]*/MaxCount*Reg10*

/RunMode*bWriteb*db[8] ;  "(6 terms, 9 symbols)

L1IDcounter[8].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[8].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[7].T = L1IDcounter[7]*MaxCount

+ L1IDcounter[7]*Reg10*/RunMode*

bWriteb*/db[7]

+ /L1IDcounter[7]*/MaxCount*Reg10*

/RunMode*bWriteb*db[7]

+ /MaxCount*/Reg10*/Reg12*add_carry-7

+ /MaxCount*RunMode*add_carry-7

+ /MaxCount*add_carry-7*/bWriteb ;  "(6 terms, 8 symbols)

L1IDcounter[7].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[7].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[6].T = L1IDcounter[4]*L1IDcounter[5]*

/MaxCount*/Reg10*/Reg12*add_carry-4

+ L1IDcounter[4]*L1IDcounter[5]*

/MaxCount*RunMode*add_carry-4

+ L1IDcounter[4]*L1IDcounter[5]*

/MaxCount*add_carry-4*/bWriteb

+ L1IDcounter[6]*MaxCount

+ L1IDcounter[6]*Reg10*/RunMode*

bWriteb*/db[6]

+ /L1IDcounter[6]*/MaxCount*Reg10*

/RunMode*bWriteb*db[6] ;  "(6 terms, 10 symbols)

L1IDcounter[6].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[6].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[5].T = L1IDcounter[4]*/MaxCount*/Reg10*/Reg12

*add_carry-4

+ L1IDcounter[4]*/MaxCount*RunMode*

add_carry-4

+ L1IDcounter[4]*/MaxCount*

add_carry-4*/bWriteb

+ L1IDcounter[5]*MaxCount

+ L1IDcounter[5]*Reg10*/RunMode*

bWriteb*/db[5]

+ /L1IDcounter[5]*/MaxCount*Reg10*

/RunMode*bWriteb*db[5] ;  "(6 terms, 9 symbols)

L1IDcounter[5].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[5].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[4].T = L1IDcounter[4]*MaxCount

+ L1IDcounter[4]*Reg10*/RunMode*

bWriteb*/db[4]

+ /L1IDcounter[4]*/MaxCount*Reg10*

/RunMode*bWriteb*db[4]

+ /MaxCount*/Reg10*/Reg12*add_carry-4

+ /MaxCount*RunMode*add_carry-4

+ /MaxCount*add_carry-4*/bWriteb ;  "(6 terms, 8 symbols)

L1IDcounter[4].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[4].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[3].T = L1IDcounter[0]*L1IDcounter[1]*

L1IDcounter[2]*/MaxCount*/Reg10*/Reg12

+ L1IDcounter[0]*L1IDcounter[1]*

L1IDcounter[2]*/MaxCount*RunMode

+ L1IDcounter[0]*L1IDcounter[1]*

L1IDcounter[2]*/MaxCount*/bWriteb

+ L1IDcounter[3]*MaxCount

+ L1IDcounter[3]*Reg10*/RunMode*

bWriteb*/db[3]

+ /L1IDcounter[3]*/MaxCount*Reg10*

/RunMode*bWriteb*db[3] ;  "(6 terms, 10 symbols)

L1IDcounter[3].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[3].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[2].T = L1IDcounter[0]*L1IDcounter[1]*

/MaxCount*/Reg10*/Reg12

+ L1IDcounter[0]*L1IDcounter[1]*

/MaxCount*RunMode

+ L1IDcounter[0]*L1IDcounter[1]*

/MaxCount*/bWriteb

+ L1IDcounter[2]*MaxCount

+ L1IDcounter[2]*Reg10*/RunMode*

bWriteb*/db[2]

+ /L1IDcounter[2]*/MaxCount*Reg10*

/RunMode*bWriteb*db[2] ;  "(6 terms, 9 symbols)

L1IDcounter[2].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[2].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[1].T = L1IDcounter[0]*/MaxCount*/Reg10*/Reg12

+ L1IDcounter[0]*/MaxCount*RunMode

+ L1IDcounter[0]*/MaxCount*/bWriteb

+ L1IDcounter[1]*MaxCount

+ L1IDcounter[1]*Reg10*/RunMode*

bWriteb*/db[1]

+ /L1IDcounter[1]*/MaxCount*Reg10*

/RunMode*bWriteb*db[1] ;  "(6 terms, 8 symbols)

L1IDcounter[1].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[1].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounter[0].D = L1IDcounter[0]*/MaxCount*/Reg10*Reg12*

/RunMode*bWriteb

+ /L1IDcounter[0]*/MaxCount*/Reg10*

/Reg12

+ /L1IDcounter[0]*/MaxCount*RunMode

+ /L1IDcounter[0]*/MaxCount*/bWriteb

+ /MaxCount*Reg10*/RunMode*bWriteb*

db[0] ;  "(5 terms, 7 symbols)

L1IDcounter[0].CLK = L1IDcounterClk ;  "(1 term, 1 symbol)

L1IDcounter[0].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

L1IDcounterClk.EQN = Reg10*/RunMode*bWriteb*nLEb2

+ /Reg10*/Reg12*qidTRIG

+ Reg12*/RunMode*bWriteb*nLEb2

+ RunMode*qidTRIG

+ /bWriteb*qidTRIG ;  "(5 terms, 6 symbols)

MaxCount.EQN = L1IDcounter[0]*L1IDcounter[10]*

L1IDcounter[11]*L1IDcounter[12]*L1IDcounter[13]*

L1IDcounter[14]*L1IDcounter[15]*L1IDcounter[16]*

L1IDcounter[17]*L1IDcounter[18]*L1IDcounter[19]*

L1IDcounter[1]*L1IDcounter[20]*L1IDcounter[21]*

L1IDcounter[22]*L1IDcounter[23]*L1IDcounter[2]*

L1IDcounter[3]*L1IDcounter[4]*L1IDcounter[5]*

L1IDcounter[6]*L1IDcounter[7]*L1IDcounter[8]*

L1IDcounter[9] ;  "(1 term, 24 symbols)

add_carry-4.EQN = L1IDcounter[0]*L1IDcounter[1]*

L1IDcounter[2]*L1IDcounter[3] ;  "(1 term, 4 symbols)

add_carry-7.EQN = L1IDcounter[4]*L1IDcounter[5]*

L1IDcounter[6]*add_carry-4 ;  "(1 term, 4 symbols)

add_carry-10.EQN = L1IDcounter[7]*L1IDcounter[8]*

L1IDcounter[9]*add_carry-7 ;  "(1 term, 4 symbols)

add_carry-13.EQN = L1IDcounter[10]*L1IDcounter[11]*

L1IDcounter[12]*add_carry-10 ;  "(1 term, 4 symbols)

add_carry-16.EQN = L1IDcounter[13]*L1IDcounter[14]*

L1IDcounter[15]*add_carry-13 ;  "(1 term, 4 symbols)

add_carry-19.EQN = L1IDcounter[16]*L1IDcounter[17]*

L1IDcounter[18]*add_carry-16 ;  "(1 term, 4 symbols)

add_carry-22.EQN = L1IDcounter[19]*L1IDcounter[20]*

L1IDcounter[21]*add_carry-19 ;  "(1 term, 4 symbols)

RegisterL1IDL[11].D = BCntIn[11]*RunMode

+ L1IDcounter[11]*/Reg10*/RunMode

+ L1IDcounter[11]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[11] ;  "(4 terms, 6 symbols)

RegisterL1IDL[11].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[11].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[10].D = BCntIn[10]*RunMode

+ L1IDcounter[10]*/Reg10*/RunMode

+ L1IDcounter[10]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[10] ;  "(4 terms, 6 symbols)

RegisterL1IDL[10].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[10].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[9].D = BCntIn[9]*RunMode

+ L1IDcounter[9]*/Reg10*/RunMode

+ L1IDcounter[9]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[9] ;  "(4 terms, 6 symbols)

RegisterL1IDL[9].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[9].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[8].D = BCntIn[8]*RunMode

+ L1IDcounter[8]*/Reg10*/RunMode

+ L1IDcounter[8]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[8] ;  "(4 terms, 6 symbols)

RegisterL1IDL[8].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[8].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[7].D = BCntIn[7]*RunMode

+ L1IDcounter[7]*/Reg10*/RunMode

+ L1IDcounter[7]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[7] ;  "(4 terms, 6 symbols)

RegisterL1IDL[7].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[7].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[6].D = BCntIn[6]*RunMode

+ L1IDcounter[6]*/Reg10*/RunMode

+ L1IDcounter[6]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[6] ;  "(4 terms, 6 symbols)

RegisterL1IDL[6].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[6].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[5].D = BCntIn[5]*RunMode

+ L1IDcounter[5]*/Reg10*/RunMode

+ L1IDcounter[5]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[5] ;  "(4 terms, 6 symbols)

RegisterL1IDL[5].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[5].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[4].D = BCntIn[4]*RunMode

+ L1IDcounter[4]*/Reg10*/RunMode

+ L1IDcounter[4]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[4] ;  "(4 terms, 6 symbols)

RegisterL1IDL[4].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[4].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[3].D = BCntIn[3]*RunMode

+ L1IDcounter[3]*/Reg10*/RunMode

+ L1IDcounter[3]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[3] ;  "(4 terms, 6 symbols)

RegisterL1IDL[3].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[3].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[2].D = BCntIn[2]*RunMode

+ L1IDcounter[2]*/Reg10*/RunMode

+ L1IDcounter[2]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[2] ;  "(4 terms, 6 symbols)

RegisterL1IDL[2].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[2].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[1].D = BCntIn[1]*RunMode

+ L1IDcounter[1]*/Reg10*/RunMode

+ L1IDcounter[1]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[1] ;  "(4 terms, 6 symbols)

RegisterL1IDL[1].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[1].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDL[0].D = BCntIn[0]*RunMode

+ L1IDcounter[0]*/Reg10*/RunMode

+ L1IDcounter[0]*/RunMode*/bWriteb

+ Reg10*/RunMode*bWriteb*db[0] ;  "(4 terms, 6 symbols)

RegisterL1IDL[0].CLK = sgbA[0] ;  "(1 term, 1 symbol)

RegisterL1IDL[0].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

sgbA[0].EQN = Reg10*/RunMode*bWriteb*nLEb2

+ /Reg10*/RunMode*/qidTRIG

+ RunMode*sgbA[2]

+ /RunMode*/bWriteb*/qidTRIG ;  "(4 terms, 6 symbols)

RegisterL1IDH[11].D = BCntIn[11]*RunMode

+ L1IDcounter[23]*/Reg12*/RunMode

+ L1IDcounter[23]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[11] ;  "(4 terms, 6 symbols)

RegisterL1IDH[11].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[11].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[10].D = BCntIn[10]*RunMode

+ L1IDcounter[22]*/Reg12*/RunMode

+ L1IDcounter[22]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[10] ;  "(4 terms, 6 symbols)

RegisterL1IDH[10].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[10].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[9].D = BCntIn[9]*RunMode

+ L1IDcounter[21]*/Reg12*/RunMode

+ L1IDcounter[21]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[9] ;  "(4 terms, 6 symbols)

RegisterL1IDH[9].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[9].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[8].D = BCntIn[8]*RunMode

+ L1IDcounter[20]*/Reg12*/RunMode

+ L1IDcounter[20]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[8] ;  "(4 terms, 6 symbols)

RegisterL1IDH[8].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[8].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[7].D = BCntIn[7]*RunMode

+ L1IDcounter[19]*/Reg12*/RunMode

+ L1IDcounter[19]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[7] ;  "(4 terms, 6 symbols)

RegisterL1IDH[7].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[7].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[6].D = BCntIn[6]*RunMode

+ L1IDcounter[18]*/Reg12*/RunMode

+ L1IDcounter[18]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[6] ;  "(4 terms, 6 symbols)

RegisterL1IDH[6].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[6].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[5].D = BCntIn[5]*RunMode

+ L1IDcounter[17]*/Reg12*/RunMode

+ L1IDcounter[17]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[5] ;  "(4 terms, 6 symbols)

RegisterL1IDH[5].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[5].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[4].D = BCntIn[4]*RunMode

+ L1IDcounter[16]*/Reg12*/RunMode

+ L1IDcounter[16]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[4] ;  "(4 terms, 6 symbols)

RegisterL1IDH[4].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[4].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[3].D = BCntIn[3]*RunMode

+ L1IDcounter[15]*/Reg12*/RunMode

+ L1IDcounter[15]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[3] ;  "(4 terms, 6 symbols)

RegisterL1IDH[3].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[3].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[2].D = BCntIn[2]*RunMode

+ L1IDcounter[14]*/Reg12*/RunMode

+ L1IDcounter[14]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[2] ;  "(4 terms, 6 symbols)

RegisterL1IDH[2].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[2].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[1].D = BCntIn[1]*RunMode

+ L1IDcounter[13]*/Reg12*/RunMode

+ L1IDcounter[13]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[1] ;  "(4 terms, 6 symbols)

RegisterL1IDH[1].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[1].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

RegisterL1IDH[0].D = BCntIn[0]*RunMode

+ L1IDcounter[12]*/Reg12*/RunMode

+ L1IDcounter[12]*/RunMode*/bWriteb

+ Reg12*/RunMode*bWriteb*db[0] ;  "(4 terms, 6 symbols)

RegisterL1IDH[0].CLK = sgbA[1] ;  "(1 term, 1 symbol)

RegisterL1IDH[0].RESET = idECR

+ /nOA_rst ;  "(2 terms, 2 symbols)

sgbA[1].EQN = Reg12*/RunMode*bWriteb*nLEb2

+ /Reg12*/RunMode*/qidTRIG

+ RunMode*sgbA[3]

+ /RunMode*/bWriteb*/qidTRIG ;  "(4 terms, 6 symbols)

L1IDL[11].D = RegisterL1IDL[11] ;  "(1 term, 1 symbol)

L1IDL[11].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[11].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[10].D = RegisterL1IDL[10] ;  "(1 term, 1 symbol)

L1IDL[10].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[10].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[9].D = RegisterL1IDL[9] ;  "(1 term, 1 symbol)

L1IDL[9].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[9].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[8].D = RegisterL1IDL[8] ;  "(1 term, 1 symbol)

L1IDL[8].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[8].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[7].D = RegisterL1IDL[7] ;  "(1 term, 1 symbol)

L1IDL[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[7].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[6].D = RegisterL1IDL[6] ;  "(1 term, 1 symbol)

L1IDL[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[6].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[5].D = RegisterL1IDL[5] ;  "(1 term, 1 symbol)

L1IDL[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[5].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[4].D = RegisterL1IDL[4] ;  "(1 term, 1 symbol)

L1IDL[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[4].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[3].D = RegisterL1IDL[3] ;  "(1 term, 1 symbol)

L1IDL[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[3].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[2].D = RegisterL1IDL[2] ;  "(1 term, 1 symbol)

L1IDL[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[2].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[1].D = RegisterL1IDL[1] ;  "(1 term, 1 symbol)

L1IDL[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[1].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDL[0].D = RegisterL1IDL[0] ;  "(1 term, 1 symbol)

L1IDL[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDL[0].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[11].D = RegisterL1IDH[11] ;  "(1 term, 1 symbol)

L1IDH[11].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[11].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[10].D = RegisterL1IDH[10] ;  "(1 term, 1 symbol)

L1IDH[10].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[10].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[9].D = RegisterL1IDH[9] ;  "(1 term, 1 symbol)

L1IDH[9].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[9].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[8].D = RegisterL1IDH[8] ;  "(1 term, 1 symbol)

L1IDH[8].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[8].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[7].D = RegisterL1IDH[7] ;  "(1 term, 1 symbol)

L1IDH[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[7].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[6].D = RegisterL1IDH[6] ;  "(1 term, 1 symbol)

L1IDH[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[6].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[5].D = RegisterL1IDH[5] ;  "(1 term, 1 symbol)

L1IDH[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[5].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[4].D = RegisterL1IDH[4] ;  "(1 term, 1 symbol)

L1IDH[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[4].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[3].D = RegisterL1IDH[3] ;  "(1 term, 1 symbol)

L1IDH[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[3].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[2].D = RegisterL1IDH[2] ;  "(1 term, 1 symbol)

L1IDH[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[2].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[1].D = RegisterL1IDH[1] ;  "(1 term, 1 symbol)

L1IDH[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[1].CLK(~) = clk4 ;  "(1 term, 1 symbol)

L1IDH[0].D = RegisterL1IDH[0] ;  "(1 term, 1 symbol)

L1IDH[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1IDH[0].CLK(~) = clk4 ;  "(1 term, 1 symbol)

sgbA[7].EQN = idECR ;  "(1 term, 1 symbol)

sgbA[6].EQN = Reg10 ;  "(1 term, 1 symbol)

sgbA[5].EQN = L1CntHighStrobe ;  "(1 term, 1 symbol)

sgbA[4].EQN = L1CntLowStrobe ;  "(1 term, 1 symbol)

PLDocument:  pld4a.doc

SOLUTIONS    Fri Nov 16 15:28:58 2001

PARTITIONING CRITERIA :

WEIGHT PRICE 10 ;

TEMPLATE = MV384_184  ;

PARTITIONING SOLUTIONS :

==> Solution  1: MV384_184

FUSEMAP FILES FOR SOLUTION 1:

Device 1 (MV384_184) : pld4a.j1

PLDocument:  pld4a.doc

PINOUT DIAGRAMS  Fri Nov 16 15:28:58 2001

Device  1 - MV384_184 -- Pinout for QFP package

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|    1  |Jtag  |                      | |   51  |Vcc   |                      |

|    2  |Biput |                      | |   52  |Biput |  sgbC[7]             |

|    3  |Biput |                      | |   53  |Biput |  sgbC[6]             |

|    4  |Biput |                      | |   54  |Biput |  sgbC[5]             |

|    5  |Biput |                      | |   55  |Biput |  sgbC[4]             |

|    6  |Biput |  BCntIn[0]           | |   56  |Biput |  sgbC[3]             |

|    7  |Biput |  BCntIn[1]           | |   57  |Biput |  sgbC[2]             |

|    8  |Biput |  BCntIn[2]           | |   58  |Biput |  sgbC[1]             |

|    9  |Biput |  BCntIn[3]           | |   59  |Biput |  sgbC[0]             |

|   10  |Vcc   |                      | |   60  |Jtag  |                      |

|   11  |GND   |                      | |   61  |GND   |                      |

|   12  |Biput |  BCntIn[4]           | |   62  |Vcc   |                      |

|   13  |Biput |  BCntIn[5]           | |   63  |Biput |  L1IDL[0]            |

|   14  |Biput |  BCntIn[6]           | |   64  |Biput |  L1IDL[1]            |

|   15  |Biput |  BCntIn[7]           | |   65  |Biput |  L1IDL[2]            |

|   16  |Biput |  BCntIn[8]           | |   66  |Biput |  L1IDL[3]            |

|   17  |Biput |  BCntIn[9]           | |   67  |Biput |  L1IDL[4]            |

|   18  |Biput |  BCntIn[10]          | |   68  |Biput |  L1IDL[5]            |

|   19  |Biput |  BCntIn[11]          | |   69  |Biput |                      |

|   20  |GND   |                      | |   70  |Biput |                      |

|   21  |Biput |                      | |   71  |GND   |                      |

|   22  |Biput |                      | |   72  |Vcc   |                      |

|   23  |Biput |                      | |   73  |Biput |                      |

|   24  |Biput |                      | |   74  |Biput |                      |

|   25  |Biput |                      | |   75  |Biput |                      |

|   26  |Biput |                      | |   76  |Biput |                      |

|   27  |Biput |                      | |   77  |Biput |                      |

|   28  |Biput |                      | |   78  |Biput |                      |

|   29  |In/CLK|                      | |   79  |Biput |  L1IDL[6]            |

|   30  |Vcc   |                      | |   80  |Biput |  L1IDL[7]            |

|   31  |GND   |                      | |   81  |GND   |                      |

|   32  |In/CLK|  idTRIG              | |   82  |Biput |  L1IDL[8]            |

|   33  |Biput |                      | |   83  |Biput |  L1IDL[9]            |

|   34  |Biput |                      | |   84  |Biput |  L1IDL[10]           |

|   35  |Biput |                      | |   85  |Biput |  L1IDL[11]           |

|   36  |Biput |                      | |   86  |Biput |                      |

|   37  |Biput |                      | |   87  |Biput |                      |

|   38  |Biput |                      | |   88  |Vcc   |                      |

|   39  |Biput |                      | |   89  |GND   |                      |

|   40  |Biput |                      | |   90  |GND   |                      |

|   41  |GND   |                      | |   91  |GND   |                      |

|   42  |Biput |                      | |   92  |GND   |                      |

|   43  |Biput |                      | |   93  |Vcc   |                      |

|   44  |Biput |  sgbD[5]             | |   94  |Biput |  L1IDH[0]            |

|   45  |Biput |  sgbD[4]             | |   95  |Biput |  L1IDH[1]            |

|   46  |Biput |  sgbD[3]             | |   96  |Biput |  L1IDH[2]            |

|   47  |Biput |  sgbD[2]             | |   97  |Biput |  L1IDH[3]            |

|   48  |Biput |  sgbD[1]             | |   98  |Biput |  L1IDH[4]            |

|   49  |Biput |  sgbD[0]             | |   99  |Biput |  L1IDH[5]            |

|   50  |GND   |                      | |  100  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  101  |Biput |                      | |  151  |GND   |                      |

|  102  |Biput |                      | |  152  |In/CLK|  nLEb2               |

|  103  |Biput |                      | |  153  |Biput |  enL1ID              |

|  104  |Biput |                      | |  154  |Biput |                      |

|  105  |Biput |                      | |  155  |Biput |                      |

|  106  |Biput |                      | |  156  |Biput |  L1_strobe           |

|  107  |Biput |                      | |  157  |Biput |  db[15]              |

|  108  |Biput |                      | |  158  |Biput |  db[14]              |

|  109  |Vcc   |                      | |  159  |Biput |  db[13]              |

|  110  |GND   |                      | |  160  |Biput |  db[12]              |

|  111  |Biput |  s2_4a[7]            | |  161  |GND   |                      |

|  112  |Biput |  s2_4a[6]            | |  162  |Biput |                      |

|  113  |Biput |  s2_4a[5]            | |  163  |Biput |                      |

|  114  |Biput |  s2_4a[4]            | |  164  |Biput |                      |

|  115  |Biput |  s2_4a[3]            | |  165  |Biput |                      |

|  116  |Biput |  s2_4a[2]            | |  166  |Biput |  db[11]              |

|  117  |Biput |  s2_4a[1]            | |  167  |Biput |  db[10]              |

|  118  |Biput |  s2_4a[0]            | |  168  |Biput |  db[9]               |

|  119  |Vcc   |                      | |  169  |Biput |  db[8]               |

|  120  |GND   |                      | |  170  |GND   |                      |

|  121  |Jtag  |                      | |  171  |Vcc   |                      |

|  122  |Biput |  nOA_rst             | |  172  |Biput |                      |

|  123  |Biput |  RunMode             | |  173  |Biput |                      |

|  124  |Biput |                      | |  174  |Biput |                      |

|  125  |Biput |                      | |  175  |Biput |                      |

|  126  |Biput |  s4a_4b[7]           | |  176  |Biput |  db[7]               |

|  127  |Biput |  s4a_4b[6]           | |  177  |Biput |  db[6]               |

|  128  |Biput |  s4a_4b[5]           | |  178  |Biput |  db[5]               |

|  129  |Biput |  s4a_4b[4]           | |  179  |Biput |  db[4]               |

|  130  |Vcc   |                      | |  180  |Jtag  |                      |

|  131  |GND   |                      | |  181  |GND   |                      |

|  132  |Biput |  bReadb              | |  182  |Vcc   |                      |

|  133  |Biput |  bWriteb             | |  183  |Biput |                      |

|  134  |Biput |  Reg10               | |  184  |Biput |                      |

|  135  |Biput |  Reg12               | |  185  |Biput |                      |

|  136  |Biput |                      | |  186  |Biput |                      |

|  137  |Biput |                      | |  187  |Biput |  db[3]               |

|  138  |Biput |                      | |  188  |Biput |  db[2]               |

|  139  |Biput |                      | |  189  |Biput |  db[1]               |

|  140  |GND   |                      | |  190  |Biput |  db[0]               |

|  141  |Biput |  idECR               | |  191  |GND   |                      |

|  142  |Biput |  SAECR2              | |  192  |Vcc   |                      |

|  143  |Biput |                      | |  193  |Biput |  sgbB[7]             |

|  144  |Biput |  ENSAECR2            | |  194  |Biput |  sgbB[6]             |

|  145  |Biput |  s4a_4b[3]           | |  195  |Biput |  sgbB[5]             |

|  146  |Biput |  s4a_4b[2]           | |  196  |Biput |  sgbB[4]             |

|  147  |Biput |  s4a_4b[1]           | |  197  |Biput |  sgbB[3]             |

|  148  |Biput |  s4a_4b[0]           | |  198  |Biput |  sgbB[2]             |

|  149  |In/CLK|  clk4                | |  199  |Biput |  sgbB[1]             |

|  150  |Vcc   |                      | |  200  |Biput |  sgbB[0]             |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  201  |GND   |                      | |  221  |Biput |  sgbA[7]             |

|  202  |Biput |                      | |  222  |Biput |  sgbA[6]             |

|  203  |Biput |                      | |  223  |Biput |  sgbA[5]             |

|  204  |Biput |                      | |  224  |Biput |  sgbA[4]             |

|  205  |Biput |                      | |  225  |Biput |  sgbA[3]             |

|  206  |Biput |                      | |  226  |Biput |  sgbA[2]             |

|  207  |Biput |                      | |  227  |Biput |  sgbA[1]             |

|  208  |Vcc   |                      | |  228  |Biput |  sgbA[0]             |

|  209  |GND   |                      | |  229  |Vcc   |                      |

|  210  |GND   |                      | |  230  |GND   |                      |

|  211  |GND   |                      | |  231  |Biput |                      |

|  212  |GND   |                      | |  232  |Biput |                      |

|  213  |Vcc   |                      | |  233  |Biput |  L1CntHighStrobe     |

|  214  |Biput |  L1IDH[6]            | |  234  |Biput |                      |

|  215  |Biput |  L1IDH[7]            | |  235  |Biput |  L1CntLowStrobe      |

|  216  |Biput |  L1IDH[8]            | |  236  |Biput |                      |

|  217  |Biput |  L1IDH[9]            | |  237  |Biput |                      |

|  218  |Biput |  L1IDH[10]           | |  238  |Biput |                      |

|  219  |Biput |  L1IDH[11]           | |  239  |Vcc   |                      |

|  220  |GND   |                      | |  240  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

DEVICE SELECTION:

Device        Manuf   Fam    Pack   Temp   Price  User1 User2

1) M5-384/184-10HC    AMD    CMOS   QFP    COM    $  0.00    0    0

2) M5-384/184-10HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

3) M5-384/184-12HC    AMD    CMOS   QFP    COM    $  0.00    0    0

4) M5-384/184-12HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

5) M5-384/184-15HC    AMD    CMOS   QFP    COM    $  0.00    0    0

6) M5-384/184-15HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

7) M5-384/184-20HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

==> M5-384/184-7HC     AMD    CMOS   QFP    COM    $  0.00    0    0

9) M5LV-384/184-10HC  AMD    V3.3   QFP    COM    $  0.00    0    0

10) M5LV-384/184-10HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

11) M5LV-384/184-12HC  AMD    V3.3   QFP    COM    $  0.00    0    0

12) M5LV-384/184-12HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

13) M5LV-384/184-15HC  AMD    V3.3   QFP    COM    $  0.00    0    0

14) M5LV-384/184-15HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

15) M5LV-384/184-20HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

16) M5LV-384/184-7HC   AMD    V3.3   QFP    COM    $  0.00    0    0

17) M5LV-384/184-7HI   AMD    V3.3   QFP    EXT    $  0.00    0    0

PLDocument:  pld4a.doc

WIRELIST     Fri Nov 16 15:28:58 2001

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| nOA_rst          | MV384_184_1       |  122  |

| bReadb           | MV384_184_1       |  132  |

| bWriteb          | MV384_184_1       |  133  |

| Reg10            | MV384_184_1       |  134  |

| Reg12            | MV384_184_1       |  135  |

| nLEb2            | MV384_184_1       |  152  |

| db[15]           | MV384_184_1       |  157  |

| db[14]           | MV384_184_1       |  158  |

| db[13]           | MV384_184_1       |  159  |

| db[12]           | MV384_184_1       |  160  |

| db[11]           | MV384_184_1       |  166  |

| db[10]           | MV384_184_1       |  167  |

| db[9]            | MV384_184_1       |  168  |

| db[8]            | MV384_184_1       |  169  |

| db[7]            | MV384_184_1       |  176  |

| db[6]            | MV384_184_1       |  177  |

| db[5]            | MV384_184_1       |  178  |

| db[4]            | MV384_184_1       |  179  |

| db[3]            | MV384_184_1       |  187  |

| db[2]            | MV384_184_1       |  188  |

| db[1]            | MV384_184_1       |  189  |

| db[0]            | MV384_184_1       |  190  |

| BCntIn[11]       | MV384_184_1       |   19  |

| BCntIn[10]       | MV384_184_1       |   18  |

| BCntIn[9]        | MV384_184_1       |   17  |

| BCntIn[8]        | MV384_184_1       |   16  |

| BCntIn[7]        | MV384_184_1       |   15  |

| BCntIn[6]        | MV384_184_1       |   14  |

| BCntIn[5]        | MV384_184_1       |   13  |

| BCntIn[4]        | MV384_184_1       |   12  |

| BCntIn[3]        | MV384_184_1       |    9  |

| BCntIn[2]        | MV384_184_1       |    8  |

| BCntIn[1]        | MV384_184_1       |    7  |

| BCntIn[0]        | MV384_184_1       |    6  |

| RunMode          | MV384_184_1       |  123  |

| idTRIG           | MV384_184_1       |   32  |

| enL1ID           | MV384_184_1       |  153  |

| clk4             | MV384_184_1       |  149  |

| L1CntLowStrobe   | MV384_184_1       |  235  |

| L1CntHighStrobe  | MV384_184_1       |  233  |

| sgbA[2]          | MV384_184_1       |  226  |

| sgbA[3]          | MV384_184_1       |  225  |

| L1_strobe        | MV384_184_1       |  156  |

| idECR            | MV384_184_1       |  141  |

| ENSAECR2         | MV384_184_1       |  144  |

| SAECR2           | MV384_184_1       |  142  |

| sgbA[0]          | MV384_184_1       |  228  |

| sgbA[1]          | MV384_184_1       |  227  |

| L1IDL[11]        | MV384_184_1       |   85  |

| L1IDL[10]        | MV384_184_1       |   84  |

| L1IDL[9]         | MV384_184_1       |   83  |

+------------------+-------------------+-------+

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| L1IDL[8]         | MV384_184_1       |   82  |

| L1IDL[7]         | MV384_184_1       |   80  |

| L1IDL[6]         | MV384_184_1       |   79  |

| L1IDL[5]         | MV384_184_1       |   68  |

| L1IDL[4]         | MV384_184_1       |   67  |

| L1IDL[3]         | MV384_184_1       |   66  |

| L1IDL[2]         | MV384_184_1       |   65  |

| L1IDL[1]         | MV384_184_1       |   64  |

| L1IDL[0]         | MV384_184_1       |   63  |

| L1IDH[11]        | MV384_184_1       |  219  |

| L1IDH[10]        | MV384_184_1       |  218  |

| L1IDH[9]         | MV384_184_1       |  217  |

| L1IDH[8]         | MV384_184_1       |  216  |

| L1IDH[7]         | MV384_184_1       |  215  |

| L1IDH[6]         | MV384_184_1       |  214  |

| L1IDH[5]         | MV384_184_1       |   99  |

| L1IDH[4]         | MV384_184_1       |   98  |

| L1IDH[3]         | MV384_184_1       |   97  |

| L1IDH[2]         | MV384_184_1       |   96  |

| L1IDH[1]         | MV384_184_1       |   95  |

| L1IDH[0]         | MV384_184_1       |   94  |

| sgbA[7]          | MV384_184_1       |  221  |

| sgbA[6]          | MV384_184_1       |  222  |

| sgbA[5]          | MV384_184_1       |  223  |

| sgbA[4]          | MV384_184_1       |  224  |

| sgbB[7]          | MV384_184_1       |  193  |

| sgbB[6]          | MV384_184_1       |  194  |

| sgbB[5]          | MV384_184_1       |  195  |

| sgbB[4]          | MV384_184_1       |  196  |

| sgbB[3]          | MV384_184_1       |  197  |

| sgbB[2]          | MV384_184_1       |  198  |

| sgbB[1]          | MV384_184_1       |  199  |

| sgbB[0]          | MV384_184_1       |  200  |

| sgbC[7]          | MV384_184_1       |   52  |

| sgbC[6]          | MV384_184_1       |   53  |

| sgbC[5]          | MV384_184_1       |   54  |

| sgbC[4]          | MV384_184_1       |   55  |

| sgbC[3]          | MV384_184_1       |   56  |

| sgbC[2]          | MV384_184_1       |   57  |

| sgbC[1]          | MV384_184_1       |   58  |

| sgbC[0]          | MV384_184_1       |   59  |

| sgbD[5]          | MV384_184_1       |   44  |

| sgbD[4]          | MV384_184_1       |   45  |

| sgbD[3]          | MV384_184_1       |   46  |

| sgbD[2]          | MV384_184_1       |   47  |

| sgbD[1]          | MV384_184_1       |   48  |

| sgbD[0]          | MV384_184_1       |   49  |

| s2_4a[7]         | MV384_184_1       |  111  |

| s2_4a[6]         | MV384_184_1       |  112  |

| s2_4a[5]         | MV384_184_1       |  113  |

| s2_4a[4]         | MV384_184_1       |  114  |

+------------------+-------------------+-------+

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| s2_4a[3]         | MV384_184_1       |  115  |

| s2_4a[2]         | MV384_184_1       |  116  |

| s2_4a[1]         | MV384_184_1       |  117  |

| s2_4a[0]         | MV384_184_1       |  118  |

| s4a_4b[7]        | MV384_184_1       |  126  |

| s4a_4b[6]        | MV384_184_1       |  127  |

| s4a_4b[5]        | MV384_184_1       |  128  |

| s4a_4b[4]        | MV384_184_1       |  129  |

| s4a_4b[3]        | MV384_184_1       |  145  |

| s4a_4b[2]        | MV384_184_1       |  146  |

| s4a_4b[1]        | MV384_184_1       |  147  |

| s4a_4b[0]        | MV384_184_1       |  148  |

+------------------+-------------------+-------+