PLDocument:  pld4b.doc

TITLE PAGE   Thu Jan 03 15:06:35 2002

A MACHXL6.2.1.11 - (c) Copyright MINC Incorporated 1987-1998

==============================================================================

TITLE    : ATLAS SCT	TIM, PLD4b:	BCID & TTID	Registers

FILE     : pld4b

DATE     : Sun Jun 07 02:53:52 1970

ENGINEER : Martin	Postranecky	/ Dominic A	Hayes

COMPANY  : University College	London

==============================================================================

MODULES  :

Document Generator     	  3.70

File Handler           	  6.2

Language Compiler      	  3.83

Architectural Optimizer	  3.110

Device Lib Scan        	  3.2

Device Library         	  3.17

Device Partition       	  3.21

Device Fusemap         	  3.4

SWITCH VALUES :

(Value in parenthesis represents batch mode switch value)

PLCOMP PRODUCT TERM LIMIT : 128

PLOPT  PRODUCT TERM LIMIT : 128

PLOPT REDUCTION  :   Espresso (1)

NODE GENERATION  :   Procedure Instantiation

Arithmetic and Relational Operators (1)

PLDocument:  pld4b.doc

EQUATIONS    Thu Jan 03 15:06:35 2002

EQUATIONS FOR SYSTEM

INPUT SIGNALS (84) :

noa_rst

clk4

runmode

bwriteb

breadb

Reg14

Reg16

Reg2A

nLEb2

ENsaBCR2

idBCR

BCntIn[11..0]

BCntStrobe

idTRIG

enBCID

TTIDin[9..0]

TTypeStrobe

enTType

sgbA[7..0]

sgbB[7..0]

sgbC[7..0]

sgbD[5..0]

s4b_5[7..0]

s4a_4b[7..0]

OUTPUT SIGNALS (41) :

db[15..0]

saBCR2

BC_Strobe

BCID[11..0]

TTID[9..0]

TT_Strobe

PHYSICAL NODE SIGNALS (63) :

dboe

bcid_offset[3..0]

shiftidBCR[15..0]

delayed_idBCR

BCIDcounter[11..0]

qBCntStrobe

RegisterTTC_BCID[11..0]

qidTRIG

RegisterBCID[11..0]

regBCIDclk

qTTypeStrobe

regTTIDclk

REDUCED EQUATIONS:

db[15].EQN = bcid_offset[3] ;  "(1 term, 1 symbol)

db[15].OE = dboe ;  "(1 term, 1 symbol)

db[14].EQN = bcid_offset[2] ;  "(1 term, 1 symbol)

db[14].OE = dboe ;  "(1 term, 1 symbol)

db[13].EQN = bcid_offset[1] ;  "(1 term, 1 symbol)

db[13].OE = dboe ;  "(1 term, 1 symbol)

db[12].EQN = bcid_offset[0] ;  "(1 term, 1 symbol)

db[12].OE = dboe ;  "(1 term, 1 symbol)

db[11].EQN = Reg14*RegisterBCID[11]

+ /Reg14*RegisterTTC_BCID[11] ;  "(2 terms, 3 symbols)

db[11].OE = dboe ;  "(1 term, 1 symbol)

db[10].EQN = Reg14*RegisterBCID[10]

+ /Reg14*RegisterTTC_BCID[10] ;  "(2 terms, 3 symbols)

db[10].OE = dboe ;  "(1 term, 1 symbol)

db[9].EQN = Reg14*RegisterBCID[9]

+ /Reg14*Reg16*TTID[9]

+ /Reg14*/Reg16*RegisterTTC_BCID[9] ;  "(3 terms, 5 symbols)

db[9].OE = dboe ;  "(1 term, 1 symbol)

db[8].EQN = Reg14*RegisterBCID[8]

+ /Reg14*Reg16*TTID[8]

+ /Reg14*/Reg16*RegisterTTC_BCID[8] ;  "(3 terms, 5 symbols)

db[8].OE = dboe ;  "(1 term, 1 symbol)

db[7].EQN = Reg14*RegisterBCID[7]

+ /Reg14*Reg16*TTID[7]

+ /Reg14*/Reg16*RegisterTTC_BCID[7] ;  "(3 terms, 5 symbols)

db[7].OE = dboe ;  "(1 term, 1 symbol)

db[6].EQN = Reg14*RegisterBCID[6]

+ /Reg14*Reg16*TTID[6]

+ /Reg14*/Reg16*RegisterTTC_BCID[6] ;  "(3 terms, 5 symbols)

db[6].OE = dboe ;  "(1 term, 1 symbol)

db[5].EQN = Reg14*RegisterBCID[5]

+ /Reg14*Reg16*TTID[5]

+ /Reg14*/Reg16*RegisterTTC_BCID[5] ;  "(3 terms, 5 symbols)

db[5].OE = dboe ;  "(1 term, 1 symbol)

db[4].EQN = Reg14*RegisterBCID[4]

+ /Reg14*Reg16*TTID[4]

+ /Reg14*/Reg16*RegisterTTC_BCID[4] ;  "(3 terms, 5 symbols)

db[4].OE = dboe ;  "(1 term, 1 symbol)

db[3].EQN = Reg14*RegisterBCID[3]

+ /Reg14*Reg16*TTID[3]

+ /Reg14*/Reg16*RegisterTTC_BCID[3] ;  "(3 terms, 5 symbols)

db[3].OE = dboe ;  "(1 term, 1 symbol)

db[2].EQN = Reg14*RegisterBCID[2]

+ /Reg14*Reg16*TTID[2]

+ /Reg14*/Reg16*RegisterTTC_BCID[2] ;  "(3 terms, 5 symbols)

db[2].OE = dboe ;  "(1 term, 1 symbol)

db[1].EQN = Reg14*RegisterBCID[1]

+ /Reg14*Reg16*TTID[1]

+ /Reg14*/Reg16*RegisterTTC_BCID[1] ;  "(3 terms, 5 symbols)

db[1].OE = dboe ;  "(1 term, 1 symbol)

db[0].EQN = Reg14*RegisterBCID[0]

+ /Reg14*Reg16*TTID[0]

+ /Reg14*/Reg16*RegisterTTC_BCID[0] ;  "(3 terms, 5 symbols)

db[0].OE = dboe ;  "(1 term, 1 symbol)

dboe.EQN(~) = /Reg14*/Reg16*/Reg2A

+ /breadb ;  "(2 terms, 4 symbols)

bcid_offset[3].D = db[15] ;  "(1 term, 1 symbol)

bcid_offset[3].CLK = nLEb2 ;  "(1 term, 1 symbol)

bcid_offset[3].CE = Reg14*bwriteb ;  "(1 term, 2 symbols)

bcid_offset[3].RESET = /noa_rst ;  "(1 term, 1 symbol)

bcid_offset[2].D = db[14] ;  "(1 term, 1 symbol)

bcid_offset[2].CLK = nLEb2 ;  "(1 term, 1 symbol)

bcid_offset[2].CE = Reg14*bwriteb ;  "(1 term, 2 symbols)

bcid_offset[2].RESET = /noa_rst ;  "(1 term, 1 symbol)

bcid_offset[1].D = db[13] ;  "(1 term, 1 symbol)

bcid_offset[1].CLK = nLEb2 ;  "(1 term, 1 symbol)

bcid_offset[1].CE = Reg14*bwriteb ;  "(1 term, 2 symbols)

bcid_offset[1].RESET = /noa_rst ;  "(1 term, 1 symbol)

bcid_offset[0].D = db[12] ;  "(1 term, 1 symbol)

bcid_offset[0].CLK = nLEb2 ;  "(1 term, 1 symbol)

bcid_offset[0].CE = Reg14*bwriteb ;  "(1 term, 2 symbols)

bcid_offset[0].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[15].D = shiftidBCR[14] ;  "(1 term, 1 symbol)

shiftidBCR[15].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[15].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[14].D = shiftidBCR[13] ;  "(1 term, 1 symbol)

shiftidBCR[14].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[14].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[13].D = shiftidBCR[12] ;  "(1 term, 1 symbol)

shiftidBCR[13].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[13].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[12].D = shiftidBCR[11] ;  "(1 term, 1 symbol)

shiftidBCR[12].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[12].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[11].D = shiftidBCR[10] ;  "(1 term, 1 symbol)

shiftidBCR[11].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[11].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[10].D = shiftidBCR[9] ;  "(1 term, 1 symbol)

shiftidBCR[10].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[10].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[9].D = shiftidBCR[8] ;  "(1 term, 1 symbol)

shiftidBCR[9].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[9].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[8].D = shiftidBCR[7] ;  "(1 term, 1 symbol)

shiftidBCR[8].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[8].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[7].D = shiftidBCR[6] ;  "(1 term, 1 symbol)

shiftidBCR[7].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[7].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[6].D = shiftidBCR[5] ;  "(1 term, 1 symbol)

shiftidBCR[6].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[6].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[5].D = shiftidBCR[4] ;  "(1 term, 1 symbol)

shiftidBCR[5].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[5].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[4].D = shiftidBCR[3] ;  "(1 term, 1 symbol)

shiftidBCR[4].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[4].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[3].D = shiftidBCR[2] ;  "(1 term, 1 symbol)

shiftidBCR[3].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[3].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[2].D = shiftidBCR[1] ;  "(1 term, 1 symbol)

shiftidBCR[2].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[2].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[1].D = shiftidBCR[0] ;  "(1 term, 1 symbol)

shiftidBCR[1].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[1].RESET = /noa_rst ;  "(1 term, 1 symbol)

shiftidBCR[0].D = idBCR ;  "(1 term, 1 symbol)

shiftidBCR[0].CLK = clk4 ;  "(1 term, 1 symbol)

shiftidBCR[0].RESET = /noa_rst ;  "(1 term, 1 symbol)

delayed_idBCR.D = bcid_offset[0]*bcid_offset[1]*

bcid_offset[2]*bcid_offset[3]*shiftidBCR[15]

+ bcid_offset[0]*bcid_offset[1]*

bcid_offset[2]*/bcid_offset[3]*shiftidBCR[7]

+ bcid_offset[0]*bcid_offset[1]*

/bcid_offset[2]*bcid_offset[3]*shiftidBCR[11]

+ bcid_offset[0]*bcid_offset[1]*

/bcid_offset[2]*/bcid_offset[3]*shiftidBCR[3]

+ bcid_offset[0]*/bcid_offset[1]*

bcid_offset[2]*bcid_offset[3]*shiftidBCR[13]

+ bcid_offset[0]*/bcid_offset[1]*

bcid_offset[2]*/bcid_offset[3]*shiftidBCR[5]

+ bcid_offset[0]*/bcid_offset[1]*

/bcid_offset[2]*bcid_offset[3]*shiftidBCR[9]

+ bcid_offset[0]*/bcid_offset[1]*

/bcid_offset[2]*/bcid_offset[3]*shiftidBCR[1]

+ /bcid_offset[0]*bcid_offset[1]*

bcid_offset[2]*bcid_offset[3]*shiftidBCR[14]

+ /bcid_offset[0]*bcid_offset[1]*

bcid_offset[2]*/bcid_offset[3]*shiftidBCR[6]

+ /bcid_offset[0]*bcid_offset[1]*

/bcid_offset[2]*bcid_offset[3]*shiftidBCR[10]

+ /bcid_offset[0]*bcid_offset[1]*

/bcid_offset[2]*/bcid_offset[3]*shiftidBCR[2]

+ /bcid_offset[0]*/bcid_offset[1]*

bcid_offset[2]*bcid_offset[3]*shiftidBCR[12]

+ /bcid_offset[0]*/bcid_offset[1]*

bcid_offset[2]*/bcid_offset[3]*shiftidBCR[4]

+ /bcid_offset[0]*/bcid_offset[1]*

/bcid_offset[2]*bcid_offset[3]*shiftidBCR[8]

+ /bcid_offset[0]*/bcid_offset[1]*

/bcid_offset[2]*/bcid_offset[3]*shiftidBCR[0] ;  "(16 terms, 20 symbols)

delayed_idBCR.CLK = clk4 ;  "(1 term, 1 symbol)

delayed_idBCR.RESET = /noa_rst ;  "(1 term, 1 symbol)

saBCR2.EQN = BCIDcounter[0]*BCIDcounter[10]*

BCIDcounter[11]*BCIDcounter[1]*BCIDcounter[2]*

BCIDcounter[3]*BCIDcounter[4]*BCIDcounter[5]*

BCIDcounter[6]*BCIDcounter[7]*BCIDcounter[8]*

BCIDcounter[9]*ENsaBCR2*/delayed_idBCR ;  "(1 term, 14 symbols)

BCIDcounter[11].T = BCIDcounter[0]*BCIDcounter[10]*

BCIDcounter[1]*BCIDcounter[2]*BCIDcounter[3]*

BCIDcounter[4]*BCIDcounter[5]*BCIDcounter[6]*

BCIDcounter[7]*BCIDcounter[8]*BCIDcounter[9]*

/delayed_idBCR

+ BCIDcounter[11]*delayed_idBCR ;  "(2 terms, 13 symbols)

BCIDcounter[11].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[11].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[10].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*BCIDcounter[3]*BCIDcounter[4]*

BCIDcounter[5]*BCIDcounter[6]*BCIDcounter[7]*

BCIDcounter[8]*BCIDcounter[9]*/delayed_idBCR

+ BCIDcounter[10]*delayed_idBCR ;  "(2 terms, 12 symbols)

BCIDcounter[10].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[10].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[9].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*BCIDcounter[3]*BCIDcounter[4]*

BCIDcounter[5]*BCIDcounter[6]*BCIDcounter[7]*

BCIDcounter[8]*/delayed_idBCR

+ BCIDcounter[9]*delayed_idBCR ;  "(2 terms, 11 symbols)

BCIDcounter[9].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[9].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[8].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*BCIDcounter[3]*BCIDcounter[4]*

BCIDcounter[5]*BCIDcounter[6]*BCIDcounter[7]*

/delayed_idBCR

+ BCIDcounter[8]*delayed_idBCR ;  "(2 terms, 10 symbols)

BCIDcounter[8].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[8].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[7].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*BCIDcounter[3]*BCIDcounter[4]*

BCIDcounter[5]*BCIDcounter[6]*/delayed_idBCR

+ BCIDcounter[7]*delayed_idBCR ;  "(2 terms, 9 symbols)

BCIDcounter[7].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[7].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[6].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*BCIDcounter[3]*BCIDcounter[4]*

BCIDcounter[5]*/delayed_idBCR

+ BCIDcounter[6]*delayed_idBCR ;  "(2 terms, 8 symbols)

BCIDcounter[6].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[6].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[5].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*BCIDcounter[3]*BCIDcounter[4]*

/delayed_idBCR

+ BCIDcounter[5]*delayed_idBCR ;  "(2 terms, 7 symbols)

BCIDcounter[5].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[5].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[4].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*BCIDcounter[3]*/delayed_idBCR

+ BCIDcounter[4]*delayed_idBCR ;  "(2 terms, 6 symbols)

BCIDcounter[4].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[4].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[3].T = BCIDcounter[0]*BCIDcounter[1]*

BCIDcounter[2]*/delayed_idBCR

+ BCIDcounter[3]*delayed_idBCR ;  "(2 terms, 5 symbols)

BCIDcounter[3].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[3].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[2].D = BCIDcounter[0]*BCIDcounter[1]*

/BCIDcounter[2]*/delayed_idBCR

+ /BCIDcounter[0]*BCIDcounter[2]*

/delayed_idBCR

+ /BCIDcounter[1]*BCIDcounter[2]*

/delayed_idBCR ;  "(3 terms, 4 symbols)

BCIDcounter[2].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[2].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[1].D = BCIDcounter[0]*/BCIDcounter[1]*

/delayed_idBCR

+ /BCIDcounter[0]*BCIDcounter[1]*

/delayed_idBCR ;  "(2 terms, 3 symbols)

BCIDcounter[1].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[1].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCIDcounter[0].D = /BCIDcounter[0]*/delayed_idBCR ;  "(1 term, 2 symbols)

BCIDcounter[0].CLK = clk4 ;  "(1 term, 1 symbol)

BCIDcounter[0].RESET = /noa_rst ;  "(1 term, 1 symbol)

qBCntStrobe.D = BCntStrobe ;  "(1 term, 1 symbol)

qBCntStrobe.CLK = clk4 ;  "(1 term, 1 symbol)

qBCntStrobe.RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[11].D = BCntIn[11] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[11].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[11].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[10].D = BCntIn[10] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[10].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[10].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[9].D = BCntIn[9] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[9].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[9].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[8].D = BCntIn[8] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[8].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[8].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[7].D = BCntIn[7] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[7].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[7].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[6].D = BCntIn[6] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[6].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[6].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[5].D = BCntIn[5] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[5].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[5].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[4].D = BCntIn[4] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[4].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[4].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[3].D = BCntIn[3] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[3].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[3].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[2].D = BCntIn[2] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[2].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[2].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[1].D = BCntIn[1] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[1].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[1].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterTTC_BCID[0].D = BCntIn[0] ;  "(1 term, 1 symbol)

RegisterTTC_BCID[0].CLK = qBCntStrobe ;  "(1 term, 1 symbol)

RegisterTTC_BCID[0].RESET = /noa_rst ;  "(1 term, 1 symbol)

qidTRIG.D = idTRIG ;  "(1 term, 1 symbol)

qidTRIG.CLK = clk4 ;  "(1 term, 1 symbol)

qidTRIG.RESET = /noa_rst ;  "(1 term, 1 symbol)

BC_Strobe.D = enBCID*qBCntStrobe*runmode

+ enBCID*qidTRIG*/runmode ;  "(2 terms, 4 symbols)

BC_Strobe.CLK = clk4 ;  "(1 term, 1 symbol)

BC_Strobe.RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[11].D = BCIDcounter[11] ;  "(1 term, 1 symbol)

RegisterBCID[11].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[11].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[10].D = BCIDcounter[10] ;  "(1 term, 1 symbol)

RegisterBCID[10].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[10].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[9].D = BCIDcounter[9] ;  "(1 term, 1 symbol)

RegisterBCID[9].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[9].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[8].D = BCIDcounter[8] ;  "(1 term, 1 symbol)

RegisterBCID[8].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[8].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[7].D = BCIDcounter[7] ;  "(1 term, 1 symbol)

RegisterBCID[7].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[7].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[6].D = BCIDcounter[6] ;  "(1 term, 1 symbol)

RegisterBCID[6].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[6].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[5].D = BCIDcounter[5] ;  "(1 term, 1 symbol)

RegisterBCID[5].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[5].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[4].D = BCIDcounter[4] ;  "(1 term, 1 symbol)

RegisterBCID[4].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[4].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[3].D = BCIDcounter[3] ;  "(1 term, 1 symbol)

RegisterBCID[3].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[3].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[2].D = BCIDcounter[2] ;  "(1 term, 1 symbol)

RegisterBCID[2].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[2].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[1].D = BCIDcounter[1] ;  "(1 term, 1 symbol)

RegisterBCID[1].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[1].RESET = /noa_rst ;  "(1 term, 1 symbol)

RegisterBCID[0].D = BCIDcounter[0] ;  "(1 term, 1 symbol)

RegisterBCID[0].CLK = regBCIDclk ;  "(1 term, 1 symbol)

RegisterBCID[0].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[11].D = RegisterBCID[11] ;  "(1 term, 1 symbol)

BCID[11].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[11].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[10].D = RegisterBCID[10] ;  "(1 term, 1 symbol)

BCID[10].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[10].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[9].D = RegisterBCID[9] ;  "(1 term, 1 symbol)

BCID[9].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[9].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[8].D = RegisterBCID[8] ;  "(1 term, 1 symbol)

BCID[8].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[8].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[7].D = RegisterBCID[7] ;  "(1 term, 1 symbol)

BCID[7].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[7].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[6].D = RegisterBCID[6] ;  "(1 term, 1 symbol)

BCID[6].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[6].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[5].D = RegisterBCID[5] ;  "(1 term, 1 symbol)

BCID[5].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[5].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[4].D = RegisterBCID[4] ;  "(1 term, 1 symbol)

BCID[4].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[4].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[3].D = RegisterBCID[3] ;  "(1 term, 1 symbol)

BCID[3].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[3].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[2].D = RegisterBCID[2] ;  "(1 term, 1 symbol)

BCID[2].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[2].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[1].D = RegisterBCID[1] ;  "(1 term, 1 symbol)

BCID[1].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[1].RESET = /noa_rst ;  "(1 term, 1 symbol)

BCID[0].D = RegisterBCID[0] ;  "(1 term, 1 symbol)

BCID[0].CLK = clk4 ;  "(1 term, 1 symbol)

BCID[0].RESET = /noa_rst ;  "(1 term, 1 symbol)

regBCIDclk.EQN = BCntStrobe*runmode

+ qidTRIG*/runmode ;  "(2 terms, 3 symbols)

qTTypeStrobe.D = TTypeStrobe ;  "(1 term, 1 symbol)

qTTypeStrobe.CLK = clk4 ;  "(1 term, 1 symbol)

qTTypeStrobe.RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[9].D = Reg16*bwriteb*db[9]*/runmode

+ /Reg16*TTID[9]*/runmode

+ TTID[9]*/bwriteb*/runmode

+ TTIDin[9]*runmode ;  "(4 terms, 6 symbols)

TTID[9].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[9].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[8].D = Reg16*bwriteb*db[8]*/runmode

+ /Reg16*TTID[8]*/runmode

+ TTID[8]*/bwriteb*/runmode

+ TTIDin[8]*runmode ;  "(4 terms, 6 symbols)

TTID[8].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[8].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[7].D = Reg16*bwriteb*db[7]*/runmode

+ /Reg16*TTID[7]*/runmode

+ TTID[7]*/bwriteb*/runmode

+ TTIDin[7]*runmode ;  "(4 terms, 6 symbols)

TTID[7].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[7].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[6].D = Reg16*bwriteb*db[6]*/runmode

+ /Reg16*TTID[6]*/runmode

+ TTID[6]*/bwriteb*/runmode

+ TTIDin[6]*runmode ;  "(4 terms, 6 symbols)

TTID[6].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[6].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[5].D = Reg16*bwriteb*db[5]*/runmode

+ /Reg16*TTID[5]*/runmode

+ TTID[5]*/bwriteb*/runmode

+ TTIDin[5]*runmode ;  "(4 terms, 6 symbols)

TTID[5].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[5].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[4].D = Reg16*bwriteb*db[4]*/runmode

+ /Reg16*TTID[4]*/runmode

+ TTID[4]*/bwriteb*/runmode

+ TTIDin[4]*runmode ;  "(4 terms, 6 symbols)

TTID[4].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[4].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[3].D = Reg16*bwriteb*db[3]*/runmode

+ /Reg16*TTID[3]*/runmode

+ TTID[3]*/bwriteb*/runmode

+ TTIDin[3]*runmode ;  "(4 terms, 6 symbols)

TTID[3].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[3].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[2].D = Reg16*bwriteb*db[2]*/runmode

+ /Reg16*TTID[2]*/runmode

+ TTID[2]*/bwriteb*/runmode

+ TTIDin[2]*runmode ;  "(4 terms, 6 symbols)

TTID[2].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[2].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[1].D = Reg16*bwriteb*db[1]*/runmode

+ /Reg16*TTID[1]*/runmode

+ TTID[1]*/bwriteb*/runmode

+ TTIDin[1]*runmode ;  "(4 terms, 6 symbols)

TTID[1].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[1].RESET = /noa_rst ;  "(1 term, 1 symbol)

TTID[0].D = Reg16*bwriteb*db[0]*/runmode

+ /Reg16*TTID[0]*/runmode

+ TTID[0]*/bwriteb*/runmode

+ TTIDin[0]*runmode ;  "(4 terms, 6 symbols)

TTID[0].CLK = regTTIDclk ;  "(1 term, 1 symbol)

TTID[0].RESET = /noa_rst ;  "(1 term, 1 symbol)

regTTIDclk.EQN = Reg16*bwriteb*nLEb2*/runmode

+ qTTypeStrobe*runmode ;  "(2 terms, 5 symbols)

TT_Strobe.D = enTType*qTTypeStrobe*runmode

+ enTType*qidTRIG*/runmode ;  "(2 terms, 4 symbols)

TT_Strobe.CLK = clk4 ;  "(1 term, 1 symbol)

TT_Strobe.RESET = /noa_rst ;  "(1 term, 1 symbol)

PLDocument:  pld4b.doc

SOLUTIONS    Thu Jan 03 15:06:35 2002

PARTITIONING CRITERIA :

WEIGHT PRICE 10 ;

TEMPLATE = MV384_184  ;

PARTITIONING SOLUTIONS :

==> Solution  1: MV384_184

FUSEMAP FILES FOR SOLUTION 1:

Device 1 (MV384_184) : pld4b.j1

PLDocument:  pld4b.doc

PINOUT DIAGRAMS  Thu Jan 03 15:06:35 2002

Device  1 - MV384_184 -- Pinout for QFP package

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|    1  |Jtag  |                      | |   51  |Vcc   |                      |

|    2  |Biput |  sgbA[0]             | |   52  |Biput |  TTIDin[0]           |

|    3  |Biput |  sgbA[1]             | |   53  |Biput |  TTIDin[1]           |

|    4  |Biput |  sgbA[2]             | |   54  |Biput |  TTIDin[2]           |

|    5  |Biput |  sgbA[3]             | |   55  |Biput |  TTIDin[3]           |

|    6  |Biput |  sgbA[4]             | |   56  |Biput |  TTIDin[4]           |

|    7  |Biput |  sgbA[5]             | |   57  |Biput |  TTIDin[5]           |

|    8  |Biput |  sgbA[6]             | |   58  |Biput |  TTIDin[6]           |

|    9  |Biput |  sgbA[7]             | |   59  |Biput |  TTIDin[7]           |

|   10  |Vcc   |                      | |   60  |Jtag  |                      |

|   11  |GND   |                      | |   61  |GND   |                      |

|   12  |Biput |  sgbB[0]             | |   62  |Vcc   |                      |

|   13  |Biput |  sgbB[1]             | |   63  |Biput |  TTIDin[8]           |

|   14  |Biput |  sgbB[2]             | |   64  |Biput |  TTIDin[9]           |

|   15  |Biput |  sgbB[3]             | |   65  |Biput |  TTID[0]             |

|   16  |Biput |  sgbB[4]             | |   66  |Biput |  TTID[1]             |

|   17  |Biput |  sgbB[5]             | |   67  |Biput |                      |

|   18  |Biput |  sgbB[6]             | |   68  |Biput |                      |

|   19  |Biput |  sgbB[7]             | |   69  |Biput |  TTID[2]             |

|   20  |GND   |                      | |   70  |Biput |                      |

|   21  |Biput |  sgbC[0]             | |   71  |GND   |                      |

|   22  |Biput |  sgbC[1]             | |   72  |Vcc   |                      |

|   23  |Biput |  sgbC[2]             | |   73  |Biput |  sgbD[0]             |

|   24  |Biput |  sgbC[3]             | |   74  |Biput |  sgbD[1]             |

|   25  |Biput |  sgbC[4]             | |   75  |Biput |  sgbD[2]             |

|   26  |Biput |  sgbC[5]             | |   76  |Biput |  sgbD[3]             |

|   27  |Biput |  sgbC[6]             | |   77  |Biput |  sgbD[4]             |

|   28  |Biput |  sgbC[7]             | |   78  |Biput |  sgbD[5]             |

|   29  |In/CLK|  idTRIG              | |   79  |Biput |                      |

|   30  |Vcc   |                      | |   80  |Biput |                      |

|   31  |GND   |                      | |   81  |GND   |                      |

|   32  |In/CLK|                      | |   82  |Biput |                      |

|   33  |Biput |                      | |   83  |Biput |                      |

|   34  |Biput |                      | |   84  |Biput |                      |

|   35  |Biput |                      | |   85  |Biput |                      |

|   36  |Biput |                      | |   86  |Biput |                      |

|   37  |Biput |  BCntIn[0]           | |   87  |Biput |                      |

|   38  |Biput |  BCntIn[1]           | |   88  |Vcc   |                      |

|   39  |Biput |  BCntIn[2]           | |   89  |GND   |                      |

|   40  |Biput |  BCntIn[3]           | |   90  |GND   |                      |

|   41  |GND   |                      | |   91  |GND   |                      |

|   42  |Biput |  BCntIn[4]           | |   92  |GND   |                      |

|   43  |Biput |  BCntIn[5]           | |   93  |Vcc   |                      |

|   44  |Biput |  BCntIn[6]           | |   94  |Biput |                      |

|   45  |Biput |  BCntIn[7]           | |   95  |Biput |                      |

|   46  |Biput |  BCntIn[8]           | |   96  |Biput |                      |

|   47  |Biput |  BCntIn[9]           | |   97  |Biput |                      |

|   48  |Biput |  BCntIn[10]          | |   98  |Biput |                      |

|   49  |Biput |  BCntIn[11]          | |   99  |Biput |                      |

|   50  |GND   |                      | |  100  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  101  |Biput |  s4b_5[0]            | |  151  |GND   |                      |

|  102  |Biput |  s4b_5[1]            | |  152  |In/CLK|  nLEb2               |

|  103  |Biput |  s4b_5[2]            | |  153  |Biput |                      |

|  104  |Biput |  s4b_5[3]            | |  154  |Biput |                      |

|  105  |Biput |  s4b_5[4]            | |  155  |Biput |                      |

|  106  |Biput |  s4b_5[5]            | |  156  |Biput |                      |

|  107  |Biput |  s4b_5[6]            | |  157  |Biput |  db[15]              |

|  108  |Biput |  s4b_5[7]            | |  158  |Biput |  db[14]              |

|  109  |Vcc   |                      | |  159  |Biput |  db[13]              |

|  110  |GND   |                      | |  160  |Biput |  db[12]              |

|  111  |Biput |  s4a_4b[0]           | |  161  |GND   |                      |

|  112  |Biput |  s4a_4b[1]           | |  162  |Biput |                      |

|  113  |Biput |  s4a_4b[2]           | |  163  |Biput |                      |

|  114  |Biput |  s4a_4b[3]           | |  164  |Biput |                      |

|  115  |Biput |  s4a_4b[4]           | |  165  |Biput |                      |

|  116  |Biput |  s4a_4b[5]           | |  166  |Biput |  db[11]              |

|  117  |Biput |  s4a_4b[6]           | |  167  |Biput |  db[10]              |

|  118  |Biput |  s4a_4b[7]           | |  168  |Biput |  db[9]               |

|  119  |Vcc   |                      | |  169  |Biput |  db[8]               |

|  120  |GND   |                      | |  170  |GND   |                      |

|  121  |Jtag  |                      | |  171  |Vcc   |                      |

|  122  |Biput |  noa_rst             | |  172  |Biput |                      |

|  123  |Biput |  runmode             | |  173  |Biput |                      |

|  124  |Biput |  idBCR               | |  174  |Biput |  TTID[9]             |

|  125  |Biput |  enBCID              | |  175  |Biput |                      |

|  126  |Biput |  enTType             | |  176  |Biput |  db[7]               |

|  127  |Biput |  ENsaBCR2            | |  177  |Biput |  db[6]               |

|  128  |Biput |                      | |  178  |Biput |  db[5]               |

|  129  |Biput |  saBCR2              | |  179  |Biput |  db[4]               |

|  130  |Vcc   |                      | |  180  |Jtag  |                      |

|  131  |GND   |                      | |  181  |GND   |                      |

|  132  |Biput |  breadb              | |  182  |Vcc   |                      |

|  133  |Biput |  bwriteb             | |  183  |Biput |                      |

|  134  |Biput |                      | |  184  |Biput |                      |

|  135  |Biput |                      | |  185  |Biput |                      |

|  136  |Biput |  Reg14               | |  186  |Biput |                      |

|  137  |Biput |  Reg16               | |  187  |Biput |  db[3]               |

|  138  |Biput |  Reg2A               | |  188  |Biput |  db[2]               |

|  139  |Biput |                      | |  189  |Biput |  db[1]               |

|  140  |GND   |                      | |  190  |Biput |  db[0]               |

|  141  |Biput |  BC_Strobe           | |  191  |GND   |                      |

|  142  |Biput |  TT_Strobe           | |  192  |Vcc   |                      |

|  143  |Biput |                      | |  193  |Biput |  BCID[0]             |

|  144  |Biput |                      | |  194  |Biput |  BCID[1]             |

|  145  |Biput |                      | |  195  |Biput |  BCID[2]             |

|  146  |Biput |                      | |  196  |Biput |  BCID[3]             |

|  147  |Biput |                      | |  197  |Biput |  BCID[4]             |

|  148  |Biput |                      | |  198  |Biput |  BCID[5]             |

|  149  |In/CLK|  clk4                | |  199  |Biput |  BCID[6]             |

|  150  |Vcc   |                      | |  200  |Biput |  BCID[7]             |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  201  |GND   |                      | |  221  |Biput |                      |

|  202  |Biput |  BCID[8]             | |  222  |Biput |                      |

|  203  |Biput |  BCID[9]             | |  223  |Biput |                      |

|  204  |Biput |  BCID[10]            | |  224  |Biput |                      |

|  205  |Biput |  BCID[11]            | |  225  |Biput |                      |

|  206  |Biput |                      | |  226  |Biput |                      |

|  207  |Biput |                      | |  227  |Biput |                      |

|  208  |Vcc   |                      | |  228  |Biput |                      |

|  209  |GND   |                      | |  229  |Vcc   |                      |

|  210  |GND   |                      | |  230  |GND   |                      |

|  211  |GND   |                      | |  231  |Biput |  BCntStrobe          |

|  212  |GND   |                      | |  232  |Biput |  TTID[8]             |

|  213  |Vcc   |                      | |  233  |Biput |  TTID[7]             |

|  214  |Biput |                      | |  234  |Biput |  TTID[3]             |

|  215  |Biput |                      | |  235  |Biput |  TTID[5]             |

|  216  |Biput |                      | |  236  |Biput |  TTID[4]             |

|  217  |Biput |                      | |  237  |Biput |  TTypeStrobe         |

|  218  |Biput |                      | |  238  |Biput |  TTID[6]             |

|  219  |Biput |                      | |  239  |Vcc   |                      |

|  220  |GND   |                      | |  240  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

DEVICE SELECTION:

Device        Manuf   Fam    Pack   Temp   Price  User1 User2

1) M5-384/184-10HC    AMD    CMOS   QFP    COM    $  0.00    0    0

2) M5-384/184-10HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

3) M5-384/184-12HC    AMD    CMOS   QFP    COM    $  0.00    0    0

4) M5-384/184-12HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

5) M5-384/184-15HC    AMD    CMOS   QFP    COM    $  0.00    0    0

6) M5-384/184-15HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

7) M5-384/184-20HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

==> M5-384/184-7HC     AMD    CMOS   QFP    COM    $  0.00    0    0

9) M5LV-384/184-10HC  AMD    V3.3   QFP    COM    $  0.00    0    0

10) M5LV-384/184-10HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

11) M5LV-384/184-12HC  AMD    V3.3   QFP    COM    $  0.00    0    0

12) M5LV-384/184-12HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

13) M5LV-384/184-15HC  AMD    V3.3   QFP    COM    $  0.00    0    0

14) M5LV-384/184-15HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

15) M5LV-384/184-20HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

16) M5LV-384/184-7HC   AMD    V3.3   QFP    COM    $  0.00    0    0

17) M5LV-384/184-7HI   AMD    V3.3   QFP    EXT    $  0.00    0    0

PLDocument:  pld4b.doc

WIRELIST     Thu Jan 03 15:06:35 2002

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| noa_rst          | MV384_184_1       |  122  |

| clk4             | MV384_184_1       |  149  |

| runmode          | MV384_184_1       |  123  |

| bwriteb          | MV384_184_1       |  133  |

| breadb           | MV384_184_1       |  132  |

| Reg14            | MV384_184_1       |  136  |

| Reg16            | MV384_184_1       |  137  |

| Reg2A            | MV384_184_1       |  138  |

| nLEb2            | MV384_184_1       |  152  |

| db[15]           | MV384_184_1       |  157  |

| db[14]           | MV384_184_1       |  158  |

| db[13]           | MV384_184_1       |  159  |

| db[12]           | MV384_184_1       |  160  |

| db[11]           | MV384_184_1       |  166  |

| db[10]           | MV384_184_1       |  167  |

| db[9]            | MV384_184_1       |  168  |

| db[8]            | MV384_184_1       |  169  |

| db[7]            | MV384_184_1       |  176  |

| db[6]            | MV384_184_1       |  177  |

| db[5]            | MV384_184_1       |  178  |

| db[4]            | MV384_184_1       |  179  |

| db[3]            | MV384_184_1       |  187  |

| db[2]            | MV384_184_1       |  188  |

| db[1]            | MV384_184_1       |  189  |

| db[0]            | MV384_184_1       |  190  |

| ENsaBCR2         | MV384_184_1       |  127  |

| idBCR            | MV384_184_1       |  124  |

| saBCR2           | MV384_184_1       |  129  |

| BCntIn[11]       | MV384_184_1       |   49  |

| BCntIn[10]       | MV384_184_1       |   48  |

| BCntIn[9]        | MV384_184_1       |   47  |

| BCntIn[8]        | MV384_184_1       |   46  |

| BCntIn[7]        | MV384_184_1       |   45  |

| BCntIn[6]        | MV384_184_1       |   44  |

| BCntIn[5]        | MV384_184_1       |   43  |

| BCntIn[4]        | MV384_184_1       |   42  |

| BCntIn[3]        | MV384_184_1       |   40  |

| BCntIn[2]        | MV384_184_1       |   39  |

| BCntIn[1]        | MV384_184_1       |   38  |

| BCntIn[0]        | MV384_184_1       |   37  |

| BCntStrobe       | MV384_184_1       |  231  |

| idTRIG           | MV384_184_1       |   29  |

| enBCID           | MV384_184_1       |  125  |

| BC_Strobe        | MV384_184_1       |  141  |

| BCID[11]         | MV384_184_1       |  205  |

| BCID[10]         | MV384_184_1       |  204  |

| BCID[9]          | MV384_184_1       |  203  |

| BCID[8]          | MV384_184_1       |  202  |

| BCID[7]          | MV384_184_1       |  200  |

| BCID[6]          | MV384_184_1       |  199  |

| BCID[5]          | MV384_184_1       |  198  |

+------------------+-------------------+-------+

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| BCID[4]          | MV384_184_1       |  197  |

| BCID[3]          | MV384_184_1       |  196  |

| BCID[2]          | MV384_184_1       |  195  |

| BCID[1]          | MV384_184_1       |  194  |

| BCID[0]          | MV384_184_1       |  193  |

| TTIDin[9]        | MV384_184_1       |   64  |

| TTIDin[8]        | MV384_184_1       |   63  |

| TTIDin[7]        | MV384_184_1       |   59  |

| TTIDin[6]        | MV384_184_1       |   58  |

| TTIDin[5]        | MV384_184_1       |   57  |

| TTIDin[4]        | MV384_184_1       |   56  |

| TTIDin[3]        | MV384_184_1       |   55  |

| TTIDin[2]        | MV384_184_1       |   54  |

| TTIDin[1]        | MV384_184_1       |   53  |

| TTIDin[0]        | MV384_184_1       |   52  |

| TTypeStrobe      | MV384_184_1       |  237  |

| TTID[9]          | MV384_184_1       |  174  |

| TTID[8]          | MV384_184_1       |  232  |

| TTID[7]          | MV384_184_1       |  233  |

| TTID[6]          | MV384_184_1       |  238  |

| TTID[5]          | MV384_184_1       |  235  |

| TTID[4]          | MV384_184_1       |  236  |

| TTID[3]          | MV384_184_1       |  234  |

| TTID[2]          | MV384_184_1       |   69  |

| TTID[1]          | MV384_184_1       |   66  |

| TTID[0]          | MV384_184_1       |   65  |

| enTType          | MV384_184_1       |  126  |

| TT_Strobe        | MV384_184_1       |  142  |

| sgbA[7]          | MV384_184_1       |    9  |

| sgbA[6]          | MV384_184_1       |    8  |

| sgbA[5]          | MV384_184_1       |    7  |

| sgbA[4]          | MV384_184_1       |    6  |

| sgbA[3]          | MV384_184_1       |    5  |

| sgbA[2]          | MV384_184_1       |    4  |

| sgbA[1]          | MV384_184_1       |    3  |

| sgbA[0]          | MV384_184_1       |    2  |

| sgbB[7]          | MV384_184_1       |   19  |

| sgbB[6]          | MV384_184_1       |   18  |

| sgbB[5]          | MV384_184_1       |   17  |

| sgbB[4]          | MV384_184_1       |   16  |

| sgbB[3]          | MV384_184_1       |   15  |

| sgbB[2]          | MV384_184_1       |   14  |

| sgbB[1]          | MV384_184_1       |   13  |

| sgbB[0]          | MV384_184_1       |   12  |

| sgbC[7]          | MV384_184_1       |   28  |

| sgbC[6]          | MV384_184_1       |   27  |

| sgbC[5]          | MV384_184_1       |   26  |

| sgbC[4]          | MV384_184_1       |   25  |

| sgbC[3]          | MV384_184_1       |   24  |

| sgbC[2]          | MV384_184_1       |   23  |

| sgbC[1]          | MV384_184_1       |   22  |

+------------------+-------------------+-------+

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| sgbC[0]          | MV384_184_1       |   21  |

| sgbD[5]          | MV384_184_1       |   78  |

| sgbD[4]          | MV384_184_1       |   77  |

| sgbD[3]          | MV384_184_1       |   76  |

| sgbD[2]          | MV384_184_1       |   75  |

| sgbD[1]          | MV384_184_1       |   74  |

| sgbD[0]          | MV384_184_1       |   73  |

| s4b_5[7]         | MV384_184_1       |  108  |

| s4b_5[6]         | MV384_184_1       |  107  |

| s4b_5[5]         | MV384_184_1       |  106  |

| s4b_5[4]         | MV384_184_1       |  105  |

| s4b_5[3]         | MV384_184_1       |  104  |

| s4b_5[2]         | MV384_184_1       |  103  |

| s4b_5[1]         | MV384_184_1       |  102  |

| s4b_5[0]         | MV384_184_1       |  101  |

| s4a_4b[7]        | MV384_184_1       |  118  |

| s4a_4b[6]        | MV384_184_1       |  117  |

| s4a_4b[5]        | MV384_184_1       |  116  |

| s4a_4b[4]        | MV384_184_1       |  115  |

| s4a_4b[3]        | MV384_184_1       |  114  |

| s4a_4b[2]        | MV384_184_1       |  113  |

| s4a_4b[1]        | MV384_184_1       |  112  |

| s4a_4b[0]        | MV384_184_1       |  111  |

+------------------+-------------------+-------+