PLDocument:  pld5.doc

TITLE PAGE    Fri Apr 19 14:17:41 2002

A MACHXL6.2.1.11 - (c) Copyright MINC Incorporated 1987-1998

==============================================================================

TITLE    : ATLAS SCT	TIM, PLD5: Serialiser &	FIFO Control

FILE     : pld5

DATE     : Mon Jun 01 05:07:04 1970

ENGINEER : Martin	Postranecky	/ Dominic A	Hayes

COMPANY  : University College	London

==============================================================================

MODULES  :

Document Generator     	  3.70

File Handler           	  6.2

Language Compiler      	  3.83

Architectural Optimizer	  3.110

Device Lib Scan        	  3.2

Device Library         	  3.17

Device Partition       	  3.21

Device Fusemap         	  3.4

SWITCH VALUES :

(Value in parenthesis represents batch mode switch value)

PLCOMP PRODUCT TERM LIMIT : 128

PLOPT  PRODUCT TERM LIMIT : 128

PLOPT REDUCTION  :   Espresso (1)

NODE GENERATION  :   Procedure Instantiation

Arithmetic and Relational Operators (1)

PLDocument:  pld5.doc

EQUATIONS     Fri Apr 19 14:17:41 2002

EQUATIONS FOR SYSTEM

INPUT SIGNALS (118) :

nOA_rst

RunMode

clk5

Reg0E

bReadb

bWriteb

nLEb2

L1_strobe

BC_strobe

fifo_L1ID_low[11..0]

fifo_L1ID_high[11..0]

fifo_BCID[11..0]

nIDef

nIDff

nIDae

nIDaf

TT_strobe

fifo_TT[9..0]

nTTef

nTTff

nTTae

nTTaf

reg32

reg34

sgbA[7..0]

sgbB[7..0]

sgbC[7..0]

sgbD[5..0]

s1_5[5..0]

s4b_5[7..0]

s5_6[7..0]

OUTPUT SIGNALS (29) :

fs1

fs0

nFIFO_rst

IDclkA

IDenA

IDclkB

IDenB

IDin

TTclkA

TTenA

TTclkb

TTenB

TTin

db_h[15..8]

db_l[7..0]

PHYSICAL NODE SIGNALS (81) :

machine_fifo_reset-0

machine_fifo_reset-1

machine_fifo_reset-2

L1_latch

BC_latch

qID_strobe

L1ID[23..0]

BCID[11..0]

IDCount[5..0]

ID_load

ID_serialiser-0

ID_serialiser-1

ID_serialiser-2

ID_serialiser-3

ID_serialiser-4

ID_serialiser-5

qTT_strobe

TT[9..0]

TTCount[5..0]

TT_load

TT_serialiser-0

TT_serialiser-1

TT_serialiser-2

TT_serialiser-3

dboe_h

dboe_l

IDin-int0

IDin-int1

REDUCED EQUATIONS:

fs1.EQN = 0 ;  "(1 term, 0 symbols)

fs0.EQN = 0 ;  "(1 term, 0 symbols)

nFIFO_rst.EQN = machine_fifo_reset-2 ;  "(1 term, 1 symbol)

machine_fifo_reset-0.D = /machine_fifo_reset-0*

/machine_fifo_reset-2 ;  "(1 term, 2 symbols)

machine_fifo_reset-0.CLK = clk5 ;  "(1 term, 1 symbol)

machine_fifo_reset-0.RESET = /nOA_rst ;  "(1 term, 1 symbol)

machine_fifo_reset-1.D = machine_fifo_reset-0*

/machine_fifo_reset-1

+ /machine_fifo_reset-0*

machine_fifo_reset-1 ;  "(2 terms, 2 symbols)

machine_fifo_reset-1.CLK = clk5 ;  "(1 term, 1 symbol)

machine_fifo_reset-1.RESET = /nOA_rst ;  "(1 term, 1 symbol)

machine_fifo_reset-2.D = machine_fifo_reset-0*

machine_fifo_reset-1

+ machine_fifo_reset-2 ;  "(2 terms, 3 symbols)

machine_fifo_reset-2.CLK = clk5 ;  "(1 term, 1 symbol)

machine_fifo_reset-2.RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1_latch.D = 1 ;  "(1 term, 0 symbols)

L1_latch.CLK = L1_strobe ;  "(1 term, 1 symbol)

L1_latch.RESET = /nOA_rst

+ qID_strobe ;  "(2 terms, 2 symbols)

BC_latch.D = 1 ;  "(1 term, 0 symbols)

BC_latch.CLK = BC_strobe ;  "(1 term, 1 symbol)

BC_latch.RESET = /nOA_rst

+ qID_strobe ;  "(2 terms, 2 symbols)

qID_strobe.D = BC_latch*L1_latch ;  "(1 term, 2 symbols)

qID_strobe.CLK = clk5 ;  "(1 term, 1 symbol)

qID_strobe.RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDclkA.EQN = clk5 ;  "(1 term, 1 symbol)

IDenA.D = qID_strobe ;  "(1 term, 1 symbol)

IDenA.CLK = IDclkA ;  "(1 term, 1 symbol)

IDenA.RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDclkB.EQN = clk5 ;  "(1 term, 1 symbol)

L1ID[23].D = fifo_L1ID_high[11] ;  "(1 term, 1 symbol)

L1ID[23].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[23].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[22].D = fifo_L1ID_high[10] ;  "(1 term, 1 symbol)

L1ID[22].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[22].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[21].D = fifo_L1ID_high[9] ;  "(1 term, 1 symbol)

L1ID[21].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[21].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[20].D = fifo_L1ID_high[8] ;  "(1 term, 1 symbol)

L1ID[20].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[20].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[19].D = fifo_L1ID_high[7] ;  "(1 term, 1 symbol)

L1ID[19].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[19].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[18].D = fifo_L1ID_high[6] ;  "(1 term, 1 symbol)

L1ID[18].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[18].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[17].D = fifo_L1ID_high[5] ;  "(1 term, 1 symbol)

L1ID[17].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[17].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[16].D = fifo_L1ID_high[4] ;  "(1 term, 1 symbol)

L1ID[16].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[16].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[15].D = fifo_L1ID_high[3] ;  "(1 term, 1 symbol)

L1ID[15].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[15].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[14].D = fifo_L1ID_high[2] ;  "(1 term, 1 symbol)

L1ID[14].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[14].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[13].D = fifo_L1ID_high[1] ;  "(1 term, 1 symbol)

L1ID[13].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[13].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[12].D = fifo_L1ID_high[0] ;  "(1 term, 1 symbol)

L1ID[12].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[12].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[11].D = fifo_L1ID_low[11] ;  "(1 term, 1 symbol)

L1ID[11].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[11].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[10].D = fifo_L1ID_low[10] ;  "(1 term, 1 symbol)

L1ID[10].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[10].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[9].D = fifo_L1ID_low[9] ;  "(1 term, 1 symbol)

L1ID[9].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[9].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[8].D = fifo_L1ID_low[8] ;  "(1 term, 1 symbol)

L1ID[8].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[8].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[7].D = fifo_L1ID_low[7] ;  "(1 term, 1 symbol)

L1ID[7].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[6].D = fifo_L1ID_low[6] ;  "(1 term, 1 symbol)

L1ID[6].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[5].D = fifo_L1ID_low[5] ;  "(1 term, 1 symbol)

L1ID[5].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[4].D = fifo_L1ID_low[4] ;  "(1 term, 1 symbol)

L1ID[4].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[3].D = fifo_L1ID_low[3] ;  "(1 term, 1 symbol)

L1ID[3].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[2].D = fifo_L1ID_low[2] ;  "(1 term, 1 symbol)

L1ID[2].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[1].D = fifo_L1ID_low[1] ;  "(1 term, 1 symbol)

L1ID[1].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

L1ID[0].D = fifo_L1ID_low[0] ;  "(1 term, 1 symbol)

L1ID[0].CLK = ID_load ;  "(1 term, 1 symbol)

L1ID[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[11].D = fifo_BCID[11] ;  "(1 term, 1 symbol)

BCID[11].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[11].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[10].D = fifo_BCID[10] ;  "(1 term, 1 symbol)

BCID[10].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[10].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[9].D = fifo_BCID[9] ;  "(1 term, 1 symbol)

BCID[9].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[9].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[8].D = fifo_BCID[8] ;  "(1 term, 1 symbol)

BCID[8].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[8].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[7].D = fifo_BCID[7] ;  "(1 term, 1 symbol)

BCID[7].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[6].D = fifo_BCID[6] ;  "(1 term, 1 symbol)

BCID[6].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[5].D = fifo_BCID[5] ;  "(1 term, 1 symbol)

BCID[5].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[4].D = fifo_BCID[4] ;  "(1 term, 1 symbol)

BCID[4].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[3].D = fifo_BCID[3] ;  "(1 term, 1 symbol)

BCID[3].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[2].D = fifo_BCID[2] ;  "(1 term, 1 symbol)

BCID[2].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[1].D = fifo_BCID[1] ;  "(1 term, 1 symbol)

BCID[1].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

BCID[0].D = fifo_BCID[0] ;  "(1 term, 1 symbol)

BCID[0].CLK = ID_load ;  "(1 term, 1 symbol)

BCID[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDenB.D = /ID_serialiser-0*/ID_serialiser-1*

/ID_serialiser-2*/ID_serialiser-3*

/ID_serialiser-4*/ID_serialiser-5*/IDenB*nIDef ;  "(1 term, 8 symbols)

IDenB.CLK = IDclkB ;  "(1 term, 1 symbol)

IDenB.RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDCount[5].T = IDCount[0]*IDCount[1]*IDCount[2]*

IDCount[3]*IDCount[4]*IDenA

+ /IDCount[0]*/IDCount[1]*/IDCount[2]

*/IDCount[3]*/IDCount[4]*/IDenA*

IDenB ;  "(2 terms, 7 symbols)

IDCount[5].CLK = clk5 ;  "(1 term, 1 symbol)

IDCount[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDCount[4].T = IDCount[0]*IDCount[1]*IDCount[2]*

IDCount[3]*IDenA

+ /IDCount[0]*/IDCount[1]*/IDCount[2]

*/IDCount[3]*/IDenA*IDenB ;  "(2 terms, 6 symbols)

IDCount[4].CLK = clk5 ;  "(1 term, 1 symbol)

IDCount[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDCount[3].T = IDCount[0]*IDCount[1]*IDCount[2]*IDenA

+ /IDCount[0]*/IDCount[1]*/IDCount[2]

*/IDenA*IDenB ;  "(2 terms, 5 symbols)

IDCount[3].CLK = clk5 ;  "(1 term, 1 symbol)

IDCount[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDCount[2].T = IDCount[0]*IDCount[1]*IDenA

+ /IDCount[0]*/IDCount[1]*/IDenA*

IDenB ;  "(2 terms, 4 symbols)

IDCount[2].CLK = clk5 ;  "(1 term, 1 symbol)

IDCount[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDCount[1].T = IDCount[0]*IDenA

+ /IDCount[0]*/IDenA*IDenB ;  "(2 terms, 3 symbols)

IDCount[1].CLK = clk5 ;  "(1 term, 1 symbol)

IDCount[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDCount[0].CLK = clk5 ;  "(1 term, 1 symbol)

IDCount[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

IDCount[0].T(~) = /IDenA*/IDenB ;  "(1 term, 2 symbols)

IDin.EQN = /ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[7]

+ /ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[15]

+ /ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-2*/ID_serialiser-3*

/ID_serialiser-4*/ID_serialiser-5

+ /ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-3*/ID_serialiser-4*

/ID_serialiser-5*L1ID[3]

+ /ID_serialiser-0*/ID_serialiser-1*

ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[9]

+ /ID_serialiser-0*/ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[17]

+ /ID_serialiser-0*/ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-3*

/ID_serialiser-4*/ID_serialiser-5*L1ID[1]

+ /ID_serialiser-0*/ID_serialiser-1*

/ID_serialiser-2*ID_serialiser-3*ID_serialiser-4*

L1ID[21]

+ /ID_serialiser-0*/ID_serialiser-1*

/ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[5]

+ /ID_serialiser-0*/ID_serialiser-1*

/ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[13]

+ ID_serialiser-1*/ID_serialiser-2*

/ID_serialiser-3*/ID_serialiser-4*

/ID_serialiser-5*L1ID[0]

+ IDin-int0

+ IDin-int1 ;  "(13 terms, 18 symbols)

ID_load.EQN = /ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-2*/ID_serialiser-3*

/ID_serialiser-4*/ID_serialiser-5 ;  "(1 term, 6 symbols)

ID_serialiser-0.CLK = clk5 ;  "(1 term, 1 symbol)

ID_serialiser-0.RESET = /nOA_rst ;  "(1 term, 1 symbol)

ID_serialiser-0.D(~) = ID_serialiser-0

+ /ID_serialiser-1*/ID_serialiser-2*

/ID_serialiser-3*/ID_serialiser-4*

/ID_serialiser-5*/IDenB ;  "(2 terms, 7 symbols)

ID_serialiser-1.D = ID_serialiser-0*/ID_serialiser-1

+ /ID_serialiser-0*ID_serialiser-1 ;  "(2 terms, 2 symbols)

ID_serialiser-1.CLK = clk5 ;  "(1 term, 1 symbol)

ID_serialiser-1.RESET = /nOA_rst ;  "(1 term, 1 symbol)

ID_serialiser-2.T = ID_serialiser-0*ID_serialiser-1 ;  "(1 term, 2 symbols)

ID_serialiser-2.CLK = clk5 ;  "(1 term, 1 symbol)

ID_serialiser-2.RESET = /nOA_rst ;  "(1 term, 1 symbol)

ID_serialiser-3.T = ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-5 ;  "(1 term, 4 symbols)

ID_serialiser-3.CLK = clk5 ;  "(1 term, 1 symbol)

ID_serialiser-3.RESET = /nOA_rst ;  "(1 term, 1 symbol)

ID_serialiser-4.T = ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*ID_serialiser-3 ;  "(1 term, 4 symbols)

ID_serialiser-4.CLK = clk5 ;  "(1 term, 1 symbol)

ID_serialiser-4.RESET = /nOA_rst ;  "(1 term, 1 symbol)

ID_serialiser-5.T = ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*ID_serialiser-3*ID_serialiser-4

+ ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*ID_serialiser-5 ;  "(2 terms, 6 symbols)

ID_serialiser-5.CLK = clk5 ;  "(1 term, 1 symbol)

ID_serialiser-5.RESET = /nOA_rst ;  "(1 term, 1 symbol)

qTT_strobe.D = TT_strobe ;  "(1 term, 1 symbol)

qTT_strobe.CLK = clk5 ;  "(1 term, 1 symbol)

qTT_strobe.RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTclkA.EQN = clk5 ;  "(1 term, 1 symbol)

TTenA.D = qTT_strobe ;  "(1 term, 1 symbol)

TTenA.CLK = TTclkA ;  "(1 term, 1 symbol)

TTenA.RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTclkb.EQN = clk5 ;  "(1 term, 1 symbol)

TT[9].D = fifo_TT[9] ;  "(1 term, 1 symbol)

TT[9].CLK = TT_load ;  "(1 term, 1 symbol)

TT[9].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[8].D = fifo_TT[8] ;  "(1 term, 1 symbol)

TT[8].CLK = TT_load ;  "(1 term, 1 symbol)

TT[8].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[7].D = fifo_TT[7] ;  "(1 term, 1 symbol)

TT[7].CLK = TT_load ;  "(1 term, 1 symbol)

TT[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[6].D = fifo_TT[6] ;  "(1 term, 1 symbol)

TT[6].CLK = TT_load ;  "(1 term, 1 symbol)

TT[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[5].D = fifo_TT[5] ;  "(1 term, 1 symbol)

TT[5].CLK = TT_load ;  "(1 term, 1 symbol)

TT[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[4].D = fifo_TT[4] ;  "(1 term, 1 symbol)

TT[4].CLK = TT_load ;  "(1 term, 1 symbol)

TT[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[3].D = fifo_TT[3] ;  "(1 term, 1 symbol)

TT[3].CLK = TT_load ;  "(1 term, 1 symbol)

TT[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[2].D = fifo_TT[2] ;  "(1 term, 1 symbol)

TT[2].CLK = TT_load ;  "(1 term, 1 symbol)

TT[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[1].D = fifo_TT[1] ;  "(1 term, 1 symbol)

TT[1].CLK = TT_load ;  "(1 term, 1 symbol)

TT[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT[0].D = fifo_TT[0] ;  "(1 term, 1 symbol)

TT[0].CLK = TT_load ;  "(1 term, 1 symbol)

TT[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTenB.D = /TT_serialiser-0*/TT_serialiser-1*

/TT_serialiser-2*/TT_serialiser-3*/TTenB*nTTef ;  "(1 term, 6 symbols)

TTenB.CLK = TTclkb ;  "(1 term, 1 symbol)

TTenB.RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTCount[5].T = TTCount[0]*TTCount[1]*TTCount[2]*

TTCount[3]*TTCount[4]*TTenA

+ /TTCount[0]*/TTCount[1]*/TTCount[2]

*/TTCount[3]*/TTCount[4]*/TTenA*

TTenB ;  "(2 terms, 7 symbols)

TTCount[5].CLK = TTclkA ;  "(1 term, 1 symbol)

TTCount[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTCount[4].T = TTCount[0]*TTCount[1]*TTCount[2]*

TTCount[3]*TTenA

+ /TTCount[0]*/TTCount[1]*/TTCount[2]

*/TTCount[3]*/TTenA*TTenB ;  "(2 terms, 6 symbols)

TTCount[4].CLK = TTclkA ;  "(1 term, 1 symbol)

TTCount[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTCount[3].T = TTCount[0]*TTCount[1]*TTCount[2]*TTenA

+ /TTCount[0]*/TTCount[1]*/TTCount[2]

*/TTenA*TTenB ;  "(2 terms, 5 symbols)

TTCount[3].CLK = TTclkA ;  "(1 term, 1 symbol)

TTCount[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTCount[2].T = TTCount[0]*TTCount[1]*TTenA

+ /TTCount[0]*/TTCount[1]*/TTenA*

TTenB ;  "(2 terms, 4 symbols)

TTCount[2].CLK = TTclkA ;  "(1 term, 1 symbol)

TTCount[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTCount[1].T = TTCount[0]*TTenA

+ /TTCount[0]*/TTenA*TTenB ;  "(2 terms, 3 symbols)

TTCount[1].CLK = TTclkA ;  "(1 term, 1 symbol)

TTCount[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTCount[0].CLK = TTclkA ;  "(1 term, 1 symbol)

TTCount[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

TTCount[0].T(~) = /TTenA*/TTenB ;  "(1 term, 2 symbols)

TTin.EQN = TT[0]*TT_serialiser-1*/TT_serialiser-2

*/TT_serialiser-3

+ TT[1]*/TT_serialiser-0*

/TT_serialiser-1*TT_serialiser-2*/TT_serialiser-3

+ TT[2]*TT_serialiser-0*

/TT_serialiser-1*TT_serialiser-2*/TT_serialiser-3

+ TT[3]*/TT_serialiser-0*

TT_serialiser-1*TT_serialiser-2

+ TT[4]*TT_serialiser-0*

TT_serialiser-1*TT_serialiser-2

+ TT[5]*/TT_serialiser-0*

/TT_serialiser-1*/TT_serialiser-2*TT_serialiser-3

+ TT[6]*TT_serialiser-0*

/TT_serialiser-1*/TT_serialiser-2*TT_serialiser-3

+ TT[7]*/TT_serialiser-0*

TT_serialiser-1*/TT_serialiser-2

+ TT[8]*TT_serialiser-0*

TT_serialiser-1*TT_serialiser-3

+ TT[9]*/TT_serialiser-0*

TT_serialiser-2*TT_serialiser-3

+ /TT_serialiser-0*TT_serialiser-1*

/TT_serialiser-2*/TT_serialiser-3 ;  "(11 terms, 14 symbols)

TT_load.EQN = /TT_serialiser-0*TT_serialiser-1*

/TT_serialiser-2*/TT_serialiser-3 ;  "(1 term, 4 symbols)

TT_serialiser-0.CLK = clk5 ;  "(1 term, 1 symbol)

TT_serialiser-0.RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT_serialiser-0.D(~) = TT_serialiser-0

+ /TT_serialiser-1*/TT_serialiser-2*

/TT_serialiser-3*/TTenB ;  "(2 terms, 5 symbols)

TT_serialiser-1.D = TT_serialiser-0*/TT_serialiser-1*

/TT_serialiser-2

+ TT_serialiser-0*/TT_serialiser-1*

/TT_serialiser-3

+ /TT_serialiser-0*TT_serialiser-1 ;  "(3 terms, 4 symbols)

TT_serialiser-1.CLK = clk5 ;  "(1 term, 1 symbol)

TT_serialiser-1.RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT_serialiser-2.D = TT_serialiser-0*TT_serialiser-1*

/TT_serialiser-2

+ /TT_serialiser-0*TT_serialiser-2

+ /TT_serialiser-1*TT_serialiser-2*

/TT_serialiser-3 ;  "(3 terms, 4 symbols)

TT_serialiser-2.CLK = clk5 ;  "(1 term, 1 symbol)

TT_serialiser-2.RESET = /nOA_rst ;  "(1 term, 1 symbol)

TT_serialiser-3.D = TT_serialiser-0*TT_serialiser-1*

TT_serialiser-2

+ /TT_serialiser-0*TT_serialiser-3

+ /TT_serialiser-2*TT_serialiser-3 ;  "(3 terms, 4 symbols)

TT_serialiser-3.CLK = clk5 ;  "(1 term, 1 symbol)

TT_serialiser-3.RESET = /nOA_rst ;  "(1 term, 1 symbol)

db_h[15].EQN = Reg0E*/nTTff ;  "(1 term, 2 symbols)

db_h[15].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[14].EQN = Reg0E*/nTTef ;  "(1 term, 2 symbols)

db_h[14].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[13].EQN = Reg0E*TTCount[5] ;  "(1 term, 2 symbols)

db_h[13].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[12].EQN = Reg0E*TTCount[4] ;  "(1 term, 2 symbols)

db_h[12].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[11].EQN = Reg0E*TTCount[3] ;  "(1 term, 2 symbols)

db_h[11].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[10].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[10].EQN(~) = Reg0E*/TTCount[2] ;  "(1 term, 2 symbols)

db_h[9].EQN = Reg0E*TTCount[1] ;  "(1 term, 2 symbols)

db_h[9].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[8].OE = dboe_h ;  "(1 term, 1 symbol)

db_h[8].EQN(~) = Reg0E*/TTCount[0] ;  "(1 term, 2 symbols)

db_l[7].EQN = /nIDff ;  "(1 term, 1 symbol)

db_l[7].OE = dboe_l ;  "(1 term, 1 symbol)

db_l[6].EQN = /nIDef ;  "(1 term, 1 symbol)

db_l[6].OE = dboe_l ;  "(1 term, 1 symbol)

db_l[5].EQN = IDCount[5] ;  "(1 term, 1 symbol)

db_l[5].OE = dboe_l ;  "(1 term, 1 symbol)

db_l[4].EQN = IDCount[4] ;  "(1 term, 1 symbol)

db_l[4].OE = dboe_l ;  "(1 term, 1 symbol)

db_l[3].EQN = IDCount[3] ;  "(1 term, 1 symbol)

db_l[3].OE = dboe_l ;  "(1 term, 1 symbol)

db_l[2].EQN = IDCount[2] ;  "(1 term, 1 symbol)

db_l[2].OE = dboe_l ;  "(1 term, 1 symbol)

db_l[1].EQN = IDCount[1] ;  "(1 term, 1 symbol)

db_l[1].OE = dboe_l ;  "(1 term, 1 symbol)

db_l[0].EQN = IDCount[0] ;  "(1 term, 1 symbol)

db_l[0].OE = dboe_l ;  "(1 term, 1 symbol)

dboe_h.EQN = Reg0E*bReadb

+ bReadb*reg32 ;  "(2 terms, 3 symbols)

dboe_l.EQN = Reg0E*bReadb ;  "(1 term, 2 symbols)

IDin-int0.EQN = BCID[0]*ID_serialiser-0*

ID_serialiser-1*/ID_serialiser-2*ID_serialiser-3

*ID_serialiser-4

+ BCID[10]*ID_serialiser-0*

/ID_serialiser-1*ID_serialiser-2*ID_serialiser-5

+ BCID[11]*/ID_serialiser-0*

ID_serialiser-1*ID_serialiser-2*ID_serialiser-5

+ BCID[1]*/ID_serialiser-0*

/ID_serialiser-1*ID_serialiser-2*ID_serialiser-3*

ID_serialiser-4

+ BCID[2]*ID_serialiser-0*

/ID_serialiser-1*ID_serialiser-2*ID_serialiser-3*

ID_serialiser-4

+ BCID[3]*/ID_serialiser-0*

ID_serialiser-1*ID_serialiser-2*ID_serialiser-3*

ID_serialiser-4

+ BCID[4]*ID_serialiser-0*

ID_serialiser-1*ID_serialiser-2*ID_serialiser-3*

ID_serialiser-4

+ BCID[5]*/ID_serialiser-0*

/ID_serialiser-1*/ID_serialiser-2*ID_serialiser-5

+ BCID[6]*ID_serialiser-0*

/ID_serialiser-1*/ID_serialiser-2*ID_serialiser-5

+ BCID[7]*/ID_serialiser-0*

ID_serialiser-1*/ID_serialiser-2*ID_serialiser-5

+ BCID[8]*ID_serialiser-0*

ID_serialiser-1*/ID_serialiser-2*ID_serialiser-5

+ BCID[9]*/ID_serialiser-0*

/ID_serialiser-1*ID_serialiser-2*ID_serialiser-5

+ ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[12] ;  "(13 terms, 19 symbols)

IDin-int1.EQN = ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[20]

+ ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-3*

/ID_serialiser-4*/ID_serialiser-5*L1ID[4]

+ ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[8]

+ ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[16]

+ ID_serialiser-0*/ID_serialiser-1*

ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[10]

+ ID_serialiser-0*/ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[18]

+ ID_serialiser-0*/ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-3*

/ID_serialiser-4*/ID_serialiser-5*L1ID[2]

+ ID_serialiser-0*/ID_serialiser-1*

/ID_serialiser-2*ID_serialiser-3*ID_serialiser-4*

L1ID[22]

+ ID_serialiser-0*/ID_serialiser-1*

/ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[6]

+ ID_serialiser-0*/ID_serialiser-1*

/ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[14]

+ /ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*ID_serialiser-3*/ID_serialiser-4

*L1ID[11]

+ /ID_serialiser-0*ID_serialiser-1*

ID_serialiser-2*/ID_serialiser-3*ID_serialiser-4

*L1ID[19]

+ /ID_serialiser-0*ID_serialiser-1*

/ID_serialiser-2*ID_serialiser-3*ID_serialiser-4*

L1ID[23] ;  "(13 terms, 19 symbols)

PLDocument:  pld5.doc

SOLUTIONS     Fri Apr 19 14:17:41 2002

PARTITIONING CRITERIA :

AVAILABLE 'amdlib.avl' ;

WEIGHT FMAX 10 ;

WEIGHT TPD 10 ;

FAMILY = CMOS  ;

TEMP_RANGE = COM  ;

TEMPLATE = MV384_184  ;

PARTITIONING SOLUTIONS :

==> Solution  1: MV384_184

FUSEMAP FILES FOR SOLUTION 1:

Device 1 (MV384_184) : pld5.j1

PLDocument:  pld5.doc

PINOUT DIAGRAMS  Fri Apr 19 14:17:41 2002

Device  1 - MV384_184 -- Pinout for QFP package

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|    1  |Jtag  |                      | |   51  |Vcc   |                      |

|    2  |Biput |                      | |   52  |Biput |                      |

|    3  |Biput |  fifo_BCID[0]        | |   53  |Biput |  fifo_L1ID_low[6]    |

|    4  |Biput |  fifo_BCID[1]        | |   54  |Biput |  fifo_L1ID_low[7]    |

|    5  |Biput |  fifo_BCID[2]        | |   55  |Biput |  fifo_L1ID_low[8]    |

|    6  |Biput |  fifo_BCID[3]        | |   56  |Biput |  fifo_L1ID_low[9]    |

|    7  |Biput |  fifo_BCID[4]        | |   57  |Biput |  fifo_L1ID_low[10]   |

|    8  |Biput |  fifo_BCID[5]        | |   58  |Biput |  fifo_L1ID_low[11]   |

|    9  |Biput |                      | |   59  |Biput |                      |

|   10  |Vcc   |                      | |   60  |Jtag  |                      |

|   11  |GND   |                      | |   61  |GND   |                      |

|   12  |Biput |                      | |   62  |Vcc   |                      |

|   13  |Biput |  fifo_BCID[6]        | |   63  |Biput |                      |

|   14  |Biput |  fifo_BCID[7]        | |   64  |Biput |  fifo_TT[0]          |

|   15  |Biput |  fifo_BCID[8]        | |   65  |Biput |  fifo_TT[1]          |

|   16  |Biput |  fifo_BCID[9]        | |   66  |Biput |  fifo_TT[2]          |

|   17  |Biput |  fifo_BCID[10]       | |   67  |Biput |  fifo_TT[3]          |

|   18  |Biput |  fifo_BCID[11]       | |   68  |Biput |  fifo_TT[4]          |

|   19  |Biput |                      | |   69  |Biput |                      |

|   20  |GND   |                      | |   70  |Biput |                      |

|   21  |Biput |                      | |   71  |GND   |                      |

|   22  |Biput |  fifo_L1ID_high[0]   | |   72  |Vcc   |                      |

|   23  |Biput |  fifo_L1ID_high[1]   | |   73  |Biput |                      |

|   24  |Biput |  fifo_L1ID_high[2]   | |   74  |Biput |  fifo_TT[5]          |

|   25  |Biput |  fifo_L1ID_high[3]   | |   75  |Biput |  fifo_TT[6]          |

|   26  |Biput |  fifo_L1ID_high[4]   | |   76  |Biput |  fifo_TT[7]          |

|   27  |Biput |  fifo_L1ID_high[5]   | |   77  |Biput |  fifo_TT[8]          |

|   28  |Biput |                      | |   78  |Biput |  fifo_TT[9]          |

|   29  |In/CLK|                      | |   79  |Biput |                      |

|   30  |Vcc   |                      | |   80  |Biput |                      |

|   31  |GND   |                      | |   81  |GND   |                      |

|   32  |In/CLK|                      | |   82  |Biput |  TTclkA              |

|   33  |Biput |                      | |   83  |Biput |  TTenA               |

|   34  |Biput |  fifo_L1ID_high[6]   | |   84  |Biput |                      |

|   35  |Biput |  fifo_L1ID_high[7]   | |   85  |Biput |                      |

|   36  |Biput |  fifo_L1ID_high[8]   | |   86  |Biput |                      |

|   37  |Biput |  fifo_L1ID_high[9]   | |   87  |Biput |  RunMode             |

|   38  |Biput |  fifo_L1ID_high[10]  | |   88  |Vcc   |                      |

|   39  |Biput |  fifo_L1ID_high[11]  | |   89  |GND   |                      |

|   40  |Biput |                      | |   90  |GND   |                      |

|   41  |GND   |                      | |   91  |GND   |                      |

|   42  |Biput |                      | |   92  |GND   |                      |

|   43  |Biput |  fifo_L1ID_low[0]    | |   93  |Vcc   |                      |

|   44  |Biput |  fifo_L1ID_low[1]    | |   94  |Biput |  TTclkb              |

|   45  |Biput |  fifo_L1ID_low[2]    | |   95  |Biput |  TTenB               |

|   46  |Biput |  fifo_L1ID_low[3]    | |   96  |Biput |                      |

|   47  |Biput |  fifo_L1ID_low[4]    | |   97  |Biput |                      |

|   48  |Biput |  fifo_L1ID_low[5]    | |   98  |Biput |                      |

|   49  |Biput |                      | |   99  |Biput |                      |

|   50  |GND   |                      | |  100  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  101  |Biput |  L1_strobe           | |  151  |GND   |                      |

|  102  |Biput |  BC_strobe           | |  152  |In/CLK|  nLEb2               |

|  103  |Biput |  TT_strobe           | |  153  |Biput |  s4b_5[3]            |

|  104  |Biput |                      | |  154  |Biput |  s4b_5[2]            |

|  105  |Biput |  nOA_rst             | |  155  |Biput |  s4b_5[1]            |

|  106  |Biput |  bWriteb             | |  156  |Biput |  s4b_5[0]            |

|  107  |Biput |  bReadb              | |  157  |Biput |  db_h[15]            |

|  108  |Biput |  Reg0E               | |  158  |Biput |  db_h[14]            |

|  109  |Vcc   |                      | |  159  |Biput |  db_h[13]            |

|  110  |GND   |                      | |  160  |Biput |  db_h[12]            |

|  111  |Biput |  nTTaf               | |  161  |GND   |                      |

|  112  |Biput |  nTTae               | |  162  |Biput |  s4b_5[7]            |

|  113  |Biput |  nTTef               | |  163  |Biput |  s4b_5[6]            |

|  114  |Biput |  nTTff               | |  164  |Biput |  s4b_5[5]            |

|  115  |Biput |  nIDae               | |  165  |Biput |  s4b_5[4]            |

|  116  |Biput |  nIDef               | |  166  |Biput |  db_h[11]            |

|  117  |Biput |  nIDff               | |  167  |Biput |  db_h[10]            |

|  118  |Biput |  nIDaf               | |  168  |Biput |  db_h[9]             |

|  119  |Vcc   |                      | |  169  |Biput |  db_h[8]             |

|  120  |GND   |                      | |  170  |GND   |                      |

|  121  |Jtag  |                      | |  171  |Vcc   |                      |

|  122  |Biput |  IDenA               | |  172  |Biput |  s5_6[3]             |

|  123  |Biput |  IDclkA              | |  173  |Biput |  s5_6[2]             |

|  124  |Biput |                      | |  174  |Biput |  s5_6[1]             |

|  125  |Biput |                      | |  175  |Biput |  s5_6[0]             |

|  126  |Biput |                      | |  176  |Biput |  db_l[7]             |

|  127  |Biput |                      | |  177  |Biput |  db_l[6]             |

|  128  |Biput |  fs0                 | |  178  |Biput |  db_l[5]             |

|  129  |Biput |  fs1                 | |  179  |Biput |  db_l[4]             |

|  130  |Vcc   |                      | |  180  |Jtag  |                      |

|  131  |GND   |                      | |  181  |GND   |                      |

|  132  |Biput |  IDclkB              | |  182  |Vcc   |                      |

|  133  |Biput |  IDenB               | |  183  |Biput |  s5_6[7]             |

|  134  |Biput |                      | |  184  |Biput |  s5_6[6]             |

|  135  |Biput |                      | |  185  |Biput |  s5_6[5]             |

|  136  |Biput |                      | |  186  |Biput |  s5_6[4]             |

|  137  |Biput |                      | |  187  |Biput |  db_l[3]             |

|  138  |Biput |                      | |  188  |Biput |  db_l[2]             |

|  139  |Biput |                      | |  189  |Biput |  db_l[1]             |

|  140  |GND   |                      | |  190  |Biput |  db_l[0]             |

|  141  |Biput |  sgbA[7]             | |  191  |GND   |                      |

|  142  |Biput |  sgbA[6]             | |  192  |Vcc   |                      |

|  143  |Biput |  sgbA[5]             | |  193  |Biput |  sgbB[7]             |

|  144  |Biput |  sgbA[4]             | |  194  |Biput |  sgbB[6]             |

|  145  |Biput |  sgbA[3]             | |  195  |Biput |  sgbB[5]             |

|  146  |Biput |  sgbA[2]             | |  196  |Biput |  sgbB[4]             |

|  147  |Biput |  sgbA[1]             | |  197  |Biput |  sgbB[3]             |

|  148  |Biput |  sgbA[0]             | |  198  |Biput |  sgbB[2]             |

|  149  |In/CLK|  clk5                | |  199  |Biput |  sgbB[1]             |

|  150  |Vcc   |                      | |  200  |Biput |  sgbB[0]             |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  201  |GND   |                      | |  221  |Biput |  sgbC[7]             |

|  202  |Biput |  IDin                | |  222  |Biput |  sgbC[6]             |

|  203  |Biput |  s1_5[3]             | |  223  |Biput |  sgbC[5]             |

|  204  |Biput |  s1_5[2]             | |  224  |Biput |  sgbC[4]             |

|  205  |Biput |  s1_5[1]             | |  225  |Biput |  sgbC[3]             |

|  206  |Biput |  s1_5[0]             | |  226  |Biput |  sgbC[2]             |

|  207  |Biput |                      | |  227  |Biput |  sgbC[1]             |

|  208  |Vcc   |                      | |  228  |Biput |  sgbC[0]             |

|  209  |GND   |                      | |  229  |Vcc   |                      |

|  210  |GND   |                      | |  230  |GND   |                      |

|  211  |GND   |                      | |  231  |Biput |                      |

|  212  |GND   |                      | |  232  |Biput |                      |

|  213  |Vcc   |                      | |  233  |Biput |  sgbD[5]             |

|  214  |Biput |  TTin                | |  234  |Biput |  sgbD[4]             |

|  215  |Biput |  reg32               | |  235  |Biput |  sgbD[3]             |

|  216  |Biput |  reg34               | |  236  |Biput |  sgbD[2]             |

|  217  |Biput |  s1_5[5]             | |  237  |Biput |  sgbD[1]             |

|  218  |Biput |  s1_5[4]             | |  238  |Biput |  sgbD[0]             |

|  219  |Biput |  nFIFO_rst           | |  239  |Vcc   |                      |

|  220  |GND   |                      | |  240  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

DEVICE SELECTION:

Device         Man  Fam Pack Temp   ICC     TPD     Fmax    Price   User

==> M5-384/184-7HC   AMD CMOS QFP  COM   650.0ma   9.5ns 105.0MHz $  0.00   0 0

2) M5-384/184-10HC  AMD CMOS QFP  COM   550.0ma  11.5ns  87.0MHz $  0.00   0 0

3) M5-384/184-12HC  AMD CMOS QFP  COM   450.0ma  14.0ns  71.0MHz $  0.00   0 0

4) M5-384/184-15HC  AMD CMOS QFP  COM   400.0ma  18.0ns  55.0MHz $  0.00   0 0

PLDocument:  pld5.doc

WIRELIST      Fri Apr 19 14:17:41 2002

+--------------------+-------------------+-------+

|       Signal       |       Device      |  Pin  |

+--------------------+-------------------+-------+

| nOA_rst            | MV384_184_1       |  105  |

| RunMode            | MV384_184_1       |   87  |

| clk5               | MV384_184_1       |  149  |

| Reg0E              | MV384_184_1       |  108  |

| bReadb             | MV384_184_1       |  107  |

| bWriteb            | MV384_184_1       |  106  |

| nLEb2              | MV384_184_1       |  152  |

| fs1                | MV384_184_1       |  129  |

| fs0                | MV384_184_1       |  128  |

| nFIFO_rst          | MV384_184_1       |  219  |

| L1_strobe          | MV384_184_1       |  101  |

| BC_strobe          | MV384_184_1       |  102  |

| IDclkA             | MV384_184_1       |  123  |

| IDenA              | MV384_184_1       |  122  |

| IDclkB             | MV384_184_1       |  132  |

| fifo_L1ID_low[11]  | MV384_184_1       |   58  |

| fifo_L1ID_low[10]  | MV384_184_1       |   57  |

| fifo_L1ID_low[9]   | MV384_184_1       |   56  |

| fifo_L1ID_low[8]   | MV384_184_1       |   55  |

| fifo_L1ID_low[7]   | MV384_184_1       |   54  |

| fifo_L1ID_low[6]   | MV384_184_1       |   53  |

| fifo_L1ID_low[5]   | MV384_184_1       |   48  |

| fifo_L1ID_low[4]   | MV384_184_1       |   47  |

| fifo_L1ID_low[3]   | MV384_184_1       |   46  |

| fifo_L1ID_low[2]   | MV384_184_1       |   45  |

| fifo_L1ID_low[1]   | MV384_184_1       |   44  |

| fifo_L1ID_low[0]   | MV384_184_1       |   43  |

| fifo_L1ID_high[11] | MV384_184_1       |   39  |

| fifo_L1ID_high[10] | MV384_184_1       |   38  |

| fifo_L1ID_high[9]  | MV384_184_1       |   37  |

| fifo_L1ID_high[8]  | MV384_184_1       |   36  |

| fifo_L1ID_high[7]  | MV384_184_1       |   35  |

| fifo_L1ID_high[6]  | MV384_184_1       |   34  |

| fifo_L1ID_high[5]  | MV384_184_1       |   27  |

| fifo_L1ID_high[4]  | MV384_184_1       |   26  |

| fifo_L1ID_high[3]  | MV384_184_1       |   25  |

| fifo_L1ID_high[2]  | MV384_184_1       |   24  |

| fifo_L1ID_high[1]  | MV384_184_1       |   23  |

| fifo_L1ID_high[0]  | MV384_184_1       |   22  |

| fifo_BCID[11]      | MV384_184_1       |   18  |

| fifo_BCID[10]      | MV384_184_1       |   17  |

| fifo_BCID[9]       | MV384_184_1       |   16  |

| fifo_BCID[8]       | MV384_184_1       |   15  |

| fifo_BCID[7]       | MV384_184_1       |   14  |

| fifo_BCID[6]       | MV384_184_1       |   13  |

| fifo_BCID[5]       | MV384_184_1       |    8  |

| fifo_BCID[4]       | MV384_184_1       |    7  |

| fifo_BCID[3]       | MV384_184_1       |    6  |

| fifo_BCID[2]       | MV384_184_1       |    5  |

| fifo_BCID[1]       | MV384_184_1       |    4  |

| fifo_BCID[0]       | MV384_184_1       |    3  |

+--------------------+-------------------+-------+

+--------------------+-------------------+-------+

|       Signal       |       Device      |  Pin  |

+--------------------+-------------------+-------+

| nIDef              | MV384_184_1       |  116  |

| nIDff              | MV384_184_1       |  117  |

| nIDae              | MV384_184_1       |  115  |

| nIDaf              | MV384_184_1       |  118  |

| IDenB              | MV384_184_1       |  133  |

| IDin               | MV384_184_1       |  202  |

| TT_strobe          | MV384_184_1       |  103  |

| TTclkA             | MV384_184_1       |   82  |

| TTenA              | MV384_184_1       |   83  |

| TTclkb             | MV384_184_1       |   94  |

| fifo_TT[9]         | MV384_184_1       |   78  |

| fifo_TT[8]         | MV384_184_1       |   77  |

| fifo_TT[7]         | MV384_184_1       |   76  |

| fifo_TT[6]         | MV384_184_1       |   75  |

| fifo_TT[5]         | MV384_184_1       |   74  |

| fifo_TT[4]         | MV384_184_1       |   68  |

| fifo_TT[3]         | MV384_184_1       |   67  |

| fifo_TT[2]         | MV384_184_1       |   66  |

| fifo_TT[1]         | MV384_184_1       |   65  |

| fifo_TT[0]         | MV384_184_1       |   64  |

| nTTef              | MV384_184_1       |  113  |

| nTTff              | MV384_184_1       |  114  |

| nTTae              | MV384_184_1       |  112  |

| nTTaf              | MV384_184_1       |  111  |

| TTenB              | MV384_184_1       |   95  |

| TTin               | MV384_184_1       |  214  |

| db_h[15]           | MV384_184_1       |  157  |

| db_h[14]           | MV384_184_1       |  158  |

| db_h[13]           | MV384_184_1       |  159  |

| db_h[12]           | MV384_184_1       |  160  |

| db_h[11]           | MV384_184_1       |  166  |

| db_h[10]           | MV384_184_1       |  167  |

| db_h[9]            | MV384_184_1       |  168  |

| db_h[8]            | MV384_184_1       |  169  |

| db_l[7]            | MV384_184_1       |  176  |

| db_l[6]            | MV384_184_1       |  177  |

| db_l[5]            | MV384_184_1       |  178  |

| db_l[4]            | MV384_184_1       |  179  |

| db_l[3]            | MV384_184_1       |  187  |

| db_l[2]            | MV384_184_1       |  188  |

| db_l[1]            | MV384_184_1       |  189  |

| db_l[0]            | MV384_184_1       |  190  |

| reg32              | MV384_184_1       |  215  |

| reg34              | MV384_184_1       |  216  |

| sgbA[7]            | MV384_184_1       |  141  |

| sgbA[6]            | MV384_184_1       |  142  |

| sgbA[5]            | MV384_184_1       |  143  |

| sgbA[4]            | MV384_184_1       |  144  |

| sgbA[3]            | MV384_184_1       |  145  |

| sgbA[2]            | MV384_184_1       |  146  |

| sgbA[1]            | MV384_184_1       |  147  |

+--------------------+-------------------+-------+

+--------------------+-------------------+-------+

|       Signal       |       Device      |  Pin  |

+--------------------+-------------------+-------+

| sgbA[0]            | MV384_184_1       |  148  |

| sgbB[7]            | MV384_184_1       |  193  |

| sgbB[6]            | MV384_184_1       |  194  |

| sgbB[5]            | MV384_184_1       |  195  |

| sgbB[4]            | MV384_184_1       |  196  |

| sgbB[3]            | MV384_184_1       |  197  |

| sgbB[2]            | MV384_184_1       |  198  |

| sgbB[1]            | MV384_184_1       |  199  |

| sgbB[0]            | MV384_184_1       |  200  |

| sgbC[7]            | MV384_184_1       |  221  |

| sgbC[6]            | MV384_184_1       |  222  |

| sgbC[5]            | MV384_184_1       |  223  |

| sgbC[4]            | MV384_184_1       |  224  |

| sgbC[3]            | MV384_184_1       |  225  |

| sgbC[2]            | MV384_184_1       |  226  |

| sgbC[1]            | MV384_184_1       |  227  |

| sgbC[0]            | MV384_184_1       |  228  |

| sgbD[5]            | MV384_184_1       |  233  |

| sgbD[4]            | MV384_184_1       |  234  |

| sgbD[3]            | MV384_184_1       |  235  |

| sgbD[2]            | MV384_184_1       |  236  |

| sgbD[1]            | MV384_184_1       |  237  |

| sgbD[0]            | MV384_184_1       |  238  |

| s1_5[5]            | MV384_184_1       |  217  |

| s1_5[4]            | MV384_184_1       |  218  |

| s1_5[3]            | MV384_184_1       |  203  |

| s1_5[2]            | MV384_184_1       |  204  |

| s1_5[1]            | MV384_184_1       |  205  |

| s1_5[0]            | MV384_184_1       |  206  |

| s4b_5[7]           | MV384_184_1       |  162  |

| s4b_5[6]           | MV384_184_1       |  163  |

| s4b_5[5]           | MV384_184_1       |  164  |

| s4b_5[4]           | MV384_184_1       |  165  |

| s4b_5[3]           | MV384_184_1       |  153  |

| s4b_5[2]           | MV384_184_1       |  154  |

| s4b_5[1]           | MV384_184_1       |  155  |

| s4b_5[0]           | MV384_184_1       |  156  |

| s5_6[7]            | MV384_184_1       |  183  |

| s5_6[6]            | MV384_184_1       |  184  |

| s5_6[5]            | MV384_184_1       |  185  |

| s5_6[4]            | MV384_184_1       |  186  |

| s5_6[3]            | MV384_184_1       |  172  |

| s5_6[2]            | MV384_184_1       |  173  |

| s5_6[1]            | MV384_184_1       |  174  |

| s5_6[0]            | MV384_184_1       |  175  |

+--------------------+-------------------+-------+