PLDocument:  pld7.doc

TITLE PAGE    Wed May 22 11:47:00 2002

A MACHXL6.2.1.11 - (c) Copyright MINC Incorporated 1987-1998

==============================================================================

TITLE    : ATLAS TIM PLD7: RAM Interface

FILE     : pld7

DATE     : Mon Jun 01 05:03:44 1970

ENGINEER : Martin Postranecky / Dominic A Hayes

COMPANY  : University College London

==============================================================================

MODULES  :

Document Generator     	  3.70

File Handler           	  6.2

Language Compiler      	  3.83

Architectural Optimizer	  3.110

Device Lib Scan        	  3.2

Device Library         	  3.17

Device Partition       	  3.21

Device Fusemap         	  3.4

SWITCH VALUES :

(Value in parenthesis represents batch mode switch value)

PLCOMP PRODUCT TERM LIMIT : 128

PLOPT  PRODUCT TERM LIMIT : 128

PLOPT REDUCTION  :   Espresso (1)

NODE GENERATION  :   Procedure Instantiation

Arithmetic and Relational Operators (1)

PLDocument:  pld7.doc

EQUATIONS     Wed May 22 11:47:00 2002

EQUATIONS FOR SYSTEM

INPUT SIGNALS (81) :

nOA_rst

RunMode

SeqClk

SinkClk

RAMselect

bWriteb

bReadb

ab[15..1]

StartSeq

StartSink

eclSpare

eclFER

eclTT

eclID

eclCAL

eclBCR

eclECR

eclL1A

nLEb2

Reg1A

Reg1C

sgbA[7..0]

sgbB[7..0]

sgbC[7..0]

sgbD[4..2]

enExtRodBusy

TIM_OK_OUT

TTC_ready_in

s6_7[7..0]

s7_8[7..0]

OUTPUT SIGNALS (81) :

db[15..0]

SeqBusy

SeqD[7..0]

seqTRIG

seqECR

seqBCR

seqCAL

seqID

seqTT

seqFER

seqSpare

SeqA[14..0]

nSeqWrEn

nSeqOE

nSeqCS

SinkBusy

SinkA[14..0]

SinkD[7..0]

nSinkWrEn

nSinkOE

nSinkCS

XSeqTRIG

XSeqClk1

XSeqClk2

PHYSICAL NODE SIGNALS (102) :

dboe

SinkGoFF

RegisterEnSeqSink[15..0]

RegisterEnd[15..0]

SeqGoFF

SeqCountStop

SeqCountEnClk

SeqCountEnReset

SeqCount[13..0]

_seqTRIG

_seqECR

_seqBCR

_seqCAL

_seqID

_seqTT

_seqFER

_seqSpare

SeqRamWrAccess

equal_to-6

equal_to-13

SinkGoDelay[4..1]

SinkCountEnClk

SinkCount[13..0]

SinkCountStop

SinkCountEnReset

SinkFFin[7..0]

SinkFFout[7..0]

equal_to-25

equal_to-26

REDUCED EQUATIONS:

db[15].EQN = RAMselect*SinkD[7]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[15]

+ /RAMselect*/Reg1A*RegisterEnd[15] ;  "(3 terms, 5 symbols)

db[15].OE = dboe ;  "(1 term, 1 symbol)

db[14].EQN = RAMselect*SinkD[6]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[14]

+ /RAMselect*/Reg1A*RegisterEnd[14] ;  "(3 terms, 5 symbols)

db[14].OE = dboe ;  "(1 term, 1 symbol)

db[13].EQN = RAMselect*SinkD[5]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[13]

+ /RAMselect*/Reg1A*RegisterEnd[13] ;  "(3 terms, 5 symbols)

db[13].OE = dboe ;  "(1 term, 1 symbol)

db[12].EQN = RAMselect*SinkD[4]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[12]

+ /RAMselect*/Reg1A*RegisterEnd[12] ;  "(3 terms, 5 symbols)

db[12].OE = dboe ;  "(1 term, 1 symbol)

db[11].EQN = RAMselect*SinkD[3]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[11]

+ /RAMselect*/Reg1A*RegisterEnd[11] ;  "(3 terms, 5 symbols)

db[11].OE = dboe ;  "(1 term, 1 symbol)

db[10].EQN = RAMselect*SinkD[2]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[10]

+ /RAMselect*/Reg1A*RegisterEnd[10] ;  "(3 terms, 5 symbols)

db[10].OE = dboe ;  "(1 term, 1 symbol)

db[9].EQN = RAMselect*SinkD[1]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[9]

+ /RAMselect*/Reg1A*RegisterEnd[9] ;  "(3 terms, 5 symbols)

db[9].OE = dboe ;  "(1 term, 1 symbol)

db[8].EQN = RAMselect*SinkD[0]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[8]

+ /RAMselect*/Reg1A*RegisterEnd[8] ;  "(3 terms, 5 symbols)

db[8].OE = dboe ;  "(1 term, 1 symbol)

db[7].EQN = RAMselect*SeqD[7]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[7]

+ /RAMselect*/Reg1A*RegisterEnd[7] ;  "(3 terms, 5 symbols)

db[7].OE = dboe ;  "(1 term, 1 symbol)

db[6].EQN = RAMselect*SeqD[6]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[6]

+ /RAMselect*/Reg1A*RegisterEnd[6] ;  "(3 terms, 5 symbols)

db[6].OE = dboe ;  "(1 term, 1 symbol)

db[5].EQN = RAMselect*SeqD[5]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[5]

+ /RAMselect*/Reg1A*RegisterEnd[5] ;  "(3 terms, 5 symbols)

db[5].OE = dboe ;  "(1 term, 1 symbol)

db[4].EQN = RAMselect*SeqD[4]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[4]

+ /RAMselect*/Reg1A*RegisterEnd[4] ;  "(3 terms, 5 symbols)

db[4].OE = dboe ;  "(1 term, 1 symbol)

db[3].EQN = RAMselect*SeqD[3]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[3]

+ /RAMselect*/Reg1A*RegisterEnd[3] ;  "(3 terms, 5 symbols)

db[3].OE = dboe ;  "(1 term, 1 symbol)

db[2].EQN = RAMselect*SeqD[2]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[2]

+ /RAMselect*/Reg1A*RegisterEnd[2] ;  "(3 terms, 5 symbols)

db[2].OE = dboe ;  "(1 term, 1 symbol)

db[1].EQN = RAMselect*SeqD[1]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[1]

+ /RAMselect*/Reg1A*RegisterEnd[1] ;  "(3 terms, 5 symbols)

db[1].OE = dboe ;  "(1 term, 1 symbol)

db[0].EQN = RAMselect*SeqD[0]

+ /RAMselect*Reg1A*

RegisterEnSeqSink[0]

+ /RAMselect*/Reg1A*RegisterEnd[0] ;  "(3 terms, 5 symbols)

db[0].OE = dboe ;  "(1 term, 1 symbol)

dboe.EQN(~) = /RAMselect*/Reg1A*/Reg1C

+ /bReadb ;  "(2 terms, 4 symbols)

SinkGoFF.D = RegisterEnSeqSink[14] ;  "(1 term, 1 symbol)

SinkGoFF.CLK = SinkClk ;  "(1 term, 1 symbol)

SinkGoFF.RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[15].D = db[15] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[15].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[15].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[15].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[14].D = db[14] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[14].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[14].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[14].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[13].D = db[13] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[13].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[13].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[13].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[12].D = db[12] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[12].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[12].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[12].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[11].D = db[11] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[11].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[11].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[11].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[10].D = db[10] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[10].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[10].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[10].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[9].D = db[9] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[9].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[9].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[9].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[8].D = db[8] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[8].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[8].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[8].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[7].D = db[7] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[7].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[7].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[6].D = db[6] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[6].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[6].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[5].D = db[5] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[5].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[5].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[4].D = db[4] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[4].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[4].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[3].D = db[3] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[3].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[3].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[2].D = db[2] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[2].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[2].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[1].D = db[1] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[1].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[1].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnSeqSink[0].D = db[0] ;  "(1 term, 1 symbol)

RegisterEnSeqSink[0].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnSeqSink[0].CE = Reg1A*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnSeqSink[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[15].D = db[15] ;  "(1 term, 1 symbol)

RegisterEnd[15].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[15].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[15].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[14].D = db[14] ;  "(1 term, 1 symbol)

RegisterEnd[14].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[14].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[14].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[13].D = db[13] ;  "(1 term, 1 symbol)

RegisterEnd[13].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[13].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[13].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[12].D = db[12] ;  "(1 term, 1 symbol)

RegisterEnd[12].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[12].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[12].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[11].D = db[11] ;  "(1 term, 1 symbol)

RegisterEnd[11].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[11].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[11].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[10].D = db[10] ;  "(1 term, 1 symbol)

RegisterEnd[10].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[10].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[10].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[9].D = db[9] ;  "(1 term, 1 symbol)

RegisterEnd[9].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[9].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[9].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[8].D = db[8] ;  "(1 term, 1 symbol)

RegisterEnd[8].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[8].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[8].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[7].D = db[7] ;  "(1 term, 1 symbol)

RegisterEnd[7].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[7].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[6].D = db[6] ;  "(1 term, 1 symbol)

RegisterEnd[6].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[6].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[5].D = db[5] ;  "(1 term, 1 symbol)

RegisterEnd[5].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[5].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[4].D = db[4] ;  "(1 term, 1 symbol)

RegisterEnd[4].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[4].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[3].D = db[3] ;  "(1 term, 1 symbol)

RegisterEnd[3].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[3].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[2].D = db[2] ;  "(1 term, 1 symbol)

RegisterEnd[2].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[2].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[1].D = db[1] ;  "(1 term, 1 symbol)

RegisterEnd[1].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[1].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

RegisterEnd[0].D = db[0] ;  "(1 term, 1 symbol)

RegisterEnd[0].CLK = SeqClk ;  "(1 term, 1 symbol)

RegisterEnd[0].CE = Reg1C*/nLEb2 ;  "(1 term, 2 symbols)

RegisterEnd[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SeqGoFF.D = RegisterEnSeqSink[10] ;  "(1 term, 1 symbol)

SeqGoFF.CLK = SeqClk ;  "(1 term, 1 symbol)

SeqGoFF.RESET = /nOA_rst ;  "(1 term, 1 symbol)

SeqCountStop.D = /equal_to-13 ;  "(1 term, 1 symbol)

SeqCountStop.CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCountStop.RESET = /nOA_rst ;  "(1 term, 1 symbol)

SeqCountEnClk.CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCountEnClk.RESET = /nOA_rst ;  "(1 term, 1 symbol)

SeqCountEnClk.D(~) = /SeqGoFF*/SinkGoFF*/StartSeq ;  "(1 term, 3 symbols)

SeqBusy.D = 1 ;  "(1 term, 0 symbols)

SeqBusy.CLK = SeqCountEnClk ;  "(1 term, 1 symbol)

SeqBusy.RESET = SeqCountEnReset ;  "(1 term, 1 symbol)

SeqCountEnReset.EQN(~) = RegisterEnSeqSink[11]*

/RegisterEnSeqSink[9]*nOA_rst

+ /RegisterEnSeqSink[9]*/SeqCountStop

*nOA_rst ;  "(2 terms, 4 symbols)

SeqCount[13].T = SeqCount[0]*SeqCount[10]*SeqCount[11]*

SeqCount[12]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*SeqCount[4]*SeqCount[5]*

SeqCount[6]*SeqCount[7]*SeqCount[8]*

SeqCount[9]*equal_to-13

+ SeqCount[13]*/equal_to-13 ;  "(2 terms, 15 symbols)

SeqCount[13].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[13].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[13].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[12].T = SeqCount[0]*SeqCount[10]*SeqCount[11]*

SeqCount[1]*SeqCount[2]*SeqCount[3]*

SeqCount[4]*SeqCount[5]*SeqCount[6]*

SeqCount[7]*SeqCount[8]*SeqCount[9]*

equal_to-13

+ SeqCount[12]*/equal_to-13 ;  "(2 terms, 14 symbols)

SeqCount[12].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[12].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[12].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[11].T = SeqCount[0]*SeqCount[10]*SeqCount[1]*

SeqCount[2]*SeqCount[3]*SeqCount[4]*

SeqCount[5]*SeqCount[6]*SeqCount[7]*

SeqCount[8]*SeqCount[9]*equal_to-13

+ SeqCount[11]*/equal_to-13 ;  "(2 terms, 13 symbols)

SeqCount[11].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[11].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[11].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[10].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*SeqCount[4]*SeqCount[5]*

SeqCount[6]*SeqCount[7]*SeqCount[8]*

SeqCount[9]*equal_to-13

+ SeqCount[10]*/equal_to-13 ;  "(2 terms, 12 symbols)

SeqCount[10].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[10].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[10].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[9].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*SeqCount[4]*SeqCount[5]*

SeqCount[6]*SeqCount[7]*SeqCount[8]*

equal_to-13

+ SeqCount[9]*/equal_to-13 ;  "(2 terms, 11 symbols)

SeqCount[9].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[9].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[9].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[8].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*SeqCount[4]*SeqCount[5]*

SeqCount[6]*SeqCount[7]*equal_to-13

+ SeqCount[8]*/equal_to-13 ;  "(2 terms, 10 symbols)

SeqCount[8].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[8].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[8].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[7].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*SeqCount[4]*SeqCount[5]*

SeqCount[6]*equal_to-13

+ SeqCount[7]*/equal_to-13 ;  "(2 terms, 9 symbols)

SeqCount[7].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[7].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[7].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[6].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*SeqCount[4]*SeqCount[5]*

equal_to-13

+ SeqCount[6]*/equal_to-13 ;  "(2 terms, 8 symbols)

SeqCount[6].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[6].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[6].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[5].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*SeqCount[4]*equal_to-13

+ SeqCount[5]*/equal_to-13 ;  "(2 terms, 7 symbols)

SeqCount[5].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[5].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[5].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[4].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

SeqCount[3]*equal_to-13

+ SeqCount[4]*/equal_to-13 ;  "(2 terms, 6 symbols)

SeqCount[4].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[4].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[4].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[3].T = SeqCount[0]*SeqCount[1]*SeqCount[2]*

equal_to-13

+ SeqCount[3]*/equal_to-13 ;  "(2 terms, 5 symbols)

SeqCount[3].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[3].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[3].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[2].D = SeqCount[0]*SeqCount[1]*/SeqCount[2]*

equal_to-13

+ /SeqCount[0]*SeqCount[2]*

equal_to-13

+ /SeqCount[1]*SeqCount[2]*

equal_to-13 ;  "(3 terms, 4 symbols)

SeqCount[2].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[2].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[2].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[1].D = SeqCount[0]*/SeqCount[1]*equal_to-13

+ /SeqCount[0]*SeqCount[1]*

equal_to-13 ;  "(2 terms, 3 symbols)

SeqCount[1].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[1].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[1].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqCount[0].D = /SeqCount[0]*equal_to-13 ;  "(1 term, 2 symbols)

SeqCount[0].CLK = SeqClk ;  "(1 term, 1 symbol)

SeqCount[0].CE = SeqBusy ;  "(1 term, 1 symbol)

SeqCount[0].RESET = RegisterEnSeqSink[9]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SeqD[7].EQN = db[7] ;  "(1 term, 1 symbol)

SeqD[7].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

SeqD[6].EQN = db[6] ;  "(1 term, 1 symbol)

SeqD[6].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

SeqD[5].EQN = db[5] ;  "(1 term, 1 symbol)

SeqD[5].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

SeqD[4].EQN = db[4] ;  "(1 term, 1 symbol)

SeqD[4].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

SeqD[3].EQN = db[3] ;  "(1 term, 1 symbol)

SeqD[3].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

SeqD[2].EQN = db[2] ;  "(1 term, 1 symbol)

SeqD[2].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

SeqD[1].EQN = db[1] ;  "(1 term, 1 symbol)

SeqD[1].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

SeqD[0].EQN = db[0] ;  "(1 term, 1 symbol)

SeqD[0].OE = SeqRamWrAccess ;  "(1 term, 1 symbol)

seqTRIG.EQN = RegisterEnSeqSink[0]*_seqTRIG ;  "(1 term, 2 symbols)

seqECR.EQN = RegisterEnSeqSink[1]*_seqECR ;  "(1 term, 2 symbols)

seqBCR.EQN = RegisterEnSeqSink[2]*_seqBCR ;  "(1 term, 2 symbols)

seqCAL.EQN = RegisterEnSeqSink[3]*_seqCAL ;  "(1 term, 2 symbols)

seqID.EQN = RegisterEnSeqSink[4]*_seqID ;  "(1 term, 2 symbols)

seqTT.EQN = RegisterEnSeqSink[5]*_seqTT ;  "(1 term, 2 symbols)

seqFER.EQN = RegisterEnSeqSink[6]*_seqFER ;  "(1 term, 2 symbols)

seqSpare.EQN = RegisterEnSeqSink[7]*_seqSpare ;  "(1 term, 2 symbols)

_seqTRIG.D = SeqBusy*SeqD[0] ;  "(1 term, 2 symbols)

_seqTRIG.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqTRIG.RESET = /nOA_rst ;  "(1 term, 1 symbol)

_seqECR.D = SeqBusy*SeqD[1] ;  "(1 term, 2 symbols)

_seqECR.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqECR.RESET = /nOA_rst ;  "(1 term, 1 symbol)

_seqBCR.D = SeqBusy*SeqD[2] ;  "(1 term, 2 symbols)

_seqBCR.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqBCR.RESET = /nOA_rst ;  "(1 term, 1 symbol)

_seqCAL.D = SeqBusy*SeqD[3] ;  "(1 term, 2 symbols)

_seqCAL.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqCAL.RESET = /nOA_rst ;  "(1 term, 1 symbol)

_seqID.D = SeqBusy*SeqD[4] ;  "(1 term, 2 symbols)

_seqID.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqID.RESET = /nOA_rst ;  "(1 term, 1 symbol)

_seqTT.D = SeqBusy*SeqD[5] ;  "(1 term, 2 symbols)

_seqTT.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqTT.RESET = /nOA_rst ;  "(1 term, 1 symbol)

_seqFER.D = SeqBusy*SeqD[6] ;  "(1 term, 2 symbols)

_seqFER.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqFER.RESET = /nOA_rst ;  "(1 term, 1 symbol)

_seqSpare.D = SeqBusy*SeqD[7] ;  "(1 term, 2 symbols)

_seqSpare.CLK = SeqClk ;  "(1 term, 1 symbol)

_seqSpare.RESET = /nOA_rst ;  "(1 term, 1 symbol)

SeqRamWrAccess.EQN = RAMselect*bWriteb ;  "(1 term, 2 symbols)

SeqA[14].EQN = 0 ;  "(1 term, 0 symbols)

SeqA[13].EQN = RAMselect*ab[14]

+ /RAMselect*SeqCount[13] ;  "(2 terms, 3 symbols)

SeqA[12].EQN = RAMselect*ab[13]

+ /RAMselect*SeqCount[12] ;  "(2 terms, 3 symbols)

SeqA[11].EQN = RAMselect*ab[12]

+ /RAMselect*SeqCount[11] ;  "(2 terms, 3 symbols)

SeqA[10].EQN = RAMselect*ab[11]

+ /RAMselect*SeqCount[10] ;  "(2 terms, 3 symbols)

SeqA[9].EQN = RAMselect*ab[10]

+ /RAMselect*SeqCount[9] ;  "(2 terms, 3 symbols)

SeqA[8].EQN = RAMselect*ab[9]

+ /RAMselect*SeqCount[8] ;  "(2 terms, 3 symbols)

SeqA[7].EQN = RAMselect*ab[8]

+ /RAMselect*SeqCount[7] ;  "(2 terms, 3 symbols)

SeqA[6].EQN = RAMselect*ab[7]

+ /RAMselect*SeqCount[6] ;  "(2 terms, 3 symbols)

SeqA[5].EQN = RAMselect*ab[6]

+ /RAMselect*SeqCount[5] ;  "(2 terms, 3 symbols)

SeqA[4].EQN = RAMselect*ab[5]

+ /RAMselect*SeqCount[4] ;  "(2 terms, 3 symbols)

SeqA[3].EQN = RAMselect*ab[4]

+ /RAMselect*SeqCount[3] ;  "(2 terms, 3 symbols)

SeqA[2].EQN = RAMselect*ab[3]

+ /RAMselect*SeqCount[2] ;  "(2 terms, 3 symbols)

SeqA[1].EQN = RAMselect*ab[2]

+ /RAMselect*SeqCount[1] ;  "(2 terms, 3 symbols)

SeqA[0].EQN = RAMselect*ab[1]

+ /RAMselect*SeqCount[0] ;  "(2 terms, 3 symbols)

nSeqWrEn.EQN(~) = SeqRamWrAccess*/nLEb2 ;  "(1 term, 2 symbols)

nSeqOE.EQN(~) = RAMselect*bReadb

+ SeqBusy*/SeqRamWrAccess ;  "(2 terms, 4 symbols)

nSeqCS.EQN = 0 ;  "(1 term, 0 symbols)

equal_to-6.EQN = RegisterEnd[0]*/SeqCount[0]

+ /RegisterEnd[0]*SeqCount[0]

+ RegisterEnd[1]*/SeqCount[1]

+ /RegisterEnd[1]*SeqCount[1]

+ RegisterEnd[2]*/SeqCount[2]

+ /RegisterEnd[2]*SeqCount[2]

+ RegisterEnd[3]*/SeqCount[3]

+ /RegisterEnd[3]*SeqCount[3]

+ RegisterEnd[4]*/SeqCount[4]

+ /RegisterEnd[4]*SeqCount[4]

+ RegisterEnd[5]*/SeqCount[5]

+ /RegisterEnd[5]*SeqCount[5]

+ RegisterEnd[6]*/SeqCount[6]

+ /RegisterEnd[6]*SeqCount[6] ;  "(14 terms, 14 symbols)

equal_to-13.EQN = RegisterEnd[10]*/SeqCount[10]

+ /RegisterEnd[10]*SeqCount[10]

+ RegisterEnd[11]*/SeqCount[11]

+ /RegisterEnd[11]*SeqCount[11]

+ RegisterEnd[12]*/SeqCount[12]

+ /RegisterEnd[12]*SeqCount[12]

+ RegisterEnd[13]*/SeqCount[13]

+ /RegisterEnd[13]*SeqCount[13]

+ RegisterEnd[7]*/SeqCount[7]

+ /RegisterEnd[7]*SeqCount[7]

+ RegisterEnd[8]*/SeqCount[8]

+ /RegisterEnd[8]*SeqCount[8]

+ RegisterEnd[9]*/SeqCount[9]

+ /RegisterEnd[9]*SeqCount[9]

+ equal_to-6 ;  "(15 terms, 15 symbols)

SinkGoDelay[4].D = SinkGoDelay[3] ;  "(1 term, 1 symbol)

SinkGoDelay[4].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkGoDelay[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkGoDelay[3].D = SinkGoDelay[2] ;  "(1 term, 1 symbol)

SinkGoDelay[3].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkGoDelay[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkGoDelay[2].D = SinkGoDelay[1] ;  "(1 term, 1 symbol)

SinkGoDelay[2].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkGoDelay[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkGoDelay[1].D = SinkGoFF ;  "(1 term, 1 symbol)

SinkGoDelay[1].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkGoDelay[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkCountEnClk.RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkCountEnClk.D(~) = /SinkGoDelay[4]*/StartSink ;  "(1 term, 2 symbols)

SinkCountEnClk.CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkBusy.D = 1 ;  "(1 term, 0 symbols)

SinkBusy.CLK = SinkCountEnClk ;  "(1 term, 1 symbol)

SinkBusy.RESET = SinkCountEnReset ;  "(1 term, 1 symbol)

SinkCount[13].T = SinkCount[0]*SinkCount[10]*

SinkCount[11]*SinkCount[12]*SinkCount[1]*

SinkCount[2]*SinkCount[3]*SinkCount[4]*

SinkCount[5]*SinkCount[6]*SinkCount[7]*

SinkCount[8]*SinkCount[9]*equal_to-26

+ SinkCount[13]*/equal_to-26 ;  "(2 terms, 15 symbols)

SinkCount[13].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[13].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[13].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[12].T = SinkCount[0]*SinkCount[10]*

SinkCount[11]*SinkCount[1]*SinkCount[2]*

SinkCount[3]*SinkCount[4]*SinkCount[5]*

SinkCount[6]*SinkCount[7]*SinkCount[8]*

SinkCount[9]*equal_to-26

+ SinkCount[12]*/equal_to-26 ;  "(2 terms, 14 symbols)

SinkCount[12].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[12].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[12].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[11].T = SinkCount[0]*SinkCount[10]*

SinkCount[1]*SinkCount[2]*SinkCount[3]*

SinkCount[4]*SinkCount[5]*SinkCount[6]*

SinkCount[7]*SinkCount[8]*SinkCount[9]*

equal_to-26

+ SinkCount[11]*/equal_to-26 ;  "(2 terms, 13 symbols)

SinkCount[11].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[11].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[11].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[10].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*SinkCount[3]*SinkCount[4]*

SinkCount[5]*SinkCount[6]*SinkCount[7]*

SinkCount[8]*SinkCount[9]*equal_to-26

+ SinkCount[10]*/equal_to-26 ;  "(2 terms, 12 symbols)

SinkCount[10].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[10].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[10].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[9].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*SinkCount[3]*SinkCount[4]*

SinkCount[5]*SinkCount[6]*SinkCount[7]*

SinkCount[8]*equal_to-26

+ SinkCount[9]*/equal_to-26 ;  "(2 terms, 11 symbols)

SinkCount[9].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[9].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[9].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[8].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*SinkCount[3]*SinkCount[4]*

SinkCount[5]*SinkCount[6]*SinkCount[7]*

equal_to-26

+ SinkCount[8]*/equal_to-26 ;  "(2 terms, 10 symbols)

SinkCount[8].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[8].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[8].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[7].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*SinkCount[3]*SinkCount[4]*

SinkCount[5]*SinkCount[6]*equal_to-26

+ SinkCount[7]*/equal_to-26 ;  "(2 terms, 9 symbols)

SinkCount[7].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[7].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[7].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[6].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*SinkCount[3]*SinkCount[4]*

SinkCount[5]*equal_to-26

+ SinkCount[6]*/equal_to-26 ;  "(2 terms, 8 symbols)

SinkCount[6].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[6].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[6].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[5].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*SinkCount[3]*SinkCount[4]*

equal_to-26

+ SinkCount[5]*/equal_to-26 ;  "(2 terms, 7 symbols)

SinkCount[5].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[5].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[5].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[4].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*SinkCount[3]*equal_to-26

+ SinkCount[4]*/equal_to-26 ;  "(2 terms, 6 symbols)

SinkCount[4].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[4].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[4].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[3].T = SinkCount[0]*SinkCount[1]*SinkCount[2]

*equal_to-26

+ SinkCount[3]*/equal_to-26 ;  "(2 terms, 5 symbols)

SinkCount[3].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[3].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[3].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[2].D = SinkCount[0]*SinkCount[1]*

/SinkCount[2]*equal_to-26

+ /SinkCount[0]*SinkCount[2]*

equal_to-26

+ /SinkCount[1]*SinkCount[2]*

equal_to-26 ;  "(3 terms, 4 symbols)

SinkCount[2].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[2].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[2].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[1].D = SinkCount[0]*/SinkCount[1]*equal_to-26

+ /SinkCount[0]*SinkCount[1]*

equal_to-26 ;  "(2 terms, 3 symbols)

SinkCount[1].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[1].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[1].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCount[0].D = /SinkCount[0]*equal_to-26 ;  "(1 term, 2 symbols)

SinkCount[0].CE = SinkBusy ;  "(1 term, 1 symbol)

SinkCount[0].RESET = RegisterEnSeqSink[13]

+ /nOA_rst ;  "(2 terms, 2 symbols)

SinkCount[0].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCountStop.D = /equal_to-26 ;  "(1 term, 1 symbol)

SinkCountStop.RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkCountStop.CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkCountEnReset.EQN(~) = RegisterEnSeqSink[11]*

/RegisterEnSeqSink[13]*nOA_rst

+ /RegisterEnSeqSink[13]*

/SinkCountStop*nOA_rst ;  "(2 terms, 4 symbols)

SinkFFin[7].D = eclSpare ;  "(1 term, 1 symbol)

SinkFFin[7].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFin[6].D = eclFER ;  "(1 term, 1 symbol)

SinkFFin[6].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFin[5].D = eclTT ;  "(1 term, 1 symbol)

SinkFFin[5].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFin[4].D = eclID ;  "(1 term, 1 symbol)

SinkFFin[4].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFin[3].D = eclCAL ;  "(1 term, 1 symbol)

SinkFFin[3].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFin[2].D = eclBCR ;  "(1 term, 1 symbol)

SinkFFin[2].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFin[1].D = eclECR ;  "(1 term, 1 symbol)

SinkFFin[1].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFin[0].D = eclL1A ;  "(1 term, 1 symbol)

SinkFFin[0].CLK = SinkClk ;  "(1 term, 1 symbol)

SinkFFin[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[7].D = SinkFFin[7] ;  "(1 term, 1 symbol)

SinkFFout[7].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[7].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkFFout[6].D = SinkFFin[6] ;  "(1 term, 1 symbol)

SinkFFout[6].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[6].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkFFout[5].D = SinkFFin[5] ;  "(1 term, 1 symbol)

SinkFFout[5].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[5].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkFFout[4].D = SinkFFin[4] ;  "(1 term, 1 symbol)

SinkFFout[4].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[4].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkFFout[3].D = SinkFFin[3] ;  "(1 term, 1 symbol)

SinkFFout[3].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[3].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkFFout[2].D = SinkFFin[2] ;  "(1 term, 1 symbol)

SinkFFout[2].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[2].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkFFout[1].D = SinkFFin[1] ;  "(1 term, 1 symbol)

SinkFFout[1].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[1].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkFFout[0].D = SinkFFin[0] ;  "(1 term, 1 symbol)

SinkFFout[0].RESET = /nOA_rst ;  "(1 term, 1 symbol)

SinkFFout[0].CLK(~) = SinkClk ;  "(1 term, 1 symbol)

SinkA[14].EQN = 0 ;  "(1 term, 0 symbols)

SinkA[13].EQN = RAMselect*ab[14]

+ /RAMselect*SinkCount[13] ;  "(2 terms, 3 symbols)

SinkA[12].EQN = RAMselect*ab[13]

+ /RAMselect*SinkCount[12] ;  "(2 terms, 3 symbols)

SinkA[11].EQN = RAMselect*ab[12]

+ /RAMselect*SinkCount[11] ;  "(2 terms, 3 symbols)

SinkA[10].EQN = RAMselect*ab[11]

+ /RAMselect*SinkCount[10] ;  "(2 terms, 3 symbols)

SinkA[9].EQN = RAMselect*ab[10]

+ /RAMselect*SinkCount[9] ;  "(2 terms, 3 symbols)

SinkA[8].EQN = RAMselect*ab[9]

+ /RAMselect*SinkCount[8] ;  "(2 terms, 3 symbols)

SinkA[7].EQN = RAMselect*ab[8]

+ /RAMselect*SinkCount[7] ;  "(2 terms, 3 symbols)

SinkA[6].EQN = RAMselect*ab[7]

+ /RAMselect*SinkCount[6] ;  "(2 terms, 3 symbols)

SinkA[5].EQN = RAMselect*ab[6]

+ /RAMselect*SinkCount[5] ;  "(2 terms, 3 symbols)

SinkA[4].EQN = RAMselect*ab[5]

+ /RAMselect*SinkCount[4] ;  "(2 terms, 3 symbols)

SinkA[3].EQN = RAMselect*ab[4]

+ /RAMselect*SinkCount[3] ;  "(2 terms, 3 symbols)

SinkA[2].EQN = RAMselect*ab[3]

+ /RAMselect*SinkCount[2] ;  "(2 terms, 3 symbols)

SinkA[1].EQN = RAMselect*ab[2]

+ /RAMselect*SinkCount[1] ;  "(2 terms, 3 symbols)

SinkA[0].EQN = RAMselect*ab[1]

+ /RAMselect*SinkCount[0] ;  "(2 terms, 3 symbols)

equal_to-25.EQN = RegisterEnd[10]*/SinkCount[10]

+ /RegisterEnd[10]*SinkCount[10]

+ RegisterEnd[11]*/SinkCount[11]

+ /RegisterEnd[11]*SinkCount[11]

+ RegisterEnd[12]*/SinkCount[12]

+ /RegisterEnd[12]*SinkCount[12]

+ RegisterEnd[13]*/SinkCount[13]

+ /RegisterEnd[13]*SinkCount[13]

+ RegisterEnd[7]*/SinkCount[7]

+ /RegisterEnd[7]*SinkCount[7]

+ RegisterEnd[8]*/SinkCount[8]

+ /RegisterEnd[8]*SinkCount[8]

+ RegisterEnd[9]*/SinkCount[9]

+ /RegisterEnd[9]*SinkCount[9] ;  "(14 terms, 14 symbols)

equal_to-26.EQN = RegisterEnd[0]*/SinkCount[0]

+ /RegisterEnd[0]*SinkCount[0]

+ RegisterEnd[1]*/SinkCount[1]

+ /RegisterEnd[1]*SinkCount[1]

+ RegisterEnd[2]*/SinkCount[2]

+ /RegisterEnd[2]*SinkCount[2]

+ RegisterEnd[3]*/SinkCount[3]

+ /RegisterEnd[3]*SinkCount[3]

+ RegisterEnd[4]*/SinkCount[4]

+ /RegisterEnd[4]*SinkCount[4]

+ RegisterEnd[5]*/SinkCount[5]

+ /RegisterEnd[5]*SinkCount[5]

+ RegisterEnd[6]*/SinkCount[6]

+ /RegisterEnd[6]*SinkCount[6]

+ equal_to-25 ;  "(15 terms, 15 symbols)

SinkD[7].EQN = RAMselect*bWriteb*db[15]

+ /RAMselect*SinkFFout[7]

+ SinkFFout[7]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[7].OE = nSinkOE ;  "(1 term, 1 symbol)

SinkD[6].EQN = RAMselect*bWriteb*db[14]

+ /RAMselect*SinkFFout[6]

+ SinkFFout[6]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[6].OE = nSinkOE ;  "(1 term, 1 symbol)

SinkD[5].EQN = RAMselect*bWriteb*db[13]

+ /RAMselect*SinkFFout[5]

+ SinkFFout[5]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[5].OE = nSinkOE ;  "(1 term, 1 symbol)

SinkD[4].EQN = RAMselect*bWriteb*db[12]

+ /RAMselect*SinkFFout[4]

+ SinkFFout[4]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[4].OE = nSinkOE ;  "(1 term, 1 symbol)

SinkD[3].EQN = RAMselect*bWriteb*db[11]

+ /RAMselect*SinkFFout[3]

+ SinkFFout[3]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[3].OE = nSinkOE ;  "(1 term, 1 symbol)

SinkD[2].EQN = RAMselect*bWriteb*db[10]

+ /RAMselect*SinkFFout[2]

+ SinkFFout[2]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[2].OE = nSinkOE ;  "(1 term, 1 symbol)

SinkD[1].EQN = RAMselect*bWriteb*db[9]

+ /RAMselect*SinkFFout[1]

+ SinkFFout[1]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[1].OE = nSinkOE ;  "(1 term, 1 symbol)

SinkD[0].EQN = RAMselect*bWriteb*db[8]

+ /RAMselect*SinkFFout[0]

+ SinkFFout[0]*/bWriteb ;  "(3 terms, 4 symbols)

SinkD[0].OE = nSinkOE ;  "(1 term, 1 symbol)

nSinkWrEn.EQN(~) = /RAMselect*/SinkClk*SinkBusy

+ SeqRamWrAccess*/nLEb2 ;  "(2 terms, 5 symbols)

nSinkOE.EQN(~) = RAMselect*bReadb ;  "(1 term, 2 symbols)

nSinkCS.EQN = 0 ;  "(1 term, 0 symbols)

XSeqTRIG.EQN = SeqBusy*SeqD[0] ;  "(1 term, 2 symbols)

XSeqClk1.EQN = SeqClk ;  "(1 term, 1 symbol)

XSeqClk2.EQN = /SinkClk ;  "(1 term, 1 symbol)

PLDocument:  pld7.doc

SOLUTIONS     Wed May 22 11:47:00 2002

PARTITIONING CRITERIA :

WEIGHT PRICE 10 ;

TEMPLATE = MV384_184  ;

PARTITIONING SOLUTIONS :

==> Solution  1: MV384_184

FUSEMAP FILES FOR SOLUTION 1:

Device 1 (MV384_184) : pld7.j1

PLDocument:  pld7.doc

PINOUT DIAGRAMS  Wed May 22 11:47:00 2002

Device  1 - MV384_184 -- Pinout for QFP package

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|    1  |Jtag  |                      | |   51  |Vcc   |                      |

|    2  |Biput |  SeqA[0]             | |   52  |Biput |  SinkA[8]            |

|    3  |Biput |  SeqA[1]             | |   53  |Biput |  SinkA[9]            |

|    4  |Biput |  SeqA[2]             | |   54  |Biput |  SinkA[10]           |

|    5  |Biput |  SeqA[3]             | |   55  |Biput |  SinkA[11]           |

|    6  |Biput |  SeqA[4]             | |   56  |Biput |  SinkA[12]           |

|    7  |Biput |  SeqA[5]             | |   57  |Biput |  SinkA[13]           |

|    8  |Biput |  SeqA[6]             | |   58  |Biput |  nSinkOE             |

|    9  |Biput |  SeqA[7]             | |   59  |Biput |  nSinkWrEn           |

|   10  |Vcc   |                      | |   60  |Jtag  |                      |

|   11  |GND   |                      | |   61  |GND   |                      |

|   12  |Biput |  SeqA[8]             | |   62  |Vcc   |                      |

|   13  |Biput |  SeqA[9]             | |   63  |Biput |  SinkD[0]            |

|   14  |Biput |  SeqA[10]            | |   64  |Biput |  SinkD[1]            |

|   15  |Biput |  SeqA[11]            | |   65  |Biput |  SinkD[2]            |

|   16  |Biput |  SeqA[12]            | |   66  |Biput |  SinkD[3]            |

|   17  |Biput |  SeqA[13]            | |   67  |Biput |  SinkD[4]            |

|   18  |Biput |  nSeqOE              | |   68  |Biput |  SinkD[5]            |

|   19  |Biput |  nSeqWrEn            | |   69  |Biput |  SinkD[6]            |

|   20  |GND   |                      | |   70  |Biput |  SinkD[7]            |

|   21  |Biput |  SeqD[0]             | |   71  |GND   |                      |

|   22  |Biput |  SeqD[1]             | |   72  |Vcc   |                      |

|   23  |Biput |  SeqD[2]             | |   73  |Biput |                      |

|   24  |Biput |  SeqD[3]             | |   74  |Biput |                      |

|   25  |Biput |  SeqD[4]             | |   75  |Biput |  enExtRodBusy        |

|   26  |Biput |  SeqD[5]             | |   76  |Biput |  sgbD[4]             |

|   27  |Biput |  SeqD[6]             | |   77  |Biput |  sgbD[3]             |

|   28  |Biput |  SeqD[7]             | |   78  |Biput |  sgbD[2]             |

|   29  |In/CLK|  SeqClk              | |   79  |Biput |  TTC_ready_in        |

|   30  |Vcc   |                      | |   80  |Biput |  TIM_OK_OUT          |

|   31  |GND   |                      | |   81  |GND   |                      |

|   32  |In/CLK|  SinkClk             | |   82  |Biput |  nSinkCS             |

|   33  |Biput |                      | |   83  |Biput |  nSeqCS              |

|   34  |Biput |                      | |   84  |Biput |  db[0]               |

|   35  |Biput |                      | |   85  |Biput |  db[1]               |

|   36  |Biput |                      | |   86  |Biput |  db[2]               |

|   37  |Biput |                      | |   87  |Biput |  db[3]               |

|   38  |Biput |                      | |   88  |Vcc   |                      |

|   39  |Biput |                      | |   89  |GND   |                      |

|   40  |Biput |                      | |   90  |GND   |                      |

|   41  |GND   |                      | |   91  |GND   |                      |

|   42  |Biput |  SinkA[0]            | |   92  |GND   |                      |

|   43  |Biput |  SinkA[1]            | |   93  |Vcc   |                      |

|   44  |Biput |  SinkA[2]            | |   94  |Biput |  db[8]               |

|   45  |Biput |  SinkA[3]            | |   95  |Biput |  db[9]               |

|   46  |Biput |  SinkA[4]            | |   96  |Biput |  db[10]              |

|   47  |Biput |  SinkA[5]            | |   97  |Biput |  db[11]              |

|   48  |Biput |  SinkA[6]            | |   98  |Biput |                      |

|   49  |Biput |  SinkA[7]            | |   99  |Biput |                      |

|   50  |GND   |                      | |  100  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  101  |Biput |  seqTRIG             | |  151  |GND   |                      |

|  102  |Biput |  seqBCR              | |  152  |In/CLK|                      |

|  103  |Biput |  seqECR              | |  153  |Biput |  bReadb              |

|  104  |Biput |  seqCAL              | |  154  |Biput |  bWriteb             |

|  105  |Biput |  seqFER              | |  155  |Biput |  nOA_rst             |

|  106  |Biput |  seqSpare            | |  156  |Biput |  RunMode             |

|  107  |Biput |  seqTT               | |  157  |Biput |                      |

|  108  |Biput |  seqID               | |  158  |Biput |                      |

|  109  |Vcc   |                      | |  159  |Biput |  SinkA[14]           |

|  110  |GND   |                      | |  160  |Biput |                      |

|  111  |Biput |  sgbC[7]             | |  161  |GND   |                      |

|  112  |Biput |  sgbC[6]             | |  162  |Biput |  Reg1A               |

|  113  |Biput |  sgbC[5]             | |  163  |Biput |  Reg1C               |

|  114  |Biput |  sgbC[4]             | |  164  |Biput |  XSeqClk2            |

|  115  |Biput |  sgbC[3]             | |  165  |Biput |                      |

|  116  |Biput |  sgbC[2]             | |  166  |Biput |                      |

|  117  |Biput |  sgbC[1]             | |  167  |Biput |  StartSeq            |

|  118  |Biput |  sgbC[0]             | |  168  |Biput |  StartSink           |

|  119  |Vcc   |                      | |  169  |Biput |  RAMselect           |

|  120  |GND   |                      | |  170  |GND   |                      |

|  121  |Jtag  |                      | |  171  |Vcc   |                      |

|  122  |Biput |  XSeqTRIG            | |  172  |Biput |                      |

|  123  |Biput |  XSeqClk1            | |  173  |Biput |                      |

|  124  |Biput |  SeqBusy             | |  174  |Biput |  SinkBusy            |

|  125  |Biput |                      | |  175  |Biput |                      |

|  126  |Biput |  db[4]               | |  176  |Biput |  db[12]              |

|  127  |Biput |  db[5]               | |  177  |Biput |  db[13]              |

|  128  |Biput |  db[6]               | |  178  |Biput |  db[14]              |

|  129  |Biput |  db[7]               | |  179  |Biput |  db[15]              |

|  130  |Vcc   |                      | |  180  |Jtag  |                      |

|  131  |GND   |                      | |  181  |GND   |                      |

|  132  |Biput |  ab[1]               | |  182  |Vcc   |                      |

|  133  |Biput |  ab[2]               | |  183  |Biput |  eclL1A              |

|  134  |Biput |  ab[3]               | |  184  |Biput |  eclECR              |

|  135  |Biput |  ab[4]               | |  185  |Biput |  eclBCR              |

|  136  |Biput |  ab[5]               | |  186  |Biput |  eclCAL              |

|  137  |Biput |  ab[6]               | |  187  |Biput |  eclID               |

|  138  |Biput |  ab[7]               | |  188  |Biput |  eclTT               |

|  139  |Biput |  SeqA[14]            | |  189  |Biput |  eclFER              |

|  140  |GND   |                      | |  190  |Biput |  eclSpare            |

|  141  |Biput |  ab[8]               | |  191  |GND   |                      |

|  142  |Biput |  ab[9]               | |  192  |Vcc   |                      |

|  143  |Biput |  ab[10]              | |  193  |Biput |  sgbB[7]             |

|  144  |Biput |  ab[11]              | |  194  |Biput |  sgbB[6]             |

|  145  |Biput |  ab[12]              | |  195  |Biput |  sgbB[5]             |

|  146  |Biput |  ab[13]              | |  196  |Biput |  sgbB[4]             |

|  147  |Biput |  ab[14]              | |  197  |Biput |  sgbB[3]             |

|  148  |Biput |  ab[15]              | |  198  |Biput |  sgbB[2]             |

|  149  |In/CLK|  nLEb2               | |  199  |Biput |  sgbB[1]             |

|  150  |Vcc   |                      | |  200  |Biput |  sgbB[0]             |

+-------+------+----------------------+ +-------+------+----------------------+

Device  1 - MV384_184 -- Pinout for QFP package (continued)

+-------+------+----------------------+ +-------+------+----------------------+

|  Pin  | Type |        Signal        | |  Pin  | Type |        Signal        |

+-------+------+----------------------+ +-------+------+----------------------+

|  201  |GND   |                      | |  221  |Biput |  sgbA[7]             |

|  202  |Biput |                      | |  222  |Biput |  sgbA[6]             |

|  203  |Biput |                      | |  223  |Biput |  sgbA[5]             |

|  204  |Biput |  s7_8[7]             | |  224  |Biput |  sgbA[4]             |

|  205  |Biput |  s7_8[6]             | |  225  |Biput |  sgbA[3]             |

|  206  |Biput |  s7_8[5]             | |  226  |Biput |  sgbA[2]             |

|  207  |Biput |  s7_8[4]             | |  227  |Biput |  sgbA[1]             |

|  208  |Vcc   |                      | |  228  |Biput |  sgbA[0]             |

|  209  |GND   |                      | |  229  |Vcc   |                      |

|  210  |GND   |                      | |  230  |GND   |                      |

|  211  |GND   |                      | |  231  |Biput |  s6_7[7]             |

|  212  |GND   |                      | |  232  |Biput |  s6_7[6]             |

|  213  |Vcc   |                      | |  233  |Biput |  s6_7[5]             |

|  214  |Biput |  s7_8[3]             | |  234  |Biput |  s6_7[4]             |

|  215  |Biput |  s7_8[2]             | |  235  |Biput |  s6_7[3]             |

|  216  |Biput |  s7_8[1]             | |  236  |Biput |  s6_7[2]             |

|  217  |Biput |  s7_8[0]             | |  237  |Biput |  s6_7[1]             |

|  218  |Biput |                      | |  238  |Biput |  s6_7[0]             |

|  219  |Biput |                      | |  239  |Vcc   |                      |

|  220  |GND   |                      | |  240  |GND   |                      |

+-------+------+----------------------+ +-------+------+----------------------+

DEVICE SELECTION:

Device        Manuf   Fam    Pack   Temp   Price  User1 User2

1) M5-384/184-10HC    AMD    CMOS   QFP    COM    $  0.00    0    0

2) M5-384/184-10HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

3) M5-384/184-12HC    AMD    CMOS   QFP    COM    $  0.00    0    0

4) M5-384/184-12HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

5) M5-384/184-15HC    AMD    CMOS   QFP    COM    $  0.00    0    0

6) M5-384/184-15HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

7) M5-384/184-20HI    AMD    CMOS   QFP    EXT    $  0.00    0    0

==> M5-384/184-7HC     AMD    CMOS   QFP    COM    $  0.00    0    0

9) M5LV-384/184-10HC  AMD    V3.3   QFP    COM    $  0.00    0    0

10) M5LV-384/184-10HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

11) M5LV-384/184-12HC  AMD    V3.3   QFP    COM    $  0.00    0    0

12) M5LV-384/184-12HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

13) M5LV-384/184-15HC  AMD    V3.3   QFP    COM    $  0.00    0    0

14) M5LV-384/184-15HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

15) M5LV-384/184-20HI  AMD    V3.3   QFP    EXT    $  0.00    0    0

16) M5LV-384/184-7HC   AMD    V3.3   QFP    COM    $  0.00    0    0

17) M5LV-384/184-7HI   AMD    V3.3   QFP    EXT    $  0.00    0    0

PLDocument:  pld7.doc

WIRELIST      Wed May 22 11:47:00 2002

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| nOA_rst          | MV384_184_1       |  155  |

| RunMode          | MV384_184_1       |  156  |

| SeqClk           | MV384_184_1       |   29  |

| SinkClk          | MV384_184_1       |   32  |

| RAMselect        | MV384_184_1       |  169  |

| db[15]           | MV384_184_1       |  179  |

| db[14]           | MV384_184_1       |  178  |

| db[13]           | MV384_184_1       |  177  |

| db[12]           | MV384_184_1       |  176  |

| db[11]           | MV384_184_1       |   97  |

| db[10]           | MV384_184_1       |   96  |

| db[9]            | MV384_184_1       |   95  |

| db[8]            | MV384_184_1       |   94  |

| db[7]            | MV384_184_1       |  129  |

| db[6]            | MV384_184_1       |  128  |

| db[5]            | MV384_184_1       |  127  |

| db[4]            | MV384_184_1       |  126  |

| db[3]            | MV384_184_1       |   87  |

| db[2]            | MV384_184_1       |   86  |

| db[1]            | MV384_184_1       |   85  |

| db[0]            | MV384_184_1       |   84  |

| bWriteb          | MV384_184_1       |  154  |

| bReadb           | MV384_184_1       |  153  |

| ab[15]           | MV384_184_1       |  148  |

| ab[14]           | MV384_184_1       |  147  |

| ab[13]           | MV384_184_1       |  146  |

| ab[12]           | MV384_184_1       |  145  |

| ab[11]           | MV384_184_1       |  144  |

| ab[10]           | MV384_184_1       |  143  |

| ab[9]            | MV384_184_1       |  142  |

| ab[8]            | MV384_184_1       |  141  |

| ab[7]            | MV384_184_1       |  138  |

| ab[6]            | MV384_184_1       |  137  |

| ab[5]            | MV384_184_1       |  136  |

| ab[4]            | MV384_184_1       |  135  |

| ab[3]            | MV384_184_1       |  134  |

| ab[2]            | MV384_184_1       |  133  |

| ab[1]            | MV384_184_1       |  132  |

| StartSeq         | MV384_184_1       |  167  |

| SeqBusy          | MV384_184_1       |  124  |

| SeqD[7]          | MV384_184_1       |   28  |

| SeqD[6]          | MV384_184_1       |   27  |

| SeqD[5]          | MV384_184_1       |   26  |

| SeqD[4]          | MV384_184_1       |   25  |

| SeqD[3]          | MV384_184_1       |   24  |

| SeqD[2]          | MV384_184_1       |   23  |

| SeqD[1]          | MV384_184_1       |   22  |

| SeqD[0]          | MV384_184_1       |   21  |

| seqTRIG          | MV384_184_1       |  101  |

| seqECR           | MV384_184_1       |  103  |

| seqBCR           | MV384_184_1       |  102  |

+------------------+-------------------+-------+

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| seqCAL           | MV384_184_1       |  104  |

| seqID            | MV384_184_1       |  108  |

| seqTT            | MV384_184_1       |  107  |

| seqFER           | MV384_184_1       |  105  |

| seqSpare         | MV384_184_1       |  106  |

| SeqA[14]         | MV384_184_1       |  139  |

| SeqA[13]         | MV384_184_1       |   17  |

| SeqA[12]         | MV384_184_1       |   16  |

| SeqA[11]         | MV384_184_1       |   15  |

| SeqA[10]         | MV384_184_1       |   14  |

| SeqA[9]          | MV384_184_1       |   13  |

| SeqA[8]          | MV384_184_1       |   12  |

| SeqA[7]          | MV384_184_1       |    9  |

| SeqA[6]          | MV384_184_1       |    8  |

| SeqA[5]          | MV384_184_1       |    7  |

| SeqA[4]          | MV384_184_1       |    6  |

| SeqA[3]          | MV384_184_1       |    5  |

| SeqA[2]          | MV384_184_1       |    4  |

| SeqA[1]          | MV384_184_1       |    3  |

| SeqA[0]          | MV384_184_1       |    2  |

| nSeqWrEn         | MV384_184_1       |   19  |

| nSeqOE           | MV384_184_1       |   18  |

| nSeqCS           | MV384_184_1       |   83  |

| StartSink        | MV384_184_1       |  168  |

| eclSpare         | MV384_184_1       |  190  |

| eclFER           | MV384_184_1       |  189  |

| eclTT            | MV384_184_1       |  188  |

| eclID            | MV384_184_1       |  187  |

| eclCAL           | MV384_184_1       |  186  |

| eclBCR           | MV384_184_1       |  185  |

| eclECR           | MV384_184_1       |  184  |

| eclL1A           | MV384_184_1       |  183  |

| SinkBusy         | MV384_184_1       |  174  |

| SinkA[14]        | MV384_184_1       |  159  |

| SinkA[13]        | MV384_184_1       |   57  |

| SinkA[12]        | MV384_184_1       |   56  |

| SinkA[11]        | MV384_184_1       |   55  |

| SinkA[10]        | MV384_184_1       |   54  |

| SinkA[9]         | MV384_184_1       |   53  |

| SinkA[8]         | MV384_184_1       |   52  |

| SinkA[7]         | MV384_184_1       |   49  |

| SinkA[6]         | MV384_184_1       |   48  |

| SinkA[5]         | MV384_184_1       |   47  |

| SinkA[4]         | MV384_184_1       |   46  |

| SinkA[3]         | MV384_184_1       |   45  |

| SinkA[2]         | MV384_184_1       |   44  |

| SinkA[1]         | MV384_184_1       |   43  |

| SinkA[0]         | MV384_184_1       |   42  |

| SinkD[7]         | MV384_184_1       |   70  |

| SinkD[6]         | MV384_184_1       |   69  |

| SinkD[5]         | MV384_184_1       |   68  |

+------------------+-------------------+-------+

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| SinkD[4]         | MV384_184_1       |   67  |

| SinkD[3]         | MV384_184_1       |   66  |

| SinkD[2]         | MV384_184_1       |   65  |

| SinkD[1]         | MV384_184_1       |   64  |

| SinkD[0]         | MV384_184_1       |   63  |

| nSinkWrEn        | MV384_184_1       |   59  |

| nSinkOE          | MV384_184_1       |   58  |

| nSinkCS          | MV384_184_1       |   82  |

| nLEb2            | MV384_184_1       |  149  |

| Reg1A            | MV384_184_1       |  162  |

| Reg1C            | MV384_184_1       |  163  |

| XSeqTRIG         | MV384_184_1       |  122  |

| XSeqClk1         | MV384_184_1       |  123  |

| XSeqClk2         | MV384_184_1       |  164  |

| sgbA[7]          | MV384_184_1       |  221  |

| sgbA[6]          | MV384_184_1       |  222  |

| sgbA[5]          | MV384_184_1       |  223  |

| sgbA[4]          | MV384_184_1       |  224  |

| sgbA[3]          | MV384_184_1       |  225  |

| sgbA[2]          | MV384_184_1       |  226  |

| sgbA[1]          | MV384_184_1       |  227  |

| sgbA[0]          | MV384_184_1       |  228  |

| sgbB[7]          | MV384_184_1       |  193  |

| sgbB[6]          | MV384_184_1       |  194  |

| sgbB[5]          | MV384_184_1       |  195  |

| sgbB[4]          | MV384_184_1       |  196  |

| sgbB[3]          | MV384_184_1       |  197  |

| sgbB[2]          | MV384_184_1       |  198  |

| sgbB[1]          | MV384_184_1       |  199  |

| sgbB[0]          | MV384_184_1       |  200  |

| sgbC[7]          | MV384_184_1       |  111  |

| sgbC[6]          | MV384_184_1       |  112  |

| sgbC[5]          | MV384_184_1       |  113  |

| sgbC[4]          | MV384_184_1       |  114  |

| sgbC[3]          | MV384_184_1       |  115  |

| sgbC[2]          | MV384_184_1       |  116  |

| sgbC[1]          | MV384_184_1       |  117  |

| sgbC[0]          | MV384_184_1       |  118  |

| sgbD[4]          | MV384_184_1       |   76  |

| sgbD[3]          | MV384_184_1       |   77  |

| sgbD[2]          | MV384_184_1       |   78  |

| enExtRodBusy     | MV384_184_1       |   75  |

| TIM_OK_OUT       | MV384_184_1       |   80  |

| TTC_ready_in     | MV384_184_1       |   79  |

| s6_7[7]          | MV384_184_1       |  231  |

| s6_7[6]          | MV384_184_1       |  232  |

| s6_7[5]          | MV384_184_1       |  233  |

| s6_7[4]          | MV384_184_1       |  234  |

| s6_7[3]          | MV384_184_1       |  235  |

| s6_7[2]          | MV384_184_1       |  236  |

| s6_7[1]          | MV384_184_1       |  237  |

+------------------+-------------------+-------+

+------------------+-------------------+-------+

|      Signal      |       Device      |  Pin  |

+------------------+-------------------+-------+

| s6_7[0]          | MV384_184_1       |  238  |

| s7_8[7]          | MV384_184_1       |  204  |

| s7_8[6]          | MV384_184_1       |  205  |

| s7_8[5]          | MV384_184_1       |  206  |

| s7_8[4]          | MV384_184_1       |  207  |

| s7_8[3]          | MV384_184_1       |  214  |

| s7_8[2]          | MV384_184_1       |  215  |

| s7_8[1]          | MV384_184_1       |  216  |

| s7_8[0]          | MV384_184_1       |  217  |

+------------------+-------------------+-------+