MCNPX Beamline Geometry December 2011

Source				[0 cm]

Collimator			[5 cm]

1st Scattering foil		[8 cm]

Al tubing (7.2cm inner dia.)	[to window]

Stopper & 2nd scattering foil 	[30 cm]

Kapton window		[35.6 cm]

Al boxing (16x16cm inner)	[45-65 cm]

Range-shifter			[50 cm]

Modulator			[~55 to 58 cm (32 steps)]

2nd collimator (not used)	[60 cm]

Al tubing			[65-110cm]

Anti-scatter collimator		[113 cm]

Monitor chambers		[115cm]

Al boxing			[110-169.2cm]

Cross-wires			[149.2cm]

Nozzle				[169.2-176.6cm]

Patient collimator		[176.2-177cm]

Wedge				[179cm]

Phantom/patient		[184cm]