| Resistor Capacitor Inductor Datasheet |  |  |
| --- | --- | --- |
| I V R V=I⋅R Ohm’s Law | I V C C=QV I=C⋅dVdt I=dQdt Q=C⋅V | V=L⋅dIdt L I V Faraday’s Law of Induction |
| In AC theory, the ratio V/I is still measured in Ohms, but is called IMPEDANCE. It is represented by a complex number Z because, in general, as well as having a MAGNITUDE, there is a PHASE difference between V and I. | V=V0sinω t I=C⋅dV0sinω tdt If I=ω C⋅V0cosω t - ∴ VI=1ωC V amplitude = V0 I amplitude =ω C⋅V0 | I=I0sinω t V=L⋅dI0sinω tdt If V=ω L⋅I0cosω t - ∴ VI=ω L V amplitude = L I0 I amplitude =I0 |
| t V V0 t I I0 V I  | t V V0 t I I0 V I  | t V V0 t I I0 V I  |
| Phase mneumonics: | CIVIL or ELI likes ICE |  |
| Resistor: V and I are in phase: V=I⋅R | Capacitor: V lags I by 90. Use operator -j to express this: V=−j⋅I⋅1ω C=I⋅1jω C | Inductor: V leads I by 90. Use operator j to express this: V=j⋅I⋅ωL=I⋅jωL |
| ZR=R Independent of frequency: | ZC=1jω C Inversely proportional to frequency: | ZL=jωL Directly proportional to frequency: |
|  R |  |  17 February 2012 martin.fry@ucl.ac.uk |