The Data Acquisition and Calibration System for the ATLAS Semiconductor Tracker

FirstAuthor, SecondAuthor, ThirdAuthor …

The semiconductor tracker (SCT) data acquisition (DAQ) system will calibrate, configure, and control the approximately six million front-end channels of the ATLAS silicon strip detector. It will provide a synchronized bunch-crossing clock to the front-end modules, communicate first-level triggers to the front-end chips, and transfer information about hit strips to the ATLAS high-level trigger system. The system has been used extensively for calibration and quality assurance during SCT barrel and endcap assembly and for performance confirmation tests after transport of the barrels and endcaps to CERN. Operating in data-taking mode, the DAQ has recorded more than ten million synchronously-triggered events during commissioning tests including more than half a million cosmic ray triggered events. In this paper we describe the components of the data acquisition system, discuss its operation in calibration and data-taking modes and present some detector performance results from these tests.

# Introduction

The ATLAS experiment is one of two general-purpose detectors at CERN’s Large Hadron Collider. The semiconductor tracker (SCT) is a silicon strip detector and forms the intermediate tracking layers of the ATLAS inner detector []. The SCT has been designed to measure four precision three-dimensional space-points for charged particle tracks with pseudo-rapidity |η| < 2.5.

Figure 1. Cross section of the ATLAS Inner Detector showing a quarter of the barrel and half of one of the two endcap regions.

The complete SCT consists of 4088 front-end modules [,]. Each module has two planes of silicon each with 768 active strips of p+ implant on n-type bulk []. The planes are offset by a small stereo angle (40 mrad), so that each module provides space-point resolutions of 17 μm perpendicular to and 580 μm parallel to its strips. The implant strips are capacitively coupled to aluminium metalisation, and are read out by application-specific integrated circuits (ASICs) known as ABCD3TA []. Each of these chips is responsible for reading out 128 channels, so twelve are required for each SCT module.

The SCT is geometrically divided into a central barrel region and two endcaps (known as ‘A’ and ‘C’). The barrel region consists of four concentric cylindrical layers (barrels). Each endcap consists of nine discs. The number of modules on each barrel layer and endcap disk is given in Table 1 and Table 2. The complete SCT has 49,056 front end ASICs and more than six million individual read-out channels.

| Barrel | B3 | B4 | B5 | B6 | Total |
| --- | --- | --- | --- | --- | --- |
| Radius / mm | 299 | 371 | 443 | 514 |  |
| Modules | 384 | 480 | 576 | 672 | 2112 |

Table 1: Radius and number of modules on each of the four SCT barrel layers.

| Disc | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ¦z¦ / mm | 847 | 934 | 1084 | 1262 | 1377 | 1747 | 2072 | 2462 | 2727 |  |
| Modules | 92 | 132 | 132 | 132 | 132 | 132 | 92 | 92 | 52 | 988 |

Table 2: Longitudinal position and number of modules for the nine discs on each SCT endcap.

For physics data-taking the data acquisition (DAQ) system must configure the front-end ASICs, communicate first-level triggers, and transfer data from the front-end chips to the ATLAS high-level trigger system.

The role of the DAQ in calibrating the detector is equally important. The SCT uses a “binary” readout architecture in which the only pulse-height information transmitted by the front-end chips is one bit per channel which denotes whether the pulse was above a preset threshold. Further information about the size of the pulse cannot be recovered later, so the correct calibration of these thresholds is central to the successful operation of the detector.

The discriminator threshold must be set at a level that guarantees uniform good efficiency while maintaining the noise occupancy at a low level. Furthermore the detector must maintain good performance even after a total ionizing dose of 21014 neutrons/cm2 of 1-MeV neutron equivalent fluence, corresponding to 10 years of operation of the LHC at its design luminosity. The performance requirements, based on track-finding and pattern-recognition considerations, are that channel hit efficiency should be greater than 99% and noise occupancy less than 510-4 even after irradiation.

During calibration, internal circuits on the front end chips can be used to inject test charges. Information about the pulse sizes is reconstructed by measuring occupancy (the average number of hits above threshold per channel per event) as a function of the front end discriminator threshold (threshold “scans”). The calibration system must initiate the appropriate scans, interpret the large volume of data obtained, and find an improved configuration based on the results.

This paper is organized as follows. In section 2 there is a description of the readout hardware. The software and control system are discussed in section 3. In section 4 there is a description of the calibration procedure. A review of the operation of and results from the data acquisition system is given in section 5. This section covers both the confirmation tests performed during the mounting of SCT modules to their carbon-fiber support structures ( “macro-assembly”) as well as more recent tests examining the performance of the completed barrel and endcaps at CERN (“detector commissioning”). We conclude in section 6. A list of some of the common abbreviations used may be found in the appendix.

# Off-detector hardware overview

The off-detector readout hardware of the SCT data acquisition (DAQ) links the SCT front-end modules with the ATLAS central trigger and DAQ system [], and provides the mechanism for their control. The principle connections to the front end modules, to the ATLAS central DAQ and between SCT-specific components are shown in Figure 2.

The SCT DAQ consists of several different modules. The Read Out Driver (ROD) module [Error: Reference source not found] performs the main control and data handling. A complementary Back Of Crate (BOC) module handles the ROD’s I/O requirements to and from the front end, and to the central DAQ. Each ROD/BOC pair deals with the control and data for 48 front-end modules. There can be up to 16 RODs and BOCs housed in a standard LHC-specification 9U VME64x crate [], occupying slots 5-12, 14-21. In slot 13 of the crate is an interface module (TIM) which accepts the Timing, Trigger and Control (TTC) signals from ATLAS and distributes them to the RODs and BOCs. The ROD Crate Controller (RCC) is a commercial 6U Single Board Computer running Linux which acts as the VME master, and hence it usually occupies the first slot in the crate. The RCC configures the other components and provides overall control of the data acquisition functions within a crate.

Figure 2. Block diagram of the SCT data acquisition hardware showing the main connections between components.

In physics data-taking mode, triggers pass from the ATLAS TTC [] to the TIM and are distributed to the RODs. Each ROD fans out the triggers via its BOC to the front end modules. The resultant hit data from the front-end modules are received on the BOC, formatted on the ROD and then returned to the BOC to be passed on to the first module of the ATLAS central DAQ, known as the ROS. The RODs can also be set up to sample and histogram events and errors from the data stream for monitoring purposes.

For calibration purposes, the SCT DAQ can operate separately from the central ATLAS DAQ. In this mode the ATLAS TTC and ROS are not used, and the TIM generates the required clock, fast commands and serialised event and trigger identifiers internally. The resultant data are not passed to the ROS, but ROD monitoring functions still sample and histogram the events. The resultant occupancy histograms are transferred over VME to the ROD Crate Controller and then over the LAN to PC servers for analysis.

## The Read-out Driver (ROD)

The Silicon Read-out Driver (ROD) [] is a 9U 400mm deep VME64x electronics board. The primary functions of the ROD are the front-end module configuration, trigger propagation and event data formatting. The secondary functions of the ROD are detector calibration and monitoring. Control commands are sent from the ROD to the front-end modules as serial data streams. These commands can be Level-1 triggers, bunch-crossing (clock) resets, event counter (trigger) resets, calibration commands or module register data.  Each ROD board is capable of controlling the configuration and processing the data read-out of up to 48 SCT front-end  modules. After formatting the data collected from the modules, the ROD builds event fragments that are transmitted to the ATLAS read-out subsystem (ROS) [] via a high speed serial optical link knows as the S-Link [].

A hybrid architecture of Field Programmable Gate Arrays (FPGAs) and Digital Signal Processors (DSPs) allows the ROD versatility to perform various roles during physics data-taking and calibrations.  Four FPGA designs are used for all of the real-time operations required for data processing at the ATLAS trigger rate.  The Formatter, Event Fragment Builder and Router FPGAs are dedicated to performing time-critical operations, such as the formatting, building and routing of event data. The Controller FPGA controls operations such as ROD setup, module configuration and trigger distribution.  A single “Master” (MDSP) and four “Slave” (SDSP) DSPs on the board are used to control and coordinate on-ROD operations, as well as for performing high-level tasks such as data monitoring and module calibration. Once configured, the ROD FPGAs handle the event data-path to ATLAS Level-2 without further assistance from the DSPs. The major data and communication paths on the ROD are shown in Figure 3.

Figure 3. An overview of the ATLAS Silicon Read-out Driver (ROD) data and communication paths.

### Operating Modes

The ROD supports the two main modes of operation: physics data-taking and detector calibrations.  The data-path through the Formatter and the Event Fragment Builder FPGAs is the same in both modes of operation. In data-taking mode the Router FPGA then transmits Event Fragments to the ROS via the S-Link and optionally also to the SDSPs for monitoring. In calibration mode the S-Link is disabled and the Router FPGA sends events to the farm of Slave DSPs for histogramming.

### Physics data-taking

Once the data-path on the ROD has been setup, event data processing is performed by the FPGAs without any intervention from the DSPs.  Triggers issued from ATLAS are relayed to the ROD via the Timing Interface Module (TIM) electronics board.  If the S-Link is receiving data from the ROD faster than it can be transferred to the ROS, back-pressure will be applied to the ROD, thereby halting the transmission of events and causing the internal ROD FIFOs to begin to fill.  Once back-pressure has been relieved, the flow of events through the S-Link resumes.  If the internal FIFOs fill to a critical limit, a ROD busy signal is sent to the TIM to stop triggers.

The Router FPGA can be set up to capture events with a user-defined pre-scale on a non-interfering basis and transmit them to the farm of SDSPs.  Histogramming these captured events and comparing them against a set of reference histograms can serve as an indicator of channels with unusually high or low occupancies and the captured data can be monitored for errors.

### Calibration

When running calibrations, the MDSP serial ports can be used to issue triggers to the modules. In calibration mode the transmission of data through the S-Link is inhibited. Instead, frames of data (256 32-bit word chunks) are passed from the Router FPGA to the SDSPs using a direct memory access transfer. Tasks running on the SDSPs flag these transferred events for processing and subsequent histogramming.  A monitoring task can be run on the SDSPs that is capable of parsing the event errors flagged by the FPGAs and reporting these errors back to the RCC. More details on the use of the ROD histogramming tasks for calibration can be found in Section 4.

### ROD Communication

The ROD contains many components, and is required to perform many different operations in real time. For smooth operation it is important that the different components have well-defined communication protocol. A system of communication registers, “primitives”, “tasks” and text-buffers is used for RCC-to-ROD and Master-to-Slave inter DSP communication and control.

The communication registers are blocks of 32-bit words at the start of the DSP’s internal memory which are regularly checked by the Master DSP (MDSP) inside of the main thread of execution running on the processor. The MDSP polls these registers, watching for requests from the RCC. These registers are also polled by the RCC and so can be used by it to monitor the status of the DSPs. Such registers are used, for example, to keep a tally of the number of tasks currently running, note if the event trapping is engaged and to report calibration test statistics. The ROD FPGA registers are mapped in the MDSP memory space.

The “primitives” are software entities which allow the MDSP to remain in control of its memory while receiving commands from the RCC. Each primitive is an encoding in a block of memory which indicates a particular command to the receiving DSP. It is through the use of primitives that reading and writing to ROD registers is possible, and through primitives that the ROD is configured and initialized. Generally each primitive is executed once by the receiving DSP. Primitives exist for reading and writing FPGA registers, reading and writing regions of MDSP memory, loading or modifying front end module configurations, and starting the SDSPs. The MDSP can send lists of primitives to the SDSPs, for example to start calibration histogramming. The DSP software is versatile enough to easily handle new primitives representing extra commands when required.

ROD functions which execute over an extended period of time are called “tasks”. These are started and stopped by the RCC (or MDSP) sending primitives and continue to execute independently of the primitive list thread. They run until complete or until they are halted by other primitives.  Examples of tasks are the event trapping and histogramming tasks. The former runs on the SDSPs to handle event trapping while the latter manages the sending of triggers as well as the processing and binning of event data.

## Back of Crate card (BOC)

The BOC transmits commands and data between the ROD and the optical fiber connections which service the front end modules, and is responsible for sending formatted data to the ROS. It also distributes the 40 MHz bunch-crossing clock to the front-end modules and to its paired ROD. A block diagram of the function of the BOC is shown in Figure 4.

Figure 4. Block diagram showing the layout and main communication paths on the BOC card.

The front end modules are controlled and read out through digital optical fibre ribbons. One fibre per module provides trigger, timing and control information. There are also two data fibres per module which are used to transfer the digital signal from the modules back to the off-detector electronics. A more detailed description of the optical system is given in [].

On the BOC each command for the front-end modules is routed via a TX plug-in as shown in Figure 4. Here the command is combined with the 40 MHz clock to generate a single Bi-Phase Mark (BPM) en coded signal which allows both clock and commands to occupy the same stream. Twelve streams are handled by each BPM12 chip []. The encoded commands are then converted from electrical to optical on a 12-way VCSEL array [Error: Reference source not found] before being transmitted to the front-end modules via a 12-way fiber array. The intensity of the laser light can be tuned in individual channels by controlling the current supplied to the laser, using a digital to analogue converter (DAC) on the BOC. This is both to cater for variations in the individual lasers, fibers and receivers and also to allow for loss of sensitivity in the receiver due to radiation damage.

The timing of the outgoing signals from the TX plug-in can be adjusted so that the clock transmitted to the front-end Modules has the correct phase relative to the passage of the particles from the collisions in LHC. This phase has to be set on a module by module basis to allow different optical fiber lengths and time-of-flight variations through the detector. It is also necessary to ensure that the Level 1 trigger is received in the correct 25 ns time bin, so that the data from the different ATLAS detectors are merged into the correct events. For this reason, there are two timing adjustments available – a coarse one in 25 ns steps, a fine one in 280 ps steps.

Incoming data from the front-end modules are accepting by the BOC in optical form, converted into electrical form and forwarded to the ROD. As each front-end module has two data streams there are up to 96 incoming streams on a BOC. The incoming data are initially converted from optical to electrical signals at a 12-way PIN diode array on the RX plug-in. These signals are then discriminated by a DRX12 chip [Error: Reference source not found]. The data for each stream is sampled at 40 MHz, with the sampling phase adjusted so that a reliable ‘1’ or ‘0’ is selected. The binary stream is synchronized with the clock supplied to the ROD to ensure that it receives the data at the correct phase to ensure reliable decoding.

After the data are checked and formatted in the ROD, they are returned to the BOC for transmitting to the first element of the ATLAS higher-level trigger system (the ROS) via the S-Link connection. There is a single S-Link connection on each BOC.

The 40 MHz clock is usually distributed from the TIM via the backplane to the BOC to the front end modules. However, in the absence of this backplane clock, a phase-locked loop on the BOC will detect this state and generate a replacement local clock. This is important not only because the ROD relies on this clock to operate, but also because the front-end modules dissipate much less heat when the clock is not present, and thermal changes could negatively impact the precision alignment of the detector.

### BOC Hardware Implementation

The BOC is a 9U, 220mm deep board and is located in the rear of the DAQ crate. It is not directly addressable via VME as it only connects to the J2 and J3 connectors on the backplane and so all configuration is done over a set-up bus via the associated ROD.

A complex programmable logic device (CPLD) is used for overall control of the BOC. Further CPLDs handle the incoming data – these have been used rather than non-programmable devices as the BOC was designed to also be usable by the ATLAS Pixel Detector, which has special requirements. As can be seen from the previous section, there is a significant amount of clock timing manipulation on the BOC. Many of these functions are implemented using the PHOS4 chip [], a quad delay ASIC which provides a delay of up to 25 ns in 1 ns units. The functions of the BOC (delays, receiver thresholds, laser currents etc.) are made available via a set of registers. These register are mapped to a region of ROD address space via the set up bus, so that they are available via VME to the DAQ. The S-Link interface is implemented by a HOLA [] daughter card.

## TTC Interface Module (TIM)

The TIM interfaces the ATLAS Level 1 Trigger system signals to the RODs In normal operation it receives clock and trigger signals from the ATLAS TTC system [] and distributes these signals to a maximum of 16 RODs and their associated BOCs within a crate. Figure 5 illustrates the principal functions of the TIM – transmitting fast commands and event identifier from the ATLAS TTC system to the RODs, and sending the clock to the BOCs (from where it is passed on to the RODs).

Figure 5. Block diagram showing a functional model of the TIM hardware.

The TIM has various programmable timing adjustments and control functions. It has a VME slave interface to give the local processor read and write access to its registers allowing it to be configured by the RCC. Several registers are regularly inspected by the RCC for trigger counting and monitoring purposes.

The incoming optical TTC signals are received on the TIM using an ATLAS standard TTCrx receiver chip [], which decodes the TTC information into electrical form. In the physics mode the priority is given to passing the bunch-crossing clock and commands to the RODs in their correct timing relationship, with the absolute minimum of delay to reduce the latency. The TTC information is passed onto the backplane of a ROD crate with the appropriate timing. The event identifier is transmitted with a serial protocol and so a FIFO buffer is used in case of rapid triggers.

For tests and calibrations, the TIM can, at the request of the local processor (RCC), generate all the required TTC information itself. It can also be connected to another TIM for stand-alone SCT multi-crate operation. In this stand-alone mode, both the clock and the commands can be generated from a variety of sources. The clock can be either generated on-board using an 80.16 MHz crystal oscillator, or transferred from external sources in either NIM or differential ECL standards. Similarly, the fast commands can be generated on the command of the RCC, or automatically by the TIM under RCC control. Fast commands can also be input from external sources in either NIM or differential ECL. These internally or externally generated commands are synchronised to whichever clock is being used at the time, to provide the correctly timed outputs. All the backplane signals are also mirrored as differential ECL outputs on the front panel to allow TIM interconnection.

A sequencer, using 832k RAM, allows long sequences of commands and identifiers to be written in by the local processor and used for testing the front-end and off-detector electronics. A ‘sink’ (receiver RAM) of the same size is also provided to allow later comparisons of commands and data sent to the RODs.

The TIM also controls the crate’s busy logic, which tells the ATLAS central trigger processor (CTP) when it must suspend sending triggers. Each ROD returns an individual busy signal to the TIM, which then produces a masked OR of the ROD busy signals in each crate. The overall crate busy is output to a separate busy module. ROD busy signals can be monitored using TIM registers.

CDF found that bond wires could break on front end modules when forces from time-varying currents in the experiment’s magnetic field excited resonant vibrations []. The risk to the ATLAS SCT modules is considered to be small [], even on the higher-current bond wires which serve the front-end optical packages. These bonds have mechanical resonances at 17 kHz so, as a precaution, the TIM will prevent fixed-frequency triggers near this frequency from being sent to the front end modules. If ten successive triggers are found at fixed frequencies above 15 kHz, a period-matching algorithm on the TIM will stop internal triggers. It will also assert a BUSY signal which should stop triggers from being sent by the ATLAS CTP. If incoming triggers continue to be sent, the TIM will enter an emergency mode and independently veto further triggers. The algorithm has been demonstrated to have a negligible effect on data-taking efficiency [].

### TIM Hardware Implementation

The TIM is a 9U, 400 mm deep board. The TTCrx receiver chip and the associated PIN diode and preamplifier developed by the RD12 collaboration at CERN [Error: Reference source not found], provide the bunch-crossing clock and the trigger identification signals. On the TIM, a mezzanine board (the TTCrq []) allows an easy replacement if required.

Communication with the BOCs is via a custom J3 backplane. The bunch-crossing clock destined for the BOCs and RODs, with the timing adjusted on the TTCrx, is passed via differential PECL drivers directly onto the point-to-point parallel impedance-matched backplane tracks. These are designed to be of identical length for all the slots in each crate to provide a synchronised timing marker. All the fast commands are also clocked directly, without any local delay, onto the backplane to minimise the TIM latency budget.

On the TIM module, a combination of FastTTL, LVTTL, ECL, PECL and LV BiCMOS devices is used. The Xilinx Spartan IIE FPGA series were chosen as the programmable logic devices. The TIM uses two of these FPGAs per board. These devices contain enough RAM resources to allow the RAMs and FIFOs to be incorporated into the FPGA.

The TIM switches between different clock sources without glitches, and in the case of a clock failure, does so automatically. To achieve this, dedicated clock-multiplexer devices have been used. These devices switch automatically to a back-up clock if the selected clock is absent. Using clock detection circuits, errors can be flagged and transmitted to all the RODs in the crate via a dedicated backplane line, allowing RODs to tag events accordingly.

# Readout Software

The complete ATLAS SCT DAQ hardware comprises many different elements: eight crates containing eight TIMs, eight Linux RCCs, 90 ROD/BOC pairs and nine rack-mounted Linux PCs. The SctRodDaq software [-] controls this hardware and provides the operator with an interface for monitoring the status of the front end modules as well as initiating and reviewing calibrations. The software can optimise the optical communication as well as testing and calibrating the front end ASICs.

It is important that the calibration can proceed rapidly, so that the entire detector can be characterized within a reasonable time. To achieve this, an iterative procedure is generally used, fixing parameters in turn. The results of each step of the calibration is analysed, and any resultant optimization completed before the subsequent step is started. Both the data-taking and the data-analysis of each step must therefore be performed as quickly as possible, and to satisfy the time constraints, parallel processes must run for both the data taking and the analysis.

Fitting Service

SctApi

Server

Analysis Service

Graphical

User

Interface

Calibration and analysis subsystem

Configuration

Service

XML files

Database

XML

files

Calibration

Controller

DCS

Key:

DAQ Process

DAQ Hardware

Archiving

Service

Data Store

Control/Data

Persistent

Storage

Non-DAQ

RODs

BOCs

TIM

SctApi

Crate Server

Rod Crate Controller

Single

Multiple

Figure 6. Control and data flow diagram for the SCT calibration and control system.

A diagram of the main software components is shown in Figure 6. The readout software comprises approximately 250 thousand lines of code, written largely in c++ and java. The hardware-communication parts of the software (SctApi) run on the RCCs and control the RODs, BOCs and TIMs over VME. They are responsible for loading configuration data, setting up the on- and off-detector hardware, performing the actions required during run state transitions, and retrieving monitoring histograms from the RODs. During calibration, they initiate calibration scans and retrieve calibration histograms from the RODs.

The analysis subsystem and user interface run on dedicated rack-mounted Linux PCs. The calibration controller is responsible for synchronizing control during calibration operation. The fitting and analysis services perform data-reduction and calculate the optimal values for calibration parameters. The archiving services write and read data information from transient objects to persistent storage on disk. Inter-process communication is based a number of ATLAS online software tools [] which provide a partition-based naming service for CORBA-compliant interfaces.

Since many operations need to be done in near real-time, most of the processes have concurrent threads of execution. For example the service which fits the occupancy histograms implements a worker/listener pattern. As new data arrives it is added to a queue by a listener thread which is then immediately free to respond to further data. Meanwhile one or more worker threads undertake the processor-intensive job of performing the fits. The fit algorithms have also been optimized for high performance since for most tests several fits are required for every read-out channel (see Section 4).

The front-end and DAQ system configuration information can be stored either in an xml file or in a relational database. It is populated with data from previous calibrations, including quality assurance tests taken during front-end module assembly, and confirmation tests performed during macro-assembly and detector commissioning. A java-based graphical user interface (Figure 7) allows the operator to launch calibration tests, to display the results of tests, to display calibration information and to compare results to reference data.

Figure 7. A view of the SCT graphical user interface. This display is showing the BOC receiver thresholds for the RX data links on all BOCs on a particular DAQ crate. The colour scale indicates the current value of the threshold. The display is set in a crate view where vertical lines of modules represent complete RODs, each of which services up to 48 modules.

# Detector setup and calibration

The front-end calibration is crucial for the correct operation of the detector because a large amount of pre-processing and data reduction occurs on the SCT’s front-end ASICs. Each front-end chip has an 8-bit DAQ which allows the threshold to be set globally across that chip. Before irradiation it is generally not difficult to find a threshold for which both the noise occupancy (< 510-4) and efficiency (> 99%) targets can be satisfied. After the module is irradiated, setting the threshold at a suitable level becomes even more important. Irradiation decreases the signal collection, and increases the noise seen by the front-end. This means that after 10-years LHC equivalent radiation the working region narrows, and to satisfy the performance requirements the channel thresholds need to be set within in a more limited range. To assure uniformity of threshold, every channel has its own 4-bit DAC (TrimDAC) which is used to compensate for channel-to-channel threshold variations. The TrimDAC steps can themselves be set to one of four different values, allowing uniformity of thresholds to be maintained even as uncorrected channel-to-channel variations increase during irradiation.

## Module communication

When first powered the SCT modules return the 40.08 MHz input clock signal on each of their two optical links with the returned signal having half frequency of the clock. Once this returned clock is observed, basic communication is confirmed.

The next step is to optimize the optical communication parameters, for example the receiver threshold and sampling phase used for the incoming data on the BOC. The front-end modules are set up to return the contents of their configuration register, rather than hit data, so a known bit-pattern can be expected. Triggers are then sent and the value of the BOC registers varied, so that operating region in which the binary stream is faithfully transmitted can be located.

## Front end calibration

Most of the calibration procedures are designed to set registers on the front-end ASICs. The most important features of these ABCD3TA chips [Error: Reference source not found] can be seen in Figure 8. The analogue front-end carries out charge integration, pulse shaping, amplitude discrimination, and latching of data. The digital pipeline stores the resultant binary 1-bit per channel information for 132 clock cycles pending the global ATLAS first level trigger (L1) decision. If a L1 trigger is received, the data are then read-out serially with token passing between the chips daisy-chained on the module. The encoding algorithm compresses the data using an algorithm which only transmits information about hit channels.

Figure 8. Simplified Block Diagram of a SCT ABCD3TA ASIC. From [1].

Every fourth channel can be tested simultaneously by applying a known voltage across a calibration capacitor from another 8-bit DAC. By injecting various known charges, and measuring the occupancy at different thresholds, the analogue properties of each channel can be determined. A complementary error function is fitted to the occupancy histogram. The threshold at which occupancy is 50% corresponds to the median of the injected charge and the sigma is a measure of the analogue output noise in mV. An example threshold scan is shown in Figure 9. During this calibration scan 500 triggers were sent per threshold point, and the charge injected was 1.5 fC.

(a) (b)

Figure 9. Occupancy as a function of front-end threshold. (a) The shading scale shows the fraction of hits, as a function of the channel number (x axis, and comparator threshold in mV (y-axis) for one all channels on one side of a barrel module (6 ASICs; 768 channels). The front-end parameters were already optimized before this scan, so the channel-to-channel and chip-to-chip occupancy variations are small. (b) Mean occupancy and complimentary error function fits for the six ASICs.

To calibrate the discriminator threshold, the DAQ system initiates threshold scans for several different values of injected charge. Example ten-point response curves for a particular module are shown in Figure 10. From the data used to create these plots, the front end gain and input noise can be calculated. The gain is the gradient of the response curve, and the input noise can be calculated by dividing the output noise (typically at 2 fC) by the gain [Error: Reference source not found].

Figure 10. Response curves, showing the value of the discriminator threshold at which the mean chip occupancy is 50% as a function of the charge injected for each of the 12 chips on one module.

Similar scans are used to optimise the TrimDAC registers. In these scans the injected charge is kept constant and threshold scans are performed for different values of the TrimDAQ registers.

Threshold scans with no injected charge are used to find the noise occupancy. The response curve allows the threshold to be calibrated in units of input charge. The parameter of interest is the noise occupancy near the 1 fC nominal working point. A variety of different noise scans can be used to test for noise pickup from cross-talk between front-end modules, and from electrical or optical activity associated with the readout [].

A full test sequence contains other procedures which verify the digital performance of the ASICs. These exercise and test the front-end trigger and bunch-crossing counter registers, the channel mask registers, pipeline cells and chip token-passing logic as described in [Error: Reference source not found,,].

# Application and results

## Barrel and endcap macro-assembly

The SctRodDaq software was used extensively to test the performance of large numbers of modules after mounting onto their support structures at the assembly sites [Error: Reference source not found,Error: Reference source not found,Error: Reference source not found,,]. Groups of up to 672 modules (full barrel 6) were tested simultaneously with single crate DAQs. The ATLAS central elements (CTP, LTP, ROS etc) were not present, so the DAQ was operated in calibration mode, with triggers generated either on the RODs or on the TIM. The hit data were histogrammed on the RODs and the S-Link data discarded. Tests were performed to measure the noise performance, confirm known problem channels, and check that no new electrical defects had been introduced during the assembly process.

Figure 11. Average input noise for chips on each of the four SCT barrels. The units on the x-axis are equivalent noise charge in electrons. These tests were performed “warm” with the mean of the modules’ temperature sensors as indicated for each barrel.

Histograms of the chip-averaged input noise values found during barrel assembly are shown in Figure 11. The noise is consistent with single-module tests performed during module production. The number of defective channels is summarised in Table 3. These numbers include channels which were found to be dead, to have unacceptably high noise, or have other defects such as missing wire bonds which made them unusable in practice.

|  | Total channels | Problem channels | Functional Channels |
| --- | --- | --- | --- |
| Barrels | 3,244,032 | 8,326 | 99.74% |
| Endcap A | 1,517,568 | 4,371 | 99.71% |
| Endcap C | 1,517,568 | 4,205 | 99.72% |

Table 3. Summary of the defective and functional channels on the barrels and on each endcap.

Similar tests were performed during endcap macro-assembly. The module-average input noise for module on endcap A is shown in Figure 12. The corresponding plot for endcap C is almost identical. On each endcap, five of the nine disks contain some modules with shorter strips (“short-middle” and “inner” modules) which have lower noise than those with standard-length strips [Error: Reference source not found]. The number of defective channels and the fractional of channels which are fully functional on the endcaps are also summarised in Table 3.

Figure 12. Average input noise per module for endcap A. These data were taken cold, with the modules’ temperature sensors at approximately 15 C.

For both the barrel and the endcaps, the noise levels are consistent with expectations. The fraction of fully functional channels is greater than 99.7% – much better than the build specification of 99% good channels.

## Commissioning and cosmic ray tests

At CERN, the SCT barrel and endcaps were each integrated [] with the corresponding sections of the transition radiation tracker (TRT). Further tests, including combined SCT/TRT cosmic ray studies, were then performed. These were the first large-scale tests of the SCT DAQ in physics mode.

For the barrel test, 468 modules, representing 22% of all modules on the four barrels, were cabled to make “top” and “bottom” sectors in φ. There was no applied magnetic field and care was taken to reproduce, as far as possible, the service routing and grounding of the final setup in the pit. All data were taken warm with module temperatures sensors at approximately 28 C.

Cosmic rays were triggered using coincident signals from a pair of scintillators located above and below the barrel. Unlike during the assembly tests, the clock, trigger identifier information was distributed to the SCT TIM and to the TRT DAQ using ATLAS Local Trigger Processor (LTP) modules. The resultant hit data were transferred from the SCT DAQ via the S-Link to a ROS and then written to disk. As well as using the cosmic trigger, noise data were recorded in physics mode under a variety of test conditions, using fixed frequency or random triggers sent from a master LTP.

To time-in the SCT with the cosmic trigger, the modules’ relative timings were calculated from known differences in optical fiber lengths. The global delay was determined using dedicated ROD monitoring histograms which recorded, as a function of the global delay, the number of coincident hits on neighbouring chips on opposing sides of each module. After timing-in, hits from cosmic rays traversing the detector could be observed on the event display (Figure 13). Comment on the noise level, when new plot available.

Figure 13. An ATLANTIS [] event display showing a cosmic ray traversing the semiconductor tracker. Hits on a single silicon plane are indicated by small squares. Double hits indicating hits on both sides of a module are shown by larger white squares. The line indicates the position of a straight track fitted to the double points. Replace with plot from Maria.

In the noise tests, the occupancies obtained were not significantly different from those found for tests made on the individual barrels before integration. No significant change in the noise occupancy was observed when running concurrently with the TRT, when running at trigger rates from 5 Hz to 50 kHz, or for synchronous versus asynchronous triggering.

(a)(b)
 (c) (d)

Figure 14. (a) Histogram of the logarithm of the average noise occupancy for each chip on the 468 modules under test. The vertical dotted line indicates the performance specification of 510-4. (b) Histogram showing the number of hits per event for a noise and a cosmic-ray triggered run. (c) Noise occupancy as a function of the channel (strip) number, averaged over all modules. The vertical dashed lines indicate the chip boundaries. The noise data are based on 80k events randomly triggered at 5 kHz with a calibrated front-end discriminator threshold of 1.0 fC. (d) Detail showing the average channel occupancy variation across a chip.

More than 450 thousand cosmic ray events and over 1.5 million synchronous noise events were recorded in the barrel tests. Figure 14a shows that the average noise occupancy was about an order of magnitude below the 510-4 specification. The distribution of noise hits per event is very well described by a Gaussian curve (Figure 14b), so there is no evidence of correlated noise. By contrast, events triggered by comic rays have a long tail showing the expected correlated hits. There is some modulation in the number of noise hits across each ASIC (Figure 14c,d) but the average occupancy is well below the specification for all channels.

A further nine million physics-mode events were recorded in the synchronous operation of 246 modules during the commissioning of endcap C. Again no significant change in noise occupancy was found for the endcap when integrated with the TRT compared to assembly tests, for synchronous versus asynchronous triggers, or for different trigger rates in the range 1 kHz to 100 kHz.

Further information about the setup and results of these commissioning tests can be found in [-]. In particular the hit-finding efficiency for cosmic-triggered tracks was found to be greater than 99% for all barrel layers after alignment.

# Conclusions

The ATLAS SCT data acquisition system has been used extensively since the autumn of 2004 for performance testing and quality assurance during assembly and commissioning of the detector. Quality assurance tests in calibration mode, made simultaneously on groups of up to 672 modules (11% of the complete SCT) have helped ensure that the barrel and both endcaps were each delivered to CERN with more than 99.7% of channels performing to specification.

Commissioning tests in physics data-taking mode have demonstrated the continuing good performance of the SCT barrel and endcaps after integration with the TRT. Over ten million events have been successfully taken with synchronous triggers, demonstrating successful operation of both the DAQ system and the SCT detector with the final ATLAS trigger and data chain.

The flexibility of the DAQ hardware and software will allow the performance of the SCT to continue to be monitored, and its calibration fine-tuned as the detector properties change during irradiation.

# Acknowledgements

We acknowledge the support of the funding authorities of the collaborating institutes, including the Spanish National Programme for Particle Physics; the Research Council of Norway; the Particle Physics and Astronomy Research Council of the United Kingdom; The Polish Ministry of Education and Science; the German Ministry of Science; the Swiss National Science Foundation; the State Secretariat for Education and Research and the Canton of Geneva; the Slovenian Research Agency and the Ministry of Higher Education, Science and Technology of the Republic of Slovenia; the Ministry of Education, Culture, Sports, Science and Technology of Japan; the Japan Society for the Promotion of Science; the Office of High Energy Physics of the United States Department of Energy; the United States National Science Foundation; the Australian Department of Education, Science and Training; the Dutch Foundation for Fundamental Research on Matter (FOM); the Ministry of Education, Youth and Sports of the Czech Republic; the National Science Council, Taiwan; the Swedish Research Council.

# Appendix

Alphabetical list of selected abbreviations:

ABCD3TA	SCT front-end application-specific chip.

BOC		Back of crate card

CTP		The ATLAS central trigger processor

DAQ		Data acquisition

ENC		Equivalent noise charge

FE		Front-end

HOLA		High speed optical link for ATLAS

LHC		The CERN Large Hadron Collider

LTP		Local trigger processor module

MDSP		Master digital signal processor.

RCC		Rod crate controller

ROD		Read-out driver

ROS		Read-out subsystem

RX		Receiver

S-Link		A serial optical link

SCT		Semiconductor tracker

SDSP		Slave digital signal processor

TTC		Trigger, timing and control

TIM		TTC interface module

TrimDAC	DACs which allow correction of individual channel thresholds

TX		Transmitter

[?]  The Atlas Inner Detector paper

[?] A. Abdesselam et al, “The barrel modules of the ATLAS semiconductor tracker”, Nucl.Instrum.Meth.A568:642-671,2006

[?] Forward modules paper

[?] Sensors paper, “The Silicon Microstrip Sensors of the ATLAS SemiConductor Tracker”

[?] F. Campabadal et al, “Design and performance of the ABCD3TA ASIC for readout of silicon strip detectors in the ATLAS semiconductor tracker”, Nucl.Instrum.Meth.A552:292-328,2005

[?] TDAQ paper

[?] http://atlas.web.cern.ch/Atlas/GROUPS/FRONTEND/documents/Crate_Technical_Specification_final.pdf

[?] B.G. Taylor “Timing distribution at the LHC” in 8th Workshop on Electronics for LHC Experiments Electronics for LHC Experiments , Colmar, France , 9-13 Sep 2002, pp 63-74

[?] Rod paper: et al “A Read-out Driver for ATLAS Silicon Detector Modules" in preparation

[?] http://atlas.web.cern.ch/Atlas/GROUPS/DAQTRIG/ReadOut/

[?] http://hsi.web.cern.ch/HSI/S-Link/

[?] A. Abdesselam et al. “The Optical Links for the ATLAS SCT” In preparation

[?] M-L Chu et al, “The Off-Detector Opto-electronics for the Optical Links of the ATLAS SemiConductor Tracker and Pixel Detector”, Nucl.Instrum.Meth.A530:293 (2004)

[?] PHOS4 chip

[?] http://hsi.web.cern.ch/HSI/S-Link/devices/hola/

[?] http://ttc.web.cern.ch/TTC/intro.html

[?] http://www.cern.ch/TTC/TTCrx_manual3.9.pdf

[?] G. Bolla et al, Wire-bond failures induced by resonant vibrations in the CDF silicon tracker, Nucl.Instrum.Meth. A518, 227 (2004)

[?] T. J. Barber et al, “Resonant bond wire vibrations in the ATLAS semiconductor tracker”, Nucl.Instrum.Meth. A538, 442-457 (2005)

[?] A. J. Barr et al., “A Fixed Frequency Trigger Veto for the ATLAS SCT”, ATL-COM-INDET-2007-002

[?] http://proj-qpll.web.cern.ch/proj-qpll/images/manualTTCrq.pdf

[?] M. J. Palmer, “Studies of Extra-Dimensional Models with the ATLAS Detector”, University of Cambridge thesis, Jan 2005

[?] B. J. Gallop, “ATLAS SCT readout and barrel macro-assembly testing and design of MAPS test structures” University of Birmingham thesis, Sep 2005

[?] The ATLAS online software. REF?

[?] P. Philips, “Functional testing of the ATLAS SCT barrels”, Nucl.Instrum.Meth.A570 292-328 (2007)

[?] L. Eklund et al, “Electrical Tests of SCT Hybrids and Modules”, ATLAS note ATL-INDET-PUB-2007-006

[?] A.J. Barr, “Calibrating the ATLAS Semiconductor Tracker Front End Electronics”, ATL-INDET-CONF-2006-001, IEEE NSS Conference Records, 16-22 Oct. 2004, 1192-1195

[?] G. Viehhauser, Conference Record of the 2004 IEEE Nuclear Science Symposium, Rome, Italy, 16–22 October 2004 (2004), pp. 1188–1191.

[?] D. Ferrère, “The ATLAS SCT endcap: From module production to commissioning” Nucl.Instrum.Meth. A570 225-229 (2007).

[?] H. Pernegger, “Integration and test of the ATLAS Semiconductor Tracker”, Nucl.Instrum.Meth. A572 108-112 (2007)

[?] G. Taylor, “Visualizing the ATLAS Inner Detector with Atlantis”, proceedings of VERTEX 2003, published in Nucl.Instrum.Meth. A549:183-187 (2005).

[?] The SCT cosmics note

[?] B.M. Demirköz, “Cosmic tests and performance of the ATLAS SemiConductor Tracker Barrels”, Nucl.Instrum.Meth. A572 43-47 (2007)