Sub-project 2: The Electronic Model

Objectives

The aim of this part of the project is to design a portable electronic model simulating the LEP and LHC accelerator systems at CERN. It should be based on the display currently at the Science Museum.

Principle Contacts

Dominic Hayes: room C19, dah@hep.ucl.ac.uk

Brian Anderson: room D26, ba@hep.ucl.ac.uk

Janet Fraser: room D109, tif@hep.ucl.ac.uk

Peter Clarke: room D12, clarke@hep.ucl.ac.uk

Jon Butterworth: room D23, jmb@hep.ucl.ac.uk

The Science Museum Model

This was built by members of the department. Dominic Hayes was the most involved in the design of the driving electronic system. Martin Postranecky, Brian Anderson, Janet Fraser and Peter Clarke were also very involved in the project.

The model’s user interface takes the form of an aerial view of the CERN area on the Swiss-French border. The image is similar to that shown in fig 1. The major differences between the two is that the picture over-leaf is more up to date and has the positions of the beam pipe rings and the border drawn onto it. The department had to estimate the shape of the rings from the positions of the detector sites.

LEDs are mounted along the circumference of the main ring as well as round the smaller ring and along the linac assemblies used in the introduction of the particles to the synchrotron. The LEDs flash in sequence to signify a particle bunch moving around the ring. Specific LEDs are used to simulate particles traveling in one direction around the ring only. This simplifies the electronics a great deal.

Beneath the picture of CERN are buttons. These perform the following functions: selecting the accelerator system - LEP or LHC - to be simulated, accelerating the particle bunches (speeding up the succession of  LED flashes), and colliding the beams.

The simulation uses two linacs to feed slow moving particles into the small ring as shown over-leaf in figure 4.

Figure 8

The blue and red lines signify the Linac sets of LEDs that introduce particles to the small ring (shown in purple) in the clockwise and anti-clockwise directions respectively.

Look carefully at Figure 8. It can be seen that CERN’s system for introducing particle bunches to the large ring is not as simple as shown above. This is not, however, relevant to what the simulation is trying to communicate. A more accurate set up would be confusing to users and would require detail not practical for the size of LED used as well as a good deal of extra electronics.

One flash is passed along the linacs at a time. The flash completes one turn of the small ring before meeting another flash from the linac relevant to its direction. The new flash then follows the first around the small ring before a third and then a fourth latch on. Once a string of four flashes is set up the bunch is transferred to the main ring and a new bunch begins to be built in the small ring. Hence, at any one time, a particle bunch in the large ring comprises four illuminated LEDs. The bunch moves around the ring as the LED on one side of it lights up while that on the other switches off.

The LEDs used have the property that a current in one direction causes the emission of one colour light and in the other direction, another colour. This is used to differentiate between different types of particles. For the LEP simulation, the clockwise LEDs shine yellow and the anti-clockwise green, representing electrons and positrons. For the LHC, the LEDs in both directions emit red light corresponding to all the beams being made up of protons.

The timing of the system is such that the particle bunches pass through each other at the detector sites. Hence, for the LEP simulation, particles are introduced until there are four bunches traveling in each direction and for the LHC until there are two in each direction. When all the bunches are established on the main ring the accelerating button lights up. Holding this down speeds up the succession of illuminated LEDs and sets up a tone which rises in pitch to give the impression of the particles’ acceleration. With each successive stage in the simulation an explanation lights up beside the machine. When the acceleration is complete the bunches are no longer distinguishable from each other. The colliding button lights up. Pressing this causes optical fibres at the detector sites to flash, accompanied by dramatic crackles.

Revision of Objectives

At the first meeting with Dominic Hayes the group’s suspicions as to the extreme complexity of the Science Museum machine’s driving electronics are confirmed. Sharing their time with other projects, it took the team around eighteen months to complete. No members of our group have any knowledge of electronic engineering. It is clear that attempting to (re)design a portable version of the model the department built is not sensible. Therefore, emphasis is put on the software side of the project. This will require many new programming skills but the problem is more approachable than building an understanding of what is in essence an entirely foreign discipline.

The group’s targets concerning the electronic simulator are cut down to building a basic understanding of the different options available for making the portable machine. These will be compared for their relative strengths and prices with a view to making as thorough a recommendation as possible as to the best way to address the problem more directly in the future.

It is expected that the two group members working on this will involve themselves in other aspects of the project, particularly towards its close.

General Restrictions on the Dimensions of the Portable Machine

The new machine should be “light, small and robust enough to be carried in the back of a car to various locations”. This is taken to mean that it should light enough and strong enough to be carried comfortably by two people and should be no larger than could fit in a typical car boot. On these bases, estimates for maximum size and mass are;

Maximum (length*height*width) ® 1m*0.75m*0.5m

Maximum mass ® 30kg

A key feature of the machine built by the department is that a separate set of LEDs are used to simulate particles traveling around the large ring in the clockwise and anti-clockwise directions. This is to simplify the driving system as opposed to being for the purposes of  having different colour flashes to signify the three different particles accelerated - single LEDs are available which can perform this function. Hence, in theory, the size of the ring could be at least halved by using a more detailed program. However, it is expected that the arrangement will prove a very important simplifying step in the machine design regardless of which driving system is adopted. Therefore, it is initially assumed that the number of LEDs on the new machine will approximate that on the Science Museum’s, around 500. Significantly fewer LEDs would upset the smoothness of a flash’s journey and limit the separation of bunches to a less pleasing distance.

Designing the Front of the Portable Machine

It is decided that the LEDs showing the paths of the accelerator particles should be mounted on an aerial view photograph of CERN. In order to obtain a recent photograph, the editor of ‘CERN Courier’, Gordon Fraser, is e-mailed at “cern.courier@cern.ch”. He suggested two possibilities and Janet Fraser offered to pick up these slides. The two suggested photographs are shown in figures 7 and 8.

Figure 9-An aerial view of Geneva

Figure 10-Another aerial view of Geneva

Both images are suitable. Figure 10 is very similar to the photograph at the Science Museum and has the advantage of being more educational as it shows the Swiss-French border. It is also considered more pleasing to the eye. However, the dimensions of the LEDs and circuit boards must also be taken into consideration in deciding which picture to use.

Choosing the LEDs for the Portable LEP and LHC Simulations

In the interactive simulation of the CERN accelerators presently at the Science Museum the paths of the particles are represented by 541 bi-colour LEDs, the breakdown of which is as follows:

For Clockwise motion (positrons) :

Linac:	9

SPS:	47

LEP:	212

For anti-clockwise motion (electrons):

Linac:	6

SPS:	56

LEP:	212

(One of the LEP LEDs is not fitted due to space constraints, but there is a driver for it.)

It is necessary to use bi-colour LEDs to distinguish between the different types of particle being accelerated.

The only way to make the display smaller without changing the number of LEDs on each part of the ring system is to use surface mount technology (SMT). This amounts to securing components directly onto printed circuit-boards as opposed to having to solder each individual element as is necessary with the dual in line (DIL) system adopted in the original design. Components made for this purpose are generally smaller than traditional ones and, more importantly, can be mounted closer together. Equally, circuit boards arranged using SMT are more robust.

The Siemens web page “http://www.siemens.de/semiconductor/products/37/37.htm” contains lots of information on the different types of LEDs available, many of which are suitable for surface-mounting. The following information (see table ??) was obtained on the types LSGT670, LSPT670, LSYT670, LOPT670 and LOGT670, which are all suitable for surface mounting (SMT) and are bi-colour like the LEDs on the Science Museum demonstration.

Figure 11: Table showing the  features of SIEMENS Multi TOPLEDÒ

(from the web site above)

These LEDs are suitable for both SMT assembly and soldering methods and are available taped on a reel (8mm) tape. With appropriate controlling it is possible to change the colour from green to yellow and from orange to super-red. This property will enable the same colour scheme as at the science museum to be used. More details on the maximum ratings and characteristics of these LEDs are given in the appendix.

Pricing the LEDs

Various electronic components catalogues are studied. LEDs of type LSG T670 - HK are found on page 820 of the April/September 1997 edition of the ‘Farnell Electronic Components Catalogue’. In this catalogue these LEDs are priced at œ0.34 each if more than 500 are bought. Therefore, if the number of LEDs is kept at 542, the cost will be around œ184.28.

The Dimensions of the LEDs

The dimensions of the LSG T670 -HK LEDs considered are shown in figure12.

Figure 12: The dimensions of the SIEMENS MultiTOPLEDÒLSG T670, LSP T670,

LSY T670 LOP T670, LOG T670 (All dimensions are in mm)

| L | S | G | T670 |
| --- | --- | --- | --- |
| LED | Emission colour 1 | Emission colour 2 | Package |
|  | cathode: pin 1 | cathode: pin 2 |  |

These dimensions are needed to determine the size to which the photograph has to be enlarged. The LEDs can be positioned next to each other, but it is best to leave a small amount of space to allow for the curvature of the ring. So, the circumference of the larger ring needs should be at least (424 x 3)mm on the enlarged photograph.

Calculating the Size of the Background Photograph

At first it was thought that the size of the background picture would just be dependent on the number of LEDs needed in each part of the accelerator’s path, but, by doing further research, it was found that the maximum dimensions of printed circuit boards would be a limiting factor. The maximum quoted size from one company is 406mm x 588 mm. The next step was thus to calculate whether or not the required number of LEDs would fit on this size board.

In order to measure the relative dimensions of the rings on each of the slides, a slide projector, string and metre rule were used. Figure 11 shows the measurements of the projected images.

Figure 13: Schematic diagrams showing the dimensions on the slides

(NB The width and height measured are of the proportion of the slide thought to be needed on the demonstration, not of the whole slide.)

Figure 13a: Slide 1

700mm

250mm

Circumference of larger ring, Cl=1500mm

Circumference of smaller ring, Cs=539mm

Figure 13b: Slide 2

340mm

823mm

Circumference of larger ring, Cl=1725mm

Circumference of smaller ring, Cs=443mm

It was not thought possible to fit the whole of either of the photographs on the printed circuit boards, so all future calculations are carried out using the above measurements, which are the parts which are vital.

For the relevant part of slide 1 to fit on the circuit board of dimensions 406mm x 588mm, the larger circle should be blown up to a circumference of:

588700∗1500mm=1260mm

A circumference of 1260mm would just about allow for 520 LEDs in the ring, so this is a reduction of 4 on the desired amount. If the photograph is blown to this size, the smaller ring would have a circumference of 452.76mm, which would allow plenty of room for the desired number of LEDS.

Similarly, for the relevant part of slide 2 to fit on the said circuit board, the larger circle should be blown up to a circumference of:

588823∗1725mm=1232.24mm

This circumference should allow for 410 LEDs in the ring, a reduction of 14 on the desired amount. Using a similar calculation, the smaller ring would have a circumference of 316.51mm, which should still allow for the103 LEDs as used in the Science Museum.

The Decision of what Slide Should be Used and the Cost of Enlargement

The decision made was that slide 1 would be the most suitable slide to use on the portable demonstration. A circuit board of 588mm * 406mm would allow for just 4 less LEDs than on the existing demonstration at the Science Museum. If slide 2 was used, a further 10 LEDs would have to be excluded and this could result in too small a separation between the bunches resulting in a less satisfying simulation.

The relevant part of the photograph containing the rings should be enlarged to about 588mm * 210 mm, which is roughly 23.52 inches * 8.40 inches. The cost of this photo enlargement is quoted as £38 from ‘Bill Johnston Photography Limited’, 5-11 Mortimer Street.

Protecting the Front of the Portable Demonstration

Obviously, if the portable simulation is to be carried back and forth between schools it will need to be hard-wearing and the front will have to be protected. Lamination of the photograph would provide some protection. Additionally, a lid for transit should be considered.

Design Options for the Driving System:

I) Using Surface Mount Technology to Build a Smaller Replica of the Original Machine.

The most obvious approach is to use the circuit design of the machine in the museum  to build a portable version. Naturally, however, investigation is necessary into how the simulator could be made smaller.

The first consideration is the possibility of cutting down the number of components on the boards. Doing this without significantly changing the circuit design amounts to combining chips. See the Science Museum machine’s circuit diagrams; appendices 1, 2 and 3. We can see from appendix 1 that the driving system comprises eight matching circuit boards numbered ESD1 to 4 and PSD1 to 4. Appendix 2 shows one of these; PSD2, and appendix 3 a detail of half of the same board. Seven different components are used on PSD2. These are as follows;

Logic ICs:

74LS164 (´10)

74LS540 (´12)

74LS32 (´2)

74LS08 (´4)

74LS541 (´2)

74LS14 (´1)

Resistor set: 150R (´10)

The simplest way to reduce the component count is to combine similar chips. From appendix 3, we can see that this could be done relatively easily in the cases of the 74LS164 logic ICs at U7 and U6, the 74LS540s at U17 and U16 and resistors RP7 and RP6. For these purposes matching components with double the number of bits are required.

It makes sense to use surface mount technology for the whole machine, not just the LEDs. Hence, SMT versions of all the components used in the original machine are sought. This proved quite difficult because the components used in the Science Museum machine are quite old-fashioned. SMT is relatively modern and so components tend to be made for the purposes of today’s systems. The equivalent logic ICs with increased numbers of bits were not found apart from in the case of  the 16-bit 74LS164. However, it is not worth redesigning the boards to cut down the overall count by just eight components.

It is decided to adopt high speed C-mos (HC) chips instead of the more old fashioned LS chips. This is purely done to make it easier to find SMT versions of the Logic ICs and resistors. The following components are selected from the Farnell catalogue, pages 1282 and 1099 (see table 2, over-leaf).

Table 8: Suggested SMT Components for a Replica of the Science Museum’s Model

| Component #. | Order Code. | Relevant Price (£ each)* | Price for Full Set (£). |
| --- | --- | --- | --- |
| MM74HC164 | 379-426 | 0.33 | 26.40 |
| MM74HC540 | 379-566 | 0.46 | 44.16 |
| MM74HC32 | 379-293 | 0.27 | 4.32 |
| MM74HC08 | 379-244 | 0.18 | 5.76 |
| MM74HC541 | 379-578 | 0.65 | 10.40 |
| MM74HC14 | 379-268 | 0.31 | 2.48 |
| 150R | 911-136 | 0.028 | 2.24 |

Total Price = £96.60

* The prices listed in this column correspond to the number of components being ordered. For example, eighty MM74HC164 drivers will be required for one machine. Ordering over fifty of these entitles you to a discount. The next discount is for over five hundred. Hence, at least six machines would have to be made for the price of each component to be noticeably cheaper.

It is felt that multi-layer boards will be required for this system. This amounts to the boards being fused together with conducting bolts that connect relevant parts of the circuits. The machine can be made very thin and robust this way. It would be necessary, however, to make the driving boards the same size as the front board with the surface mounted LEDs (406 ´ 588mm).

Estimating the price of assembling the circuit boards this way is difficult. Manufacturers give quotes according to the specific job. Price rises markedly with circuit complexity, number of layers required (in our case nine plus an integrated button and control set up) and how many links there are between layers. The department recently had 2 matching 200 ´ 300mm 4 layer boards of moderate complexity made for about £400 each. Making a multi-layered board to the original circuit design can be estimated to cost of the order £5000. Importantly, however, each subsequent machine will be much cheaper.

The advantages of this approach are as follows;

- It requires a minimal amount of work - most of the engineering has already be done.

- The department’s machine has been very successful. No break down has occurred since its instillation, during which time it has been used by some 50,000 visitors. Of  these, over half followed the demonstration to its conclusion. This is despite the display’s fairly obscure situation within the museum.

- As discussed, the price of making the boards and buying components will not turn out to be as high as quoted above because more than one machine will be made. It is not yet known how many will be required (probably of order five) but certainly there will be some scope for ordering in bulk.

There are, however, some disadvantages to approaching the problem this way;

- It is, none-the-less an expensive option. To achieve a sufficiently robust and small machine multi-layered circuit boards would have to be used. There are no facilities within the department to build these so they would have to be produced externally.

- Effective though the Science Museum’s machine is, its technology is dated. Today, single chips can be bought which perform the same functions as complex arrangements of chips such as those listed above. These are discussed briefly in the following section.

- The system is inflexible - once built practically no modification can take place.

II) Using Programmable Chips

Programmable chips have functions are permanently programmed onto them. Instead of using a large number of simple chips which can be integrated into all kinds of circuits, relatively few complex and specific ones are used.

Traditional programmable chips comprise an array of AND gates that feed an array of OR gates. The programming amounts to connecting the two Programmable Logic Arrays to perform the desired function. These systems are known as PLDs. Today, large numbers of PLDs are combined onto one chip to make CPLDs (Complex PLDs), which can be equivalent to up to 50,000 logic gates.

However, far more powerful are the modern FPGAs. These differ from PLDs because the pattern of links between gates is such that each gate can be used a number of times during each operation. This way, whole systems can be set up on a single chip.

It is difficult to quantify the relative price of building the machine this way to the way described above. It would seem that an altogether new approach would need to be taken to the driving system. The program would have to be begun afresh. However, once the design is completed, the production costs would clearly be far lower. The final machine would also have the advantage of containing fewer components and so be smaller and lighter. Perhaps it would be useful to print on both sides of the LED board but certainly expensive multi-layered boards would not need to be commissioned. Hence, the amount of design work that has to be done by the members of the team is much more important than external production to the cost of the project.

Programmable chips have the disadvantage over computer programs that their function cannot be changed after being built.

III) Running the LEDs Directly from a Computer

Figure 14. A Simple Box Diagram to Illustrate a Possible Computer Run Driver.

Serial Receiver

Portable System

Computer

Serial Port

Memory

Pattern Generator

Clock

500 LED Array

The principles behind a driving system like the one shown above can be quite simply explained. The pattern generator would have an output of 500 bits, “1” corresponding to a specific LED being illuminated and “0” to it being turned off. The computer’s memory keeps track of the state of all the bits. The information is relayed to the LEDs themselves through a serial line between the computer and the portable machine. A clocking device regulates how long the LEDs stay in any one state before getting their next set of instructions from the pattern generator. This device could be part of the portable machine with a button accelerating the bunches by shortening the time between the receipt of successive sets of bits, or could be incorporated into the program.

The program generating the bit pattern could be the same one as used for the CD-ROM simulation required. However, unless the program had been made with the portable model specifically in mind, it would need a certain amount of modification. In particular, the number of bits would have to match the number of LEDs.

The advantages of building the simulator this way are as follows:

- It markedly reduces the amount of work that has to be done for the project as the same program can be used to drive the LED display as the simulation on the computer screen.

- There are far fewer expensive components to buy. All that is required is the circuit board with the 500 surface mounted LEDs discussed above and a serial port. The timer would be integrated into the program for consistency.

- This dramatic fall in component count will serve to make the machine lighter and therefore more easily portable.

- There is scope for making explanations of what is happening appear on the computer screen to coincide with the simulation, just as takes place in the Science Museum.

- The system will be far easier to change and update.

There are, however, some important reservations to the approach:

ù The computer program will most likely need to be made more sophisticated to run the LEDs. This is because the scale of a monitor screen compared to the LED ring will mean that less detail will tend to be adopted for the computer simulation. Hence, the true fall in workload may not be as obvious as hoped.

ù The machine will now be reliant on there being a modern computer available. However, these are now very widespread and, in exceptional circumstances, a lap-top can always accompany the display.

ù Explanations appearing on the screen will be relatively obsolete for the portable model compared to that in the Science Museum seeing as the machine is intended to be accompanied by a demonstrator.

ù It is felt that selecting functions, particularly in the cases of accelerating and colliding the particles, will be less exciting via a cumbersome computer interface than by pressing bright buttons on the machine itself. A computer’s presence will make the overall set-up look less attractive and detract attention away from the display. Good presentation is paramount to keeping an audience’s attention.

Conclusion

Only a limited understanding of the LEP/LHC simulator at the Science Museum and the alternatives for making it portable has been achieved. However, on the basis of what has been learned, the group would suggest the following course of action;

- Irrespective of which driving system is chosen, the display should be based around a 406 ´ 588mm printed circuit board with the image of Slide 1 mounted onto it such that the 541 SMT LEDs follow the large and small rings as well as two imaginary linacs. Suitable LEDs would be the LSG T670-HK type from the Farnell catalogue. The front of the machine may be larger than the size of the circuit boards to accomodate more of CERN’s surroundings.

- The LEDs should be driven by a modified version of the computer program written for the CD-ROM simulation. This would be cheap in terms of parts for the machine and of time put into the project by the department as the computer program must be written anyway.

The possibility of using programmable chips must be more carefully considered than the group had the expertise to do before any decision is made. Were it possible to transfer the computer program onto these chips then this would be preferable because no distracting computer interface would need be involved. However, if the system is to be connected to a computer, it would be wasteful not to coordinate the display with information appearing on the screen, if only to act as a prompt for the demonstrator.

Research also needs to be done into the possibilities available for integrating the original machine’s sound effects into the portable system. These add a great deal to the Science Museum demonstration. Whilst not being particularly relevant to what really takes place at CERN, it is felt that they play a very important role in captivating people sufficiently to fully explore the simulation. Running the LEDs by a computer with a sound blaster would be a convenient way to achieve such results.