# Proposal to prevent Fixed Frequency Triggers from reaching FE in case CTP does not react to ROD-BUSY feedback

Version 1.0, Martin Postranecky 22/12/05

## Current state

- When TIM-3C detects "fixed frequency" triggers above ~15 kHz

( presetable ), it asserts ROD-BUSY feedback to CTP for 1 msec

( presetable ) after 10x ( presetable ) same frequency triggers

( +/- 1kHz, presetable )

- It then expects CTP to stop sending the same frequency triggers

for this 1 msec ( presetable ) period

- The TIM-3C "Status Register" ( address 0x0C ) Bit 7 is set to

'1' when ROD-BUSY is asserted

- The TIM-3C "Busy Status3 Register" ( address 0x5A ) Bit 10

is also set to '1' when ROD-BUSY is asserted

- The TIM-3C "Busy Status3 Register" ( address 0x5A ) Bit 14

is set to '1' when FFTV is asserted

- No bits in TIM-3C "RODBusy Status Register" ( address 0x20 )

are set to '1' when only FFTV is asserted

- No bits in TIM-3C "RODBusy Latch Register" ( address 0x22 )

are set to '1' when only FFTV is asserted

- No bits in TIM-3C "RODBusy Monitor Register" ( address 0x24 )

are set to '1' when only FFTV is asserted

## Proposal for Local Emergency Action by TIM-3C if this doesn't happen :

- If, after a short interval ( in the order of one/few usec - TO BE

DETERMINED - intended to allow for any processing on LTP and CTP

and for any transmission delays ), after the ROD-BUSY feedback to CTP

has been asserted, TIM-3C detects another trigger within the 1 msec

'FFTV busy' period, it will then prevent any/all further triggers

from reaching the RODs in its crate, for the same 1 msec ( presetable )

period

- It will, at the same time, assert the TTC<7> ( currently 'SPARE' )

line to all RODs to alert them of this unilateral/local trigger veto

- it will set the TIM-3C "Busy Status3 Register" ( address 0x5A )

Bit 13 (?) to '1' when this unilateral/local trigger veto is

operational

- it will de-assert the "TIM-OK" backplane signal ( which is sent by TIM

to all BOCs & RODs in the crate, and indicates - in RunMode - that the

TTC clock is selected and 'good' and that triggers are being received

from TTC )

NOTE 1 : The RUN CONTROL should be checking the TIM-3C "Busy Status3

-------- Register" ( address 0x5A ) Bit 14 and issue warning if this

has been asserted more often than once per run / per day.

NOTE 2 : The RUN CONTROL should be checking the TIM-3C "Busy Status3

-------- Register" ( address 0x5A ) Bit 13 (?) and issue warning if

this has been asserted at any time during a run.

NOTE 3 : This "Local Emergency Action" will happen at any time the

-------- TIM-3C is set to 'Run Mode' and receiving triggers via the

TTCrq opto-link, ie. it will operate with both CTP-issued

triggers and with local LTP-issued triggers

NOTE 4 : If this "Local Emergency Action" is asserted, the trigger L1ID

-------- numbers on all local RODs will get out of sync. from ATLAS-SCT

L1IDs

QUESTION 1 : Should this "Local Emergency Action" by any one TIM-3C

------------ in any one crate be signalled to all SCT-ROD crates ( HOW ? )

in case some TIMs do not detect same number of FF triggers