Inner Detector ROD Crate VME Problem

July 14, 2004    John Joseph, John Hill and Richard Jared

Problem:

The host VME interface to the ROD crate fails in program I/O or DMA access.

The standard crate test configuration of the DMA/Programmed IO failure is shown below:

VME slot 1: 1 GHz or 800MHz Concurrent Technology CPU

VME slot 2: Nation Instruments VME controller

VME slot 3: V Metro bus analyzer

VME slot 4: blank

VME slot 5 to 12: RODs

VME slot 13: TIM

VME slot 14 to 21: RODs

Failure occurs in slots 5, 6, 7, 8 and sometimes 9 depending on the speed of the SBC in use.  The faster SBC causes failures in the higher number slots. Reducing the number of RODs in the crate also reduces the failure rate. A low number of RODs in the crate eliminates the failures. For example if RODs are loaded into slot 5, 6 and 7 only, or in slots 10 up to 21, the SBC does not fail on any VME access. The Cypress VME interface chips on the ROD have loading of 10pf per input on inputs or transceiver line. Handshake (control) signal traces are short (some data and address lines are longer, about 4 inches). Removing the National Instruments VME controller and/or V Metro bus analyzer affects the error rate.  The implication is that loading of the back plane with cards affects the microstructure of the failures. No data or address errors have been detected.

Testing of the VME bus activity with the V Metro bus analyzer indicates correct operation. Observation of the handshake signals with an oscilloscope shows correct operation.  The rising edges of the handshake signals show ringing. The number of cards loading the crate affects the ringing. The exact magnitude of the ringing could not be observed due to available locations of grounds on the backplane.

DMA (Block Write) Failure Description:

After some number of writes, sigma large, of 1024 word transfers the Universe chip on the CCT SBC host processor stops VME access to the crate.  Exception 13536 is issued and the CPU returns to a prompt.  VME signal address sync remains low (incorrect state).  The internals of the CPU work.  If a VME access is issued at this time, the SBC will freeze and a power cycle is required for recovery to occur.  Resetting the CPU does not correct the problem or return the CPU to a normal operating mode.  Block reads do not cause this problem. Nation Instrument DMA was not used for comparison because National Instrument in their literature states that they will not work with a fast PC driving the interface.

Programmed IO Failure Description:

After some number of writes, sigma large, of 1024 word transfers, the Universe chip on the CCT SBC host processor stops VME access to the crate.  No exception is issued, the PCI activity light is off, and the internals of the CPU do not work. Resetting the CPU does not correct the problem or return the CPU to a normal operating mode.  Removing the ROD that was the recipient of the failed access does not return the CPU to a normal operating mode. A power cycle of the crater is required for recovery to occur. Programmed IO reads do not cause this problem. Block writes/reads with the Nation Instruments VME controller operated in programmed IO do not cause this problem.

Proposed Correction:  (see background below)

Change the VME interface chips to low capacity (~5pf) versions).

This change is to reduce the noise (ringing) on the VME bus that may be affecting the Universe chip. This is roughly equivalent to reducing the number of RODs in the crate to eliminate the errors.

Change the VME interface chips on the handshake lines (AS*, DS0*, DS1* and WRITE*) to low capacity Schmitt Triggers.

This change provides clean signals to the ROD slave controller.

Change the Cypress VME chip to a FPGA implementation supporting A32 D32 data cycles, DMA and interrupt generation (Currently RODs only support these operations.).

This change will provide maximum flexibility to filter signal and provide detailed control of the VME handshake and resultant data cycles. The FPGA interface can be upgraded if needs are required by ATLAS.

Background:

The ROD was tested for VME block transfer and programmed IO operation with a full crate of 16 RODs for one week as part of the initial check out.  Unfortunately these same tests as used later at Cambridge did not find the error for reasons unknown. A Crate of RODs was sent to Cambridge in April for extensive testing as part of the system test. A problem was detected in the VME transactions in the lower slots (mainly 5 or 6) of the crate.

Since the problem could not be duplicated at LBL with the few RODs available, 10 were returned to LBL to find a solution to the problem.  The problems above were found to be observable. Universe and Concurrent Technology were contacted by e-mail to see if they could shed some light on the errors. Universe replied that it could not be the cause of the problem. Concurrent technology did not reply. Careful evaluation of the bus showed that the back plane has noise (ringing). The observed timing of the handshake signal showed no defect. It was first attempted to program the Cypress VME chip to wait long times to let the bus settle.  This affected the frequency of the failure but did not correct it.  The second attempt was to filter the input to the control lines AS*, DS0*, DS1* and WRITE*.  The cypress bus interface chips have 10 pf of capacitance. Resistors of 100 then 330 ohm resistor were put in series with the 4 primary interface lines (time constant of 1 or 3.3 ns).  Ether value of resistor eliminated the problem. Noise was substantially reduced (for 300 ohm resistors) and scope traces showed the expected VME time distribution. A bus analyzer was then used to monitor the transaction.  No defects were detected.   Extensive DMA and programmed IO testing was performed with no errors.   It seamed that the problem was corrected.  The RODs were then sent back to Cambridge for more testing.

Cambridge still observed the improper operation of DMA and programmed IO transaction.  Since the problem did not show in the LBL crate, John Joseph went to Cambridge to understand/correct the errors.  The first attempt was to increase the filtering (resistor valued were changed from 330 to 1.3k ohms) on the 4 control lines to 13 ns this decreased the errors rate but did not correct the problem. The second attempt was to place Schmitt Triggers on 3 of the 4 control lines this reduced the error rate (1.5 million block transfers) but did not eliminate the errors. It is obvious that the Cambridge system is more sensitive to the VME errors. The errors were as described above.  It became clear that the combination of Cypress 960 with the concurrent Technology processor would not work with a crate of RODs.  A few RODs in the crate always worked. Sixteen ROD showed the error in slot 5 and 6 mainly dependent on bus loading. Failures occurred in slots up to ten with higher frequency CPU ( 1 GHz). It is noted, the relatively slower National Instruments card as the host always worked. A Mustard card (PLL VME interface) in slot 3 could be read and written to with no errors.  The conclusion if that the VME bus is too noisy with the 10 pf load per slot for operation with the Cypress VME slave controller, Concurrent Technology processor and backplane. It is not known what the exact nature of the failure is.

Our solution  is to change the one thing under our control the Cypress 960 chip set.  We will replace the IO driver and receivers with low capacitance chips and use an existing FPGA on the ROD to perform the VME interface function.  This will provide maximum flexibility in achieving correct VME functionality.