SCT Grounding + Shielding

Wednesday 5th December 2001

RAL

Present

Joern Grosse-Knetter, Stephen Haywood, Tim Jones, Tony Smith, Ned Spencer, Jason Tarrant

Present for some of time

Debbie Greenfield, John Matheson, Val O’Shea

# System Test Results – Joern Grosse-Knetter

## Pickup Studies, using cooling unit as noise source

The cooling unit pickup is described at:

http://asct186.home.cern.ch/asct186/barrel_web/check_shield_2001-10.html

Important conclusion: the barrel shielding configuration can reject the cooling unit noise, but only if the shields, foils, and plates are very well connected.

Showed

- Input Noise

- Noise Occupancy – pickup from air-conditioning

Able to see and fix effect of bad shielding connections from LM Tapes connections.

Common causes for bad connections which cause noise pick-up:

- Ageing/badly stuck copper tapes

- Oxidised Al surfaces

The connection between PP1 and the unit shield (sector) is very important. PP1 provides a commoning point between cables, and the unit shield needs a low impedance tie to PP1 for this to be effective.  So the repair of this connection would understandably produce better pickup shielding.

When the experimenters reinstated the shield connection on a bundle of six tapes between PP1-PP2, they gained the second pickup improvement. The cable shield foils are clearly needed, and reliable connection is essential.

How well does the system test sector match the real arrangement in the detector? There is a plate underneath – shorting out end-plates, in parallel with Thermal Shield – this is required to match impedances if you don’t have a full barrel. Ned believes it is about right for these results to be useful. The barrel sector system test needs to be rebuilt if it is to usefully emulate the final system pickup properties.

Tim: How can we check quality of connections ? Pass largish current (1A) through inside enclosure, as well as connection to thermal shield, and measure DC voltage changes. The connection resistances should not dominate the voltage drop contributions.

## Pickup Studies using applied current in thermal shield

Conclusion: When the shielding elements are connected with the best conductivity, and a 100 mA 5 MHz current forced through the thermal shield, the modules show very little pickup. In runs with looser, higher resistance shield connections, the configuration without shunt shields and with a tie to the cooling tubes rejects the pickup better.

The results are at:

http://asct186.home.cern.ch/asct186/barrel_web/comp_shield_2001-10.html

Two basic sets of measurements made:

- Default: almost Ned’s

- Option: almost Tony’s

Use set-up where all problems have been fixed.

Little difference seen for

- Noise from 3-point noise scan

- Peaks in noise Occupancy

The only big difference which is seen occurs when noise is deliberately injected at the same time as there exists a poor connection to Thermal Shield – shorted Shunt-shield is better. However this is just one example and must explore other situations. Further, the set-ups differed from the complete proposals of Ned and Tony and for Ned’s scheme, some significant connections were missing for the connections representing those off the Barrel, i.e. between the Thermal Shield and PPF1.

# End-cap Discussions

Look from outside to inside.

What follows is a logical rather than temporal ordering of the discussion.

## General Points

### Joining Techniques

Good techniques for joining aluminium plates and foils need to be developed. The best system will be had from shielding connections being low resistance, particularly over time. Chromate conversion or Alochrome prevents oxide build-up on aluminium. Alochrome, chromate conversion, is actually described as a semiconductor, so it can make poorer conductivity joints than shiny aluminium.

Will Alochrome foils work well? The task is to create a good Faraday cage using the foils, bulkheads and trays available. The biggest challenge in using this material efficiently is the connection surfaces between the metal parts, particularly the aluminium.

When used on flanges with an EMI metallic gasket, like an elastomeric with wire mesh, Alochrome is punctured by the metal points, and direct metal contact occurs. The chromate conversion keeps these contact points from deteriorating from corrosion. So Alochrome on abrasive gasketed aluminium joints can give the best long-term performance.

What we do not know is how conductive Alochrome surfaces in light contact are. (Ned: actually I do not see Alochromed foil shields in very many applications on the web.)

### Material Budget

Throughout, there were discussions about the thickness and amount of material:

- Watch X0: there is the obvious material and then bits needed to support/fasten.

- Ensure satisfactory mechanical performance

- Ensure satisfactory electrical performance

Ned is concerned that the aluminium material, particularly foils and plates, be joined very conductively. Adding thickness for improved shielding is secondary in importance.

At various times in the meeting, Stephen asked for justification of various material thicknesses since we will have to justify X0 to our collaborators. Tony said it was impossible. At high frequency, once one goes beyond skin-depth transmission, relatively little is gained from thicker foils. Ned agreed that we should not mindlessly add material.

If foils exert too much stress on structures, rather than being fixed, they could be Kapton-backed and allowed to float on surfaces with loose fixings.

## PPF1 to Shield

Ned strongly recommends that we ensure good connectivity between PPF1 and the Shield. Explanation: common the cables at the shield, not inside the shield. The best place to do this is PP1.

One possibility is a 0.5 mm thick aluminium Cable-tray from PPF1 to Shield. The main purpose of the tray is to make the bundle of 62 tapes thermally neutral. The tray is ~70mm  ~15mm in cross-section.

Trays should be isolated from the Squirrel Cage, since the SCT End-cap is an isolated system, with only a single-point earth tie. The TRT owns the Squirrel Cage electrically and SCT would have only this single-point tie for each End-cap in common with TRT.

We would need 4 cable-trays per quadrant. Foils and Trays should be tightly connected at point of contact with the thermal shield. The trays will be continuous electrically with conventional cable shields on other side of PPF1, using PPF1 as the path.

Heater wires will go in the same trays. This implies that the heater power will be thoroughly isolated: it will have its earth reference through the unit shield single-point tie.

Jason showed the Tape routing: at Disc, there are 120-way connectors; but use 240-way at PPF1 to save space. This requires some crossing – however, this cross-over is not practical, especially with Cable-trays.

Alternatively, we could consider a Heat-spreader as in the Barrel. Could have a foil/plate – 100um would suffice for shielding purposes – but for heat spreading, may need more.

Tim is worried about cooling of Tapes – this will be done with C6F14 (mono-phase, used for Thermal Enclosures). We think this is Andy Nichols’ responsibility. Some associated questions:

- How much material is need to provide thermal neutrality of the cable array?

- Who is responsible for strategy of cooling/heating?

- What are the temperature and performance specifications?

- How do we incorporate good shielding design co-ordination into the mechanical design?

Jason sketched out how the outer Aluminium skin of the Outer Thermal Enclosure, OTE, could connect over the top of the annular ring of the foam Feed-through (see below) to the incoming cables from PPF1 and round to the back of the Rear Wing.

Tape questions:

- How will tapes actually be wrapped?

- How will they be bent – especially as a large bundle?

- Will tapes be cut to exactly right length? How will we cope with uncertainties – x cm too short is no good!

The proposal for the layout between the Barrel and PPB1 is:

- Covering sheet of Al-Kapton over Tapes

- Insulation of cover facing TRT Barrel End-plate

- Conducting surface of cover contacting the Heat-spreader Plate

- Between these two, are the wrapped Tapes – Al of wrapping on outside

Tim is very worried about the space available at PPF1, in particular for active components, namely the capacitors.

## Service Feed-throughs at Thermal Shield

Feed-throughs are in the  “top-right” corner of End-cap, between OTE and Rear Wing. Consist of two annular rings of foam to seal services coming in, and sealed with RTB (must avoid cold gas leaks). The foam annulus is ~ 10cm in R to allow services to warm up on exiting the thermal enclosure. Concern has been expressed at FDR about acetic acid from RTB potentially damaging Silicon Detectors.

In order to shield all these big slots some conductive gasketing is needed that is also moderately gas-tight.

At http://www.hollandshielding.com/pdf/doc.pdf is an excellent catalogue of shielding product descriptions. Page 4 shows foam wrapped with reinforced aluminium foil, Amucor-shield 6800. The foil/foam assembly can have appropriate conductive adhesive, so that placement can be straightforward.

At above link, page 21 shows a shielded cable entry system. For round services, the foamed opening should be 3 times the tube diameter, which is too large a cavity in the bulkhead. The foam needs to be more carefully shaped to get the height reduced from 3 diameter of the service.

Conductive filled RTV silicone caulk using silver powder is available from Chomerics. They also have silver plated aluminium powder filler. This does not look attractive, because of high material and activation of silver.

This service entry bulkhead assembly needs very high RF conductivity. The various pieces of aluminium could be gasketed together with a product like Amucor-shield (page 4) in a groove. The aluminium-jacketed foam can be as small as 1mm2mm.

On page 12 is Flatshield 5700, aluminium wires embedded in a silicone sheet. Instead of a groove, this sheet would clamp between two aluminium pieces to provide a gas seal with electrical shielding.

## Thermal Shield Foils Acting as Faraday Cage

### General Points

Tim asked if he can remove material from foils, for example foil with 50% holes – Ned is not keen on holes.

Tim asked whether we can use Heaters as a shield. Tony said they are not complete foil – this is needed for low impedance. Also difficult to tie edges together.

Jason explained that the foils on surfaces of enclosures have four functions:

- Spread heat

- Even-out heat distribution which would otherwise result from different performance of heaters

- Faraday cage

- High conductivity path to common out ground loops on power cables and services

The Shields which are closer to where Services are, namely on the OTE and Rear Wing, need to be thicker than those which are further from Services since these foils are the commoning path to shunt the service ground loops.

### Inner Thermal Enclosure (ITE)

As a baseline, Ned would like at least a 25um (current design is 50um) skin which is not perforated - any extra metal helps shielding, although this is the most sensitive region for X0 (low R and large grazing angle). Ned says watch out for EM fields from beam – 400 mA at 40 MHz (we are most sensitive to 5 MHz). Probably not too much of a problem, but plenty of uncertainties. We need to get more information to estimate whether we have a good design for beam pipe pickup rejection.

There may be two skins on CF cylinder – needed to reduce diffusion. Thinner foil need only be few um. Two skins will provide better shielding then the same material in one skin.

### Rear Wing

Ned would prefer that End-plate foil should be continuous sheet. This would be much closer to a Faraday cage, and would optimally use the material that is attenuating the physics by being there.

Tim asks about using the foil on the inner surface of Rear Wing. Connections from inner Shield of Rear Wing need to pass through gaps in Support Cylinder (these exist for pipes) to make electrical connections to Trays. Jason reluctant to extend foil from inside up into feed-through region because it will cool in a region where trying to warm cables to avoid condensation problems. Ned proposes that could take strips out at level of 25% of circumference, and with a spacing of a few cm, this would be sufficient for electrical purposes.

### Outer Thermal Enclosure (OTE)

- Inner foil for spreading heat – this is the most useful electrically.

- Outer foil spreads heat and holds heaters down.

- Pitch of pipes on inner foil is 2/56.

Current design has 50 um on the outer skin – Ned would prefer 75 um.

How does one electrically connect the 4 quadrants of the OTE outer foil?

The quadrants of the OTE are supported by thin ribs separated in z. They connect the quadrants – the gap is filled with a strip of insulation and gunk. Ned expressed some concern about this gap: the shielding will not be continuous across the gap – although the quadrants could be connected by straps.

At http://www.hollandshielding.com/pdf/doc.pdf, page 6 is Ultra Soft Shield 7400, which is a very low closure force EMI gasket. Also on page 14 is their sketch of V-shape gaskets which have more compression range than a foam gasket. Make these joints really conductive and we are actually using material efficiently that has to be there anyway.

### Front Wing

A foil backed by a Kapton sheet will seal the complete front surface across the Wing and the Infill.

## Connections between Shields

Tony is worried about damage to foils, especially tabs used for making connections – these could be made thicker or given a Kapton backing. The foil joints should either have a tight crimp or foam compression.

### ITE to Rear Wing

Between Rear Wing and ITE, propose to use an electrically-conducting L-shaped clamp to connect foils. Cannot use flange - otherwise cannot insert Wheels.

If the clamp is foam gasketed, then it would be removable with no compromise of conductivity.

### Rear Wing to OTE

Principle shield is on outside of Wing, although one should common all conducting surfaces. Can connect all 4 foils (2 inner, 2 outer) from OTE and Rear Wing at the same location inside Feed-through. This is not so straightforward at 3 and 9 O’clock due to wings. This is the most important junction in the shielding, since the ground loops in all the services are shorted here. To create a good system, we need to be meticulous in optimising conductivity between these 4 foils.

### OTE to Front Wing

The outer surfaces will be connected with straps. A continuous Ultra Soft Shield7400, page 7, gasketed flange would be much better than straps.

It is less clear what to do for inner surface of Wing since don’t want to allow heat paths in region where need to keep cool. Can connect inner skin and annular Heat-spreader, but then these would not be connected to outside parts surfaces. This is an important challenge: connecting these skins makes a better system.

### Front Wing to ITE

Jason has a nice scheme for connection of shields by wrapping outer foil around inner radius and bolting the structures together with a flange so that foil makes contact with inner surfaces.

There could be a small problem during maintenance caused by a small cold region, unprotected by heaters. Could this area be jacketed with foam during maintenance?

## Connection to Earth

We need to designate the external point for the End-cap single-earth tie. The Cryostat would be ideal – but it is difficult to make a solid connection to this – we cannot drill into it. A low impedance conductor should exist between this earth point and the End-cap. A tape 2-3 cm braid would be better than a cable since it has smaller inductance. (Although one could consider using the cooling pipes, it is advantageous to have a single connection which can then be tested.)

Tony asked how we handle fault currents? Lightening strikes should not be an issue, but there are power supplies around with large currents.

## Inside the Shield

### Support Cylinder

What we are trying to achieve is not so much shielding as commoning – to ensure that conductive surfaces do not float but are tied to a common potential. What is particularly important here is to provide low impedance path in region of Tapes to avoid pick-up.

In current thinking, Tapes are not wrapped once they enter the Faraday cage.

Ned proposed a 50 um Al skin around Cylinder - this is not great due to CTE mismatch with CF skin. Tony proposed a 50um foil runner under each group of 3 tapes. So get 450um at end (X0!) – and what about packing factor ?

A simpler solution would be to have a foil over the top of the Tapes which is connected to the conductors at the Feed-through. A practical connection to each respective PPF0 on the Discs is needs to be designed.

How is Cylinder referenced ? The two skins should be electrically contacted – but it is not obvious how well this will work due to the directionality and finite length of fibres.

### Discs

The power supply placement and more importantly the power cable routing poses the biggest G&S challenge. What is very bad about the Discs electrically is the fact that the left and right halves are powered by cables taking very different paths- either side of UX cavern! This arises from requirement to space out routing geometrically to fill space evenly. There is no room for a crossover so that a disk has a compact cable bundle.

This is the motivation for commoning at PP1, avoiding putting the ground loops across the detectors.

Tony would like to cover the complete Disc with 1 cm foil rings at the position of the Module support Blocks and the PPF0’s might be a satisfactory compromise.

#### Shield Associated with Disk 1

There should be a shield to limit capacitance between Modules and Faraday cage foil, unit shield foil,  on Front Wing.

Tim proposed this could be supported from the Module supports associated with the lower Modules and held ~3 mm from the upper Modules. It only needs to cover the 12 cms of the outer Modules, and Tony though a mesh would suffice.

#### Shield Associated with Disk 9

This would be similar to the one for Disk 1. This shield may not be necessary given the distance between modules and rear foil. This should be tested if the extra material is of concern.

### On Disk

It is understood that we would go with Tony’s proposal of 1 cm annuli of Al foil associated with the mounting points at the primary blocks as well as near the patch Panels.

### Making Connections between Conductors

Jason Worried about joining Al foils. Ned doesn’t like idea of Silver Epoxy (also activation) unreliable connection. The Barrel (Nickerson) believe that if the surfaces are Alochromed, then it is sufficient to clamp them together.

Important connections need good EMI gaskets. Alochrome is discussed above, in Section 2.1. The effectiveness of all these foils is limited by the quality of the aluminium joints.

# Follow-up

AN	Questions about cooling of Cables outside the Shields.

JT	Consider stresses in Support Cylinder due to 50 um foil.

JT	Measure conductivity of samples of CF skins, as used in Support Cylinder.

???	Define cartoons which will show what we have agreed, by Xmas.

Stephen Haywood (10/12/01)

Feedback on Draft Minutes from: JG-K, NS