|  | ATLAS SCT – End-cap Grounding and Shielding |  |  |
| --- | --- | --- | --- |
| ATLAS Project Document No: | Institute Document No. | Created : 29/10/01 21:05 | Page: 1 of 7 |
| ATL-IS-EN-XXXX |  | Modified : 30/10/01 | Rev. No .: A |

| Engineering Note ATLAS SCT – End-cap Grounding and Shielding This document analyses the available documentation concerning the grounding and shielding for the ATLAS SCT end-cap and attempts to identify how the proposed schemes can be practically implemented within the context of the end-cap engineering design presented at the end-cap structures FDR. |  |  |
| --- | --- | --- |
| Prepared by: Tim Jones, Liverpool | Checked by: | Approved by: |
| Distribution List ATLAS SCT End-cap Engineering Working Group |  |  |

| History of Changes |  |  |  |
| --- | --- | --- | --- |
| Rev. No. | Date | Pages | Description of changes |
| A | 29.10.01 |  | First draft |

Table of Contents

0	Scope	4

1	Grounding and Shielding Resources	4

Ref	4

2	Comparison of Schemes and Impact on Engineering Design	4

2.1	Module Interface	5

2.2	Disks	5

2.3	PPF0	6

2.4	Support Cylinder	6

2.5	Thermal Enclosures and Feed-throughs	7

3	Conclusions	7

# Scope

This document aims to compare and contrast the ATLAS SCT end-cap grounding and shielding schemes proposed by Ned Spencer and Tony Smith and to assess the possible impact of their implementation on the design of the end-cap structures (support cylinder, disks and thermal enclosures).

These two proposals have been developed from an electrical engineers view-point and they are both vague as to the precise methods are to be implemented. From an electrical point-of-view both schemes have some merit. The final choice of scheme will depend on what is discovered during the multi-module ‘system-tests’ that are planned as part of the end-cap module development programme. Delays in this activity mean that it will not be possible to make an informed decision on the two schemes for six to twelve months.

This note concentrates on the implications of the two schemes on the main structural components (disks, support cylinder and thermal enclosures) which are being reviewed. The two schemes make definite proposals on the implementation of grounding and shielding for SCT end-cap beyond their exit from the thermal enclosure – these are NOT considered as part of this note. Section 1 summarises the resources available that the initiators of the two schemes have made available. Section 2 compares the two schemes starting at the module and then considering the disk, PPF0, the support cylinder and power tapes and  the thermal enclosure.

# Grounding and Shielding Resources

This section summarises the resources available from Ned Spencer (NS) and Tony Smith (TS) concerning their SCT end-cap grounding and shielding  proposals.

| Ref | Document Title | Date & Author | Location |
| --- | --- | --- | --- |
| 1 | ATLAS SCT/Pixel Grounding and Shielding Note | 22/11/99 Ned Spencer | http://scipp.ucsc.edu/groups/atlas/elect-doc/SCT_GND_SHIELD.pdf |
| 2 | Responses to Tony Smith Proposal for Forward Grounding/Shielding | 31/08/01 Ned Spencer | e-mail |
| 3 | Forward Grounding & Shielding | 6/09/01 N.S. + SJH ? | e-mail |
| 4 | End-cap Grounding and Shielding Design | 1/10/01 NS | e-mail |
| 5 | Grounding and Shielding for the ATLAS SCT end-cap | 12/04/01 Tony Smith | EDMS |
| 6 | End-cap grounding Schematic | 4/10/01 Tony Smith | e-mail |

The subsequent sections are based on a detailed study of the proposals summarised in 4, 5 and 6.

# Comparison of Schemes and Impact on Engineering Design

This section compares the two possible grounding and shielding implementations working out from the module and considering each component in turn.

## Module Interface

TS wants to make a DC electrical connection from module ground to the cooling block. This is implemented by a small Cu/kapton flex attached to the module. The copper plating is soldered to module ground and the connection to the mounting block/cooling structure is made using an electrically conducting mounting nut/washer/location pin/mounting block. [See (5) – Proposed feature – 1].

NS wants to implement the shunt-shield. An insulator/conductor/insulator sandwich is incorporated into the module construction. The conductor layer is electrically connected to module ground. There must be no electrical contact between the mounting block and any part of the module. Any force spreading washer used as part of the module mounting components must either be an electrical insulator or must bear down on an insulator (e.g. the ‘precision washer” ?).

## Disks

TS makes several explicit proposals concerning the disks. NS makes no proposals but seems to acknowledge that TS’s proposals are reasonable.

TS explicitly proposes to equip the disk with a 50m thick, 10mm wide annular foil for each ring at the radius of the main cooling block. The cooling blocks are electrically connected to the foil. [See (5) – Proposed feature 3].

TS proposed to make an electrical connection to the disk at each cooling block location [See (5) – Proposed feature 2]. However in the later drawing [6] this is not the case. Instead there is an annular foil ring at the radius of the PPF0 which is electrically connected at 8 points to the disk skins.

The annular foils are to be electrically connected with radial foil strips to each of PPF0 patch panel card [See (5) – Proposed feature – 4].

Based on TS’s proposals the following may be a possible physical implementation.

- The annular and radial foils are manufactured in sections using the same process as for the Ljubljana low mass tapes. The ends need copper plating. The connection between the annular and the radial sections is made by soldering a similar bridging foil on top.

- The annular foils are sandwiched between the cooling blocks and their associated gluon. The electrical connection between the foil and the block is made using silver loaded epoxy

- The radial foils run from each ring to each PPF0 foil ring. Foils from the front side need to pass to the PPF0-side.

- Close to the PPF0 ring there need to be 8 locations on the PPF0-side of the disk to which an electrical contact can be made. This contact needs to be passed to the front side. A small diameter hole is drilled through the disk and a close-fitting wire is inserted and held in place using small blobs of silver loaded epoxy.

Both TS and NS agree that disk 1 is a special case. They both propose a foil (50m – [see [4]) covering the active area located between the disk and the front TE. They differ in how it should be connected to disk 1. NS proposes to connect them at the inner radius at 8 points, TS proposes that the connection is made at the outer radius but does not believe that the connection point is a big issue. In fact the best place to couple this skin to is the cooling blocks themselves.

TS makes no explicit reference to disk 9 whilst NS does. NS proposes that the actively cooled heat spreader on the inside of the rear TE is electrically connected to the annular foils on disk 9 at 8 equi-spaced points at the inner radius. There should be no electrical contact to other conductors. There may be complications concerning electrical breaks in the pipes for the active cooling. It may be more convenient to attach an extra Al/kapton foil to the inside of the thermal enclosure to perform this function.

## PPF0

On the disk side PPF0 is a single PCB with up to 9 wiggly tapes terminating in a connector on the side closest to the disk and 3 120-way board-to-board connectors mounted on the side further from the disk. The incoming power tape bundles contain up to 3 modules and terminate via a solder connection to a board that has the mating half of the 120-way board-to-board connector on the opposite side.

TS proposes that the incoming 3-module tape bundles are enclosed by a foil screen and that this is connected via PPF0 to the PPF0 annular foil on the disk. There is no (explicit mention of) a foil screen on the support cylinder. [See (5) Proposed feature 5 and (6)]. However he does propose an electrical contact between the support cylinder skin and the PPF0 annular ring on the disk.

NS proposes that the support cylinder has a 75 m foil mounted on but electrically isolated from the outer skin. This replaces the shielding wraps on the tape bundles within the end-cap. He proposes an electrical connection between power tape DGND and the support tube foil.

For TS’s proposal the following may be a possible implementation.

- Terminate the screen of the 3-tape bundle in two short 10 mm wide Al/kapton tape.

- Connect one of these tapes to a ZIF connector mounted on a metal foil that is in electrical contact with the outer skin of the support cylinder close to the aperture.

- Equip the disk based PPF0 PCB with a ZIF connector corresponding to the second Al/Kapton tape and connect this with a plane on the PCB to a point close to the PPF0 annular foil ring.

- Connect the PPF0 PCB to the PPF0 annular foil with a 1cm wide Al/kapton tape terminating in a ZIF connector soldered to the PPF0 annular foil.

And for NS’s scheme –

- A 1 cm wide Al/Kapton tape is electrically connected to the outer cylinder foil close to the PPF0 aperture.

- Equip the disk based PPF0 PCB with a ZIF connector corresponding to the Al/kapton tape and connect this with a plane to each of the three groups of (one or more contiguous) DGND pins and the PPF0 annular foil contact point as in (2) above.

- Connect the PPF0 PCB to the PPF0 annular foil with a 1cm wide Al/kapton  tape terminating in a ZIF connector soldered to the PPF0 annular foil.

The differences between the TS and NS proposals can be resolved in the precise design of the PPF0 PCB. NS proposes that there are capacitors on some of the lines to ground. The space envelope for the PCB needs to be checked with respect to the capacitor sizes.

## Support Cylinder

Both TS and NS make specific reference to what happens to the services as the pass along the outer support cylinder. In general they both seem to aim to electrically connect all the electrically conducting material as well as possible.

The big difference is that whilst TS proposes that the 3-tape bundles are wrapped in foil screens, NS dispenses with these are replaces them with a foil screen mounted on but electrically isolated from the outer skin of the support cylinder.

However whilst TS proposes connecting the cooling tubes, tape-bundle screens and heat spreader plates [See (5) – Proposed Features 6] together “as well as possible” he does not say how this should be done. One may envisage a series of foil bands running around the support cylinder close to each set of disk service apertures. These may then need connecting together length-wise. Depending on the width (in z) of these bands there is a clear tendency towards a complete foil screen. TS agrees that a complete foil screen is probably OK.

Provision for a complete foil screen on the outer surface of the support cylinder should be made. The possible impact on CTE needs to be estimated.

## Thermal Enclosures and Feed-throughs

TS and NS propose that all the tape bundle screens are electrically connected to the thermal enclosure at the feedthroughs [See (5) Proposed Feature 7 and (4) “The Rear Thermal …”]. TS seems to assume that each 3 module tape bundle has its own screen up to PPF1 [See (6)] whilst NS proposes to wrap up to 62 modules in a single screen [See (4)].

TS proposes to electrically connect the metallic parts of the outer TE to the PPF0 annular foil on the disks. [See (6)] This seems impossible to me ! NS makes no explicit mention of any requirements for the grounding / shielding of the outer TE.

TS proposes that at the point where the cooling pipes and power tapes diverge there is an electrical break in the cooling pipe.

# Conclusions

The design of the structural components of the SCT end-cap mechanics is decoupled to a great extent from the differing requirements of grounding and shielding. The main areas needing consideration are:-

- Provision of electrical contact points (8) on disks.

- Provision of foil screen on outside of support cylinder.

In addition there are several other areas that influence the design of the disk services and PPF0:-

- Design of annluar foils, joining techniques and attachment of annular foils on disks between cooling blocks and their associated support pads.

- Design on PPF0 on-disk PCB and the connection to the annular foils.

- Design of contact points on outside of support cylinder for connection to PPF0.

- Assembly methods.