Off Detector Electronics Status  (January 29, 2001)

Read Out Drivers (ROD)

Two RODs have been fabricated and are under test.

The currents test are with booting and read/write to various location.

Booting works part of the time. It appears the DSP is the cause of the

boot problem.

The  VME interface can read/write to the program manager (governs the boot of

the ROD) FPGA.

The VME interface can read/write to the resource interface (the control function

of the ROD)  FPGA.

The VME interface can read/write via the DSP host port to internal program

memory, FLASH memory and the resource interface FPGA.

The DSP program can write to internal program memory, internal data memory,

SDRAM and FLASH memory and the resource interface FPGA.

Problems exist in the writing (read works) from VME via the DSP host port to

internal data memory and external SDRAM.

Preliminary test of the writing to the other FPGAs (VME to DSP host port

to resource interface FPGA to other FPGAs) of the ROD works

(normal set up and control path).

Timing Interface Module  (TIM)

The TIM has the necessary functionality debugged for the start of the Cambridge test.

Renewed debugging will commence 30 days before   additional functionality is needed at Cambridge.

Back Of Crate (BOC) (optical interface)

The VME  card (RIG) to read and write the BOC bus is functional.

Testing has started on the BOC bus.

The supply of optical components for the test phase is in hand and facilities to test the optical devices are available.