July 19, 1999

Draft, Draft, Draft

Interface Specification

ROD Back Plane and Crate Card Utilization

The back planes provide the interconnection between the ROD Crate Controller (RCC), Timing Interface Module (TIM) , Read Out Driver (ROD)and the Back Of the Crate (BOC) (optical interface card) and the power supplies.  This interface specification defines the assignment of signal and power to the back plane pins. The card utilization of the crate is also defined because this effects the clock and busy signals.

The signals that must be connected to the ROD BOC and TIM are listed below:

Note. All signal are 3V levels unless indicated other wise.

ROD  BOC TIM

XC-0 to XC-47    Module control streams      48  48   0

RD-0 to R95     Link data streams            96  96   0

BOC set up buss                              20  20   0

SD-0 to SD-7  8 Data lines

SA-0 to SA-9  10 Address lines

WRN           Read/write (write = low)

# STBN          Strobe (Active low)  STBN

S-link                                       43  43   0

CNTRL/DATA         Data mode selection

SLD-0 toSLD-32     32 bit data word

SL-CLK             Slink in data clock

SL-WRITE-EN        Write data word

X-OFF              Link full (ROD must

stop in 2 words)

SL-RESET           Reset s-link

SL-Test            Test pattern generation

SL-UD-0 to SL-UD-3 TBD

SL-BAD             Link is down

ROD busy             1 pin on ROD             1  0    16

16 pins on TIM

Clk40+ and Clk40-    System Clock (Diff PECL) 2  2    32

2 pins on ROD and BOC

and 32 pins on TIM

TTC-0 to TCC 7       Trigger control bus      7  0     7

Total Signal PINS   ROD BOC TIM

217 209  48

The power voltages are listed below:

In standard    User defined

VME   VMEx

+12V           J1/P1 pins           1               0

-12V           J1/P1 pins           1               0

+5V  6 pins    J1/P1 pins           3               0

J2/P2 pins           3               0

J3/P3 pins           0               0

+3.3V  10 pins J1/P1 pins           0               0

J2/P2 pins           0    10         0

J3/P3 pins           0               0

Ground 46 pins J1/P1 pins           7    18         0

J2/P2 pins           4    17         8

J3/P3 pins           0               26

Pin diode bias (25V min)  J3/P3     0     0         1

Pin diode ground          J3/P3     0     0         1

Total power pins(19), standard grounds pins(46)and user defined ground pins 36.  Total count of potential signal return pins (power and ground pins) is 101 pins.

Note need to write justification for the number of ground pins.

Note. There are 272 pins available for our definition J2 and J3.

Note. The current number of unassigned pins is 19.

P1/J1 Connector for TIM, ROD and Crate Processor Slots

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | Std Sig | Std Sig | Std Sig | Std Sig | Std VPC |
| 2 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std Gnd |
| 3 | Std Sig | Std Sig | Std Sig | Std Sig | ## Std +V1 |
| 4 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +V2 |
| 5 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 6 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std –V1 |
| 7 | Std Sig | Std Sig | Std Sig | Std Sig | ## Std –V2 |
| 8 | ## Std Gnd | Std Sig | Std Sig | Std Sig | Std Sig |
| 9 | Std Sig | Std Gnd | Std Sig | Std Gnd | Std Sig |
| 10 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std GA0 |
| 11 | Std Sig | Std Sig | Std Sig | Std Sig | ## Std GA1 |
| 12 | ## Std Gnd | Std Sig | Std Gnd | Std Sig | ## Std +3.3V |
| 13 | Std Sig | Std Sig | ## Std +5 | Std Sig | ## Std GA2 |
| 14 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3.3V |
| 15 | Std Sig | ## Std Gnd | Std Sig | Std Sig | ## Std GA3 |
| 16 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3.3V |
| 17 | Std Sig | Std Gnd | Std Sig | Std Sig | # Std GA4 |
| 18 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3.3V |
| 19 | Std Sig | ## Std Gnd | Std Sig | Std Sig | Std Sig |
| 20 | # Std Gnd | Std Sig | ## Std Gnd | Std Sig | ## Std +3.3V |
| 21 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 22 | ## Std Gnd | Std Sig | ## Std Gnd | Std Sig | ## Std +3.3V |
| 23 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 24 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3.3V |
| 25 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 26 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3.3V |
| 27 | # Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 28 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3.3V |
| 29 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 30 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3.3V |
| 31 | Std Sig | ## Std –12V | ## Std +5VS | ## STD +12V | Std Gnd-1 |
| 32 | ## Std Gnd | ## Std +5 | ## Std +5 | ## Std +5 | Std VPC |

P2/J2 Connector ROD and BOC slots

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | XC-0 | XC-16 | ## Std +5V | SD-0 | ## GND |
| 2 | Std Gnd | XC-17 | ## Std Gnd | SD-1 | SLD_0 |
| 3 | XC-1 | XC-18 | Std Sig | SD-2 | SLD_1 |
| 4 | Std Gnd | XC-19 | Std Sig | SD-3 | SLD_2 |
| 5 | XC-2 | XC-20 | Std Sig | SD-4 | SLD_3 |
| 6 | Std Gnd | XC-21 | Std Sig | SD-5 | SLD_4 |
| 7 | XC-3 | XC-22 | Std Sig | SD-6 | SLD_5 |
| 8 | Std Gnd | XC-23 | Std Sig | SD-7 | SLD_6 |
| 9 | XC-4 | XC-24 | Std Sig | ## Gnd | SLD_7 |
| 10 | Std Gnd | XC-25 | Std Sig | # SA-0 | ## Gnd |
| 11 | XC-5 | XC-26 | Std Sig | SA-1 | SLD_8 |
| 12 | Std Gnd | XC-27 | ## Std Gnd | SA-2 | SLD_9 |
| 13 | XC-6 | XC-28 | ## Std +5V | SA-3 | SLD_10 |
| 14 | Std Gnd | XC-29 | Std Sig | SA-4 | SLD_11 |
| 15 | XC-7 | XC-30 | Std Sig | SA-5 | SLD_12 |
| 16 | Std Gnd | # XC-31 | Std Sig | SA-6 | ## Gnd |
| 17 | XC-8 | XC-32 | Std Sig | SA-7 | SLD_13 |
| 18 | Std Gnd | XC-33 | Std Sig | SA-8 | SLD_14 |
| 19 | XC-9 | XC-34 | Std Sig | SA-9 | SLD_15 |
| 20 | Std Gnd | XC-35 | Std Sig | ## Gnd | SLD_16 |
| 21 | XC-10 | XC-36 | Std Sig | WRN | SLD_17 |
| 22 | Std Gnd | XC-37 | ## Std Gnd | STBN | SLD_18 |
| 23 | XC-11 | XC-38 | Std Sig |  | SLD_19 |
| 24 | Std Gnd | XC-39 | Std Sig |  | SLD_20 |
| 25 | XC-12 | XC-40 | Std Sig |  | ## Gnd |
| 26 | ## Std Gnd | XC-41 | Std Sig |  | SLD_21 |
| 27 | XC-13 | XC-42 | Std Sig | ## Gnd | SLD_22 |
| 28 | ## Std Gnd | XC-43 | Std Sig | SLD_31 | SLD_23 |
| 29 | XC-14 | XC-44 | Std Sig | SLD_30 | SLD_24 |
| 30 | ## Std Gnd | XC-45 | Std Sig | SLD_29 | SLD_25 |
| 31 | XC-15 | XC-46 | ## Std Gnd | SLD_28 | SLD_26 |
| 32 | ## Std Gnd | XC-47 | ## Std +5V | SLD_27 | ## Gnd |

P3/J3 Connector BOC and ROD

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | ## Gnd | RD-27 | RD-53 | Gnd |  |
| 2 | RD-0 | Gnd | RD-54 | RD-80 |  |
| 3 | RD-1 | RD-28 | RD-55 | RD-81 |  |
| 4 | RD-2 | RD-29 | RD-56 | RD-82 |  |
| 5 | RD-3 | RD-30 | Gnd | RD-83 |  |
| 6 | RD-4 | RD-31 | RD-57 | RD-84 |  |
| 7 | RD-5 | RD-32 | RD-58 | Gnd |  |
| 8 | Gnd | Gnd | RD-59 | RD-85 |  |
| 9 | RD-6 | RD-33 | RD-60 | RD-86 |  |
| 10 | RD-7 | RD-34 | RD-61 | RD-87 |  |
| 11 | RD-8 | RD-35 | Gnd | RD-88 |  |
| 12 | RD-9 | RD-36 | RD-62 | RD-89 |  |
| 13 | RD-10 | RD-37 | RD-63 | Gnd |  |
| 14 | RD-11 | Gnd | RD-64 | RD-9 |  |
| 15 | Gnd | RD-38 | RD-65 | RD-91 | PD-Bias |
| 16 | RD-12 | RD-39 | RD-66 | RD-92 | PD-Gnd |
| 17 | RD-13 | RD-40 | Gnd | RD-93 | ## Gnd |
| 18 | RD-14 | RD-41 | RD-67 | RD-94 | CLK-40+ |
| 19 | RD-15 | RD-42 | RD-68 | Gnd | CLK-40- |
| 20 | RD-16 | Gnd | RD-69 | RD-95 | ## Gnd |
| 21 | Gnd | RD-43 | RD-70 |  | SL-BAD |
| 22 | RD-17 | RD-44 | RD-71 | ROD-Busy | SL-UD3 |
| 23 | RD-18 | RD-45 | Gnd | ## Gnd | Sl-UD2 |
| 24 | RD-19 | RD-46 | RD-72 | TTC-7 | SL-UD1 |
| 25 | RD-20 | RD-47 | RD-73 | TTC-6 | SL-UD0 |
| 26 | RD-21 | Gnd | RD-74 | TTC-5 | ## Gnd |
| 27 | Gnd | RD-48 | RD-75 | TTC-4 | SL_TEST |
| 28 | RD-22 | RD-49 | RD-76 | TTC-3 | SL-RESET |
| 29 | RD-23 | RD-50 | Gnd | TTC-2 | X-OFF |
| 30 | RD-24 | RD-51 | RD-77 | TTC-1 | SL-W-EN |
| 31 | RD-25 | RD-52 | RD-78 | TTC-0 | SL-CLK |
| 32 | RD-26 | Gnd | RD-79 | ## Gnd | ## Gnd |

Note. TCC-0 to TCC-7 and ROD busy are not used on the BOC

P2/J2 Connector for TIM and Crate Processor

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | ## Std +5V |  |  |
| 2 | Std Gnd |  | ## Std Gnd |  |  |
| 3 |  |  | Std Sig |  |  |
| 4 | Std Gnd |  | Std Sig |  |  |
| 5 |  |  | Std Sig |  |  |
| 6 | Std Gnd |  | Std Sig |  |  |
| 7 |  |  | Std Sig |  |  |
| 8 | Std Gnd |  | Std Sig |  |  |
| 9 |  |  | Std Sig |  |  |
| 10 | Std Gnd |  | Std Sig |  |  |
| 11 |  |  | Std Sig |  |  |
| 12 | Std Gnd |  | ## Std Gnd |  |  |
| 13 |  |  | ## Std +5V |  |  |
| 14 | Std Gnd |  | Std Sig |  |  |
| 15 |  |  | Std Sig |  |  |
| 16 | Std Gnd |  | Std Sig |  |  |
| 17 |  |  | Std Sig |  |  |
| 18 | Std Gnd |  | Std Sig |  |  |
| 19 |  |  | Std Sig |  |  |
| 20 | Std Gnd |  | Std Sig |  |  |
| 21 |  |  | Std Sig |  |  |
| 22 | Std Gnd |  | ## Std Gnd |  |  |
| 23 |  |  | Std Sig |  |  |
| 24 | Std Gnd |  | Std Sig |  |  |
| 25 |  |  | Std Sig |  |  |
| 26 | ## Std Gnd |  | Std Sig |  |  |
| 27 |  |  | Std Sig |  |  |
| 28 | ## Std Gnd |  | Std Sig |  |  |
| 29 |  |  | Std Sig |  |  |
| 30 | ## Std Gnd |  | Std Sig |  |  |
| 31 |  |  | ## Std Gnd |  |  |
| 32 | ## Std Gnd |  | ## Std +5V |  |  |

P3/J3 Connector TIM slot

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | ### Gnd |  | ROD-Busy6 |  |  |
| 2 | CLK-40+ slot21 | ## Gnd |  |  |  |
| 3 | CLK-40- slot21 | CLK-40+ slot8 |  |  |  |
| 4 | CLK-40+ slot20 | CLK-40- slot8 |  |  |  |
| 5 | CLK-40- slot20 | CLK-40+ slot7 |  |  |  |
| 6 | CLK-40+ slot19 | CLK-40- slot7 |  |  |  |
| 7 | CLK-40- slot19 |  |  |  |  |
| 8 | Gnd |  |  |  |  |
| 9 | CLK-40+ slot18 | ## Gnd |  |  |  |
| 10 | CLK-40- slot18 | CLK-40+ slot6 |  |  |  |
| 11 | CLK-40+ slot17 | CLK-40- slot6 |  |  |  |
| 12 | CLK-40- slot17 |  |  |  |  |
| 13 | CLK-40+ slot16 |  |  |  |  |
| 14 | CLK-40- slot16 | ## Gnd |  |  |  |
| 15 | Gnd | ROD-Busy21 |  |  |  |
| 16 | CLK-40+ slot15 | ROD-Busy20 |  |  |  |
| 17 | CLK-40- slot15 | ROD-Busy19 |  |  |  |
| 18 | CLK-40+ slot14 | ROD-Busy18 |  |  |  |
| 19 | CLK-40- slot14 | ROD-Busy17 |  |  |  |
| 20 |  | # Gnd |  |  |  |
| 21 | Gnd | ROD-Busy16 |  |  |  |
| 22 | CLK-40+ slot13 | ROD-Busy15 |  |  |  |
| 23 | CLK-40- slot13 | ROD-Busy14 |  |  |  |
| 24 | CLK-40+ slot12 | ROD-Busy13 |  | ## Gnd |  |
| 25 | CLK-40- slot12 | ROD-Busy12 |  | TTC-7 |  |
| 26 | Gnd | ## Gnd |  | TTC-6 |  |
| 27 | CLK-40+ slot11 | ROD-Busy11 |  | TTC-5 |  |
| 28 | CLK-40- slot11 | ROD-Busy10 |  | TTC-4 |  |
| 29 | CLK-40+ slot10 | ROD-Busy9 |  | TTC-3 |  |
| 30 | CLK-40- slot10 | ROD-Busy8 |  | TTC-1 |  |
| 31 | CLK-40+ slot9 | ROD-Busy7 |  | TTC-0 |  |
| 32 | CLK-40- slot9 | ## Gnd |  | ## Gnd |  |

Note.  The ground pins have been selected so that no damage will occur if the card in put in any crate slot.

Note.  The signal pin selection will cause damage to the TIM of BOC. There are two solutions available.

The first is to have 72 links. Then there can be no driver outputs connected together when a TIM is place in the incorrect slot.

The second is to key the TIM so that it can not be plugged into a ROD slot.

The crate has 16 slots for the RODs, 1 slot for the TIM, I slot for the 6U to 9U divider and 2 slots for the crate processor.  The table below shows the assignments.

| Slot Number | Item in slot |
| --- | --- |
| 1 | Crate processor |
| 2 | Reserved for crate processor use |
| 3 | TBD |
| 4 | Divider for 6U to 9U card sizes |
| 5 | TIM |
| 6 | ROD and BOC |
| 7 | ROD and BOC |
| 8 | ROD and BOC |
| 9 | ROD and BOC |
| 10 | ROD and BOC |
| 11 | ROD and BOC |
| 12 | ROD and BOC |
| 13 | ROD and BOC |
| 14 | ROD and BOC |
| 15 | ROD and BOC |
| 16 | ROD and BOC |
| 17 | ROD and BOC |
| 18 | ROD and BOC |
| 19 | ROD and BOC |
| 20 | ROD and BOC |
| 21 | ROD and BOC |