Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization

| Change Record |  |  |
| --- | --- | --- |
| Date | by | Change |
| 03-May-2000 | MJG | Limited distribution as discussion document prior to phone conference on 05-May |
| 09-May-2000 | MJG | First general release: +3v3 and +5V available on J3P3, slot compatible power and ground |
| 10-May-2000 | MJG | Card sizes, currents, and numbers added at end. Signal classes added to signal summary on Page 3 |
| 16-May-2000 | RCJ | Added sign off page, 1 bit buss, deleted P0 reference and changed ROD DC power |
| 19-May_2000 | RCJ | Added correction from J. Lane. Note TCC and ROD busy active Lo. P5-6 TCC bus terminated on slot 21. P3 row Z changed 3v3 to +5V pins 19-24-29. Updated TIM power for +-12V |
| 22-May-2000 | RCJ | Added Crate power to power utilization table, Suggestion of J. Hill |
| 22-May-2000 | RCJ | 1 Bussed added to table 9 A21 ROD and BOC slots 6-21, corrected typo on –12V power |
| 23-Jun-2000 | RCJ | Changed the location of the TIM in the crate to slot 13. The following changes were made. Added TTC 0-7 B bus and changed name of TTC 0-7 to TTC 0-7 A on the TIM. Changed clock pin number on TIM. Changed name of signal on C21 TIM to bussed and changed ROD busy slot 14 to slot 5 on TIM. |

## Sign off

| Name | Subsystem | Date |
| --- | --- | --- |
| Roy Wastie | Backplane | 16-May-2000 |
| John Lane | TIM | 19-May-2000 |
| Maurice Goodrick | BOC | 18-May-2000 |
| Richard Jared | ROD | 17-May-2000 |
| John Hill | SCT RCC | 10-May-2000 |
| Tom Meyer | Pixel RCC | 19-May-2000 |

## Introduction

The back planes provide the interconnection between the ROD Crate Controller (RCC), Timing Interface Module (TIM), Read Out Driver (ROD), the Back Of the Crate (BOC) (optical interface card) and the power supplies.  This interface specification defines the assignment of signal and power to the backplane pins. The card utilization of the crate is also defined because this effects the clock and busy signals.

More information on the off-detector electronics can be found at:

http://www-wisconsin.cern.ch/~atlas/off-detector/off-detector.html

General notes:

- This document is based on an earlier, much modified document, the latest version of which can be found at http://www.hep.phy.cam.ac.uk/~goodrick/AtBOC/OldBackplane.pdf

- All signals are 3V levels, asserted high, unless indicated otherwise. The S-Link signals use a final “#” to indicate active low. Elsewhere, a final “N” is used for the same purpose. In addition, TTC[7:0] and ROD-Busy are active LOW.

- S-Link UDW[1:0] lines hard wired on BOC. S-Link LRL[3:0] lines not routed.

- To limit congestion, a maximum of 4 routed signals have been allocated per horizontal row on slots 6-21 of P3J3.

- The use of series resistors on the outputs of drivers to limit noise generation is recommended for at least some S-Link signals (L-UCLK, L-UWEN#, L-UCTRL#), and for the XC[47:0] (on the ROD) and RD[95:0] (on BOC).

- TIM, ROD and BOC do not support live insertion.

- Power pins are used extensively to screen fast signals and to provide good AC return paths; so it is important that they be well by-passed to ground on both ROD and BOC.

- Tables 8-12 re-present the data given in Tables 5-7 to allow pin usage at different slots to be compared.

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization – Tables and Conventions

| Table | Description |
| --- | --- |
| 1 | P1 Connector as used by TIM, Crate Processor and RODs @ Slots 1-21 |
| 2 | P2/J2 Connector as used by TIM and Crate Processor @ Slots 1-5 |
| 3 | P2/J2 Connector as used by ROD @ Slots 6-21 |
| 4 | P2/J2 Connector as used by BOC @ Slots 6-21 |
| 5 | P3/J3 Connector as used by TIM @ Slot 5 |
| 6 | P3/J3 Connector as used by ROD.. plugs into Slots 6-21 |
| 7 | P3/J3 as used by BOC .. plugs into transition (i.e. Back-Of-Crate) slots 6-21 |
| 8 | P3/J3 Connectors: use of Row Z |
| 9 | P3/J3 Connectors: use of Row A |
| 10 | P3/J3 Connectors: use of Row B |
| 11 | P3/J3 Connectors: use of Row C |
| 12 | P3/J3 Connectors: use of Row D |
| Key to Colours used in Tables for Signal Names |  |
| NC | No Connection |
| Thru Pin | Through Connection: not bussed |
| RED | Signal Output |
| GREEN | Signal Input |
| BROWN | Bi-Directional |
| BLACK | Ground/Power |
| [n] | See Note [n] |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization – Signal Summary

| Summary of Signals |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Signal Group/ Name |  | Conn. | Description | Direction | Class |
| Control | XC[47:0] | P2 | Transmit Data | ROD > BOC | LV,Hi,r |
| RX Data | RD[95:0] | P3 | Receive Data | ROD < BOC | LV,Hi,r |
| SetUp Bus | SD[7:0] | P2 | SetUpBus Data | ROD <> BOC | LV,Hi,Bi |
|  | SA[9:0] | P2 | SetUpBus Address | ROD > BOC | LV,Hi |
|  | SWRN | P2 | SetUpBus Write Line (Lo) | ROD > BOC | LV,Lo |
|  | STBN | P2 | SetUpBus Strobe (Lo) | ROD > BOC | LV,Lo |
|  | SBUSY | P2 | SetUpBus Busy status line | ROD < BOC | LV,Hi |
| S-Link | L-UD[31:0] | P2 | Link User Data | ROD > S-Link | LV,Hi,r |
|  | L-URESET# | P3 | Link User Reset | ROD > S-Link | LV,Lo,r |
|  | L-UWEN# | P3 | Link Write Enable | ROD > S-Link | LV,Lo,r |
|  | L-UTEST# | P3 | Link Test mode control line | ROD > S-Link | LV,Lo,r |
|  | L-UCLK | P3 | Link User Clock | ROD > S-Link | LV,Hi,r |
|  | L-UCTRL# | P3 | Link User Control | ROD > S-Link | LV,Lo,r |
|  | L-LDOWN# | P3 | Link Failure line (Lo) | ROD < S-Link | LV,Lo,r |
|  | L-LFF# | P3 | Link Full Flag (Lo) | ROD < S-Link | LV,Lo,r |
| Clock Distrib | CLK40+[21:5] | P3 | Balanced PECL Clock to BOC slots (+ copy to TIM) | TIM > BOCs | D5vP,Hi |
|  | CLK40-[21:5] | P3 |  | TIM > BOCs | D5vP,Lo |
|  | RCLK40+ | P3 | Buffered copy of CLK40 for ROD | BOC > ROD | D5vP,Hi |
|  | RCLK40- | P3 |  | BOC > ROD | D5vP,Lo |
| TIM Signals | TTC[7:0] | P3 | Timing, Trigger and Control Bus | TIM > RODs | LV,Lo,T |
|  | ROD-Busy | P3 | ROD asking for trigger throttle | RODs > TIM | LV,Lo,1 |
|  | TIM-Busy Out | P3 | OR of ROD-Busy’s | Goes nowhere | LV,Hi |
| Misc | ROD-sense | P3 | Module Mis-Location Interlock | Special | 2 |
|  | Laser-Ilock | P3 | Laser-Access Gate Open on >=1 BOC | BOC > | 3 |

|  | Class Descriptions |
| --- | --- |
| Class | Description |
| LV | Driver meets or exceeds LV-TTL spec., Receiver can manage with LV-TTL inputs |
| D5vP | Differential PECL: i.e. ECL running between Ground and +5V. Note 220R resistor to Ground at Driver end: needs 100R between lines at receiver. |
| Hi | Active HIGH signal sense |
| Lo | Active LOW signal sense |
| Bi | Bi-Directional |
| T | Terminated on Backplane at slot 21 |
| r | Consideration should be given to having a series resistor (of c. 30 Ω) at driver end to limit slew rate, ground bounce and EMI. |
| 1 | Resistive pull-up on TIM, so empty ROD slot is not seen as BUSY. |
| 2 | Pin sensed by TIM and BOC: if HIGH, they disable their outputs. Pulled High by TIM and BOC, grounded by ROD and on backplane at slot 5. |
| 3 | Electrical specification for this bussed line is still under consideration. |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization – Power and Ground Summary

| Summary of Power and Ground Pins |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Connector | # Pins | Description | Notes |
| Std Gnd | P1 | 26 | Backplane Plane | Not accessible to BOC |
| Std +3v3 | P1 | 10 | Backplane Plane | Not accessible to BOC |
| Std +5V | P1 | 3 | Backplane Plane | Not accessible to BOC |
| Std VPC | P1 | 2 | Connected to +5V | Not accessible to BOC |
| Std +5VS | P1 | 1 | +5v Stand-By conn. to +5V | Not accessible to BOC |
| Std +12V | P1 | 1 |  | Not accessible to BOC |
| Std -12V | P1 | 1 |  | Not accessible to BOC |
| Std +V1 | P1 | 1 | Ret. For –V1 | Not accessible to BOC |
| Std +V2 | P1 | 1 | Ret. For –V2 | Not accessible to BOC |
| Std -V1 | P1 | 1 | Nom. 48V | Not accessible to BOC |
| Std -V2 | P1 | 1 | Nom. 48V | Not accessible to BOC |
| Std Gnd | P2 | 20 | Backplane Plane |  |
| Std +5V | P2 | 3 | Backplane Plane |  |
| Std VPC | P2 | 1 | Connected to +5V |  |
| Std Gnd (ret) | P2 | 1 | Return for VPC: conn to Gnd plane |  |
| Gnd | P2 | 7 | User-Defined (Thru Pins) |  |
| Gnd | P3 | 22 | Backplane Plane |  |
| +3v3 | P3 | 10 | Backplane Plane | BOC and ROD use |
| +5V | P3 | 8 | Backplane Plane |  |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P1

Table 1: P1 Connector as used by TIM, Crate Processor and RODs @ Slots 1-21

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | Std Sig | Std Sig | Std Sig | Std Sig | ###### Std VPC |
| 2 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std Gnd |
| 3 | Std Sig | Std Sig | Std Sig | Std Sig | ## Std +V1 |
| 4 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +V2 |
| 5 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 6 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std -V1 |
| 7 | Std Sig | Std Sig | Std Sig | Std Sig | ## Std -V2 |
| 8 | ## Std Gnd | Std Sig | Std Sig | Std Sig | Std Sig |
| 9 | Std Sig | Std Gnd | Std Sig | ## Std Gnd | Std Sig |
| 10 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std GA0 |
| 11 | Std Sig | ## Std Gnd | Std Sig | Std Sig | ## Std GA1 |
| 12 | ## Std Gnd | Std Sig | # Std Sig | Std Sig | ## Std +3v3 |
| 13 | Std Sig | Std Sig | ## Std Sig | Std Sig | ## Std GA2 |
| 14 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3v3 |
| 15 | Std Sig | ## Std Gnd | Std Sig | Std Sig | ## Std GA3 |
| 16 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3v3 |
| 17 | Std Sig | ## Std Gnd | Std Sig | Std Sig | # Std GA4 |
| 18 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3v3 |
| 19 | Std Sig | ## Std Gnd | Std Sig | Std Sig | Std Sig |
| 20 | # Std Gnd | Std Sig | ## Std Gnd | Std Sig | ## Std +3v3 |
| 21 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 22 | ## Std Gnd | Std Sig | ## Std Sig | Std Sig | ## Std +3v3 |
| 23 | Std Sig | Std Sig | Std Gnd | Std Sig | Std Sig |
| 24 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3v3 |
| 25 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 26 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3v3 |
| 27 | # Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 28 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3v3 |
| 29 | Std Sig | Std Sig | Std Sig | Std Sig | Std Sig |
| 30 | ## Std Gnd | Std Sig | Std Sig | Std Sig | ## Std +3v3 |
| 31 | Std Sig | ## Std -12V | ## Std +5VS | ## Std +12V | ## Std Gnd |
| 32 | ## Std Gnd | ## Std +5V | ## Std +5V | ## Std +5V | ###### Std VPC |

Note that there are no connections to the Back-Of-Crate for P1

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P2/J2

Table 2: P2/J2 Connector as used by TIM and Crate Processor @ Slots 1-5

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | ## Std +5V |  | ## Gnd |
| 2 | Std Gnd |  | ## Std Gnd |  |  |
| 3 |  |  | Std Sig |  |  |
| 4 | Std Gnd |  | Std Sig |  |  |
| 5 |  |  | Std Sig |  |  |
| 6 | Std Gnd |  | Std Sig |  |  |
| 7 |  |  | Std Sig |  |  |
| 8 | Std Gnd |  | Std Sig |  |  |
| 9 |  |  | Std Sig | ## Gnd |  |
| 10 | Std Gnd |  | Std Sig |  | ## Gnd |
| 11 |  |  | Std Sig |  |  |
| 12 | Std Gnd |  | ## Std Gnd |  |  |
| 13 |  |  | ## Std +5V |  |  |
| 14 | Std Gnd |  | Std Sig |  |  |
| 15 |  |  | Std Sig |  |  |
| 16 | Std Gnd |  | Std Sig |  | ## Gnd |
| 17 |  |  | Std Sig |  |  |
| 18 | Std Gnd |  | Std Sig |  |  |
| 19 |  |  | Std Sig |  |  |
| 20 | Std Gnd |  | Std Sig | ## Gnd |  |
| 21 |  |  | Std Sig |  |  |
| 22 | Std Gnd |  | ## Std Gnd |  |  |
| 23 |  |  | Std Sig |  |  |
| 24 | Std Gnd |  | Std Sig |  |  |
| 25 |  |  | Std Sig |  | ## Gnd |
| 26 | ## Std Gnd |  | Std Sig |  |  |
| 27 |  |  | Std Sig | ## Gnd |  |
| 28 | ## Std Gnd |  | Std Sig |  |  |
| 29 |  |  | Std Sig |  |  |
| 30 | ## Std Gnd |  | Std Sig |  |  |
| 31 |  |  | ## Std Gnd |  | Std Gnd (VPC ret) |
| 32 | ## Std Gnd |  | ## Std +5V |  | ## Std VPC |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P2/J2

Table 3: P2/J2 Connector as used by ROD @ Slots 6-21

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | XC46 | XC47 | ## Std +5V | SD0 | ## Gnd |
| 2 | Std Gnd | XC45 | ## Std Gnd | SD1 | L-UD0 |
| 3 | XC43 | XC44 | Std Sig | SD2 | L-UD1 |
| 4 | Std Gnd | XC42 | Std Sig | SD3 | L-UD2 |
| 5 | XC40 | XC41 | Std Sig | SD4 | L-UD3 |
| 6 | Std Gnd | XC39 | Std Sig | SD5 | L-UD4 |
| 7 | XC37 | XC38 | Std Sig | SD6 | L-UD5 |
| 8 | Std Gnd | XC36 | Std Sig | SD7 | L-UD6 |
| 9 | XC34 | XC35 | Std Sig | ## Gnd | L-UD7 |
| 10 | Std Gnd | XC33 | Std Sig | # SA0 | ## Gnd |
| 11 | XC31 | XC32 | Std Sig | SA1 | L-UD8 |
| 12 | Std Gnd | XC30 | ## Std Gnd | SA2 | L-UD9 |
| 13 | XC28 | XC29 | ## Std +5V | SA3 | L-UD10 |
| 14 | Std Gnd | XC27 | Std Sig | SA4 | L-UD11 |
| 15 | XC25 | XC26 | Std Sig | SA5 | L-UD12 |
| 16 | Std Gnd | # XC24 | Std Sig | SA6 | ## Gnd |
| 17 | XC22 | XC23 | Std Sig | SA7 | L-UD13 |
| 18 | Std Gnd | XC21 | Std Sig | SA8 | L-UD14 |
| 19 | XC19 | XC20 | Std Sig | SA9 | L-UD15 |
| 20 | Std Gnd | XC18 | Std Sig | ## Gnd | L-UD16 |
| 21 | XC16 | XC17 | Std Sig | SWRN | L-UD17 |
| 22 | Std Gnd | XC15 | ## Std Gnd | STBN | L-UD18 |
| 23 | XC13 | XC14 | Std Sig | SBUSY | L-UD19 |
| 24 | Std Gnd | XC12 | Std Sig |  | L-UD20 |
| 25 | XC10 | XC11 | Std Sig |  | ## Gnd |
| 26 | ## Std Gnd | XC9 | Std Sig | L-UD22 | L-UD21 |
| 27 | XC7 | XC8 | Std Sig | ## Gnd | L-UD23 |
| 28 | ## Std Gnd | XC6 | Std Sig | L-UD25 | L-UD24 |
| 29 | XC4 | XC5 | Std Sig | L-UD27 | L-UD26 |
| 30 | ## Std Gnd | XC3 | Std Sig | L-UD29 | L-UD28 |
| 31 | XC1 | XC2 | ## Std Gnd | L-UD30 | Std Gnd (VPC ret) |
| 32 | ## Std Gnd | XC0 | ## Std +5V | L-UD31 | ## Std VPC |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P2/J2

Table 4: P2/J2 Connector as used by BOC @ Slots 6-21

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | XC46 | XC47 | ## Std +5V | SD0 | ## Gnd |
| 2 | Std Gnd | XC45 | ## Std Gnd | SD1 | L-UD0 |
| 3 | XC43 | XC44 | Std Sig | SD2 | L-UD1 |
| 4 | Std Gnd | XC42 | Std Sig | SD3 | L-UD2 |
| 5 | XC40 | XC41 | Std Sig | SD4 | L-UD3 |
| 6 | Std Gnd | XC39 | Std Sig | SD5 | L-UD4 |
| 7 | XC37 | XC38 | Std Sig | SD6 | L-UD5 |
| 8 | Std Gnd | XC36 | Std Sig | SD7 | L-UD6 |
| 9 | XC34 | XC35 | Std Sig | ## Gnd | L-UD7 |
| 10 | Std Gnd | XC33 | Std Sig | # SA0 | ## Gnd |
| 11 | XC31 | XC32 | Std Sig | SA1 | L-UD8 |
| 12 | Std Gnd | XC30 | ## Std Gnd | SA2 | L-UD9 |
| 13 | XC28 | XC29 | ## Std +5V | SA3 | L-UD10 |
| 14 | Std Gnd | XC27 | Std Sig | SA4 | L-UD11 |
| 15 | XC25 | XC26 | Std Sig | SA5 | L-UD12 |
| 16 | Std Gnd | # XC24 | Std Sig | SA6 | ## Gnd |
| 17 | XC22 | XC23 | Std Sig | SA7 | L-UD13 |
| 18 | Std Gnd | XC21 | Std Sig | SA8 | L-UD14 |
| 19 | XC19 | XC20 | Std Sig | SA9 | L-UD15 |
| 20 | Std Gnd | XC18 | Std Sig | ## Gnd | L-UD16 |
| 21 | XC16 | XC17 | Std Sig | SWRN | L-UD17 |
| 22 | Std Gnd | XC15 | ## Std Gnd | STBN | L-UD18 |
| 23 | XC13 | XC14 | Std Sig | SBUSY | L-UD19 |
| 24 | Std Gnd | XC12 | Std Sig |  | L-UD20 |
| 25 | XC10 | XC11 | Std Sig |  | ## Gnd |
| 26 | ## Std Gnd | XC9 | Std Sig | L-UD22 | L-UD21 |
| 27 | XC7 | XC8 | Std Sig | ## Gnd | L-UD23 |
| 28 | ## Std Gnd | XC6 | Std Sig | L-UD25 | L-UD24 |
| 29 | XC4 | XC5 | Std Sig | L-UD27 | L-UD26 |
| 30 | ## Std Gnd | XC3 | Std Sig | L-UD29 | L-UD28 |
| 31 | XC1 | XC2 | ## Std Gnd | L-UD30 | Std Gnd (VPC ret) |
| 32 | ## Std Gnd | XC0 | ## Std +5V | L-UD31 | ## Std VPC |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 5: P3/J3 Connector as used by TIM @ Slot 13

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | ## NC | ## Gnd |  | ## +3v3 | ROD-Busy5 1 |
| 2 | NC |  | ###### Gnd | # ROD-Busy6 | +3v3 |
| 3 | NC | CLK40+ slot5 3 | ## CLK40+ slot14 3 | # +3v3 | ROD-Busy7 |
| 4 | Gnd | CLK40- slot5 3 | CLK40- slot14 3 | ROD-Busy8 | +3v3 |
| 5 | NC | ###### Gnd |  | Gnd |  |
| 6 | NC | CLK40+ slot6 3 | CLK40+ slot15 3 | ROD-Busy9 | +3v3 |
| 7 | NC | ## CLK40- slot6 3 | ## CLK40- slot15 3 | +3v3 | ROD-Busy10 |
| 8 | NC | ###### CLK40+ slot7 3 | CLK40+ slot16 3 | +3v3 | +3v3 |
| 9 | Gnd |  | ## Gnd | ROD-Busy11 | +3v3 |
| 10 | NC | CLK40- slot7 3 | CLK40- slot16 3 | Gnd | +3v3 |
| 11 | NC | ###### Gnd |  | ROD-Busy12 | Laser-Ilock 4 |
| 12 | NC | CLK40+ slot8 3 | CLK40+ slot17 3 | ROD-Busy14 | Gnd |
| 13 | NC | ## CLK40- slot8 3 | ## CLK40- slot17 3 | ## ROD-Busy15 | ROD-sense |
| 14 | Gnd |  | ###### Gnd | ROD-Busy16 | ## ROD-Busy17 |
| 15 | NC | CLK40+ slot9 3 | ## CLK40+ slot18 3 | ###### Gnd | ROD-Busy18 |
| 16 | NC | CLK40- slot9 3 | CLK40- slot18 3 | ROD-Busy19 | ROD-Busy20 |
| 17 | NC | Gnd |  | ROD-Busy21 | ## +5V |
| 18 | NC | CLK40+ slot10 3 | CLK40+ slot19 3 |  | CLK40+ _Ret |
| 19 | +5V | CLK40- slot10 3 | ## CLK40- slot19 3 |  | CLK40- _Ret |
| 20 | NC | # Gnd |  | ###### Gnd | +5V |
| 21 | NC | CLK40+ slot11 3 | ## CLK40+ slot20 3 | Bussed | TIM-Busy Out 6 |
| 22 | NC | CLK40- slot11 3 | CLK40- slot20 3 |  | +5V |
| 23 | NC |  | ## Gnd | TTC-7 B | TTC-7 A |
| 24 | +5V | CLK40+ slot12 3 | CLK40+ slot21 3 | TTC-6 B | TTC-6 A |
| 25 | NC | CLK40- slot12 3 | ## CLK40- slot21 3 | ###### Gnd | TTC-5 A |
| 26 | NC | ## Gnd |  | TTC-5 B | TTC-4 A |
| 27 | NC | CLK40+ slot13 2 | ## CLK40- slot13 2 | TTC-4 B | +5V |
| 28 | NC |  | ###### Gnd | TTC-3 B | TTC-3 A |
| 29 | +5V |  |  | TTC-2 B | TTC-2 A |
| 30 | NC |  |  | Gnd | TTC-1 A |
| 31 | NC | ###### Gnd |  | TTC-1 B | TTC-0 A |
| 32 | NC |  |  | ## TTC-0 B | ## +5V |

Note: TTC-0 to TTC-7 outputs from TIM are bussed to ROD Slots 5-12 and 14-21

terminated on backplane at slot 5 and 21. They as active low.   TTC 0-7 B are signals for ROD slots 5-12; TTC 0-7 A are signals for ROD slots 14-21.

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 6: P3/J3 Connector as used by ROD.. plugs into Slots 5-12 and 14-21

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | ## RD94 | Gnd | RD95 | +3v3 | L-UWEN# |
| 2 | RD92 | RD93 | Gnd | L-URESET# | +3v3 |
| 3 | RD89 | RD90 | RD91 | +3v3 | L-UCLK |
| 4 | Gnd | RD87 | RD88 | L-UTEST# | +3v3 |
| 5 | RD85 | Gnd | RD86 | Gnd | L-UCTRL# |
| 6 | RD82 | RD83 | RD84 | L-LDOWN# | +3v3 |
| 7 | RD79 | RD80 | RD81 | +3v3 | L-LFF# |
| 8 | RD76 | RD77 | RD78 | +3v3 | +3v3 |
| 9 | Gnd | RD74 | Gnd | RD75 | +3v3 |
| 10 | RD71 | RD72 | RD73 | Gnd | +3v3 |
| 11 | RD68 | Gnd | RD69 | RD70 | Laser-Ilock |
| 12 | RD64 | RD65 | RD66 | RD67 | Gnd |
| 13 | RD60 | RD61 | RD62 | RD63 | ROD-sense |
| 14 | Gnd | RD58 | Gnd | RD59 |  |
| 15 | RD55 | RD56 | RD57 | Gnd | RCLK40+ |
| 16 | RD51 | RD52 | RD53 | RD54 | RCLK40- |
| 17 | RD48 | Gnd | RD49 | RD50 | ## +5V |
| 18 | RD44 | RD45 | RD46 | RD47 | NC |
| 19 | +5V | RD41 | RD42 | RD43 | NC |
| 20 | RD39 | Gnd | RD40 | Gnd | +5V |
| 21 | RD36 | Bussed | RD37 | RD38 | ROD-Busy |
| 22 | RD32 | RD33 | RD34 | RD35 | +5V |
| 23 | RD29 | RD30 | Gnd | RD31 | TTC-7 |
| 24 | +5V | RD26 | RD27 | RD28 | TTC-6 |
| 25 | RD23 | RD24 | RD25 | Gnd | TTC-5 |
| 26 | RD20 | Gnd | RD21 | RD22 | TTC-4 |
| 27 | RD16 | RD17 | RD18 | RD19 | +5V |
| 28 | RD13 | RD14 | Gnd | RD15 | TTC-3 |
| 29 | +5V | RD10 | RD11 | RD12 | TTC-2 |
| 30 | RD7 | RD8 | RD9 | Gnd | TTC-1 |
| 31 | RD4 | Gnd | RD5 | ###### RD6 | TTC-0 |
| 32 | RD0 | ###### RD1 | RD2 | ###### RD3 | ## +5V |

Note: TTC-0 to TTC-7 outputs from TIM are bussed to all RODs and

terminated on backplane at slot 21. They are active LOW.

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 7: P3/J3 as used by BOC .. plugs into transition (i.e. Back-Of-Crate) slots 5-12 and 14-21

| Pin | Row Z | Row A | Row B | Row C | Row D |
| --- | --- | --- | --- | --- | --- |
| 1 | ## RD94 | Gnd | RD95 | +3v3 | L-UWEN# |
| 2 | RD92 | RD93 | Gnd | L-URESET# | +3v3 |
| 3 | RD89 | RD90 | RD91 | +3v3 | L-UCLK |
| 4 | Gnd | RD87 | RD88 | L-UTEST# | +3v3 |
| 5 | RD85 | Gnd | RD86 | Gnd | L-UCTRL# |
| 6 | RD82 | RD83 | RD84 | L-LDOWN# | +3v3 |
| 7 | RD79 | RD80 | RD81 | +3v3 | L-LFF# |
| 8 | RD76 | RD77 | RD78 | +3v3 | +3v3 |
| 9 | Gnd | RD74 | Gnd | RD75 | +3v3 |
| 10 | RD71 | RD72 | RD73 | Gnd | +3v3 |
| 11 | RD68 | Gnd | RD69 | RD70 | Laser-Ilock |
| 12 | RD64 | RD65 | RD66 | RD67 | Gnd |
| 13 | RD60 | RD61 | RD62 | RD63 | ROD-sense |
| 14 | Gnd | RD58 | Gnd | RD59 |  |
| 15 | RD55 | RD56 | RD57 | Gnd | RCLK40+ |
| 16 | RD51 | RD52 | RD53 | RD54 | RCLK40- |
| 17 | RD48 | Gnd | RD49 | RD50 | ## +5V |
| 18 | RD44 | RD45 | RD46 | RD47 | CLK40+ |
| 19 | +5V | RD41 | RD42 | RD43 | CLK40- |
| 20 | RD39 | Gnd | RD40 | Gnd | +5V |
| 21 | RD36 | Bussed | RD37 | RD38 | NC |
| 22 | RD32 | RD33 | RD34 | RD35 | +5V |
| 23 | RD29 | RD30 | Gnd | RD31 | NC |
| 24 | +5V | RD26 | RD27 | RD28 | NC |
| 25 | RD23 | RD24 | RD25 | Gnd | NC |
| 26 | RD20 | Gnd | RD21 | RD22 | NC |
| 27 | RD16 | RD17 | RD18 | RD19 | +5V |
| 28 | RD13 | RD14 | Gnd | RD15 | NC |
| 29 | +5V | RD10 | RD11 | RD12 | NC |
| 30 | RD7 | RD8 | RD9 | Gnd | NC |
| 31 | RD4 | Gnd | RD5 | ###### RD6 | NC |
| 32 | RD0 | ###### RD1 | RD2 | ###### RD3 | ## +5V |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 8: P3/J3 Connectors: use of Row Z

| Pin | TIM @ Slot 13 | Backplane @ Slot 13 | Backplane @ Slots 5-12 and 14-21 | ROD @ Slots 5-12 and 14-21 | BOC @ rear of Slots 5-12 and 14-21 |
| --- | --- | --- | --- | --- | --- |
| 1 | ## NC | ## Thru Pin | ## Thru Pin | ## RD94 | ## RD94 |
| 2 | NC | ## Thru Pin | ## Thru Pin | RD92 | RD92 |
| 3 | NC | ## Thru Pin | ## Thru Pin | RD89 | RD89 |
| 4 | Gnd | Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 5 | NC | ## Thru Pin | ## Thru Pin | RD85 | RD85 |
| 6 | NC | ## Thru Pin | ## Thru Pin | RD82 | RD82 |
| 7 | NC | ## Thru Pin | ## Thru Pin | RD79 | RD79 |
| 8 | NC | ## Thru Pin | ## Thru Pin | RD76 | RD76 |
| 9 | Gnd | Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 10 | NC | ## Thru Pin | ## Thru Pin | RD71 | RD71 |
| 11 | NC | ## Thru Pin | ## Thru Pin | RD68 | RD68 |
| 12 | NC | ## Thru Pin | ## Thru Pin | RD64 | RD64 |
| 13 | NC | ## Thru Pin | ## Thru Pin | RD60 | RD60 |
| 14 | Gnd | Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 15 | NC | ## Thru Pin | ## Thru Pin | RD55 | RD55 |
| 16 | NC | ## Thru Pin | ## Thru Pin | RD51 | RD51 |
| 17 | NC | ## Thru Pin | ## Thru Pin | RD48 | RD48 |
| 18 | NC | ## Thru Pin | ## Thru Pin | RD44 | RD44 |
| 19 | +5V | +5V.. Plane | +5V.. Plane | +5V | +5V |
| 20 | NC | ## Thru Pin | ## Thru Pin | RD39 | RD39 |
| 21 | NC | ## Thru Pin | ## Thru Pin | RD36 | RD36 |
| 22 | NC | ## Thru Pin | ## Thru Pin | RD32 | RD32 |
| 23 | NC | ## Thru Pin | ## Thru Pin | RD29 | RD29 |
| 24 | +5V | +5V.. Plane | +5V .. Plane | +5V | +5V |
| 25 | NC | ## Thru Pin | ## Thru Pin | RD23 | RD23 |
| 26 | NC | ## Thru Pin | ## Thru Pin | RD20 | RD20 |
| 27 | NC | ## Thru Pin | ## Thru Pin | RD16 | RD16 |
| 28 | NC | ## Thru Pin | ## Thru Pin | RD13 | RD13 |
| 29 | +5V | +5V .. Plane | +5V.. Plane | +5V | +5V |
| 30 | NC | ## Thru Pin | ## Thru Pin | RD7 | RD7 |
| 31 | NC | ## Thru Pin | ## Thru Pin | RD4 | RD4 |
| 32 | NC | ## Thru Pin | ## Thru Pin | RD0 | RD0 |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 9: P3/J3 Connectors: use of Row A

| Pin | TIM @ Slot 13 | Backplane @ Slot 13 | Backplane @ Slots 5-12 and 14-21 | ROD @ Slots 5-12 and 14-21 | BOC @ rear of Slots 5-12 and 14-21 |
| --- | --- | --- | --- | --- | --- |
| 1 | ## Gnd | ## Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 2 |  |  | ## Thru Pin | RD93 | RD93 |
| 3 | CLK40+ slot 5 | ## To J3D18, slot 5 | ## Thru Pin | RD90 | RD90 |
| 4 | CLK40- slot 5 | To J3D18, slot 5 | ## Thru Pin | RD87 | RD87 |
| 5 | ###### Gnd | ## Gnd .. Plane | ## Gnd .. Plane | Gnd | Gnd |
| 6 | CLK40+ slot 6 | ## To J3D18, slot 6 | Thru Pin | RD83 | RD83 |
| 7 | ## CLK40- slot 6 | To J3D18, slot 6 | ## Thru Pin | RD80 | RD80 |
| 8 | ###### CLK40+ slot 7 | To J3D18, slot 7 | ## Thru Pin | RD77 | RD77 |
| 9 |  |  | ## Thru Pin | RD74 | RD74 |
| 10 | CLK40- slot 17 | To J3D18, slot 7 | ## Thru Pin | RD72 | RD72 |
| 11 | ###### Gnd | ## Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 12 | CLK40+ slot 8 | ## To J3D18, slot 8 | ## Thru Pin | RD65 | RD65 |
| 13 | ## CLK40- slot 8 | To J3D18, slot 8 | ## Thru Pin | RD61 | RD61 |
| 14 |  |  | ## Thru Pin | RD58 | RD58 |
| 15 | CLK40+ slot 9 | ## To J3D18, slot 9 | ## Thru Pin | RD56 | RD56 |
| 16 | CLK40- slot 9 | To J3D18, slot 9 | Thru Pin | RD52 | RD52 |
| 17 | Gnd | ## Gnd .. Plane | ## Gnd .. Plane | Gnd | Gnd |
| 18 | CLK40+ slot 10 | ## To J3D18, slot 10 | ## Thru Pin | RD45 | RD45 |
| 19 | CLK40- slot1 0 | To J3D18, slot 10 | ## Thru Pin | RD41 | RD41 |
| 20 | # Gnd | ###### Gnd .. Plane | ## Gnd .. Plane | Gnd | Gnd |
| 21 | CLK40+ slot 11 | ## To J3D18, slot 11 | Bussed | Bussed | Bussed |
| 22 | CLK40- slot 11 | To J3D18, slot 11 | ## Thru Pin | RD33 | RD33 |
| 23 |  |  | ## Thru Pin | RD30 | RD30 |
| 24 | CLK40+ slot 12 | ## To J3D18, slot 12 | ## Thru Pin | RD26 | RD26 |
| 25 | CLK40- slot 12 | To J3D18, slot 12 | ## Thru Pin | RD24 | RD24 |
| 26 | ## Gnd | ###### Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 27 | CLK40+ slot 13 | ## To J3D18, slot 13 | ## Thru Pin | RD17 | RD17 |
| 28 |  |  | ## Thru Pin | RD14 | RD14 |
| 29 |  |  | ## Thru Pin | RD10 | RD10 |
| 30 |  |  | ## Thru Pin | RD8 | RD8 |
| 31 | ###### Gnd | ## Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 32 |  |  | ###### Thru Pin | ###### RD1 | ###### RD1 |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 10: P3/J3 Connectors: use of Row B

| Pin | TIM @ Slot 13 | Backplane @ Slot 13 | Backplane @ Slots 5-12 and 14-21 | ROD @ Slots 5-12 and 14-21 | BOC @ rear of Slots 5-12 and 14-21 |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | ## Thru Pin | RD95 | RD95 |
| 2 | ###### Gnd | ###### Gnd .. Plane | ## Gnd .. Plane | Gnd | Gnd |
| 3 | ## CLK40+ slot14 | ## To J3D19, slot 14 | Thru Pin | RD91 | RD91 |
| 4 | CLK40- slot14 | To J3D19, slot 14 | ## Thru Pin | RD88 | RD88 |
| 5 |  |  | ## Thru Pin | RD86 | RD86 |
| 6 | CLK40+ slot15 | ## To J3D19, slot 15 | ## Thru Pin | RD84 | RD84 |
| 7 | ## CLK40- slot15 | To J3D19, slot 15 | ## Thru Pin | RD81 | RD81 |
| 8 | CLK40+ slot16 | To J3D19, slot 16 | Thru Pin | RD78 | RD78 |
| 9 | ## Gnd | ## Gnd .. Plane | ## Gnd .. Plane | Gnd | Gnd |
| 10 | CLK40- slot16 | To J3D19, slot 16 | ## Thru Pin | RD73 | RD73 |
| 11 |  |  | ## Thru Pin | RD69 | RD69 |
| 12 | CLK40+ slot17 | ## To J3D19, slot 17 | ## Thru Pin | RD66 | RD66 |
| 13 | ## CLK40- slot17 | To J3D19, slot 17 | Thru Pin | RD62 | RD62 |
| 14 | ###### Gnd | ###### Gnd .. Plane | ## Gnd .. Plane | Gnd | Gnd |
| 15 | ## CLK40+ slot18 | ## To J3D19, slot 18 | ## Thru Pin | RD57 | RD57 |
| 16 | CLK40- slot18 | To J3D19, slot 18 | ## Thru Pin | RD53 | RD53 |
| 17 |  |  | ## Thru Pin | RD49 | RD49 |
| 18 | CLK40+ slot19 | ## To J3D19, slot 19 | Thru Pin | RD46 | RD46 |
| 19 | ## CLK40- slot19 | To J3D19, slot 19 | ## Thru Pin | RD42 | RD42 |
| 20 |  |  | ## Thru Pin | RD40 | RD40 |
| 21 | ## CLK40+ slot20 | ## To J3D19, slot 20 | ## Thru Pin | RD37 | RD37 |
| 22 | CLK40- slot20 | To J3D19, slot 20 | ## Thru Pin | RD34 | RD34 |
| 23 | ## Gnd | ## Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 24 | CLK40+ slot21 | ## To J3D19, slot 21 | ## Thru Pin | RD27 | RD27 |
| 25 | ## CLK40- slot21 | To J3D19, slot 21 | ## Thru Pin | RD25 | RD25 |
| 26 |  |  | ## Thru Pin | RD21 | RD21 |
| 27 | ## CLK40- slot13 | ## To J3D19, slot 13 | ## Thru Pin | RD18 | RD18 |
| 28 | ###### Gnd | ###### Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 29 |  |  | ## Thru Pin | RD11 | RD11 |
| 30 |  |  | ## Thru Pin | RD9 | RD9 |
| 31 |  |  | ## Thru Pin | RD5 | RD5 |
| 32 |  |  | ## Thru Pin | RD2 | RD2 |

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 11: P3/J3 Connectors: use of Row C

| Pin | TIM @ Slot 13 | Backplane @ Slot 13 | Backplane @ Slots 5-12 and 14-21 | ROD @ Slots 5-12 and 14-21 | BOC @ rear of Slots 5-12 and 14-21 |
| --- | --- | --- | --- | --- | --- |
| 1 | ## +3v3 | ## +3v3 .. Plane | ## +3v3 .. Plane | +3v3 | +3v3 |
| 2 | # ROD-Busy6 | # From J3D21, slot 7 | ## Thru Pin | L-URESET# | L-URESET# |
| 3 | # +3v3 | # +3v3 .. Plane | ## +3v3 .. Plane | +3v3 | +3v3 |
| 4 | ROD-Busy8 | ## From J3D21, slot 9 | ## Thru Pin | L-UTEST# | L-UTEST# |
| 5 | Gnd | # Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 6 | ROD-Busy9 | # From J3D21, slot 10 | ## Thru Pin | L-LDOWN# | L-LDOWN# |
| 7 | +3v3 | ## +3v3 .. Plane | ## +3v3 .. Plane | +3v3 | +3v3 |
| 8 | +3v3 | ## +3v3 .. Plane | ## +3v3 .. Plane | +3v3 | +3v3 |
| 9 | ROD-Busy11 | # From J3D21, slot 12 | ## Thru Pin | RD75 | RD75 |
| 10 | Gnd | # Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 11 | ROD-Busy12 | ## From J3D21, slot 13 | ## Thru Pin | RD70 | RD70 |
| 12 | ROD-Busy14 | # From J3D21, slot 14 | ## Thru Pin | RD67 | RD67 |
| 13 | ## ROD-Busy15 | ## From J3D21, slot 15 | ## Thru Pin | RD63 | RD63 |
| 14 | ROD-Busy16 | ## From J3D21, slot 16 | ## Thru Pin | RD59 | RD59 |
| 15 | ###### Gnd | # Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 16 | ROD-Busy19 | # From J3D21, slot 19 | ## Thru Pin | RD54 | RD54 |
| 17 | ROD-Busy21 | ## From J3D21, slot 21 | ## Thru Pin | RD50 | RD50 |
| 18 |  |  | ## Thru Pin | RD47 | RD47 |
| 19 |  |  | ## Thru Pin | RD43 | RD43 |
| 20 | ###### Gnd | ###### Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 21 | Bussed |  | ## Thru Pin | RD38 | RD38 |
| 22 |  |  | ## Thru Pin | RD35 | RD35 |
| 23 | TCC-7 B | TCC-7 B bussed | ## Thru Pin | RD31 | RD31 |
| 24 | TCC-6 B | TCC-6 B bussed | ## Thru Pin | RD28 | RD28 |
| 25 | ###### Gnd | Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 26 | TCC-5 B | TCC-5 B bussed | ## Thru Pin | RD22 | RD22 |
| 27 | TCC-4 B | TCC-4 B bussed | ## Thru Pin | RD19 | RD19 |
| 28 | TCC-3 B | TCC-3 B bussed | ## Thru Pin | RD15 | RD15 |
| 29 | TCC-2 B | TCC-2 B bussed | ## Thru Pin | RD12 | RD12 |
| 30 | Gnd | Gnd .. Plane | Gnd .. Plane | Gnd | Gnd |
| 31 | TCC-1 B | TCC-1 B bussed | ## Thru Pin | ###### RD6 | ###### RD6 |
| 32 | TCC-0 B | TCC-0 B bussed | ## Thru Pin | ###### RD3 | ###### RD3 |

Note TTC 0-7 B are bussed to ROD slots 5-12 and terminated at slot 5

Interface Specification

##### SCT Off Detector Crate Backplane and Crate Card Utilization of P3/J3

Table 12: P3/J3 Connectors: use of Row D

| Pin | TIM @ Slot 13 | Backplane @ Slot 13 | Backplane @ Slots 5-12 | Backplane @ Slots 14-21 | ROD @ Slots 5-12 and 14-21 | BOC @ rear of Slots 5-12 and 14-21 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ROD-Busy5 | From J3D21, slot6 | Thru Pin | Thru Pin | L-UWEN# | L-UWEN# |
| 2 | +3v3 | +3v3 .. Plane | +3v3 .. Plane | +3v3 .. Plane | +3v3 | +3v3 |
| 3 | ROD-Busy7 | From J3D21, slot8 | Thru Pin | Thru Pin | L-UCLK | L-UCLK |
| 4 | +3v3 | +3v3 .. Plane | +3v3 .. Plane | +3v3 .. Plane | +3v3 | +3v3 |
| 5 |  |  | Thru Pin | Thru Pin | L-UCTRL# | L-UCTRL# |
| 6 | +3v3 | +3v3 .. Plane | +3v3 .. Plane | +3v3 .. Plane | +3v3 | +3v3 |
| 7 | ROD-Busy10 | From J3D21, slot11 | Thru Pin | Thru Pin | L-LFF# | L-LFF# |
| 8 | +3v3 | +3v3 .. Plane | +3v3 .. Plane | +3v3 .. Plane | +3v3 | +3v3 |
| 9 | +3v3 | +3v3 .. Plane | +3v3 .. Plane | +3v3 .. Plane | +3v3 | +3v3 |
| 10 | +3v3 | +3v3 .. Plane | +3v3 .. Plane | +3v3 .. Plane | +3v3 | +3v3 |
| 11 | Laser-Ilock | Laser-Ilock-Bussed | Laser-ILock-Bussed | Laser-ILock-Bussed | Laser-Ilock | Laser-Ilock |
| 12 | Gnd | ## Gnd .. Plane | ## Gnd .. Plane | ## Gnd .. Plane | Gnd | Gnd |
| 13 | ROD-sense | ###### Gnd .. Plane | Thru Pin | Thru Pin | ROD-sense | ROD-sense |
| 14 | ## ROD-Busy17 | ## From J3D21, slot17 | ## Thru Pin | ## Thru Pin |  |  |
| 15 | ROD-Busy18 | From J3D21, slot18 | Thru Pin | Thru Pin | RCLK40+ | RCLK40+ |
| 16 | ROD-Busy20 | From J3D21, slot20 | Thru Pin | Thru Pin | RCLK40- | RCLK40- |
| 17 | ## +5V | ## +5V .. Plane | ## +5V .. Plane | ## +5V .. Plane | ## +5V | ## +5V |
| 18 | CLK40+ _Ret | CLK40+ Routed | CLK40+ Routed | CLK40+ Routed | NC | CLK40+ |
| 19 | CLK40- _Ret | CLK40- Routed | CLK40- Routed | CLK40- Routed | NC | CLK40- |
| 20 | ## +5V | ## +5V .. Plane | ## +5V .. Plane | ## +5V .. Plane | ## +5V | ## +5V |
| 21 | TIM-Busy Out | TIM-Busy Out | ROD-Busy-Routed | ROD-Busy-Routed | ROD-Busy | NC |
| 22 | ## +5V | ## +5V .. Plane | ## +5V .. Plane | ## +5V .. Plane | ## +5V | ## +5V |
| 23 | TTC-7 A | TTC-7 A Bus | TTC-7 B- Bussed | TTC-7 A Bus | TTC-7 | NC |
| 24 | TTC-6 A | TTC-6 A Bus | TTC-6 B Bussed | TTC-6 A Bus | TTC-6 | NC |
| 25 | TTC-5 A | TTC-5 A Bus | TTC-5 B Bussed | TTC-5 A Bus | TTC-5 | NC |
| 26 | TTC-4 A | TTC-4 A Bus | TTC-4 B Bussed | TTC-4 A Bus | TTC-4 | NC |
| 27 | ## +5V | ## +5V .. Plane | ## +5V .. Plane | ## +5V .. Plane | ## +5V | ## +5V |
| 28 | TTC-3 A | TTC-3 A Bus | TTC-3 B Bussed | TTC-3 A Bus | TTC-3 | NC |
| 29 | TTC-2 A | TTC-2 A Bus | TTC-2 B Bussed | TTC-2 A Bus | TTC-2 | NC |
| 30 | TTC-1 A | TTC-1 A Bus | TTC-1 B Bussed | TTC-1 A Bus | TTC-1 | NC |
| 31 | TTC-0 A | TTC-0 A Bus | TTC-0 B Bussed | TTC-0 A Bus | TTC-0 | NC |
| 32 | ## +5V | ## +5V .. Plane | ## +5V .. Plane | ## +5V .. Plane | ## +5V | ## +5V |

Note  TTC 0-7 A are bussed to slots 14-21 and terminated at slot 21

Interface Specification

##### SCT Off Detector Crate Backplane: Utilization of Slots and Power Requirements

The crate has 16 slots for the RODs (16 ea.) with the BOCs (16 ea.) plugged into the card cage behind the backplane, 1 slot for the TIM, 1 slot for the 6U to 9U divider and 2 slots for the crate processor.  The table below shows the assignments.

| Slot Number | Item in slot |
| --- | --- |
| 1 | Crate processor, Front 6Ux160 mm |
| 2 | Reserved for crate processor use, Front 6Ux160mm |
| 3 | TBD, Front card 6Ux160 mm |
| 4 | Divider for 6U to 9U card sizes |
| 5 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 6 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 7 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 8 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 9 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 10 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 11 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 12 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 13 | TIM Terminate all ROD-Busy on TIM |
| 14 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 15 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 16 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 17 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 18 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 19 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 20 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |
| 21 | ROD and BOC Front 9Ux400 mm, Back 9Ux240 mm |

Power Utilization:

| Item | ###### Number Of Cards | 5 Volts | 3.3 Volts | -12 volts | +12 Volts |
| --- | --- | --- | --- | --- | --- |
| ROD | 16 | 9 A each 144 A total | 11 A each 176 A total | 0 | 0 |
| BOC | 16 | 4.5 A each 72 A total | 3 A each 48 A total | 0 | 0.1A each 1.6 A total |
| TIM | 1 | 15 A each | 8 A each | 1.5A each | 1.5 |
| RCC | 1 | 3.5 A each | 0 | 1 A each | 0 |
| Total Amps: |  | 234 A | 232 A | 2.5A | 3.1 A |
| Power |  | 1170W | 765W | 30W | 37W |

Crate Total Power    2002W

Interface Specification

##### SCT Off Detector Crate Backplane: General Information

Number of RODs and BOCs (1 BOC needed per ROD) :

The number of RODs and BOCs is reasonably stable but changes of a few percent could be needed.

| Application | Number Needed |
| --- | --- |
| SCT barrel | 44 |
| SCT forward | 48 |
| Total SCT | 92 |
| Pixel barrel | 49 |
| Pixel forward disks 1-3 | 13 |
| Pixel forward disk 4 | 9 |
| Pixel B-layer | 39 |
| Total Pixels | 110 |
| Grand Total RODs + BOCs | 202 |

Number of crates:

The number of crates should be stable.

| Application | Number Needed |
| --- | --- |
| SCT | 8 |
| Pixel | 8 |
| ###### Total SCT and Pixel Crates | 16 |

1  These ROD-Busy signals are routed from Slots 5-12 and 14-21, P3J3, pin 21. They are active LOW, LV-TTL totem-pole signals.

42This copy of CLK-40 signal is routed to Slot13, P3J3, pins D18 and D19: this path is the same length as CLK-40 signals destined for other slots.

3 These copies of the CLK-40 signal are routed to the BOCs at slots 5-12 and 14-21; they all have the same electronic length, and are 100 ohm differential  impedance.

4The state of the bussed Laser-Ilock line appears as a bit in a Set-Up bus register. It is also available to RODs and the TIM.

5  ROD-Sense grounded on ROD, pulled high through resistor on TIM and BOC. Grounded on backplane at slot 5.

6 TIM-Busy Out is provided by TIM, but is not routed on the backplane at slot 13.

3

2

BOC 3v3 power is only drawn exclusively from these 3v3 supply pins – the RODs may also draw some of their 3v3 power from them.

The state of the bussed Laser-Ilock line appears as a bit in a Set-Up bus register. It is also available to RODs and the TIM.

ROD-Sense grounded on ROD, pulled high through resistor on TIM and BOC. Grounded on backplane at slot 13.

RCLK-40 is buffered copy of CLK-40 signal received by BOC on P3J3, pins D18 and D19.

These ROD-Busy signals are routed to pins on P3J3 @ Slot5. They are active LOW, LV-TTL totem-pole signals.

BOC 3v3 power is only drawn exclusively from these 3v3 supply pins – the RODs may also draw some of their 3v3 power from them.

The state of the bussed Laser-Ilock line appears as a bit in a Set-Up bus register. It is also available to RODs and the TIM.

ROD-Sense grounded on ROD, pulled high through resistor on TIM and BOC. Grounded on backplane at slot 5.

RCLK-40 is buffered copy of CLK-40 signal received by BOC on P3J3, pins D18 and D19.