Exerpts from the operator manual for error counting

EFB data engine

Format of errors in the event trailer.

The next step is to process all of the errors in the pipeline.  A static mask determines if a link is to be processed.  The errors processed for each link are shown below:

| Mask Bit Location | Error Type |
| --- | --- |
| 0 | Header bit error |
| 1 | Trailer bit error |
| 2 | FE Flagged error |
| 3 | Sync error |
| 4 | Condensed Mode Hit Pattern Error (SCT) / MCC Error (Pixel) |
| 5 | L1ID error |
| 6 | BCID error |
| 7 | Timeout error |
| 8 | Almost full |
| 9 | Data overflow |
| 10 | Non-Sequential Chip Number in Event Data |
| 11 | Invalid FE Chip (SCT)/Invalid Pixel Row Number (row > 159) |
| 12 | Invalid Pixel Column Number (column > 17) |

For each link in the event the error bits above are counted, and ANDed with the complement of the Error Mask register and ORed with errors from all of the links in a fragment.  The result is that the number and type of errors in a fragment will be recorded and inserted into the Event Trailer.

These can be scaled on a per bin basis.

S-Link Event header Output Format (sent to S-link and DSP)

The Router adds the UCTRL bit in the Router output format to indicate to the ROS the location of the beginning and end of fragment markers.  The count of errors and types of errors are placed in the trailer for the event. There is no information on the links that had errors. It is possible to scale these errors for a bin.  The format is shown below:

# Event Header

| Word | Contents | Comment |
| --- | --- | --- |
| 0 | 0xB0F00000 + UCTRL | Beginning of fragment marker |
| 1 | 0xEE1234EE | Start of header |
| 2 | 0x9 | Header size |
| 3 | TBD | Format version number |
| 4 | TBD | Source identifier |
| 5 | L1 ID | Level 1 ID |
| 6 | BC ID | Bunch crossing ID |
| 7 | L1 TT | ATLAS Level 1 trigger type |
| 8 | DET | Detector event type ROD or TIM |

# Event Trailer

| Word | Contents | Comment |
| --- | --- | --- |
| 0 | Error count | Status 1: count of words with error |
| 1 | Error flags | Status 2: Bit error see EFB errors |
| 2 | 0x2 | Number of status words |
| 3 | nData | Count of data words |
| 4 | 0x1 | Status block position: 0/1 = before/after data |
| 5 | 0xE0F00000 + UCTRL | End of fragment marker |

End of Block (sent to DSP)

The Router generates an end of block word that is located in the last location of the last frame of a trapped event.  The word contains trap information that is required by the DSP for signal processing.  The format is as follows:

| Contents | Comment |
| --- | --- |
| 01000000000t11dewwwwwwwwwwwwwwww | t = trap num, d = NOT data loss (FIFO full) e = NOT error (one or more links has an error) w = event word count |

S-link Module Output Format  (sent to DSP and S-link)

The Router modifies the output data format of the module data before it is sent out on the S-Link Bus.  The L1ID and BCID is replaced with error codes and the link number as shown below:

Router Output – SCT Format (bits 31:0 bits  for all words)

| Name | Bits [15:0] or [31:16] | EFB Output |
| --- | --- | --- |
| Header | 001ptlbKxMMMMMMM | 001pLLLLBBBBBBBB |
| Trailer | 010zhvxxxxxxxxxx |  |
| 1 hit condensed | 1FFFFCCCCCCCxfx0 |  |
| 2 hits condensed | 1FFFFCCCCCCCsfx1 |  |
| 1st hit cluster expanded | 1FFFFCCCCCCC0DDD |  |
| 1 hit cluster expanded | 1xxxxxxx0xxx1DDD |  |
| 2 hit cluster expanded | 1xxxxxxx1DDD1DDD |  |
| Flagged error | 000xxxxxxFFFFEEE |  |
| Raw data | 011nnnxxWWWWWWWW |  |

Key :

x= do not care  (The ROD fills these with 0’s)

B=BCID

W=raw data

C=cluster base address

D=3 bit hit data

E=ABC error code

F=FE number

h=header trailer limit error

L=L1ID

n=count of raw data

f=error in condensed mode data, 1st hit

s=error in condensed mode data, 2nd hit

p=preamble error

z=trailer bit error

v=data overflow error

t=time out error

K=condensed mode

l=L1 error

b=BCID error

M=link number

On a per link per bin basis the p, t, l, b, f, x, z  and h could be scaled but would take time.

We are changing the DSP format to provide the error data format for all DSP data. This will give link information that can be easily scaled on a per bin and per link basis.  We prefer to scale these numbers because it can be fast rather than the information derived from above.  This change has not been made in the VHDL or C code at this time.

Router Error Event Format (DSP only)

# Event Header

| Word | Contents | Comment |
| --- | --- | --- |
| 0 | 0xB0F00000 | Beginning of fragment marker |
| 1 | 0xEE1234EE | Start of header |
| 2 | L1 ID | Level 1 ID |
| 3 | BC ID | Bunch crossing ID |
| 4 | L1 TT | ATLAS Level 1 trigger type |
| 5 | DET | Detector event type ROD or TIM |

# Event Trailer

| Word | Contents | Comment |
| --- | --- | --- |
| 0 | Error count | Status 1: count of words with error |
| 1 | Error flags | Status 2: Bit error see EFB errors |
| 2 | Value | Error status for links 0-15 |
| 3 | Value | Error status for links 16-31 |
| 4 | Value | Error status for links 32-47 |
| 5 | Value | Error status for links 48-63 |
| 6 | Value | Error status for links 64-79 |
| 7 | Value | Error status for links 80-95 |
| 8 | 0xE0F00000 | End of fragment marker |

As the data is processed in the Router, six 32-bit words are built allowing 2-bits of status per link.  The two bits encode the following cases:

| Bit Value | Status |
| --- | --- |
| 00 | No Data |
| 01 | Good Data |
| 10 | BCID or L1ID error |
| 11 | Time Out error |

We can change the Time Out  error to any other error or all combinations. All combinations will give full error coverage without details of specific errors for the code 11.