L1ID extension proposal

History:

15 November 2001 First draft. Philippe Farthouat

Motivation:

A 24-bit L1ID formed in the TTCrx is associated with each event. In the front-end electronics, a subset of the L1ID is also implemented allowing a synchronization check at the level of the RODs.

The L1ID is formed by a 24-bit counter incremented by L1A and reset by ECR.

24-bit L1ID allows to cover 223 seconds (167 s) at L1A rate of 75 kHz (100 kHz) which was deemed sufficient to uniquely characterized an event in the ROD-ROB system until a level-2 decision is made.

It may happen that there will be relatively frequent (order of 0.1 Hz, and not more than 1 Hz) ECR’s issued to cure front-end problems. If it is the case, we may be in a situation where several events in the ROD-ROB system will have the same L1ID which is not acceptable.

Proposal:

It is proposed to expand the L1ID to 32 bits in concatenating with the current 24-bit L1ID an 8-bit ECRID as shown in figure 1.

| Bits 31 to 24 | Bits 23 to 0 |
| --- | --- |
| ECRID | TTC L1ID |

Figure 1. Extended L1ID

The ECRID will be formed at the ROD level by an 8-bit counter incremented by ECR and reset by a VME command under software control at the beginning of a run.

Justification:

The synchronization between the front-end electronics and the RODs is not affected as the 24 low order bits are unchanged with respect to the current scheme. With this scheme, the minimum time during which there won’t be several events with the same L1ID in the ROD-ROB system is 256 seconds.