# 07 December 1999

TTC  INTERFACE  MODULE   ( TIM ) :

MODULE  CONNECTIONS  to the  BACKPLANE

Extracted and adapted from:

“SCT Off Detector Crate Back Planes and Crate Card Utilization,   Version 1.0,	8 Oct. 1999”

# Update Log :

07 Dec. 1999, MP-UCL : Removed unused VME signals

01 Dec. 1999, MP-UCL : Added Jaux & JBL minor changes

30 Nov. 1999, MP-UCL : Simplify & JBL minor corrections

29 Nov. 1999, MP-UCL : TIM backplane connections only

Introduction

This document is a description of the TIM connectors to the backplane.

The back planes provide the interconnection between the ROD Crate Controller (RCC), Timing Interface Module (TIM) , Read Out Driver (ROD), the Back Of the Crate (BOC) (optical interface card) and the power supplies.

The signals that must be connected to the TIM are listed below.

Note: All signals are 3V levels unless indicated otherwise

ROD  BOC	TIM

#### ROD-Sense			 To identify ROD to BOC	 1  1	1

ROD-Busy             1 pin on ROD             1  0    16

16 pins on TIM

CLK-40+ and CLK-40-  System Clock (Diff.PECL) 4  4    36

4 pins on ROD and BOC

36 pins on TIM

TTC-0 to TTC-7       Trigger control bus      8  0     8

TIM-Busy Out 		 OR-ed Masked busy out			1(2)

Total Signal PINS   ROD BOC TIM

221 212  62

The power voltages used by TIM prototype are listed below:

In standard    User defined

VME   VMEx	 ( TIM only )

+12V           J1/P1 pins           0               0

-12V           J1/P1 pins           1               0

+5V     6 pins J1/P1 pins           3               0

J2/P2 pins           3               0

J3/P3 pins           0               0

+3.3V  10 pins J1/P1 pins           0    10         0

J2/P2 pins           0    	       0

J3/P3 pins           0               0

Ground 82 pins J1/P1 pins           8    18         0

J2/P2 pins           4    17         0

J3/P3 pins           0               27

Pin diode bias (+5V)  	 J3/P3     0     0      	  1 (3)

Pin diode ground          J3/P3     0     0      	  1 (3)

Note: (2) This signal will probably be on the front panel

(3) Probably not required because the BOC may use

the  standard +5 volts.

P1 Connector on TIM module :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | D00 |  | D08 | VPC | 1 |
| 2 | ## Gnd | D01 |  | D09 | ## Gnd | 2 |
| 3 |  | D02 |  | D10 |  | 3 |
| 4 | ## Gnd | D03 |  | D11 |  | 4 |
| 5 |  | D04 |  | D12 |  | 5 |
| 6 | ## Gnd | D05 |  | D13 |  | 6 |
| 7 |  | D06 |  | D14 |  | 7 |
| 8 | ## Gnd | D07 |  | D15 |  | 8 |
| 9 |  | Gnd |  | Gnd | GAP* | 9 |
| 10 | ## Gnd |  |  |  | ## GA0* | 10 |
| 11 |  | Gnd |  | BERR* | ## GA1* | 11 |
| 12 | ## Gnd | DS1* |  |  | ## +3.3V | 12 |
| 13 |  | DS0* |  | LWORD* | ## GA2* | 13 |
| 14 | ## Gnd | WRITE* |  | AM5 | ## +3.3V | 14 |
| 15 |  | ## Gnd |  | A23 | ## GA3* | 15 |
| 16 | ## Gnd | DTACK* | AM0 | A22 | ## +3.3V | 16 |
| 17 |  | Gnd | AM1 | A21 | # GA4* | 17 |
| 18 | ## Gnd |  | AM2 | A20 | ## +3.3V | 18 |
| 19 |  | ## Gnd | AM3 | A19 |  | 19 |
| 20 | # Gnd | IACK* | ## Gnd | A18 | ## +3.3V | 20 |
| 21 |  |  |  | A17 |  | 21 |
| 22 | ## Gnd |  |  | A16 | ## +3.3V | 22 |
| 23 |  | AM4 | Gnd | A15 |  | 23 |
| 24 | ## Gnd | A07 |  | A14 | ## +3.3V | 24 |
| 25 |  | A06 |  | A13 |  | 25 |
| 26 | ## Gnd | A05 |  | A12 | ## +3.3V | 26 |
| 27 |  | A04 |  | A11 |  | 27 |
| 28 | ## Gnd | A03 |  | A10 | ## +3.3V | 28 |
| 29 |  | A02 |  | A09 |  | 29 |
| 30 | ## Gnd | A01 |  | A08 | ## +3.3V | 30 |
| 31 |  | ## -12V |  |  | Gnd | 31 |
| 32 | ## Gnd | ## +5V | ## +5V | ## +5V | VPC | 32 |

GND:		26 pins

+5V		 3 pins

+3.3V	10 pins

-12V		 1 pin

P2 Connector on TIM module :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | ## +5V |  |  | 1 |
| 2 | Gnd |  | ## Gnd |  |  | 2 |
| 3 |  |  |  |  |  | 3 |
| 4 | Gnd |  | A24 | -5.2V (#) |  | 4 |
| 5 |  |  | A25 |  |  | 5 |
| 6 | Gnd |  | A26 |  |  | 6 |
| 7 |  | -5.2V (#) | A27 |  |  | 7 |
| 8 | Gnd |  | A28 |  |  | 8 |
| 9 |  |  | A29 |  |  | 9 |
| 10 | Gnd |  | A30 |  |  | 10 |
| 11 |  |  | A31 |  |  | 11 |
| 12 | Gnd |  | ## Gnd |  |  | 12 |
| 13 |  | -5.2V (#) | ## +5V |  |  | 13 |
| 14 | Gnd |  |  |  |  | 14 |
| 15 |  |  |  |  |  | 15 |
| 16 | Gnd |  |  |  |  | 16 |
| 17 |  |  |  |  |  | 17 |
| 18 | Gnd |  |  |  |  | 18 |
| 19 |  | -5.2V (#) |  | -5.2V (#) |  | 19 |
| 20 | Gnd |  |  |  |  | 20 |
| 21 |  |  |  |  |  | 21 |
| 22 | Gnd |  | ## Gnd |  |  | 22 |
| 23 |  |  |  |  |  | 23 |
| 24 | Gnd |  |  |  |  | 24 |
| 25 |  |  |  |  |  | 25 |
| 26 | ## Gnd |  |  |  |  | 26 |
| 27 |  |  |  |  |  | 27 |
| 28 | ## Gnd |  |  |  |  | 28 |
| 29 |  |  |  |  |  | 29 |
| 30 | ## Gnd |  |  |  |  | 30 |
| 31 |  |  | ## Gnd |  | Gnd | 31 |
| 32 | ## Gnd |  | ## +5V |  | VPC | 32 |

GND:		21 pins

+5V:		 3 pins

-5.2V  	 5 pins

Note :	(#)	-5.2V supply pins on TIM P2 are for prototype testing purposes in VME/VXI-type crates at UCL or CERN only, and will not be provided on the final version. All the above -5.2V pins have links for isolation

P3 Connector on TIM module :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ### Gnd |  | ### Gnd | ## ROD-Busy6 |  | 1 |
| 2 |  | ## Gnd |  | # ROD-Busy7 | ROD-Sense(1) | 2 |
| 3 |  | CLK-40+ slot5 | ## CLK-40- slot5 | # ROD-Busy8 |  | 3 |
| 4 |  | CLK-40+ slot6 | CLK-40- slot6 | ROD-Busy9 |  | 4 |
| 5 |  |  | ## Gnd | ROD-Busy10 |  | 5 |
| 6 |  | CLK-40+ slot7 | CLK-40- slot7 | ROD-Busy11 |  | 6 |
| 7 |  | ## CLK-40+ slot8 | ## CLK-40- slot8 | Gnd |  | 7 |
| 8 | Gnd | CLK-40+ slot9 | CLK-40- slot9 | ROD-Busy12 |  | 8 |
| 9 |  | ## Gnd |  | ROD-Busy13 |  | 9 |
| 10 |  | CLK-40+ slot10 | CLK-40- slot10 | ROD-Busy14 |  | 10 |
| 11 |  |  | ## Gnd | ROD-Busy15 |  | 11 |
| 12 |  | CLK-40+ slot11 | CLK-40- slot11 | ROD-Busy16 | PD-Bias (3) | 12 |
| 13 |  | ## CLK-40+ slot12 | ## CLK-40- slot12 | Gnd | PD-Gnd (3) | 13 |
| 14 |  | ## Gnd |  | ROD-Busy17 | Gnd | 14 |
| 15 | Gnd | CLK-40+ slot13 | ## CLK-40- slot13 | ROD-Busy18 |  | 15 |
| 16 |  | CLK-40+ slot14 | CLK-40- slot14 | ROD-Busy19 |  | 16 |
| 17 |  |  | ## Gnd | ROD-Busy20 | Gnd | 17 |
| 18 |  | CLK-40+ slot15 | CLK-40- slot15 | ROD-Busy21 | CLK-40+ (5) | 18 |
| 19 |  | CLK-40+ slot16 | ## CLK-40- slot16 | Gnd | CLK-40- (5) | 19 |
| 20 |  | # Gnd |  |  | Gnd | 20 |
| 21 | Gnd | CLK-40+ slot17 | ## CLK-40- slot17 |  |  | 21 |
| 22 |  | CLK-40+ slot18 | CLK-40- slot18 | TIM-Busy Out |  | 22 |
| 23 |  |  | ## Gnd | ## Gnd |  | 23 |
| 24 |  | CLK-40+ slot19 | CLK-40- slot19 | ## TTC-7 |  | 24 |
| 25 |  | CLK-40+ slot20 | ## CLK-40- slot20 | TTC-6 | Gnd | 25 |
| 26 | Gnd | ## Gnd |  | TTC-5 |  | 26 |
| 27 |  | CLK-40+ slot21 | ## CLK-40- slot21 | TTC-4 |  | 27 |
| 28 |  |  |  | TTC-3 |  | 28 |
| 29 |  |  | ## Gnd | TTC-2 |  | 29 |
| 30 |  |  |  | TTC-1 |  | 30 |
| 31 |  |  |  | TTC-0 |  | 31 |
| 32 |  | ## Gnd |  | ## Gnd | ## Gnd | 32 |

GND :	27 pins

Note:	The ground pins have been selected so that no damage will occur if the card is put in any slot

Note:	The signal pin selection will cause damage to the TIM or BOC. There are two solutions available :

- The first is to have ROD-Sense (1)

- The second is to key the TIM so that it can not be plugged into a ROD slot and vice versa

Note:	(1)	ROD-Sense: Grounded on ROD, pulled high

on TIM and BOC

(4)	CLK-40 +/- slotNN: from TIM to BOC only, all clock lines have the same electronic length and are 100 ohm impedance

(5)	CLK-40 +/- : return to TIM. This is the return signal from pin 3 Row A and B above. It is the same electrical length as all the other clock lines from TIM to BOC, and the same 100 ohm impedance

(3)	PIN Diode Bias Voltage to BOC only ( +5V )

( probably not required )

Note:	-	TTC-0 to TTC-7 and ROD-Busy are not used

on the BOC. These lines are terminated on the back plane.

Impedance about 50 ohms with card plugged in

Jaux Connector on TIM :

Note :	Jaux not soldered in – pads only provided

| Pin | Row A | Row B | Row C | Pin |
| --- | --- | --- | --- | --- |
| 1 |  | Gnd |  | 1 |
| 2 |  | Gnd |  | 2 |
| 3 |  | Gnd |  | 3 |
| 4 |  | Gnd |  | 4 |
| 5 |  | Gnd |  | 5 |
| 6 |  | Gnd |  | 6 |
| 7 |  |  |  | 7 |
| 8 |  |  |  | 8 |
| 9 | -5.2V (#) | -5.2V (#) | -5.2V (#) | 9 |
| 10 | -5.2V (#) | -5.2V (#) | -5.2V (#) | 10 |

GND:		6 pins

-5.2V  	6 pins

Note :	(#)	-5.2V supply pins on TIM Jaux are for prototype testing purposes in VME/VXI-type crates at UCL or CERN only, and will not be provided on the final version. All the above -5.2V pins have links for isolation

The crate has 16 slots for the RODs, 1 slot for the TIM, I slot for the 6U to 9U divider and 2 slots for the crate processor.  The table below shows the assignments.

| Slot Number | Item in slot |
| --- | --- |
| 1 | Crate processor |
| 2 | Reserved for crate processor use |
| 3 | TBD |
| 4 | Divider for 6U to 9U card sizes |
| 5 | TIM |
| 6 | ROD and BOC |
| 7 | ROD and BOC |
| 8 | ROD and BOC |
| 9 | ROD and BOC |
| 10 | ROD and BOC |
| 11 | ROD and BOC |
| 12 | ROD and BOC |
| 13 | ROD and BOC |
| 14 | ROD and BOC |
| 15 | ROD and BOC |
| 16 | ROD and BOC |
| 17 | ROD and BOC |
| 18 | ROD and BOC |
| 19 | ROD and BOC |
| 20 | ROD and BOC |
| 21 | ROD and BOC |