# 30 November 1999

TIMING  INTERFACE  MODULE   ( TIM ) :

MODULE  CONNECTIONS  to the  BACKPLANE

Extracted and adapted from:

“SCT Off Detector Crate Back Planes and Crate Card Utilization”

Version 1.0,	8 Oct. 1999

# Update Log

30 Nov. 1999, JBL &  MP-UCL

29 Nov. 1999, MP-UCL : TIM backplane connections only

The back planes provide the interconnection between the ROD Crate Controller (RCC), Timing Interface Module (TIM) , Read Out Driver (ROD), the Back Of the Crate (BOC) (optical interface card) and the power supplies.

The signals that must be connected to the TIM are listed below:

Note: All signals are 3V levels unless indicated other wise

ROD  BOC	TIM

#### ROD-Sense			 To identify ROD to BOC	 1  1	1

ROD-Busy             1 pin on ROD             1  0    16

16 pins on TIM

Clk-40+ and Clk-40-  System Clock (Diff PECL) 4  4    36

4 pins on ROD and BOC

and 36 pins on TIM

TTC-0 to TCC-7       Trigger control bus      8  0     8

TIM-Busy Out 		OR-ed Masked busy out			1 (2)

Total Signal PINS   ROD BOC TIM

221 212  62

The power voltages are listed below:

In standard    User defined

VME   VMEx	 ( TIM only )

+12V           J1/P1 pins           0               0

-12V           J1/P1 pins           1               0

+5V  6 pins    J1/P1 pins           3               0

J2/P2 pins           3               0

J3/P3 pins           0               0

+3.3V  14 pins J1/P1 pins           0    10         0

J2/P2 pins           0    	       0

J3/P3 pins           0               0

Ground 82 pins J1/P1 pins           8    18         0

J2/P2 pins           4    17         0

J3/P3 pins           0               27

Pin diode bias (5V)  	 J3/P3     0     0      	  1 (3)

Pin diode ground          J3/P3     0     0      	  1 (3)

Note: (2) This signal will probably be on the front panel

(3) Probably not required because the BOC may use

the  standard +5 volts.

J1 Connector on TIM :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | D00 |  | D08 | VPC | 1 |
| 2 | ## Gnd | D01 |  | D09 | ## Gnd | 2 |
| 3 |  | D02 |  | D10 |  | 3 |
| 4 | ## Gnd | D03 |  | D11 |  | 4 |
| 5 |  | D04 |  | D12 |  | 5 |
| 6 | ## Gnd | D05 |  | D13 |  | 6 |
| 7 |  | D06 |  | D14 |  | 7 |
| 8 | ## Gnd | D07 |  | D15 |  | 8 |
| 9 |  | Gnd |  | Gnd | GAP* | 9 |
| 10 | ## Gnd |  |  |  | ## GA0* | 10 |
| 11 |  | Gnd |  | BERR* | ## GA1* | 11 |
| 12 | ## Gnd | DS1* |  |  | ## +3.3V | 12 |
| 13 |  | DS0* |  | LWORD* | ## GA2* | 13 |
| 14 | ## Gnd | WRITE* |  | AM5 | ## +3.3V | 14 |
| 15 |  | ## Gnd |  | A23 | ## GA3* | 15 |
| 16 | ## Gnd | DTACK* | AM0 | A22 | ## +3.3V | 16 |
| 17 |  | Gnd | AM1 | A21 | # GA4* | 17 |
| 18 | ## Gnd | AS* | AM2 | A20 | ## +3.3V | 18 |
| 19 |  | ## Gnd | AM3 | A19 |  | 19 |
| 20 | # Gnd | IACK* | ## Gnd | A18 | ## +3.3V | 20 |
| 21 |  |  |  | A17 |  | 21 |
| 22 | ## Gnd |  |  | A16 | ## +3.3V | 22 |
| 23 |  | AM4 | Gnd | A15 |  | 23 |
| 24 | ## Gnd | A07 |  | A14 | ## +3.3V | 24 |
| 25 |  | A06 |  | A13 |  | 25 |
| 26 | ## Gnd | A05 |  | A12 | ## +3.3V | 26 |
| 27 |  | A04 |  | A11 |  | 27 |
| 28 | ## Gnd | A03 |  | A10 | ## +3.3V | 28 |
| 29 |  | A02 |  | A09 |  | 29 |
| 30 | ## Gnd | A01 |  | A08 | ## +3.3V | 30 |
| 31 |  | ## -12V |  |  | Gnd | 31 |
| 32 | ## Gnd | ## +5V | ## +5V | ## +5V | VPC | 32 |

GND:		26 pins

+5V		 3 pins

+3.3V	10 pins

-12V		 1 pin

J2 Connector on TIM :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | ## +5V |  |  | 1 |
| 2 | Gnd |  | ## Gnd |  |  | 2 |
| 3 |  |  | Retry* |  |  | 3 |
| 4 | Gnd |  | A24 | -5.2V (#) |  | 4 |
| 5 |  |  | A25 |  |  | 5 |
| 6 | Gnd |  | A26 |  |  | 6 |
| 7 |  | -5.2V (#) | A27 |  |  | 7 |
| 8 | Gnd |  | A28 |  |  | 8 |
| 9 |  |  | A29 |  |  | 9 |
| 10 | Gnd |  | A30 |  |  | 10 |
| 11 |  |  | A31 |  |  | 11 |
| 12 | Gnd |  | ## Gnd |  |  | 12 |
| 13 |  | -5.2V (#) | ## +5V |  |  | 13 |
| 14 | Gnd |  | D16 |  |  | 14 |
| 15 |  |  | D17 |  |  | 15 |
| 16 | Gnd |  | D18 |  |  | 16 |
| 17 |  |  | D19 |  |  | 17 |
| 18 | Gnd |  | D20 |  |  | 18 |
| 19 |  | -5.2V (#) | D21 | -5.2V (#) |  | 19 |
| 20 | Gnd |  | D22 |  |  | 20 |
| 21 |  |  | D23 |  |  | 21 |
| 22 | Gnd |  | ## Gnd |  |  | 22 |
| 23 |  |  | D24 |  |  | 23 |
| 24 | Gnd |  | D25 |  |  | 24 |
| 25 |  |  | D26 |  |  | 25 |
| 26 | ## Gnd |  | D27 |  |  | 26 |
| 27 |  |  | D28 |  |  | 27 |
| 28 | ## Gnd |  | D29 |  |  | 28 |
| 29 |  |  | D30 |  |  | 29 |
| 30 | ## Gnd |  | D31 |  |  | 30 |
| 31 |  |  | ## Gnd |  | Gnd | 31 |
| 32 | ## Gnd |  | ## +5V |  | VPC | 32 |

GND:		21 pins

+5V:		 3 pins

-5.2V  	 5 pins

Note :	(#)	-5.2V supply pins on TIM J2 are for a prototype testing purposes in VME/VXI-type crates at UCL or CERN only, and will not be provided on the final version. All the above -5.2V pins have links for isolation

J3 Connector on TIM :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ### Gnd |  | ### Gnd | ## ROD-Busy6 |  | 1 |
| 2 |  | ## Gnd |  | # ROD-Busy7 | ROD-Sense(1) | 2 |
| 3 |  | CLK-40+ slot5 | ## CLK-40- slot5 | # ROD-Busy8 |  | 3 |
| 4 |  | CLK-40+ slot6 | CLK-40- slot6 | ROD-Busy9 |  | 4 |
| 5 |  |  | ## Gnd | ROD-Busy10 |  | 5 |
| 6 |  | CLK-40+ slot7 | CLK-40- slot7 | ROD-Busy11 |  | 6 |
| 7 |  | ## CLK-40+ slot8 | ## CLK-40- slot8 | Gnd |  | 7 |
| 8 | Gnd | CLK-40+ slot9 | CLK-40- slot9 | ROD-Busy12 |  | 8 |
| 9 |  | ## Gnd |  | ROD-Busy13 |  | 9 |
| 10 |  | CLK-40+ slot10 | CLK-40- slot10 | ROD-Busy14 |  | 10 |
| 11 |  |  | ## Gnd | ROD-Busy15 |  | 11 |
| 12 |  | CLK-40+ slot11 | CLK-40- slot11 | ROD-Busy16 | PD-Bias (3) | 12 |
| 13 |  | ## CLK-40+ slot12 | ## CLK-40- slot12 | Gnd | PD-Gnd (3) | 13 |
| 14 |  | ## Gnd |  | ROD-Busy17 | Gnd | 14 |
| 15 | Gnd | CLK-40+ slot13 | ## CLK-40- slot13 | ROD-Busy18 |  | 15 |
| 16 |  | CLK-40+ slot14 | CLK-40- slot14 | ROD-Busy19 |  | 16 |
| 17 |  |  | ## Gnd | ROD-Busy20 | Gnd | 17 |
| 18 |  | CLK-40+ slot15 | CLK-40- slot15 | ROD-Busy21 | CLK-40+ (5) | 18 |
| 19 |  | CLK-40+ slot16 | ## CLK-40- slot16 | Gnd | CLK-40- (5) | 19 |
| 20 |  | # Gnd |  |  | Gnd | 20 |
| 21 | Gnd | CLK-40+ slot17 | ## CLK-40- slot17 |  |  | 21 |
| 22 |  | CLK-40+ slot18 | CLK-40- slot18 | TIM-Busy Out |  | 22 |
| 23 |  |  | ## Gnd | ## Gnd |  | 23 |
| 24 |  | CLK-40+ slot19 | CLK-40- slot19 | ## TTC-7 |  | 24 |
| 25 |  | CLK-40+ slot20 | ## CLK-40- slot20 | TTC-6 | Gnd | 25 |
| 26 | Gnd | ## Gnd |  | TTC-5 |  | 26 |
| 27 |  | CLK-40+ slot21 | ## CLK-40- slot21 | TTC-4 |  | 27 |
| 28 |  |  |  | TTC-3 |  | 28 |
| 29 |  |  | ## Gnd | TTC-2 |  | 29 |
| 30 |  |  |  | TTC-1 |  | 30 |
| 31 |  |  |  | TTC-0 |  | 31 |
| 32 |  | ## Gnd |  | ## Gnd | ## Gnd | 32 |

GND :	27 pins

Note:	The ground pins have been selected so that no damage will occur if the card is put in any slot

Note:	The signal pin selection will cause damage to the TIM or BOC. There are two solutions available :

- The first is to have ROD-Sense (1)

- The second is to key the TIM so that it can not be plugged into a ROD slot and vice versa

Note:	(1)	ROD-Sense: Grounded on ROD, pulled high

on TIM and BOC

(4)	CLK-40 +/- slotNN: from TIM to BOC only, all clock lines have the same electronic length and are 100 ohm impedance.

(5)	CLK-40 +/- : return to TIM. This is the return signal from pin 3 Row A and B above. It is the same electrical length as all the other clock lines from TIM to BOC, and the same 100 ohm impedance

(3)	PIN Diode Bias Voltage to BOC only ( +5V )

( probably not required )

Note:	-	TTC-0 to TTC-7 and ROD-Busy are not used

on the BOC. These lines are terminated on the back plane

Impedance about 50 ohms with card plugged in

The crate has 16 slots for the RODs, 1 slot for the TIM, I slot for the 6U to 9U divider and 2 slots for the crate processor.  The table below shows the assignments.

| Slot Number | Item in slot |
| --- | --- |
| 1 | Crate processor |
| 2 | Reserved for crate processor use |
| 3 | TBD |
| 4 | Divider for 6U to 9U card sizes |
| 5 | TIM |
| 6 | ROD and BOC |
| 7 | ROD and BOC |
| 8 | ROD and BOC |
| 9 | ROD and BOC |
| 10 | ROD and BOC |
| 11 | ROD and BOC |
| 12 | ROD and BOC |
| 13 | ROD and BOC |
| 14 | ROD and BOC |
| 15 | ROD and BOC |
| 16 | ROD and BOC |
| 17 | ROD and BOC |
| 18 | ROD and BOC |
| 19 | ROD and BOC |
| 20 | ROD and BOC |
| 21 | ROD and BOC |