# 03 July 2000

TTC  INTERFACE  MODULE   ( TIM ) :

MODULE  CONNECTIONS  to the  BACKPLANE

# Update Log :

##### 03 Jul. 2000, MP-UCL : Corrected & changed as per comments from

##### ROD Workshop, UCL, 15-16 June 2000

##### 09 Jun. 2000, MP-UCL : Changed as per final “SCT Off Detector Crate

##### Backplane” document, dated 22 May 2000

07 Dec. 1999, MP-UCL : Removed unused VME signals

01 Dec. 1999, MP-UCL : Added Jaux & JBL minor changes

30 Nov. 1999, MP-UCL : Simplify & JBL minor corrections

# 29 Nov. 1999, MP-UCL : TIM backplane connections only

This document is a description of the actual TIM0 module connections to the ROD Crate Backplane.

It follows the Interface Specification Document, as shown in the

“SCT Off Detector Crate Back Planes and Crate Card Utilization”

Version 1.0,	 8 Oct. 1999

Version 2.n	22 May  2000

P1 Connector on TIM module :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | D00 |  | D08 | +5V | 1 |
| 2 | ## Gnd | D01 |  | D09 | ## Gnd | 2 |
| 3 |  | D02 |  | D10 |  | 3 |
| 4 | ## Gnd | D03 |  | D11 |  | 4 |
| 5 |  | D04 |  | D12 |  | 5 |
| 6 | ## Gnd | D05 |  | D13 |  | 6 |
| 7 |  | D06 |  | D14 |  | 7 |
| 8 | ## Gnd | D07 |  | D15 |  | 8 |
| 9 |  | Gnd |  | Gnd | GAP* | 9 |
| 10 | ## Gnd |  |  |  | ## GA0* | 10 |
| 11 |  | Gnd |  | BERR* | ## GA1* | 11 |
| 12 | ## Gnd | DS1* |  |  | ## +3.3V | 12 |
| 13 |  | DS0* |  | LWORD* | ## GA2* | 13 |
| 14 | ## Gnd | WRITE* |  | AM5 | ## +3.3V | 14 |
| 15 |  | ## Gnd |  | A23 | ## GA3* | 15 |
| 16 | ## Gnd | DTACK* | AM0 | A22 | ## +3.3V | 16 |
| 17 |  | Gnd | AM1 | A21 | # GA4* | 17 |
| 18 | ## Gnd |  | AM2 | A20 | ## +3.3V | 18 |
| 19 |  | ## Gnd | AM3 | A19 |  | 19 |
| 20 | # Gnd | IACK* | ## Gnd | A18 | ## +3.3V | 20 |
| 21 |  | IACKIN* |  | A17 |  | 21 |
| 22 | ## Gnd | IACKOUT* |  | A16 | ## +3.3V | 22 |
| 23 |  | AM4 | Gnd | A15 |  | 23 |
| 24 | ## Gnd | A07 | IRQ(7)* | A14 | ## +3.3V | 24 |
| 25 |  | A06 | IRQ(6)* | A13 |  | 25 |
| 26 | ## Gnd | A05 | IRQ(5)* | A12 | ## +3.3V | 26 |
| 27 |  | A04 | IRQ(4)* | A11 |  | 27 |
| 28 | ## Gnd | A03 | IRQ(3)* | A10 | ## +3.3V | 28 |
| 29 |  | A02 | IRQ(2)* | A09 |  | 29 |
| 30 | ## Gnd | A01 | IRQ(1)* | A08 | ## +3.3V | 30 |
| 31 |  | ## -12V | ## +5V | ## +12V | Gnd | 31 |
| 32 | ## Gnd | ## +5V | ## +5V | ## +5V | ## +5V | 32 |

GND:		26 pins

+5V		 6 pins

+3.3V	10 pins

-12V		 1 pin

+12V		 1 pin

P2 Connector on TIM module :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | ## +5V |  |  | 1 |
| 2 | Gnd |  | ## Gnd |  |  | 2 |
| 3 |  |  |  |  |  | 3 |
| 4 | Gnd |  | A24 | -5.2V (#) |  | 4 |
| 5 |  |  | A25 |  |  | 5 |
| 6 | Gnd |  | A26 |  |  | 6 |
| 7 |  | -5.2V (#) | A27 |  |  | 7 |
| 8 | Gnd |  | A28 |  |  | 8 |
| 9 |  |  | A29 |  |  | 9 |
| 10 | Gnd |  | A30 |  |  | 10 |
| 11 |  |  | A31 |  |  | 11 |
| 12 | Gnd |  | ## Gnd |  |  | 12 |
| 13 |  | -5.2V (#) | ## +5V |  |  | 13 |
| 14 | Gnd |  |  |  |  | 14 |
| 15 |  |  |  |  |  | 15 |
| 16 | Gnd |  |  |  |  | 16 |
| 17 |  |  |  |  |  | 17 |
| 18 | Gnd |  |  |  |  | 18 |
| 19 |  | -5.2V (#) |  | -5.2V (#) |  | 19 |
| 20 | Gnd |  |  |  |  | 20 |
| 21 |  |  |  |  |  | 21 |
| 22 | Gnd |  | ## Gnd |  |  | 22 |
| 23 |  |  |  |  |  | 23 |
| 24 | Gnd |  |  |  |  | 24 |
| 25 |  |  |  |  |  | 25 |
| 26 | ## Gnd |  |  |  |  | 26 |
| 27 |  |  |  |  |  | 27 |
| 28 | ## Gnd |  |  |  |  | 28 |
| 29 |  |  |  |  |  | 29 |
| 30 | ## Gnd |  |  |  |  | 30 |
| 31 |  |  | ## Gnd |  | Gnd | 31 |
| 32 | ## Gnd |  | ## +5V |  | +5V | 32 |

GND:		21 pins

+5V:		 4 pins

-5.2V  	 5 pins

Note :	(#)	-5.2V supply pins on TIM P2 are for prototype testing purposes in VME/VXI-type crates at UCL or CERN only, and will not be provided on the final version. All the above -5.2V pins have links for isolation

P3 Connector on TIM module :

| Pin | Row Z | Row A | Row B | Row C | Row D | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | ## Gnd |  | ## +3.3v | # ROD-Busy6 | 1 |
| 2 |  |  | Gnd | # ROD-Busy7 | ## +3.3v | 2 |
| 3 |  | CLK-40+ slot5 | ## CLK-40- slot5 | ## +3.3v | # ROD-Busy8 | 3 |
| 4 | Gnd | CLK-40+ slot6 | CLK-40- slot6 | ROD-Busy9 | ## +3.3v | 4 |
| 5 |  | Gnd |  | ## Gnd |  | 5 |
| 6 |  | CLK-40+ slot7 | CLK-40- slot7 | ROD-Busy10 | ## +3.3v | 6 |
| 7 |  | ## CLK-40+ slot8 | ## CLK-40- slot8 | ## +3.3v | ROD-Busy11 | 7 |
| 8 |  | CLK-40+ slot9 | CLK-40- slot9 | ## +3.3v | ## +3.3v | 8 |
| 9 | Gnd |  | ## Gnd | ROD-Busy12 | ## +3.3v | 9 |
| 10 |  | CLK-40+ slot10 | CLK-40- slot10 | ## Gnd | ## +3.3v | 10 |
| 11 |  | Gnd |  | ROD-Busy13 | Laser- Interlock | 11 |
| 12 |  | CLK-40+ slot11 | CLK-40- slot11 | ROD-Busy14 | Gnd | 12 |
| 13 |  | ## CLK-40+ slot12 | ## CLK-40- slot12 | ROD-Busy15 | ROD-Sense(1) | 13 |
| 14 | Gnd |  | Gnd | ## ROD-Busy16 | ROD-Busy17 | 14 |
| 15 |  | CLK-40+ slot13 | ## CLK-40- slot13 | ## Gnd | ROD-Busy18 | 15 |
| 16 |  | CLK-40+ slot14 | CLK-40- slot14 | ROD-Busy19 | ROD-Busy20 | 16 |
| 17 |  | Gnd |  | ROD-Busy21 | +5V | 17 |
| 18 |  | CLK-40+ slot15 | CLK-40- slot15 |  | CLK-40+ (5) | 18 |
| 19 | +5V | CLK-40+ slot16 | ## CLK-40- slot16 |  | CLK-40- (5) | 19 |
| 20 |  | # Gnd |  | ## Gnd | +5V | 20 |
| 21 |  | CLK-40+ slot17 | ## CLK-40- slot17 |  | TIM-Busy Out | 21 |
| 22 |  | CLK-40+ slot18 | CLK-40- slot18 |  | +5V | 22 |
| 23 |  |  | ## Gnd | ## TTC-7B | ## TTC-7A | 23 |
| 24 | +5V | CLK-40+ slot19 | CLK-40- slot19 | ## TTC-6B | TTC-6A | 24 |
| 25 |  | CLK-40+ slot20 | ## CLK-40- slot20 | ## Gnd | TTC-5A | 25 |
| 26 |  | ## Gnd |  | # TTC-5B | TTC-4A | 26 |
| 27 |  | CLK-40+ slot21 | ## CLK-40- slot21 | TTC-4B | ## +5V | 27 |
| 28 |  |  | Gnd | TTC-3B | TTC-3A | 28 |
| 29 | +5V |  |  | TTC-2B | TTC-2A | 29 |
| 30 |  |  |  | ## Gnd | TTC-1A | 30 |
| 31 |  | Gnd |  | # TTC-1B | TTC-0A | 31 |
| 32 |  |  |  | ## TTC-0B | ## +5V | 32 |

GND :	22 pins

+5V :	 8 pins

+3.3V :	10 pins ( ARE NOT USED BY TIM )

## Note:	The TIM module must only be used in its dedicated slot 5

Note:	The ground pins have been selected so that no damage will occur if the card is put in any slot

Note:	The signal pin selection may cause damage to the TIM or BOC.

Note:	(1)	ROD-Sense: Grounded on all RODs and on backplane in

slot 5 only, pulled high on TIM and BOC by 1k resistor

(4)	CLK-40 +/- slotNN: from TIM to BOC only, all clock lines have the same electronic length and are 100 ohm impedance

(5)	CLK-40 +/- : return to TIM. This is the return signal from pin 3 Row A and B above. It is the same electrical length as all the other clock lines from TIM to BOC, and the same 100 ohm impedance

- All CLK-40 +/- are differential PECL signal pairs.

- They all have 270R resistor to GND on TIM and have 100R resistor between differential pairs on BOC

Note:	-	TTC-0 to TTC-7 are LOW GOING TTL signals bussed to all

RODs. They are not used on BOCs.

These lines are terminated on the backplane in slot 21

Impedance about 50 ohms with all ROD cards plugged in

Note:	-	ROD-BUSY(6-21) are LOW GOING LV signals, point-to-

point from all RODs. They are not used on BOCs.

These lines are pulled high on TIM by 10k resistor

to +3V3

Paux ( “Jaux” ) Connector on TIM :

Note :	Paux not soldered in – pads only provided

| Pin | Row A | Row B | Row C | Pin |
| --- | --- | --- | --- | --- |
| 1 |  | Gnd |  | 1 |
| 2 |  | Gnd |  | 2 |
| 3 |  | Gnd |  | 3 |
| 4 |  | Gnd |  | 4 |
| 5 |  | Gnd |  | 5 |
| 6 |  | Gnd |  | 6 |
| 7 |  |  |  | 7 |
| 8 |  |  |  | 8 |
| 9 | -5.2V (#) | -5.2V (#) | -5.2V (#) | 9 |
| 10 | -5.2V (#) | -5.2V (#) | -5.2V (#) | 10 |

GND:		6 pins

-5.2V  	6 pins

Note :	(#)	-5.2V supply pins on TIM Paux are for prototype testing purposes in VME/VXI-type crates at UCL or CERN only, and will not be provided on the final version. All the above -5.2V pins have links for isolation