Resonant Bond Wire Vibrations in the ATLAS SemiConductor Tracker

# Introduction

# Calculations of Resonant Bond Wire Vibrations

# Experimental Study of Resonant Bond Wire Vibrations

# Trigger Veto Algorithms

As a result of the combination of the favourable orientation with respect to the magnetic field (barrel opto-package) and short bond wires (forward opto-package and ASICs) the amplitudes of any resonant vibrations will be very small. Although they would not be expected to lead to any immediate failures, metal fatigue is still a concern for the longer term operation. Since the production of the components is well advanced the option of adding a potting compound at the feet of the wire bonds to damp the oscillations is not a practical option. A large additional safety factor can be obtained if one can avoid any fixed frequency trigger operation as explained in Section 4.1. A proposed algorithm to veto fixed frequency trigger operation is described in 4.2.

## Frequency Spectrum of Random Triggers

In normal ATLAS data taking, Physics triggers will occur at random times so that the frequency spectrum associated with currents in the read out circuitry will have a very broad frequency spectrum. A very simple Monte Carlo simulation of the time structure of the readout was used to calculate this frequency spectrum. L1 triggers were generated at an average frequency of 75 kHz (the maximum trigger rate expected in ATLAS). The current pulse associated with the resulting readout was generated from a uniform distribution between 0 and 10 s, although it was checked that the frequency spectrum was very insensitive to the nature of this distribution. The calculated frequency spectrum is shown in . The frequency spectrum is very wide and the fraction of the power contained in a resonance at f=18 kHz and df=100 Hz is only xx%. Therefore the danger due to resonant wire bond vibrations from Physics triggers is greatly reduced compared to operation with a fixed frequency clock operating at a wire bond resonant frequency.

## Proposed Trigger Veto Algorithm

Although fixed frequency running will not occur in normal Physics operation in ATLAS, there is a danger that it will occur in special runs during development and tests of the trigger system. Therefore a fixed frequency trigger veto algorithm will be implemented in the SCT TTC (Timing, Trigger and Control) Interface Module (TIM).

A TIM is located in each SCT Read-Out Driver (ROD) crate, where it interfaces and fans-out triggers from the TTC to the ROD’s via the crate backplane. It also aggregates the busy signals from each ROD card, and transfers this back up the chain to the TTC system. This is, therefore, an ideal place to locate an SCT sub-detector specific fixed frequency trigger veto (FFTV) system.

As the TIM design is into its final pre-production phase, the FFTV is required to fit into the existing architecture. The TIM’s main functions are implemented in programmable logic chips (FPGA’s) and, as such, it has no on-board processing. To avoid a large scale redesign, it is best the the FFTV be built using spare logic resources inside an existing FPGA chip. To ensure the system could be integrated easily, it was felt that the focus should be on keeping it as small as possible.

### Fixed Frequency Detection

Using Fourier analysis seemed like an obvious choice as FFT’s are widely used in programmable-logic designs. Early trials were with a user-customisable FFT ‘core’ supplied ready-made with Xilinx firmware development software (***ref:  http://www.xilinx.com/ipcenter/catalog/logicore/docs/vfft32.pdf). Testing the design showed it to be resource hungry - using over 50% of a large FPGA - and over specified for the application. To keep resource utilisation to a minimum, focus was shifted toward a more application specific algorithm.

CDF originally used a simple algorithm to veto against dangerous triggers: If the number of triggers in a fixed period exceeds a pre-defined limit, then further triggers were vetoed for a specified safe period of time (***ref? http://www-pnp.physics.ox.ac.uk/~weidberg/Resonant_Bond_Wire_Problem.doc?***). Although effective, this provided little means to tune the system, and did not seem to address problems relating to non-uniform, yet similar, trigger distribution. None the less, this was a good starting point for the development of the SCT system.

### Repetitive Period Monitoring

To detect fixed-frequencies it is easier to think in terms of repetitive-periods, and work with these values on an FPGA. By counting clocks between each trigger and storing the results, successive trigger periods are checked for equality (i.e. a match). Each time there is match, a match-counter is incremented, with transgression of a pre-defined limit generating a veto. If there is no match the match-counter is decremented.

In the cases where triggers stop, the period-counter continues to user-defined maximum and rolls-over to zero. Each time it rolls-over the match-counter is decremented. This sets the decay-rate of the match-counter.

As mentioned earlier in this paper, dangerous trigger rates are those at around ~17kHz (***ref? ***correct?) By setting a limit for the period-counter, above which no action is taken, a low-pass filter is created allowing lower frequency and harmless triggers to run at full efficiency. Adding a lower period-count value – below which no action is taken – creates a band-pass filter. Setting these maxima and minima allow the system to be tuned to a particular band of trigger frequencies.

### Period Matching Accuracy

To ensure trigger-periods with only a slight mismatch are still detected, the delta-period value of successive triggers was calculated and period-match decided on the size of this number. Increasing this threshold allows a relatively wide range of triggers to be matched. This could be called something like a dynamic-centred band-pass filter.

### Operation

Operation is best demonstrated using the output from the ModelSim shown below

In all cases the system was configured with the following values:

Period Upper Limit 		2850 (71ms = 14kHz)

Period Lower Limit 		2200 (55ms = 18kHz)

Period Roll-over 		4100 (102ms)

Delta-Period			127 (+/- 3ms)

Period Match Threshold	5

Veto Duration			8100 (200ms)

Figure 4.1*** - Demonstrates the Veto generated after 5 successive triggers, as well as trigger matching non identical trigger periods. Note that during a Veto, the period-counter continues counting beyond the scope of this plot.

Figure 4.2*** - Demonstrates a pair of repetitive triggers - running at equal rates, but offset – still generating a Veto.

Figure 4.3*** - Demonstrates trigger below and above threshold being ignored. Note that short periods are ignored and used as part of longer period components. (*** this may not be the correct action, but it is require to the functionality in 4.2 above***)

### Fitting Into Hardware

The system as described was built as a user configurable module (either through compile-time constants, or registers accessible to the user). Compiling and fitting it for a Xilinx Spartan II 600E FPGA, as used on the TIM, generated a configuration utilising just 2% of logic-resources.

# Conclusions

Resonant bond wire vibrations were studied by imaging the bond wire with a high magnification CCD camera. A novel way of detecting resonant bond wire vibrations in a magnetic field was developed based on measuring the back emf. The two methods gave consistent results but the back emf method was sensitive to lower amplitude vibrations.

Resonant bond wire vibrations will occur if there is a time varying current perpendicular to a magnetic field. If the bond wires are longer than about 2mm and the magnetic field is in the plane of the loop of the bond wire currents of a few mA with a frequency close to the resonant frequency of the wire bond will lead to large amplitude vibrations. These large amplitude vibrations are very dangerous and will typically cause the wire bond to fail after a few minutes operation. The problem turns out to be less severe in the ATLAS SCT because of a combination of the favourable orientation of the wire bonds with the magnetic field (barrel opto-package) and very short wire bonds (End Cap). A large additional safety factor can be obtained by vetoing fixed frequency running and a suitable algorithm has been developed and will be implemented in ATLAS.

# Acknowledgements

Financial support from the UK Particle Physics and Astronomy Research Council is gratefully acknowledged. We would like to thank Dr Joel Goldstein (RAL) for explaining the resonant wire bond vibration problems seen in CDF.

References