Resonant Bond Wire Vibrations in the ATLAS SemiConductor Tracker

D. G. Charlton

Birmingham University, UK

T.J. Barber

Cambridge University, UK

A.R. Weidberg

Physics Department, Oxford University, UK

W. Murray

Rutherford Appleton Laboratory, UK

Abstract

If time varying currents are passed through bond wires in a magnetic field, there can be dangerous mechanical resonances which can lead to the breaking of the bond wires. The results of analytic calculations and an FEA analysis are described. The results of experimental investigations using wire bonds on test PCBs are given. The possible effects within the SCT were investigated and a fixed frequency trigger veto algorithm designed to minimise the dangers of breaking wire bonds was developed.

PACS: 42.88, 04.40N, 85.40, 85.60.

Keywords: LHC; ASICs; Resonances.

# Introduction

In the presence of a magnetic field, B, a wire of length l, carrying a current, I, will experience a Lorentz force, F=B I l. For a typical bond wires of length a 2 mm carrying a currents of  10 mA in a magnetic field of 2T the magnitude of this force is 4 10-5 N, which is very much lower than the minimum force required to break a wire bond, which is of the order of 0.1 N. Therefore even relatively large DC currents do not represent any significant danger. However if the current is varying with time at a frequency close to one of the mechanical resonance frequencies of a bond wire, then a strong resonant vibration will start, which will usually lead to the failure of the bond-wire after a period of a few seconds to minutes. Such effects were seen by the CDF collaboration in the SVXII detector and several detector modules failed because of the resulting broken wire bonds. CDF implemented administrative measures to avoid any dangerous fixed frequency running conditions and have not subsequently seen any more such problems[].

Optical links will be used in the ATLAS SemiConductor Tracker (SCT) and Pixel detector[,] to transmit data from the detector modules to the off-detector electronics and to distribute the Timing, Trigger and Control (TTC) data from the counting room to the front-end electronics[]. The currents from the VDC ASIC to the on-detector VCSELs form 25 ns long pulses when a data “1” is being transmitted. Therefore there will be a burst of current during the transmission of event data after receipt of a level 1 (L1) trigger. There is a potential danger of resonant wire bond vibrations if  L1s are issued at a fixed frequency close to a resonant frequency.

In Section 2 a general theoretical analysis of resonant wire bond vibrations is given. A short summary of analytic calculations is given and then the results of more detailed FEA calculations are given. Experimental measurements of resonant wire bond vibrations for test wire bonds are given in Section 3. The results of searches for resonances with test structures with wire bonds identical to those that will be used in ATLAS are also presented in Section 3. In order to minimise the danger due to resonant wire bond vibrations an algorithm was developed to veto dangerous fixed frequency triggers and this is presented in Section 4. Finally some conclusions are drawn in Section 5.

# Calculations of Resonant Bond Wire Vibrations

The results of simple analytic calculations are given in Section 2.1 and the results of more detailed FEA calculations are given in Section 2.2.

## Analytic Calculations

In a simple analytical model of a bond wire, consider a straight beam of material, length l, which is clamped at both ends. If the wire were constrained to oscillate in only one dimension, it would be expected that the first two normal modes of oscillation have the form as shown in Figure 1. For a higher frequency mode, n, there would be n-1 nodes, but the shapes would be similar.

l

l

1st Resonance

2nd Resonance Resonance

Figure 1 Shape of the first two normal modes.

To find the shape and frequencies of the equations, we can consider the wire as a loaded beam[]. It can be shown that the differential equation for the vertical displacement, y, of a beam of length l is:

where F is the force per unit length acting on the bar and D is its flexural rigidity,

equal to:

where E is the Young’s modulus of the material, I its moment of inertia and  the Poisson’s ratio. Solving the differential equation with boundary conditions for a beam clamped at both ends:

gives the solution of:

This is the shape of the first normal mode as sketched above. The maximum displacement occurs at x=l/2 and has a value of:

Rearranging this equation (5) gives the restoring force per unit length for a given vertical displacement. Equating this with the subsequent acceleration yields the equation for simple harmonic motion.

where A is the cross sectional area of the wire and  is the density. For a wire of circular cross sectional area and diameter d and substituting a value of I = d4/64 for the moment of inertia gives a frequency of oscillation of:

For the second normal mode there is a similar dependence on length and diameter, but the term inside the square root is slightly different.

The main conclusions from these analytic calculations are that the amplitude of the resonant vibration scales proportional to the square of the length of the wire and that the fundamental frequency scales as the inverse fourth power of the length. Therefore longer bond wires are expected to experience more dangerous vibrations than shorter ones.

## FEA Calculations

A number of bond wires were constructed using the JL Analyser FEA package[]. The parameters used for constructing these models where the bond length, diameter and the loop height. A simple quadratic curve was fitted to these variables to give an estimation of the bond shape, with a 0.1mm foot length at either end.  The model was then meshed as a circular beam and given the following properties consistent with wire bonds for ATLAS:

| Young’s Modulus | 69Gpa |
| --- | --- |
| Density | 2710 kgm-3 |
| Poisson’s Ratio | 0.330 |

A side view of the bond model is shown in Figure 2.

Figure 2 FEA model for bond wire.

To mimic attachment to the bond pad, restrictions in all degrees of freedom where imposed on both of the bond feet. Frequency analysis was performed on a number of bonds to find the frequencies of the normal modes. The shapes of the first five modes are shown in figure

Figure 3 Shapes of some normal modes.

The shapes of the odd numbered modes are those predicted using the analytic beam model earlier (compare with Figure 1). The even numbered modes correspond to displacements in the plane of the wire. As there will be Lorentz forces in both directions due to the different orientations of hardware in the ATLAS B-field, both types will be considered.

A plot of the normal mode frequencies versus bond wire length is shown in Error: Reference source not found and a good fit can be obtained to  as expected from the analytic calculations of Section 2.1. The FEA calculations also showed that the normal mode frequencies have a direct proportionality with bond diameter as expected from the analytic calculations. The FEA calculations also predicted some variation of the resonant frequency with bond loop height. For a range of bond-heights between 0.2 mm and 0.3 mm the expected change in resonant frequency was ~9%.

Figure 4 Plot of FEA calculated resonant frequency versus bond wire length.

Some FEA calculations were also carried out with ANSYS, a more advanced FEA package, using a more realistic bond wire, with l=2.6mm and h = 1.4mm. This showed the same shapes of normal modes as before, with a fundamental frequency at 13.5kHz. A spectrum analysis could also be performed using a sinusoidal Lorentz driving force of 5.2e-5N. The forces were applied in both the Y (perpendicular to the plane of the bond wire loop) and Z (in the plane of the bond wire loop) directions to simulate the possible orientations in the magnetic field in ATLAS. The responses are shown in figures Figure 5 and Figure 6.

These show that the Y & Z direction forces will excite the even and odd numbered modes respectively. The plots show that the Y direction amplitude is roughly two orders of magnitude smaller than that for the Z. In the Y direction case the amplitude is an order of magnitude less than the diameter of a bond wire, so will probably not be a problem in ATLAS. The fundamental of the Z direction response shows a much larger magnitude and should be easy to detect and would be expected to lead to bond wires breaking if the resonance was excited.

Figure 5 Frequency response in the Y direction.

Figure 6 Frequency response in the Z direction.

## Summary of Calculations

The main conclusions from the analytic and FEA calculations are that The resonant frequency depends on the length of the bond wire asand the amplitude scales as . Therefore shorter bond wires will have very high resonant frequencies with very small amplitudes whereas longer bond wires will have lower resonant frequencies and much larger amplitude oscillations.

# Experimental Study of Resonant Bond Wire Vibrations

The first very simple tests were carried out using a video camera to image test wire bonds, placed inside the Birmingham superconducting magnet facility. The magnetic field was limited to 0.5T to avoid degradation of the image quality but the magnitude of the current was increased to 40 mA to compensate. The test wire bonds were made with the same length as those used in the ATLAS SCT barrel opto-package to connect the VDC ASIC to the VCSELs. The  magnetic field was in the plane of the loop of the bond wire, i.e. perpendicular to that which will be used in ATLAS. The frequency of the current through the bonds were scanned in the range 10 to 100 kHz and strong resonances were visible at a frequency around 15 kHz. When the set-up was left running on resonance, the wire bonds were observed to suddenly fail after a few minutes. This showed that there was a potential problem with resonant wire bond breaking in ATLAS.

More detailed studies were carried out at the Rutherford Appleton Laboratory using a 1.8T dipole magnetic field. Two different techniques were used to study the wire bond vibrations:

- The wire bonds were viewed directly with a CCD camera equipped with a high magnification lens. The images were read out at an adjustable frequency as the frequency of the current through the wire bond was scanned. Since the resonance frequencies were much higher than the inverse of the time per frame, any vibrations of the wire bond would lead to the image of the wire bond being fuzzy. A very crude estimate of the amplitude of the vibration could be obtained from the width of the fuzzy image of the wire bond.

- The presence of a vibrating wire bond in the magnetic field leads to a back e.m.f. across the wire bond. A step pulse was applied to the wire bond with an amplitude of 1V and a duration of 1ms. The voltage across the wire bond was monitored with an oscilloscope to look at the resulting free oscillations at the end of the pulse. The signals were averaged over a large number (1000 to 4000) sweeps to reduce the effects of noise. A Fast Fourier Transform (FFT) was performed by the oscilloscope. Any resonant wire bond vibrations would show up as peak(s) in the resulting FFT spectrum.

The results with test wire bonds are discussed in Section 3.1and the search for resonant wire bind vibrations with ATLAS SCT components are presented in Section 3.3.

## Test Wire Bonds

The first set of experiments uses test bond wires to help classify the general nature of the oscillations and resonant frequencies. Short bond wires (lengths less than 1 mm) were used initially and as expected no resonances large enough to be seen with the video camera were observed. Clear evidence for resonant vibrations were obtained using the back e.m.f. method. The response of the system with and without the field is

shown in Figure 7, averaged over 1000 sweeps. The response shown is from track 4 on the test board. (The “track” quoted throughout section 4 is simply an arbitrary experiment number.)

Figure 7  Response measured on a short bond wire to a transient pulse with the magnetic field on and off.

With the field on there appears to be a high frequency oscillation superimposed onto a lower frequency signal (~1kHz). This low frequency is unlikely to be bond wire oscillations, but could be due to other components of the board reacting to the magnetic field. The most likely candidates are the wires used to connect the signal cable to the actual bonds. From the simulations, a signal in the 1kHz range could be caused by a 5cm length of copper wire, of comparable to the lengths of actual wires used. The higher frequency signal is likely to be a bond wire oscillating. An FFT of the two signals is shown in figure 4.2.

Figure 8 FFT spectrum of back emf signal from a short bond wire (with and without magnetic field).

Figure 9 FFT of back emf signal from a short bond wire.

There is a clear peak in the FFT which does not appear with the field switched off. The peak also shifts from positive to negative at its highest point, implying a phase change across this region. The peak occurs at 89.50.5kHz which is consistent with a bond wire with a length of 0.85mm and a loop height of 0.3mm.

To investigate this further, the wire above and three others were tested in the field but this time averaged over 4000 sweeps to decrease errors. Four wires were tested and their frequencies are shown in Table 1. In two cases the negative side of the peak was not seen; this may be because the resonance is too narrow for the FFT resolution so only the positive peak is displayed. There was a significant variation in amplitudes of the peaks compared to the surrounding level. Visual inspection of the wires after the experiment revealed that the smaller amplitude wire had been distorted which could be the cause.

Table 1 Resonant frequencies for short bond wires

| Track Number | Frequency / kHz | Amplitude / dB |
| --- | --- | --- |
| 3 | 74 | 5.1 |
| 4 | 90 | 5.8 |
| 5 | 75 | 2.7 |
| 6 | 69 | 12.2 |
| Mean: | 77 | 6.5 |
| Std. Dev: | 9 | 4.0 |

The standard deviation corresponds to a frequency variation of ~12%, giving a length variation of 6%. Assuming all of this error comes from the placement of two bond feet at each end of the wire, the accuracy of the bonding machine would be ~ 27um.

An FFT of the voltage with no bond wire, (i.e. a shorted track) showed no peak in the spectrum, but still contained the low frequency resonance, implying it is not a bond wire effect. Also changing the orientation in the field to give a force in the plane of the wire loop also showed no peak. This is expected as a different set of modes are excited by this orientation of field, with much lower amplitudes (see section 2.3). It would therefore be expected that the peaks moved and would be too small to be detected.

Longer bonds were also tested to try and obtain visual confirmation of the resonance. The back e.m.f method was used tio locate the approximate position of the resonant frequencies before trying to view them directly. As expected these bonds show lower resonant frequencies with much larger amplitudes due to their length. An example of the back e.m.f. FFT for one of the bonds is shown in Figure 10.

Figure 4.4

Figure 10 FFT of back emf signal for a long wire bond.

The resonant frequency is now much clearer at 17.00.2 kHz and there is a hint of a first harmonic at 34.60.2 kHz. The resonant frequencies of four wires were measured using this method and are summarized in Table 2

Table 2 Resonant frequencies for long bond wires.

| Bond Track | Frequency / kHz |
| --- | --- |
| 5 | 17.0 |
| 6 | 17.8 |
| 7 | 18.6 |
| 8 | 18.0 |
| Mean: | 17.9 |
| St. Dev: | 0.7 |

The FEA prediction for a bond wire of 2.3mm and a 0.3mm loop height is 18.0 kHz, and so is in very good agreement. Once again, the mean and standard deviation correspond to a bond foot placement error of 22um.

The bond wire on track 7 was scanned with a constant current sine wave of 10mA from 15kHz to 20kHz and the wire width appeared to increase as it reached resonance. Closer to the resonant frequency the wire is clearly oscillating, as the extremes of movement appear bright with a dark area between. The largest amplitude of oscillations occurred at 17.4 kHz, which is significantly lower than predicted by the back e.m.f. method. This may be because the system is not linear and the low frequency components in the pulse may affect the bond wire resonance. Photos of the oscillating bonds are shown in

When trying to scan other bonds with sinusoidal currents the resonance seemed to disappear. After showing a clear peak with the e.m.f. method and initial oscillations close to resonance a scan over the frequencies showed no difference in width. Returning to the e.m.f. method revealed the peak had disappeared. This happened with the three remaining bonds and the cause is still unknown. It is possible that initial fatigue due to driving at resonance or close to it for a short time may prevent oscillations occurring. The wires may have warmed making them more malleable or

cracks may prevent oscillation.

Figure 11 photo of wire bond with a current of a frequency of 15 kHz, away from resonance.

Figure 12 photograph of a wire bond with a current of frequency of 17 kHz, on resonance

## Q Factor of Resonances

The quality factor of an oscillation (Q – factor) is defined by:

where  is the angular frequency, <U> is the average energy stored per cycle, <P> is the average power dissipated per cycle. The Q factor for the resonances of the 2.3mm bonds can be evaluated by three different methods.

### Transient Response

The transient responses of the track 3 (overbonded) and 5 (normal) bonds as measured using the back e.m.f. method were fitted to a series of three decaying sinusoidal oscillations (corresponding to damped SHM solutions) of the form:

E.M.F. / V

Time after pulse / s

A plot of the fit is shown in .

Figure 13Fit of transient response signal.

The fit shows some low frequency components at around 1kHz, probably due to other wires in the system, and a higher frequency component at around 17kHz, due to the bond wire. The Q value in this case can be derived as follows:

A summary of the high frequency components’ properties are shown in Table 3. Both of the frequencies agree with the previous measurements from the FFT peaks within experimental error.

Table 3 Summary of fit values.

| Track | Frequency / kHz | Gamma / Hz | Quality Factor |
| --- | --- | --- | --- |
| 5 | 17.3±0.4 | 77.2±1.9 | 112±4 |

### Width of Power Spectrum

The Q - factor can also be given by:

where 0 is the resonant frequency and  is the full width half maximum (FWHM) of the peak in the power spectrum. This can be evaluated from the FFT curves obtained from the back e.m.f. method. A summary of the results is shown in Table 4.

Table 4 Summary of fit values. The error on all frequency measurements is ±50Hz.

| Track | Frequency f0 / kHz | Bandwidth f / Hz | Q Factor |
| --- | --- | --- | --- |
| 1 | 17.37 | 250 | 69±14 |
| 2 | 13.81 | 200 | 69±18 |
| 3 | 17.30 | 340 | 51±8 |
|  |  | Mean: | 63 |
|  |  | St. Dev: | 10 |
| 5 | 16.90 | 320 | 56±9 |
| 6 | 17.76 | 270 | 65±12 |
| 7 | 18.60 | 150 | 124±41 |
| 8 | 17.99 | 190 | 95±25 |
|  |  | Mean: | 85 |
|  |  | St. Dev: | 40 |

### Amplitude versus Frequency Fitting

Photo analysis of track 7 at frequencies close to resonance gives an estimate of the actual amplitude of the oscillating wires. The square of this amplitude gives a value proportional to the power spectrum of oscillations that can be fitted to the Lorentzian peak predicted from driven SHM solutions:

where H is the peak height, 0 the resonant frequency, γ the FWHM of the peak. The best fit gives values of f0 = 17.4 kHz, f = 100Hz and so Q = 172 ± 43. This is in agreement with the FFT method in the previous section for track 7. The Plot of the data points and the fitted curve are shown in and indicate a good fit at frequencies close to and far from resonance, but seems to underestimate between the two extremes. This may indicate that there may be other factors in the system not accounted for by the Lorentzian.

In conclusion, the quality factor of the oscillations is 137±38 from a combination of methods and bond wires. All but tracks 7 & 8 have been ignored from the FFT peak method. At this precision there seems to be no difference between over-bonded and normal wires.

## Wire Bonds in ATLAS SCT Components

Having investigated the phenomenon of bond wire resonance in test bond wires, more specific tests were done on pieces of ATLAS hardware that could be at risk from resonance. This includes any bond wires that could carry signals at constant frequency with high enough current to excite the bonds.

The wires bonds that are vulnerable to resonant wire bond vibrations are those that carry current from the VCSEL driver ASIC (VDC)[] to the VCSELs[] in the opto-packages[,]. In the barrel opto package (mounted on a Cu/kapton flex circuit) the magnetic field will be perpendicular to the wire loop (“barrel” mode), so will excite the even modes as in section 2.2, which are likely to have very small amplitude. The forward opto packages (mounted onto the forward SCT hybrids) have bonds with the magnetic field in the plane of the wire loop, so excite the more the dangerous odd modes (“forward” mode).

### Barrel Opto-package

The bonds that carry analogue output signals from the VDC chip to the VCSEL chip on the barrel opto packages were monitored with the camera and scanned over a frequency range from 16 kHz to 20 kHz with a 10mA square wave, to simulate a worst case ATLAS scenario. Both the “forward” and “barrel” orientations of field were tested although in ATLAS only the “barrel” case will occur. As the current through the bonds is controlled by a chip it was not possible to monitor the voltage across them directly and so the back e.m.f. method could not be used.

In “forward” mode two sets of resonant frequencies were found for each wire, a fundamental frequency, f, and a resonance at f / 3. The lower frequency square wave will contain a Fourier component at the fundamental and hence should excite it with lower amplitude. The fundamental frequencies of the two wires were 17.2 kHz and 18.0 kHz (±0.1kHz). This is slightly higher than that expected from the FEA calculations which predicted a fundamental frequency in the range 13.2 kHz to 14.0 kHz corresponding to the length range of 2.74mm to 2.65mm with a loop height of 0.3mm. The loop height variation may be responsible for this discrepancy.

In “barrel” mode no resonance could be seen up to 50 kHz despite a predicted a resonant frequency between 33 kHz and 36 kHz. An amplitude of one bond wire would have been detected, so one can conclude that the amplitude of any resonances must be smaller than 25 m.

### Forward Opto-Package

The two bond wires to the VCSEL chips are in the “forward” orientation but are fairly short (less than 1mm) so are expected to have high frequency, low amplitude oscillations. The wires were driven directly through a resistor to give 25mA. Only 10mA is expected in normal ATLAS operating conditions.

Figure 5.3 Forward Opto-Package photographs. a) shows the overall package with the wires out of view. b) shows the bond wires from the gold tracks to the VCSEL chips.

F

B

I

a)

b)

Figure 14 SCT EC opto-package. (a) view of opto-package and 3 fibres. (b) wire bonds from PCB tracks to the VCSELs.

The back e.m.f. method showed no peaks up to a frequency of 500 kHz. This puts an upper limit on resonant peaks at 4uV compared to the 80uV peak for the 2.3mm bonds. Visual scans up to 150 kHz revealed no oscillations with amplitude greater than about ¼ of a bond wire diameter. Four wires were tested in total.

The wires are estimated to be 0.67mm long with a 0.21mm loop height. Predictions from FEA calculations give an extrapolated fundamental at 131±11 kHz. This is encouraging as there are unlikely to be triggers sent at above 100 kHz so any resonances above this will only be hit with higher frequency, lower amplitude Fourier components. This, combined with the low amplitude resonance, indicates that these bonds are unlikely to be at risk.

### Forward K5 Hybrid

Similar methods were used to investigate the LVDS bonds from the VDC chip on the K5 forward hybrid. The back e.m.f. method revealed no resonances of more than 4 V up to 500kHz. Visual scans were made of four different wires and also showed no resonance in the region between 50 kHz and 150 kHz. A typical ATLAS 10mA square wave current was used to pulse the wires.

The wires are ~0.7mm long which predicts a resonant frequency of   118±10 kHz. Their short length also means that a low amplitude would be expected, around 100 times smaller than the 2.3 mm test bonds due to the fourth power relationship. As the largest amplitude seen with the long wires was ~16 wire diameters, the largest corresponding amplitude for the K5 bonds is less than 0.2 diameters. As the bond wire showed up as 4 pixels wide on the CCD image, it is not surprising nothing was seen. On this scale it would be very hard to locate the exact resonant frequencies to test the bond wire durability. Prolonged exposure to low amplitude resonant pulse may still, however, be enough to break the bonds by metal fatigue.

# Trigger Veto Algorithms

As a result of the combination of the favourable orientation with respect to the magnetic field (barrel opto-package) and short bond wires (forward opto-package and ASICs) the amplitudes of any resonant vibrations will be very small. Although they would not be expected to lead to any immediate failures, metal fatigue is still a concern for the longer term operation. Since the production of the components is well advanced the option of adding a potting compound at the feet of the wire bonds to damp the oscillations is not a practical option. A large additional safety factor can be obtained if one can avoid any fixed frequency trigger operation as explained in Section 4.1. A proposed algorithm to veto fixed frequency trigger operation is described in 4.2.

## Frequency Spectrum of Random Triggers

In normal ATLAS data taking, Physics triggers will occur at random times so that the frequency spectrum associated with currents in the read out circuitry will have a very broad frequency spectrum. A very simple Monte Carlo simulation of the time structure of the readout was used to calculate this frequency spectrum. L1 triggers were generated at an average frequency of 75 kHz (the maximum trigger rate expected in ATLAS). The current pulse associated with the resulting readout was generated from a uniform distribution between 0 and 10 s, although it was checked that the frequency spectrum was very insensitive to the nature of this distribution. The calculated frequency spectrum is shown in . The frequency spectrum is very wide and the fraction of the power contained in a resonance at f=18 kHz and df=100 Hz is only xx%. Therefore the danger due to resonant wire bond vibrations from Physics triggers is greatly reduced compared to operation with a fixed frequency clock operating at a wire bond resonant frequency.

## Proposed Trigger Veto Algorithm

Although fixed frequency running will not occur in normal Physics operation in ATLAS, there is a danger that it will occur in special runs during development and tests of the trigger system. Therefore a fixed frequency trigger veto algorithm will be implemented in the SCT TTC (Timing, Trigger and Control) Interface Module (TIM).

A TIM is located in each SCT Read-Out Driver (ROD) crate, where it interfaces and fans-out triggers from the TTC to the ROD’s via the crate backplane. It also aggregates the busy signals from each ROD card, and transfers this back up the chain to the TTC system. This is, therefore, an ideal place to locate an SCT sub-detector specific fixed frequency trigger veto (FFTV) system.

As the TIM design is into its final pre-production phase, the FFTV is required to fit into the existing architecture. The TIM’s main functions are implemented in programmable logic chips (FPGA’s) so it has no on-board processing. To avoid a large scale redesign the system, it would best be built using spare logic resources inside an FPGA chip. To ensure the system could be integrated easily, it was felt that the focus should be on keeping it as small as possible.

### Fixed Frequency Detection

Using Fourier transforms seemed like an obvious choice as FFT’s are widely used in programmable-logic designs. Early trials with a user-customisable FFT ‘core’ that Xilinx supplies ready-made with it’s firmware development package (***ref:  http://www.xilinx.com/ipcenter/catalog/logicore/docs/vfft32.pdf) showed it to be resource hungry - using over 50% of a large FPGA - and over specified for the application. To keep resource utilisation to a minimum, focus was shifted toward a more application specific algorithm.

CDF originally used a simple algorithm to veto against dangerous triggers: If the number of triggers in a fixed period exceeds a pre-defined limit, then further triggers are vetoed for a specified safe period of time (***ref? http://www-pnp.physics.ox.ac.uk/~weidberg/Resonant_Bond_Wire_Problem.doc?***). Although effective, this provides little means to tune the system, nor does it seem to address problems relating to non-uniform yet similar trigger distribution. None the less, this is an excellent starting point for the system.

### Repetitive Period Monitoring

To detect fixed-frequencies it is easier to think in terms of repetitive-periods, and work with these values on an FPGA. By counting clocks between each trigger and storing the results, successive trigger periods are checked for equality (i.e. a match). Each time there is match, a match-counter is incremented, with transgression of a pre-defined limit generating a veto. If there is no match, the match-counter is reset to zero. Depending on the nature of the bond-wire resonance, it may be better to decrement the counter with each mismatch, and thereby avoid low frequency random triggers inhibiting detection.

As mentioned earlier in this paper, dangerous triggers rates are those higher than ~15kHz (***ref?). With a 40 MHz clock, only a 12-bit period-counter is required (maximum = 4096 * 25ns clock period = 102,400ns = 9.77 kHz). If this counter overflows, all detection is abandoned until the next trigger. In doing so, the period-counter acts as a low-pass filter, with its maximum the cut-off point.

Add to this a minimum period-count value - lower than which no action is taken – and a band-pass filter is created. Setting these maxima and minima allow the system to be tuned to a particular band of trigger frequencies.

### Period Matching Accuracy

In an attempt to ensure trigger-periods with only a slight mismatch (by, say, a single bit) are still seen as equal, the lowest order bits of the trigger-period value are masked out of the equality test. Although this was not perfect (consider the difference between 0xFF and 0x100) it does provide additional functionality at no logic overhead.

An improvement on this is to find the delta-period of successive triggers, and decide equality based on the size of this number. Increasing this threshold allows a relatively wide range of triggers to be matched. This could be called something like a dynamic centred band-pass filter.

If the wire-bond danger frequency is better understood, then the centre frequency (i.e. the previous period) can be preset and all incoming triggers compared to this.

### Operation

Operation is best demonstrated using the output from the simulator (below).

The system was configured with period_max = 1000, period_min = 400 (25 ns units) and period_match_threshhold = 4.

Triggers of differing periods are applied, some regular some not.

### Fitting Into Hardware

The system described above was built as a user configurable module (either through compile-time constants, or registers accessible by software). Compiling and fitting it for a Xilinx Spartan II 600E FPGA, as used on the TIM, generated a configuration utilising a maximum of 100 logic cells, out of an available 6900.

With such low logic overhead, it is conceivable that an array of these modules could be used in parallel, each configured to cover a different frequency band. Indeed, if the period matching removed, the bandwidth reduced and tens of modules used it would be possible to generate a large granularity trigger frequency spectrum.

# Conclusions

Resonant bond wire vibrations were studied by imaging the bond wire with a high magnification CCD camera. A novel way of detecting resonant bond wire vibrations in a magnetic field was developed based on measuring the back emf. The two methods gave consistent results but the back emf method was sensitive to lower amplitude vibrations.

Resonant bond wire vibrations will occur if there is a time varying current perpendicular to a magnetic field. If the bond wires are longer than about 2mm and the magnetic field is in the plane of the loop of the bond wire currents of a few mA with a frequency close to the resonant frequency of the wire bond will lead to large amplitude vibrations. These large amplitude vibrations are very dangerous and will typically cause the wire bond to fail after a few minutes operation. The problem turns out to be less severe in the ATLAS SCT because of a combination of the favourable orientation of the wire bonds with the magnetic field (barrel opto-package) and very short wire bonds (End Cap). A large additional safety factor can be obtained by vetoing fixed frequency running and a suitable algorithm has been developed and will be implemented in ATLAS.

# Acknowledgements

Financial support from the UK Particle Physics and Astronomy Research Council is gratefully acknowledged. We would like to thank Dr Joel Goldstein (RAL) for explaining the resonant wire bond vibration problems seen in CDF.

References

See talk by Reid Mumford at url: http://ssd-rd.web.cern.ch/ssd-rd/bond/talks/reid_cdf.pdf

J. Goldstein, private communication.

ATLAS Inner Detector Technical Design Report, CERN/LHCC/97-16/17.

ATLAS Pixel Detector  Technical Design Report, CERN/LHCC/98-13.

D.G. Charlton et al., System Test of Radiation Hardness of Optical Links for the ATLAS SemiConductor Tracker, Nucl. Instr. and Meth. A443  (2000) 430.

Eric Weisstein’s World of Physics, http://scienceworld.wolfram.com/physics/beam.html .

See www.autofea.com for program details.

VDC reference

P.K. Teng, VCSEL paper.

M-L Chu, The SCT Barrel Opto-package, EDMS note.

M-L Chu, The SCT End Cap Opto-package, EDMS note .

Corresponding author. Email: t.weidberg1@physics.ox.ac.uk

Calculated from a 20mA current, 2T field and 2.6mm of wire, using F = BIl