## First PCB interface

- Connectors

Connectors used are JST SHL 20-pin 1mm-pitch connectors. References are detailed in the table below.

| Part of connector | ### Reference |
| --- | --- |
| Contact | SSHL-003T-PO.2 |
| Housing | SHLP-20V-S-B |
| Shrouded header | SM20B-SHLS-TF |

Two 20-pin connectors will be used to connect the PCB to the read-out. The I/O description is given below.

- Input/output

##### Calibration connector

| Number of pin | Description |
| --- | --- |
| 12 | 6 LVDS calibration timing (1 per cal. channel) |
| 6 | Outgoing calibration currents (Imax=80mA) |
| 1 | Ground (0V) |
| 1 | Vss (-5V) |

##### Readout and control connector

| Number of pin | Description |
| --- | --- |
| 2 | LVDS Shift register clock |
| 2 | LVDS Hold signal (/track) |
| 2 | LVDS Shift register input |
| 2 | LVDS shift register reset |
| 6 | Analog output (1 per channel) |
| 2 | PIN diode voltage bias (-200V) |
| 2 | Ground (0V) |
| 2 | Vss (-5V) |

###### Calibration connector pinout

| Pin # | Sig Name | Description |
| --- | --- | --- |
| 1 | Tcalib0 | Channel 0 positive LVDS cal. timing signal |
| 2 | Tcalib0* | Channel 0 negative LVDS cal. timing signal |
| 3 | Tcalib1 | Channel 1 positive LVDS cal. timing signal |
| 4 | Tcalib1* | Channel 1 negative LVDS cal. timing signal |
| 5 | Tcalib2 | Channel 2 positive LVDS cal. timing signal |
| 6 | Tcalib2* | Channel 2 negative LVDS cal. timing signal |
| 7 | Tcalib3 | Channel 3 positive LVDS cal. timing signal |
| 8 | Tcalib3* | Channel 3 negative LVDS cal. timing signal |
| 9 | Tcalib4 | Channel 4 positive LVDS cal. timing signal |
| 10 | Tcalib4* | Channel 4 negative LVDS cal. timing signal |
| 11 | Tcalib5 | Channel 5 positive LVDS cal. timing signal |
| 12 | Tcalib5* | Channel 5 negative LVDS cal. timing signal |
| 13 | GND | Ground (0V) |
| 14 | #### Icalib0 | #### Channel 0 Calibration current |
| 15 | Icalib1 | #### Channel 1 Calibration current |
| 16 | Icalib2 | #### Channel 2 Calibration current |
| 17 | Icalib3 | #### Channel 3 Calibration current |
| 18 | Icalib4 | #### Channel 4 Calibration current |
| 19 | Icalib5 | #### Channel 5 Calibration current |
| 20 | VSS | Vss (-5V) |

| Pin # | Sig Name | Description |
| --- | --- | --- |
| 1 | GND | Ground |
| 2 | VSS | Vss (-5V) |
| 3 | HPD_BIAS | Diode bias (-200V) |
| 4 | HPD_BIAS | Diode bias (-200V) |
| 5 | VSS | Vss (-5V) |
| 6 | GND | Ground (0V) |
| 7 | Vout_5 | Chip 5 output voltage |
| 8 | Vout_4 | Chip 4 output voltage |
| 9 | Vout_3 | Chip 3 output voltage |
| 10 | Vout_2 | Chip 2 output voltage |
| 11 | Vout_1 | Chip 1 output voltage |
| 12 | Vout_0 | Chip 0 output voltage |
| 13 | SR_RST* | Shift registers negative LVDS reset |
| 14 | #### SR_RST | Shift registers positive LVDS reset |
| 15 | SR_IN* | Shift registers negative LVDS input |
| 16 | SR_IN | Shift registers positive LVDS input |
| 17 | SR_CLK* | Shift registers negative LVDS clock |
| 18 | SR_CLK | Shift registers positive LVDS clock |
| 19 | HOLD* | #### Output signal negative LVDS hold |
| 20 | HOLD | #### Output signal positive LVDS hold |

Readout and control connector pinout