# IMPORTANT NOTICE

# Due to a mistake of the packaging company, the pin assignment of the latest production run is different to the original description given in the old specifications.

# The orientation of the chips was erroneously rotated by 180 degrees.

# Datasheet / Rev 1.2 contains the new pin-assignment.

Revised pin-assignment:

| Pin # | Name | Description |
| --- | --- | --- |
| 15 | DO2 | data_out 2 |
| 16 | GND | ground |
| 17 | SDA | I2C data |
| 18 | VDD | supply 3.3V |
| 19 | SCL | I2C clock |
| 20 | A5 | I2C address |
| 21 | A4 | I2C address |
| 22 | A3 | I2C address |
| 23 | A2 | I2C address |
| 24 | NC | not connected |
| 25 | NC | not connected |
| 26 | GND | ground |
| 27 | PHI2 | clock output |
| 28 | GND | ground |
| 1 | DO1 | data_out 1 |
| 2 | VDD | supply 3.3 V |
| 3 | DO0 | data_out 0 |
| 4 | GND | ground |
| 5 | DI0 | data_in 0 |
| 6 | DI1 | data_in 1 |
| 7 | VDD | supply 3.3 V |
| 8 | PHI1 | clock input |
| 9 | GND | ground |
| 10 | DI2 | data_in 2 |
| 11 | DI3 | data_in 3 |
| 12 | GND | ground |
| 13 | DO3 | data_out 3 |
| 14 | VDD | supply 3.3V |

# CERN - MICROELECTRONICS GROUP

# PHOS4 – 4 Channel delay generation ASIC with 1ns resolution

# Datasheet	/ Rev 1.2

Purpose

The purpose of the ASIC is to provide 4 calibrated delay lines for digital signals, which can be programmed independently via an I2C interface. One of 25 delay taps, spaced 1ns apart, can be chosen for each of the 4 signal channels and the clock channel. The chip requires a 40 MHz clock as timing reference.

Architecture

The device contains a delay locked loop (DLL) to provide the reference control voltage for the four independent delay units. The delay units have the same layout as the DLL in order to achieve identical electrical properties in both the reference delay unit (=DLL) and in the signal delay units.

Technology

0.8 m CMOS, AMS CYX

Functional Block diagram

Delay-locked loop and delay-lines

The DLL consists of a multi-stage variable delay line, a phase detector and a loop filter. The DLL uses a starved inverter design to incorporate a variable delay element, a phase comparator, a charge pump and a loop filter. The delay lines consist of 25 delay stages. A 25:1 multiplexing stage follows the delay-line elements. The delay value is stored in a five bit register connected to the multiplexer. The outputs of the DLL delayline are also fed into a multiplexer, allowing to delay the clock signal.

I2C-Bus interface

The values of the delay-registers can be written over the I2C-bus interface. The I2C interface uses the external signals SCL and SDA for data transfer. Of the seven device address bits (A6-A0) four bits (A5-A2) correspond to the hard-wired device address, while A6 is supposed to be zero and A1 and A0 are ignored. The data word is  divided into two sections, the upper three bits (S2-S0) select one of the delay channels, the lower five bits specify the delay or are used to set the output to zero or one. (See following tables.)

A write-cycle has the following format:

| S | A6 | A5 | A4 | A3 | A2 | A1 | A0 | 0 | A | S2 | S1 | S0 | D4 | D3 | D2 | D1 | D0 | NA | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

S: Start condition (see [1])

A6 : MSB of device address, must be 0

A5-A2: 4 bit device address, defined by pins A5-A2.

A1-A0: lower bits of device address, ignored

A: Acknowledge cycle

D4-D0: 5 bit delay data:

| D4-D0 value | Significance |
| --- | --- |
| 0-24 | Delay in ns |
| 25 | Constant 0 at output |
| 26 | Phase detector output (for test purposes) |
| 27 | Constant 1 at output |

S2-S0: 3 bit device select:

| S2-S0 value | Significance |
| --- | --- |
| 0 | Delay channel 0 |
| 1 | Delay channel 1 |
| 2 | Delay channel 2 |
| 3 | Delay channel 3 |
| 4 | CLK channel |

NA: “Not Acknowledge” cycle

P: Stop condition

Example

To program a delay value of 15 nanoseconds on delay channel 3, the data byte which has to be transmitted calculates to 3*32+15=79. For setting the output of the clock channel to zero the data byte to be transmitted is 4*32+25=153.

Reset procedure

Since the chip does not have a reset signal, it has to be generated from the I2C interface. The first negative transition on the I2C clock line is used to trigger the reset procedure. Therefore, the first data transmission will not have any effect on the delay registers. It is to note that the clock has to be active during reset. Before a reset, or if there is no clock at the corresponding input, the chip can go into a state where current consumption is relatively high, i.e. >30mA. Therefore it is recommended to reset the chip immediately after power-up with an arbitrary transmission on the I2C bus, i.e. not necessarily addressing any of the chips on the bus.

External signals

- GND				Ground

- VDD: 			3.3 V supply (nominal)

- PHI1 (input): 			40 MHz external clock signal

- PHI2 (output):		40 MHz de-skewed output clock

- DI<3:0>(input): 		Digital input signals

- DO<3:0>(output): 		Delayed digital output signals

- SDA (bidirectional): 		I2C-Data

- SCL (input): 			I2C-Clock

- A<6:3> (input): 		Upper four bits of I2C-Address

recommended operating conditions

|  |  | MIN | NOM | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| VDD | Supply voltage | 3.0 | 3.3 | 3.6 | V |
| VIH | High-level Input voltage | 2.0 |  |  | V |
| VIL | Low-level Input voltage |  |  | 0.8 | V |
| TA | Operation free-air temperature | -20 | 25 | 75 | C |
| trf | Input transition rise or fall time |  | 2 | 5 | ns |
| CL | Capacitive load at output |  | 15 | 50 | pF |
| IL | Continuous output current |  |  | 4 | mA |

absolute maximum ratings over operating free-air temperature range

|  | MIN | MAX | UNIT |
| --- | --- | --- | --- |
| Supply voltage, VDD | 0 | 5.5 | V |
| Input voltage range | 0 | VDD+0.3 | V |
| Continuous output current, IL |  | 10 | mA |

Current consumption

|  |  | MIN | NOM | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| P0 | DC bias current @ 40 Mhz | 2 | 5 | 10 | mA |
| Pch0 | Current/channel, output sw. off, 40Mhz | 1 | 2 | 4 | mA |
| Pch | Current/channel, 15pF total load, 40 MHz | 3 | 4 | 6 | mA |
| Pch/f | Current/channel/Mhz , 15 pF total load |  | 0.1 |  | mA |

The overall current consumption is calculated by adding up the DC bias current to the currents in the active channels. If, for example, the clock output is switched off, and one channel has a mean transition frequency of 2 Mhz, then the total current calculates to P0+ Pch0 + 2 Pch /f = 7.2 mA (with a total load of 15pF, no DC current, 40 Mhz input clock, nominal case).

Timing requirements

|  |  | MIN | NOM | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| t | Clock cycle time |  | 25 |  | ns |
| tp | Pulse width in signal channels 0-3 | 3 |  |  | ns |

For timing on I2C signals SDA, SCL see [1].

timing characteristics

|  |  | MIN | NOM | MAX | UNIT |
| --- | --- | --- | --- | --- | --- |
| t0 | minimum signal delay (tap 0) | 8 | 10.5 | 14 | ns |
| t | delay step, t=25ns | 0.92 | 1 | 1.08 | ns |
| t | differential non-linearity rms |  | 26 |  | ps |
| te0 | integral non-linearity, Channel 0 and 3 |  | 500 |  | ps |
| te1 | integral non-linearity, Channel 1 and 2 |  | 250 |  | ps |
| t | output clock jitter (max. @ tap 24) | 25 |  | 50 | ps |

Pin assignment

| Pin # | Name | Description |
| --- | --- | --- |
| 1 | DO1 | data_out 1 |
| 2 | VDD | supply 3.3 V |
| 3 | DO0 | data_out 0 |
| 4 | GND | ground |
| 5 | DI0 | data_in 0 |
| 6 | DI1 | data_in 1 |
| 7 | VDD | supply 3.3 V |
| 8 | PHI1 | clock input |
| 9 | GND | ground |
| 10 | DI2 | data_in 2 |
| 11 | DI3 | data_in 3 |
| 12 | GND | ground |
| 13 | DO3 | data_out 3 |
| 14 | VDD | supply 3.3V |
| 15 | DO2 | data_out 2 |
| 16  Not that in the production run the pin assignment has changed due to erroneous packaging. See notice on first page ! | GND | ground |
| 17 | SDA | I2C data |
| 18 | VDD | supply 3.3V |
| 19 | SCL | I2C clock |
| 20 | A5 | I2C address |
| 21 | A4 | I2C address |
| 22 | A3 | I2C address |
| 23 | A2 | I2C address |
| 24 | NC | not connected |
| 25 | NC | not connected |
| 26 | GND | ground |
| 27 | PHI2 | clock output |
| 28 | GND | ground |

References

[1] Signetics I2C-bus specifications, January 1992

Read-access to the registers is not supported.

Although a higher supply voltage, e.g. 4V can be used, correct operation is not guaranteed with a reference clock of 40 Mhz and has to be tested individually. The same applies for supply voltages less than 3.0 V, and for reference clock frequencies different from the nominal case of 40 Mhz. The general trend is that higher clock frequencies require higher supply voltages.