ABSTRACT

The design and the functionality of the Clock and Control (CC) hardware and the firmware for the EuXFEL DAQ system are described. The CC board is responsible for synchronising the DAQ system to the overall system timing. The hardware consists of a DESY designed MTCA.4 board and a custom Rear Transition Module (RTM). Each RTM board provides the capability to control up to 16 Front End Modules (FEM)  for a 1 Mpixel 2D detector. The firmware for the CC board generates the clock, data and veto signals for the FEMs and it receives the status as a feedback. AC-coupled LVDS links running through standard CAT6 RJ45 cables provide the FEM connection.

SUMMARY

The European free electron laser (EuXFEL) facility will generate coherent and intense X-ray flashes at up to 27000 flashes per second with a ~4.5 MHz bunch delivery rate. Each flash is intense enough to produce to produce a full diffractive picture of scattering targets. [1] The detectors for the 2D camera systems will be able to capture up to 5000 images/second with a resolution of 1MPixel. There are three detector designs being developed namely, AGIPD, LPD and DSSC. These 3 differ in terms of the functionality and the technologies involved in the design [2] , however the overall structure which consists of sensor modules with an ASIC and front end modules (FEM) is similar. [3] [4] The FEM electronics provide the read-out data for the Train Builder (TB) board and control the ASIC.

The clock and control (CC) system provides the synchronising clock and bunch and train related information to the FEM electronics. It also receives status feedback from and provide the veto signal to the FEMs to reject some of the detector data.

The CC system consists of a MTCA.4 AMC board and a rear transition module (RTM). The MTCA.4 AMC board (DAMC2) [5] is designed by DESY as a multi-purpose FPGA hardware platform for various projects in DESY and provides the processing capability needed for the CC functionality. We have developed our own custom RTM board according to the MTCA.4 standard [6] (named physics uRTM) which connects to the DAMC2 through two thirty-pair advanced differential fabric (ADF) connectors.

The connections between the CC system and the FEM boards are realised on AC-coupled LVDS links on CAT5 or better RJ45 cables. The RTM board provides the number of channels to support up to 16 FEM modules for 1MPixel 2D detector. Each channel comprises 4 LVDS pairs on an RJ45 connector. The names and the description of these signals are listed below. [10]

- Output clock (FAST clock): This is a ~99 MHz clock derived from the 4.5 MHz bunch clock

- Output data (FAST data) : This provides the trigger start signal to the FEMs along with the train number ID information

- Status : This is a status feedback from the FEMs

- Veto : This signal is the bunch reject data encoded on a either 99 or 4.5 MHz clock.

The description of the additional circuitry in order to use AC-coupling on the data signal on the FEM side and the status signal on the CC side will be presented.

The CC system is designed to provide scalability to support higher Mpixel detectors by using extra AMC and uRTM board pairs in the same crate with one pair designated as master and sharing the clock and the data through the MTCA.4 backplane.

The CC system receives clock and system related information from the timing receiver (TR) board which along with the timing transmitter provides synchronisation for all the different modules in EuXFEL facility. The TR board is to be housed in the same crate as the CC system. The crate is designated as the CC crate which also houses the CPU board and the MCH. The links to the TR board are through the crate backplane which consist of 8 differential pairs in MLVDS standard in bussed configuration and two high quality clock lines (TCLKA/B) in star mesh configuration. [7]

The firmware block diagram for the CC board is shown in figure x. The FAST TX module generates the FAST data to the FEMs. The telegram RX module receives and decodes the information coming from TR board on a clock and data pair through the backplane. The PCIe block provides the connection to the crate CPU to receive configuration data from the control software. Brief description of the control software will be provided [8]. There is a separate module for generating the veto data. The RocketIO module makes use of the SFPs on the DAMC2 board and provides an optional veto source input to the firmware. All the modules in the firmware are integrated through a bus and a register block called Internal Interface (II) [9] which is developed by DESY.

Each CC board pair takes up two slots on the crate which comes in either 6 or 12 slot configuration. [9] The DAMC2 part of the pair is a full-height single size  however the 2X8 stacked RJ45 connector on the uRTM makes it a full-height double size board.

The CC hardware/firmware system is designed to provide the flexibility, extendibility and scalability to support possible future upgrades to the DAQ. It also reduces the cost and effort for the development of such system by using a general-purpose MTCA.4 FPGA board as its processing platform and an RTM to provide the custom functionality.