# A Physics TCA Solution for the EuXFEL Clock and Control System

Erdem Motuk, Martin Postranecky, Matt Warren, Matthew Wing

University College London

ABSTRACT

The development of the Clock and Control (CC) hardware and firmware for the EuXFEL DAQ system is presented. The system will exploit the advances in data handling provided by the new telecommunication crate architecture standard for physics (MTCA.4). The CC board is responsible for synchronising the DAQ system to the overall system timing. The hardware consists of a DESY designed MTCA.4 board and a custom Rear Transition Module (RTM). Each RTM board controls up to 16 Front End Modules (FEMs) for a 1 Megapixel 2D detector. The firmware for the CC board generates the clock, data and veto signals for the FEMs and it receives the status as a feedback. AC-coupled LVDS links running through standard CAT6 RJ45 cables provide the FEM connection. The CC hardware and firmware are designed to provide the flexibility, extendibility and scalability to support possible future upgrades to the DAQ or larger detectors. It also reduces the cost and effort for the development of such system by using a general-purpose MTCA.4 FPGA board as its processing platform and an RTM to provide the custom functionality.

SUMMARY

The European X-ray Free Electron laser (EuXFEL) facility will generate 27000 coherent and intense X-ray flashes per second with a ~4.5 MHz bunch delivery rate. Each flash is intense enough to produce to produce a full diffractive picture of scattering targets [1]. The detectors for the 2D camera systems will be able to capture up to 5000 images/second with a resolution of 1 MegaPixel. There are three detector designs being developed namely, AGIPD, LPD and DSSC. These three differ in terms of the functionality and the technologies involved in the design [2], however the overall structure which consists of sensor modules with an ASIC and Front End Modules (FEMs) is similar [3]. The FEM electronics provide the read-out data for the Train Builder (TB) and control the ASICs.

The common Clock and Control (CC) system provides the synchronising clock and bunch and train related information to the FEM electronics. It also receives status feedback from FEMs and distributes the veto signals to the FEMs to reject some of the detector data.

The CC system consists of a MTCA.4 Advanced Mezzanine Card (AMC) board and a Rear Transition Module (RTM). The MTCA.4 AMC board (DAMC2) [4] is designed by DESY as a multi-purpose FPGA hardware platform for various projects in DESY and provides the processing capability needed for the CC functionality. We have developed our own custom RTM board according to the MTCA.4 standard [5] (named physics RTM) which connects to the DAMC2 through two thirty-pair Advanced Differential Fabric (ADF) connectors.

The connections between the CC system and the FEM boards are realised on AC-coupled LVDS links on CAT6 RJ45 cables. The RTM board provides the number of channels to support up to 16 FEM modules for a 1 MPixel 2D detector. Each channel comprises 4 LVDS pairs on an RJ45 connector. The names and the description of these signals are listed below.

- Output clock (FAST clock): This is a ~99 MHz clock derived from the 4.5 MHz bunch clock.

- Output data (FAST data): This provides the trigger start signal and train ID data to the FEMs.

- Veto: This signal is the bunch reject data encoded on a either 99 or 4.5 MHz clock.

- Status: This is a status feedback from the FEMs

The development and testing of the additional receiver circuitry required to use AC-coupling on the data signal on the FEMs and the status signal on the CCs will also be described.

The CC system receives clock and system related information from the Timing Receiver (TR) board which along with the timing transmitter provides synchronisation for all the different modules in EuXFEL facility. A dedicated CC crate will house one TR module plus the crate the CPU board and the MTCA Carrier Hub (MCH). The links to the TR board are through the crate backplane which consist of 8 differential pairs in MLVDS standard in bussed configuration and two high quality clock lines (TCLKA/B) in star mesh configuration [6].

The firmware block diagram for the CC board is shown in Fig. 1. The FAST TX module generates the FAST data to the FEMs. The telegram RX module receives and decodes the information coming from TR board on a clock and data pair through the backplane. The PCIe block provides the connection to the crate CPU to receive configuration data from the control software. Brief description of the control software will be provided [7]. There is a separate module for generating the veto data. The RocketIO module makes use of the SFPs on the DAMC2 board and provides an optional veto source input to the firmware. All the modules in the firmware are integrated through a bus and a register block called Internal Interface (II) [8] which is developed by DESY.

The CC system is designed to provide scalability to support multi-Megapixel detectors by using extra AMC and uRTM board pairs in the same crate with one pair designated as master and sharing the clock and the data through the MTCA.4 backplane. Each CC board pair takes up two slots (double full-size board) on the crate which comes in either 6 or 12 slot configuration. [9]

The CC hardware and firmware is designed to provide the flexibility, extendibility and scalability to support possible future upgrades to the DAQ or larger detectors. It also reduces the cost and effort for the development of such system by using a general-purpose MTCA.4 FPGA board as its processing platform and an RTM to provide the custom functionality.

Fig.1: Firmware block diagram

Fig.2: Assembly drawing for the RTM (to be replaced with an actual picture)

REFERENCES

[1] European XFEL GmbH, http://www.xfel.eu/  ;

M. Altarelli et al., The European X-Ray free-electron laser, technical design report, DESY 2006-097,

http://www.xfel.eu/sites/site xfel-gmbh/content/e63617/e79991/e68669/european-xfel-tdr eng.pdf  .

[2] P. Goettlicher for the AGIPD-consortium, Electronics for the European XFEL: AGIPD a high frame rate camera in TWEPP 2010, iopscience.iop.org/1748-0221/5/12/C12015

[3] M. Porro et al., Expected performance of the DEPFET sensor with signal compression: A large format X-ray imager with mega-frame readout capability for the European XFEL, Nucl. Instrum. Meth. 624 (2010) 509

[4] P. Vetrov, Versatile AMC for XFEL machine control, in ATCA workshop presentation of IEEE-Real-Time conference , May 24-28, 2010, Lisboa, Portugal,

[5] Micro telecommunications computing architecture, short form specification (2006)

http://www.picmg.org/pdf/MicroTCA Short Form Sept 2006.pdf  ,MicroTCA application guide (2010), http://www.picmg.org/pdf/MicroTCA_Application_Guide.pdf

[6] K. Rehlich, New TCA hardware developments for the European XFEL  in ATCA workshop presentation of IEEE-Real-Time conference , May 24-28, 2010, Lisboa, Portugal,

[7] C. Youngman, private communication

[8] K. T. Pozniak. Internal Interface, TESLA Report 2005-22, http://tesla.desy.de/new_pages/TESLA_Reports/2005/pdf_files/tesla2005-22.pdf

[9] Schroff GmbH, AdvancedTCA Packaging Solutions,  http://web.schroff.de/webcat/subgroup/pdf/schroff_cat_39601422_atca_packaging_solutions_2008_e.pdf

[10] Schematics for the UCL RTM board and the test boards,  http://www.hep.ucl.ac.uk/~mp/XFEL-CC_DAUGHTER-CARD-A_pc3461m1_SCHEMATICS.pdf ,  http://www.hep.ucl.ac.uk/~mp/XFEL-CC_RTM_SCHEMATICS_pc3462m_schematics.pdf