BACKPLANE INTERFACES  :

This slide shows the standards adopted for the backplane interface.

The 40MHz clock is distributed to all BOC cards as 16 individual differential PECL pairs via the J3 backplane. The devices chosen for this task are the MOTOROLA MC100E111JC 1:9 PECL DIFFERENTIAL CLOCK DRIVERs, which guarantee channel-to-channel skew to be below 50 ps.

Each line of the differential pair has a 270R load resistor at the transmitter, and is terminated by 100R between the differential pair at the BOC receiver end.

The eight active-low TTC(0-7) outputs are bussed to RODs on two separate backplane buses, each for 8x RODs. The devices chosen, PHILIPS N74ABT574D, are Advanced BiCMOS Bus Interface drivers, which guarantees VOL of below 0.55V while sinking 64mA, and VOH of above 2V while sourcing 32mA.

With TIM is slot 13 in the middle of the crate, the TTC(0-7)A bus is terminated on the backplane in slot 21, and the TTC(0-7)B bus in slot 5.

Finally, the 16x ROD BUSY signals are received from individual RODs by PHILIPS N74LVT16244B LV Bus receivers. The active-low ROD BUSY lines are pulled up to +3V3 on the TIM, thus indicating empty ROD slots as NOT BUSY.