DESCRIPTION OF BUSY's on TIM-3 :

1) On FPGA2, there are internal busys, which get asserted while any of the internally-generated TTC(x) functions are being generated.

They should be as long as the SCT serial code for each 'fast command' :

L1A :			3 clocks

BCR, FER, ECR, Spare :		7 clocks

CAL :                   		27 + pipeline-delay-to-trigger + 3 clocks

2) "OA_BUSY" is an OR of all above BUSYs and of "TESTBUSYON"

( "OA_BUSY" in the Status Register 6 bit 2 should NOT get asserted when "TIM-OK" goes LOW = ie. not OK )

3) "TESTBUSYON" is a busy line which is enabled by Commands Register 1 bit 13 "EnTestBusy", and cleared by Commands Register 1 bit 14 "ClrTestBusy" and/or by "OA_RESET"

"TESTBUSYON" is set by the next L1A after being enabled, and should prevent any internally-generated or externally-received fast commands after this L1A being acted upon

"TESTBUSYON" is also output to the Status Register 6 bit 14

4) "INTBUSY" is the "OA_BUSY" shown in the Status Register 6 bit 2

( "INTBUSY" & "OA_BUSY" in the Status Register 6 bit 2 should NOT get asserted when "TIM-OK" goes LOW = ie. not OK )

5) "VBUSY" is set in Commands Register 1 bit 7

6) "EXTBUSY" is the OR of externally-input "NIMEXTBUSY" and "ECLEXTBUSY" from the front panel

It is also a bit 0 in Status Register 6

7) Front panel signal "NIMEXTBUSYIN" is enabled onto the "NIMEXTBUSY" line by selecting link LK7 pins 1+2

( NOTE : If this selection in is used, the "EXTRODBUSYIN" should be grounded by selecting link LK7 pins 3+4 )

8) Front panel signal "NIMEXTBUSYIN" is enabled onto the "EXTRODBUSYIN"

line by selecting link LK7 pins 2+3

9) "BUSYOUT" is the OR of

a) "OA_BUSY" ( same as "INTBUSY" ) enabled by "EnIntBUSY" in Enables Register 0 bit 7

b) "EXTBUSY" enabled by "EnExtBUSY" in Enables Register 0 bit 15

c) "VBUSY" from Commands Register 1 bit 7

d) "RODBUSY", enabled by "EnRODBUSY" in TTC Enable Register 12 bit 7

It is also a bit 3 in Status Register 6

10) "BUSYOUTB" is the buffered "BUSYOUT" sent to 'TIM BUSY' LED

11) "NIMBUSYOUT" is a front panel output of the "BUSYOUTB" signal ( buffered version of "BUSYOUT" ), and can be selected by link LK6 to be either +ve or -ve going

12) "ECLBUSYOUT" is a front panel output, which is selected by link PL41 to be either "BUSYOUTB" signal as above ( PL41 pins 1+2 ), or the "MEXTBUSYOUT" signal ( PL41 pins 2+3 )

13) "MEXTBUSYOUT" is the "EXTBUSY" signal enabled by "EnExtBUSY" in Enables Register 0 bit 15

It is also a bit 1 in Status Register 6

14) "VRODBUSY" is set in Commands Register 1 bit 8

15) "RODBUSYOUT" is a front panel output, which is selected by link LK1 to be either NIM-type signal "ROD_CRATE_BUSYOUTB3" ( LK1 pins 2+3 ) – which can be selected either +ve or -ve going by link LK4 - or TTL-type signal ( LK1 pins 1+2 )

( "ROD_CRATE_BUSYOUT" should NOT get asserted when "TIM-OK" goes LOW = ie. is not OK )

( it should BE asserted while the TIM is in "SAMODE" )

16) "ROD_CRATE_BUSYOUT" is the OR of

a) masked OR of all the ROD-BUSY(x) signals received from TIM backplane

( see ROD Busy Register 16 and ROD Mask Register 15 )

b) "EXTRODBUSYIN" input from the front panel ( see par.8 ) and enabled by the "EnRODBUSY" bit 7 the TTC Enables Register 12

c) "VRODBUSY from Commands Register 1 bit 8

d) "SAMODE"  from Status register 6 bit 11

14) "VRODBUSY" is set in Commands Register 1 bit 8

15) "RODBUSYOUT" is a front panel output, which is selected by link LK1 to be either NIM-type signal "ROD_CRATE_BUSYOUTB3" ( LK1 pins 2+3 ) – which can be selected either +ve or -ve going by link LK4 - or TTL-type signal ( LK1 pins 1+2 )

( "ROD_CRATE_BUSYOUT" should NOT get asserted when "TIM-OK" goes LOW = ie. is not OK )

( it should BE asserted while the TIM is in "SAMODE" )

16) "ROD_CRATE_BUSYOUT" is the OR of

a) masked OR of all the ROD-BUSY(x) signals received from TIM backplane

( see ROD Busy Register 16 and ROD Mask Register 15 )

b) "EXTRODBUSYIN" input from the front panel ( see par.8 ) and enabled by the "EnRODBUSY" bit 7 the TTC Enables Register 12

c) "VRODBUSY from Commands Register 1 bit 8

d) "SAMODE"  from Status register 6 bit 11

17) "RODBUSYOUT" is the "ROD_CRATE_BUSYOUT" set in bit 7 of Status Register 6

18) "ROD_CRATE_BUSYOUTB1" is the "ROD_CRATE_BUSYOUT" sent to TIM backplane J3 pin 21D

19) "ROD_CRATE_BUSYOUTB2" is sent to the 'ROD BUSY' LED

20) BurstBUSY, SeqBUSY, SinkBUSY - see Status Register 6