TIM-3  CLOCK SWITCHING :

To ensure a safe and un-interruptible supply of 40 MHz clocks to all SCT  FE modules, there a fail-safe network of two Zero Delay Glitch-Free Clock multiplexers/PLLs ( ICS 581—02 ).

The first multiplexer select between the X-tal generated internal clock or the externally supplied clock, with the X-tal clock being the default.

The second and final multiplexer is used to select the TTC-generated or stand-alone clock. If there is a failure of the TTC clock, this device switches automatically to the stand-alone clock within 3 clock periods.

Failure of any selected clock is also indicated by the de-assertion

of the TIM-OK line on the backplane.