TIM-3 CLOCKS FLOW :

This slide show the clock system of the TIM. In SA MODE, an 80.1573 MHz on-board crystal clock generator provides the required 40.0786 MHz INTERNAL CLOCK.

There is a provision for accepting 40MHz EXTERNAL CLOCK in either NIM or differential ECL standard. Required clocks can be selected in the ENABLES register.

When in the RUN MODE, the TTC supplies the machine clock via the TTCrx ASIC.

There is a couple of delay lines on TIM, allowing for independent adjustments of ROD SETUP ( for TTC outputs timing ) and TIM SETUP ( of internal timings ). These delay lines are set by DIL switches as 6-bits in 0.5 nsec steps, giving more than adequate coverage over the 25nsec clock.

There is an additional delay adjustments available for the STAND-ALONE clock, set by the CLOCK DELAY register as 6-bits of 0.5nsec steps, which can be used to scan.

There are four additional delay lines, used to provide a TRIGGER WINDOW width and delay, which can be used with to reject random triggers.