TIM FUNCTIONAL MODEL shows clearly the main blocks of TIM :

- TTC INTERFACE with its TTCrx ASIC, producing all the Timing,

Trigger and Command signals received from the CTP while in

the RUN MODE

- STAND-ALONE generator of the same signals in the SA MODE.

- STAND-ALONE SEQUENCER which allows long command

sequences to be down-loaded from the host in the SA MODE

- THE EVENT FIFOS AND SERIALISERS to allow IDs of multiple

triggers to be stored and output sequentially

- BACKPLANE MAPPING logic

- TTC INTERFACE with its TTCrx ASIC, producing all the timing,

trigger and command signals received from the FLT while in the

RUN MODE

- STAND-ALONE generator of the same signals in the SA MODE.

- STAND-ALONE SEQUENCER which allows long command

sequences to be down-loaded from the host in the SA MODE

- THE EVENT FIFOS AND SERIALISERS to allow IDs of multiple

triggers to be stored and output sequentially

- BACKPLANE MAPPING logic

TIM SCHEMATICS

- This slide shows the schematic representation of the circuitry

of the STAND-ALONE generator, together with the suite of NIM and

ECL inputs and outputs.

- There is an 80.1273MHz X-tal oscillator producing an internal 40.0636MHz

clock. Either this, or an externally generated 40MHz clock can be selected to

Provide the STAND-ALONE clock.

- The fast commands ( trigger and resets ) can be generated internally,

either repetitively at a single selected frequency ( 50Hz - 600kHz ), with

trigger also available as a random trigger with selected average frequency

or by VME commands from the host by writing to the COMMAND register.

- All the TTC-type commands, plus the 40MHz clock, can also be input

from external sources in either NIM or ECL standard, using front panel

connectors.

- All the STAND-ALONE commands are synchronised to the selected

Clock and send to the BACKPLANE MAPPING logic to produce the

TTC(0-7) output signals.

- In addition, the trigger command can be delayed by a fully

Programmable delay of up to 143 clock periods to accommodate the

FE data pipeline length.

- There is a further capability of providing a "burst" of a

fully-programmable ( between 0-16k ) number of either external or internal

triggers or calibration triggers.

- The selection of any/all of the above externally input or internally

generated signals is controlled by an VME accessed ENABLES register,

allowing for a full flexibility.

- The L1ID counter and register allows any FLT number to be down-loaded

from the host interface in STAND-ALONE mode prior to issuing triggers.

- The FLTN counter will increment with each subsequent trigger, loading the new

number into the FLTN register. In the RUN mode, this register is loaded from

the TTC.

- Similarly, the BCID counter and register provides the suitably offset

BCN for every trigger, in both STAND-ALONE and RUN modes.

The BCNTIN register stores the BCN received from the TTC during run.

- Finally, the TTID is stored in the TTYPE register.

-The SEQUENCER RAM can be loaded from the host with up to 32k long

sequence of commands to allow for various tests of the system.

- There is an matching SINK RAM which allows us to store up to 32k of

commands being issued by the TIM to RODs. Again, this will be invaluable

in testing the hardware and timing.

- The slide also shows the BACKPLANE MAPPING, together with the

Backplane outputs synchronising drivers, the backplane PECL clock drivers,

the two FIFOs and serialisers for trigger L1ID, BCID and TTID data,

plus the TTCrx receiver and interface. As you can see, three sets of signals

from three sources - STAND-ALONE, TTC and SEQUENCER - are received

by the MAPPING module.

- Finally, the basic ROD BUSY system receives ROD BUSY signals from all

RODs in the crate and provides a masked-OR output to the FLT in either NIM or

TTL standard ( selectable by a link ).

TIM CLOCKS FLOW :

- This slide show the clock system of the TIM. In SA MODE, an 80.1273 MHz

on-board crystal clock generator provides the required 40.0636 MHz

INTERNAL CLOCK.

- There is a provision for accepting 40MHz EXTERNAL CLOCK in either NIM or

differential ECL standard. Required clocks can be selected in the ENABLES

register.

- When in the RUN MODE, the TTC supplies the machine clock via the

TTCrx ASIC.

- There is a number of delay lines on TIM, allowing for independent adjustments

of ROD SETUP ( for TTC outputs timing ) and TIM SETUP ( of internal timings ).

- These delay lines are set by DIL switches as 6-bits in 0.5 nsec steps,

giving more than adequate coverage over the 25nsec clock.

- There is an additional delay adjustments available for the STAND-ALONE

clock, set by the CLOCK DELAY register as 6-bits of 0.5nsec steps, which

can be used to scan.

- There are four additional delay lines, used to provide a TRIGGER WINDOW

width and delay, which can be used with reject random triggers.

TIM CLOCK SWITCHING

- To ensure a safe and un-interruptible supply of 40 MHz clocks

to all SCT  FE modules, there is a fail-safe network of two Zero Delay

Glitch-Free Clock multiplexers/PLLs ( ICS 581—02 ).

- The second ( and final ) multiplexer is used to select the

TTC-generated or stand-alone clock. If there is a failure of

this TTC clock, this device switches automatically to the

stand-alone clock within 3 clock periods.

- The first multiplexer select between the X-tal generated internal

clock or the externally supplied clock, with the X-tal being the

default.

- Failure of any selected clock is also indicated by the de-assertion

of the TIM-OK line on the backplane.

TIM - OK :

"TIM-OK" is a +5V "high-when-OK" output, and will be set low by

either the failure of the TTC clock as received by TIM, or also by

the failure of the TIM x-tal clock oscillator when in stand-alone

mode.

Additionally, "TIM-OK" will also be set low in case of failure of

the +3V3 and/or +5V0 supply on TIM ( by blown fuses ?).

( "INTBUSY" & "OA_BUSY" in the Status Register 6 bit 2 should NOT

get asserted when "TIM-OK" goes LOW = ie. is not OK )

( "ROD_CRATE_BUSYOUT" should NOT get asserted when "TIM-OK"

goes LOW = ie. is not OK )

DEFAULT :

After a switch-on and after each "O/A Reset", the internal x-tal clock

will be automatically selected to produce "CLKIN"

"TIM-OK" output is set to '1', and "TIM-OK" bit 13 in the

Status Register 6 is also set to "1"

ERROR CONDITIONS :

1) when the "SetRunMode" bit 12 in Commands Register 1 is set to '1' :

- if "TTCClkOn" line, AND/OR the "TTC-READY_IN" line go to '0', ( ie. the

TTC clock is not present for more than 4 clock periods, or the TTC is

not in a correct state ) :

Set "TIM-OK" output to '0', and set "TIM-OK" bit 13 in the Status

Register 6 to "0"

When this 'error' conditions end, the "TIM-OK "output and "TIM-OK"

bit 13  in the Status Register 6, should be reset back to "1"

2) when the "SetRunMode" bit 12 in Commands Register 1 is set to '0' :

AND when "EnExtClk" bit 8 in Commands Register 1 is set to '1' :

- if "SACLKON" line goes to '0' ( ie. the "CLKIN" is not present, due to

either the absence of any external clock on inputs, or any of the inputs

being set high, for more than 4 clock periods ), go to default state :

Set "TIM-OK" output to '0', and set "TIM-OK" bit 13 in the Status

Register 6 also to "0"

When this 'error' conditions end, the "TIM-OK "output and "TIM-OK"

bit 13  in the Status Register 6, should be reset back to "1”

DESCRIPTION OF BUSY's on TIM-3

1) On FPGA2, there are internal busys, which get asserted while any of the

internally-generated TTC(x) functions are being generated.

They should be as long as the SCT serial code for each 'fast command' :

L1A :                   3 clocks

BCR, FER, ECR, spare :  7 clocks

CAL :                   27 + pipeline-delay-to-trigger + 3 clocks

2) "OA_BUSY" is an OR of all above BUSYs and of "TESTBUSYON"

- ( "OA_BUSY" in the Status Register 6 bit 2 should NOT get

asserted when "TIM-OK" goes LOW = ie. not OK )

3) "TESTBUSYON" is a busy line which is enabled by Commands Register 1

bit 13 "EnTestBusy", and cleared by Commands Register 1 bit 14

"ClrTestBusy" and/or by "OA_RESET"

"TESTBUSYON" is set by the next L1A after being enabled, and should

prevent any internally-generated or externally-received fast commands

after this L1A being acted upon

"TESTBUSYON" is also output to the Status Register 6 bit 14

4) "INTBUSY" is the "OA_BUSY" shown in the Status Register 6 bit 2

- ( "INTBUSY" & "OA_BUSY" in the Status Register 6 bit 2 should NOT get

asserted when "TIM-OK" goes LOW = ie. not OK )

5) "VBUSY" is set in Commands Register 1 bit 7

6) "EXTBUSY" is the OR of externally-input "NIMEXTBUSY" and

"ECLEXTBUSY" from the front panel

It is also a bit 0 in Status Register 6

7) Front panel signal "NIMEXTBUSYIN" is enabled onto the "NIMEXTBUSY"

line by selecting link LK7 pins 1+2

8) Front panel signal "NIMEXTBUSYIN" is enabled onto the "EXTRODBUSYIN"

line by selecting link LK7 pins 2+3

In case of par. 7) the "EXTRODBUSYIN" should be grounded by selecting

link LK7 pins 3+4

9) "BUSYOUT" is the OR of

a) "OA_BUSY" ( same as "INTBUSY" ) enabled

by "EnIntBUSY" in Enables Register 0 bit 7

b) "EXTBUSY" enabled by "EnExtBUSY" in

Enables Register 0 bit 15

c) "VBUSY" from Commands Register 1 bit 7

d) "RODBUSY", enabled by "EnRODBUSY" in

TTC Enable Register 12 bit 7

It is also a bit 3 in Status Register 6

10) "BUSYOUTB" is the buffered "BUSYOUT" sent to 'TIM BUSY' LED

11) "NIMBUSYOUT" is a front panel output of the "BUSYOUTB" signal

( buffered version of "BUSYOUT" ), and can be selected by link LK6

to be either +ve or -ve going

12) "ECLBUSYOUT" is a front panel output, which is selected by link PL41

to be either "BUSYOUTB" signal as above ( PL41 pins 1+2 ), or the

"MEXTBUSYOUT" signal ( PL41 pins 2+3 )

13) "MEXTBUSYOUT" is the "EXTBUSY" signal enabled by "EnExtBUSY" in

Enables Register 0 bit 15

It is also a bit 1 in Status Register 6

14) "VRODBUSY" is set in Commands Register 1 bit 8

15) "RODBUSYOUT" is a front panel output, which is selected by link LK1 to

be either NIM-type signal "ROD_CRATE_BUSYOUTB3" ( LK1 pins 2+3 ) - which

can be selected either +ve or -ve going by link LK4 -

or TTL-type signal ( LK1 pins 1+2 )

- ( "ROD_CRATE_BUSYOUT" should NOT get asserted when "TIM-OK"

goes LOW = ie. is not OK )

-  ( it should BE asserted while the TIM is in "SAMODE" )

16) "ROD_CRATE_BUSYOUT" is the OR of

a) masked OR of all the ROD-BUSY(x) signals received from

TIM backplane ( see ROD Busy Register 16 and ROD Mask

Register 15 )

b) "EXTRODBUSYIN" input from the front panel ( see par.8 )

and enabled by the "EnRODBUSY" bit 7 the TTC Enables

Register 12

c) "VRODBUSY from Commands Register 1 bit 8

d) "SAMODE"  from Status register 6 bit 11

14) "VRODBUSY" is set in Commands Register 1 bit 8

15) "RODBUSYOUT" is a front panel output, which is selected by link LK1 to

be either NIM-type signal "ROD_CRATE_BUSYOUTB3" ( LK1 pins 2+3 ) - which

can be selected either +ve or -ve going by link LK4 -

or TTL-type signal ( LK1 pins 1+2 )

- ( "ROD_CRATE_BUSYOUT" should NOT get asserted when "TIM-OK"

goes LOW = ie. is not OK )

-  ( it should BE asserted while the TIM is in "SAMODE" )

16) "ROD_CRATE_BUSYOUT" is the OR of

a) masked OR of all the ROD-BUSY(x) signals received from

TIM backplane ( see ROD Busy Register 16 and ROD Mask

Register 15 )

b) "EXTRODBUSYIN" input from the front panel ( see par.8 )

and enabled by the "EnRODBUSY" bit 7 the TTC Enables

Register 12

c) "VRODBUSY from Commands Register 1 bit 8

d) "SAMODE"  from Status register 6 bit 11

17) "RODBUSYOUT" is the "ROD_CRATE_BUSYOUT" set in bit 7 of Status

Register 6

18) "ROD_CRATE_BUSYOUTB1" is the "ROD_CRATE_BUSYOUT" sent to TIM

backplane J3 pin 21D

19) "ROD_CRATE_BUSYOUTB2" is sent to the 'ROD BUSY' LED

20) BurstBUSY, SeqBUSY, SinkBUSY - see Status Register 6

FRONT PANEL INTERFACES :

This slide show the standards adopted for the inputs and outputs of external

clocks and commands. The NIM inputs and outputs are provided with LEMO 00

connectors, and ECL CLOCKS are input and output on two-pin LEMO 0B

connectors.

The complete set of six TTC(0-5) type commands, together with BUSY and

CLOCK, is output on one half of dual 16-pin IDC connector as 8x differential

ECL pairs.

Same external commands can also be input on the second half of this

IDC dual connector. This allows one TIM to operate as a master & drive

another TIM slave. At least four TIM slaves can be driven by one TIM master

using a daisy-chain ribbon cable and removing the ECL differential termination

on the first three modules.

Few words about the reasons for adopting the ECL and NIM standards for

external inputs and outputs, requiring the additional -5V2 supply. As you may

know, the CERN VMEbus Steering Committee recommended, in their paper

"RECOMMENDED PRACTICES FOR THE USE OF VMEBUS IN PHYSICS

APPLICATION", that differential ECL is the preferred method for front panel

connectivity, and NIM for single-ended coaxial connections. It is also the case,

that, for historical reasons, most of the off-the-shelf equipment available in

CERN for eg. System tests uses NIM and differential ECL. For those reasons,

and to make TIM as widely usable as possible, we have decided to retain the

ECL and NIM standards as currently used by all the CLOAC modules.

This required the provision of -5V2 supply. We have provided an on-board

DC-DC converter providing -5V from +/-12V supplies. The DC-DC converter

can be disabled by a single link. Thus the total -5V2 supplied circuitry, which is

only used for tests with externally generated clock and commands, can be

disabled.

BACKPLANE INTERFACES  :

This slide shows the standards adopted for the backplane interface.

The 40MHz clock is distributed to all BOC cards as 16 individual

differential PECL pairs via the J3 backplane. The devices chosen for

this task are the MOTOROLA MC100E111JC 1:9 PECL DIFFERENTIAL

CLOCK DRIVERs, which guarantee channel-to-channel skew to be below

50 ps.

Each line of the differential pair has a 270R load resistor at the transmitter,

and is terminated by 100R between the differential pair at the BOC receiver

end.

The eight active-low TTC(0-7) outputs are bussed to RODs on two separate

backplane buses, each for 8x RODs. The devices chosen, PHILIPS

N74ABT574D, are Advanced BiCMOS Bus Interface drivers, which guarantees

VOL of below 0.55V while sinking 64mA, and VOH of above 2V while sourcing

32mA.

With TIM is slot 13 in the middle of the crate, the TTC(0-7)A bus is

terminated on the backplane in slot 21, and the TTC(0-7)B bus in slot 5.

Finally, the 16x ROD BUSY signals are received from individual RODs by

PHILIPS N74LVT16244B LV Bus receivers. The active-low ROD BUSY

lines are pulled up to +3V3 on the TIM, thus indicating empty ROD slots

as NOT BUSY.

MP-UCL, 28 May 2004