HARDWARE AND FUNCTIONAL DESCRIPTION

DRAFT NOTES FOR THE TIM  FDR/PRR

MP-UCL, 20 June 2004

TIM FUNCTIONAL MODEL shows the main blocks of TIM :

-TTC INTERFACE with its TTCrx ASIC, producing all the timing, trigger and command signals received from the FLT while in the RUN MODE

-STAND-ALONE generator of the same signals in the SA MODE.

-STAND-ALONE SEQUENCER which allows long command sequences to be down-loaded from the host in the SA  MODE

-THE EVENT FIFOS AND SERIALISERS to allow IDs of multiple triggers to be stored and output sequentially

-BACKPLANE MAPPING logic

TIM SCHEMATICS

This slide shows the schematic representation of the circuitry of the STAND-ALONE generator, together with the suite of NIM and ECL inputs and outputs.

There is an 80.1573MHz X-tal oscillator producing an internal 40.0786MHz clock.

Either this, or any externally generated 40MHz clock can be selected to provide the STAND-ALONE clock.

The fast commands ( trigger and resets ) can be generated internally, either repetitively at a single selected frequency ( 50Hz - 600kHz ), with trigger also available as a random trigger with selected average frequency, or by VME commands from the host by writing to the COMMAND register.

All the TTC-type commands, plus the 40MHz clock, can also be input from external sources in either NIM or differential ECL standard, using front panel connectors.

All the STAND-ALONE commands are synchronised to the selected clock and send to the BACKPLANE MAPPING logic to produce the TTC(0-7) output signals.

In addition, the trigger command can be delayed by a fully programmable delay of up to 143 clock periods to accommodate the FE data pipeline length.

There is a further capability of providing a "burst" of a fully-programmable            ( between 0-16k ) number of either external or internal triggers or calibration triggers.

The selection of any/all of the above externally input or internally generated signals is controlled by an VME accessed ENABLES register, allowing for a full flexibility.

The L1ID counter and register allows any FLT number to be down-loaded from the host interface in STAND-ALONE mode prior to issuing triggers.

The FLTN counter will increment with each subsequent trigger, loading the new number into the FLTN register. In the RUN mode, this register is loaded from the TTC.

Similarly, the BCID counter and register provides the suitably offset BCN for every trigger, in both STAND-ALONE and RUN modes.

The BCNTIN register stores the BCN received from the TTC during run.

Finally, the TTID is stored in the TTYPE register.

The SEQUENCER RAM can be loaded from the host with up to 32k long sequence of commands to allow for various tests of the system.

There is an matching SINK RAM which allows us to store up to 32k of commands being issued by the TIM to RODs. Again, this is invaluable in testing the hardware and timing.

The slide also shows the BACKPLANE MAPPING, together with the backplane outputs synchronising drivers, the backplane PECL clock drivers, the two FIFOs and serialisers for trigger L1ID, BCID and TTID data, plus the TTCrx receiver and interface. As you can see, three sets of signals from three sources - STAND-ALONE, TTC and SEQUENCER - are received by the MAPPING module.

Finally, the basic ROD BUSY system receives ROD BUSY signals from all RODs in the crate and provides a masked-OR output to the FLT in either NIM or TTL standard ( selectable by a link ).

TIM-3 CLOCKS FLOW :

This slide show the clock system of the TIM. In SA MODE, an 80.1573 MHz on-board crystal clock generator provides the required 40.0786 MHz INTERNAL CLOCK.

There is a provision for accepting 40MHz EXTERNAL CLOCK in either NIM or differential ECL standard. Required clocks can be selected in the ENABLES register.

When in the RUN MODE, the TTC supplies the machine clock via the TTCrx ASIC.

There is a couple of delay lines on TIM, allowing for independent adjustments of ROD SETUP ( for TTC outputs timing ) and TIM SETUP ( of internal timings ). These delay lines are set by DIL switches as 6-bits in 0.5 nsec steps, giving more than adequate coverage over the 25nsec clock.

There is an additional delay adjustments available for the STAND-ALONE clock, set by the CLOCK DELAY register as 6-bits of 0.5nsec steps, which can be used to scan.

There are four additional delay lines, used to provide a TRIGGER WINDOW width and delay, which can be used with to reject random triggers.

TIM-3  CLOCK SWITCHING :

To ensure a safe and un-interruptible supply of 40 MHz clocks to all SCT  FE modules, there a fail-safe network of two Zero Delay Glitch-Free Clock multiplexers/PLLs ( ICS 581—02 ).

The first multiplexer select between the X-tal generated internal clock or the externally supplied clock, with the X-tal clock being the default.

The second and final multiplexer is used to select the TTC-generated or stand-alone clock. If there is a failure of the TTC clock, this device switches automatically to the stand-alone clock within 3 clock periods.

Failure of any selected clock is also indicated by the de-assertion

of the TIM-OK line on the backplane.

TIM - OK :

"TIM-OK" is a +5V  TTL "high-when-OK" output, and is set low by either the failure of the TTC clock as received by TIM, or also by the failure of the TIM X-tal clock oscillator when in stand-alone mode.

Additionally, "TIM-OK" will also be set low in case of failure of the +3V3 and/or +5V0 supply on TIM ( eg. by blown fuses ).

( "INTBUSY" & "OA_BUSY" in the Status Register 6 bit 2 do NOT get asserted when "TIM-OK" goes LOW = ie. is not OK )

( "ROD_CRATE_BUSYOUT" is NOT asserted when "TIM-OK" goes LOW = ie. is not OK )

DEFAULT :

After a switch-on and after each "O/A Reset", the internal X-tal clock is automatically selected to produce "CLKIN"

"TIM-OK" output is set to '1', and "TIM-OK" bit 13 in the Status Register 6 is also set to "1"

ERROR CONDITIONS :

1) when the "SetRunMode" bit 12 in Commands Register 1 is set to '1' :

- if "TTCClkOn" line, AND/OR the "TTC-READY_IN" line go to '0', ( ie. The TTC clock is not present for more than 4 clock periods, or the TTC is not in a correct state ) :

Set "TIM-OK" output to '0', and set "TIM-OK" bit 13 in the Status Register 6 to "0"

When this 'error' conditions end, the "TIM-OK "output and "TIM-OK" bit 13  in the Status Register 6, should be reset back to "1"

2) when the "SetRunMode" bit 12 in Commands Register 1 is set to '0', AND when "EnExtClk" bit 8 in Commands Register 1 is set to '1' :

- if "SACLKON" line goes to '0' ( ie. the "CLKIN" is not present, due to either the absence of any external clock on inputs, or any of the inputs being set high, for more than 4 clock periods ), go to default state.

Set "TIM-OK" output to '0', and set "TIM-OK" bit 13 in the Status Register 6 also to "0"

When this 'error' conditions end, the "TIM-OK "output and "TIM-OK" bit 13  in the Status Register 6, should be reset back to "1”

DESCRIPTION OF BUSY's on TIM-3 :

1) On FPGA2, there are internal busys, which get asserted while any of the internally-generated TTC(x) functions are being generated.

They should be as long as the SCT serial code for each 'fast command' :

L1A :				3 clocks

BCR, FER, ECR, Spare :	7 clocks

CAL :              		27 + pipeline-delay-to-trigger + 3 clocks

2) "OA_BUSY" is an OR of all above BUSYs and of "TESTBUSYON"

( "OA_BUSY" in the Status Register 6 bit 2 should NOT get asserted when "TIM-OK" goes LOW = ie. not OK )

3) "TESTBUSYON" is a busy line which is enabled by Commands Register 1 bit 13 "EnTestBusy", and cleared by Commands Register 1 bit 14 "ClrTestBusy" and/or by "OA_RESET"

"TESTBUSYON" is set by the next L1A after being enabled, and should prevent any internally-generated or externally-received fast commands after this L1A being acted upon

"TESTBUSYON" is also output to the Status Register 6 bit 14

4) "INTBUSY" is the "OA_BUSY" shown in the Status Register 6 bit 2

( "INTBUSY" & "OA_BUSY" in the Status Register 6 bit 2 should NOT get asserted when "TIM-OK" goes LOW = ie. not OK )

5) "VBUSY" is set in Commands Register 1 bit 7

6) "EXTBUSY" is the OR of externally-input "NIMEXTBUSY" and "ECLEXTBUSY" from the front panel

It is also a bit 0 in Status Register 6

7) Front panel signal "NIMEXTBUSYIN" is enabled onto the "NIMEXTBUSY" line by selecting link LK7 pins 1+2

( NOTE : If this selection in is used, the "EXTRODBUSYIN" should be grounded by selecting link LK7 pins 3+4 )

8) Front panel signal "NIMEXTBUSYIN" is enabled onto the "EXTRODBUSYIN"

line by selecting link LK7 pins 2+3

9) "BUSYOUT" is the OR of

a) "OA_BUSY" ( same as "INTBUSY" ) enabled by "EnIntBUSY" in Enables Register 0 bit 7

b) "EXTBUSY" enabled by "EnExtBUSY" in Enables Register 0 bit 15

c) "VBUSY" from Commands Register 1 bit 7

d) "RODBUSY", enabled by "EnRODBUSY" in TTC Enable Register 12 bit 7

It is also a bit 3 in Status Register 6

10) "BUSYOUTB" is the buffered "BUSYOUT" sent to 'TIM BUSY' LED

11) "NIMBUSYOUT" is a front panel output of the "BUSYOUTB" signal ( buffered version of "BUSYOUT" ), and can be selected by link LK6 to be either +ve or -ve going

12) "ECLBUSYOUT" is a front panel output, which is selected by link PL41 to be either "BUSYOUTB" signal as above ( PL41 pins 1+2 ), or the "MEXTBUSYOUT" signal ( PL41 pins 2+3 )

13) "MEXTBUSYOUT" is the "EXTBUSY" signal enabled by "EnExtBUSY" in Enables Register 0 bit 15

It is also a bit 1 in Status Register 6

14) "VRODBUSY" is set in Commands Register 1 bit 8

15) "RODBUSYOUT" is a front panel output, which is selected by link LK1 to be either NIM-type signal "ROD_CRATE_BUSYOUTB3" ( LK1 pins 2+3 ) – which can be selected either +ve or -ve going by link LK4 - or TTL-type signal ( LK1 pins 1+2 )

( "ROD_CRATE_BUSYOUT" should NOT get asserted when "TIM-OK" goes LOW = ie. is not OK )

( it should BE asserted while the TIM is in "SAMODE" )

16) "ROD_CRATE_BUSYOUT" is the OR of

a) masked OR of all the ROD-BUSY(x) signals received from TIM backplane

( see ROD Busy Register 16 and ROD Mask Register 15 )

b) "EXTRODBUSYIN" input from the front panel ( see par.8 ) and enabled by the "EnRODBUSY" bit 7 the TTC Enables Register 12

c) "VRODBUSY from Commands Register 1 bit 8

d) "SAMODE"  from Status register 6 bit 11

14) "VRODBUSY" is set in Commands Register 1 bit 8

15) "RODBUSYOUT" is a front panel output, which is selected by link LK1 to be either NIM-type signal "ROD_CRATE_BUSYOUTB3" ( LK1 pins 2+3 ) – which can be selected either +ve or -ve going by link LK4 - or TTL-type signal ( LK1 pins 1+2 )

( "ROD_CRATE_BUSYOUT" should NOT get asserted when "TIM-OK" goes LOW = ie. is not OK )

( it should BE asserted while the TIM is in "SAMODE" )

16) "ROD_CRATE_BUSYOUT" is the OR of

a) masked OR of all the ROD-BUSY(x) signals received from TIM backplane

( see ROD Busy Register 16 and ROD Mask Register 15 )

b) "EXTRODBUSYIN" input from the front panel ( see par.8 ) and enabled by the "EnRODBUSY" bit 7 the TTC Enables Register 12

c) "VRODBUSY from Commands Register 1 bit 8

d) "SAMODE"  from Status register 6 bit 11

17) "RODBUSYOUT" is the "ROD_CRATE_BUSYOUT" set in bit 7 of Status Register 6

18) "ROD_CRATE_BUSYOUTB1" is the "ROD_CRATE_BUSYOUT" sent to TIM backplane J3 pin 21D

19) "ROD_CRATE_BUSYOUTB2" is sent to the 'ROD BUSY' LED

20) BurstBUSY, SeqBUSY, SinkBUSY - see Status Register 6

MP-UCL, 20 June 2004