TIM - OK :

"TIM-OK" is a +5V  TTL "high-when-OK" output, and is set low by either the failure of the TTC clock as received by TIM, or also by the failure of the TIM X-tal clock oscillator when in stand-alone mode.

Additionally, "TIM-OK" will also be set low in case of failure of the +3V3 and/or +5V0 supply on TIM ( eg. by blown fuses ).

( "INTBUSY" & "OA_BUSY" in the Status Register 6 bit 2 do NOT get asserted when "TIM-OK" goes LOW = ie. is not OK )

( "ROD_CRATE_BUSYOUT" is NOT asserted when "TIM-OK" goes LOW = ie. is not OK )

DEFAULT :

After a switch-on and after each "O/A Reset", the internal X-tal clock is automatically selected to produce "CLKIN"

"TIM-OK" output is set to '1', and "TIM-OK" bit 13 in the Status Register 6 is also set to "1"

ERROR CONDITIONS :

1) when the "SetRunMode" bit 12 in Commands Register 1 is set to '1' :

- if "TTCClkOn" line, AND/OR the "TTC-READY_IN" line go to '0', ( ie. The TTC clock is not present for more than 4 clock periods, or the TTC is not in a correct state ) :

Set "TIM-OK" output to '0', and set "TIM-OK" bit 13 in the Status Register 6 to "0"

When this 'error' conditions end, the "TIM-OK "output and "TIM-OK" bit 13  in the Status Register 6, should be reset back to "1"

2) when the "SetRunMode" bit 12 in Commands Register 1 is set to '0', AND when "EnExtClk" bit 8 in Commands Register 1 is set to '1' :

- if "SACLKON" line goes to '0' ( ie. the "CLKIN" is not present, due to either the absence of any external clock on inputs, or any of the inputs being set high, for more than 4 clock periods ), go to default state.

Set "TIM-OK" output to '0', and set "TIM-OK" bit 13 in the Status Register 6 also to "0"

When this 'error' conditions end, the "TIM-OK "output and "TIM-OK" bit 13  in the Status Register 6, should be reset back to "1”