TIM SCHEMATICS

This slide shows the schematic representation of the circuitry of the STAND-ALONE generator, together with the suite of NIM and ECL inputs and outputs.

There is an 80.1573MHz X-tal oscillator producing an internal 40.0786MHz clock.

Either this, or any externally generated 40MHz clock can be selected to provide the STAND-ALONE clock.

The fast commands ( trigger and resets ) can be generated internally, either repetitively at a single selected frequency ( 50Hz - 600kHz ), with trigger also available as a random trigger with selected average frequency, or by VME commands from the host by writing to the COMMAND register.

All the TTC-type commands, plus the 40MHz clock, can also be input from external sources in either NIM or differential ECL standard, using front panel connectors.

All the STAND-ALONE commands are synchronised to the selected clock and send to the BACKPLANE MAPPING logic to produce the TTC(0-7) output signals.

In addition, the trigger command can be delayed by a fully programmable delay of up to 143 clock periods to accommodate the FE data pipeline length.

There is a further capability of providing a "burst" of a fully-programmable            ( between 0-16k ) number of either external or internal triggers or calibration triggers.

The selection of any/all of the above externally input or internally generated signals is controlled by an VME accessed ENABLES register, allowing for a full flexibility.

The L1ID counter and register allows any FLT number to be down-loaded from the host interface in STAND-ALONE mode prior to issuing triggers.

The FLTN counter will increment with each subsequent trigger, loading the new number into the FLTN register. In the RUN mode, this register is loaded from the TTC.

Similarly, the BCID counter and register provides the suitably offset BCN for every trigger, in both STAND-ALONE and RUN modes.

The BCNTIN register stores the BCN received from the TTC during run.

Finally, the TTID is stored in the TTYPE register.

The SEQUENCER RAM can be loaded from the host with up to 32k long sequence of commands to allow for various tests of the system.

There is an matching SINK RAM which allows us to store up to 32k of commands being issued by the TIM to RODs. Again, this is invaluable in testing the hardware and timing.

The slide also shows the BACKPLANE MAPPING, together with the backplane outputs synchronising drivers, the backplane PECL clock drivers, the two FIFOs and serialisers for trigger L1ID, BCID and TTID data, plus the TTCrx receiver and interface. As you can see, three sets of signals from three sources - STAND-ALONE, TTC and SEQUENCER - are received by the MAPPING module.

Finally, the basic ROD BUSY system receives ROD BUSY signals from all RODs in the crate and provides a masked-OR output to the FLT in either NIM or TTL standard ( selectable by a link ).