# A Physics uTCA Solution for the EuXFEL Clock and Control System

E. Motuk, Member, IEEE, M. Postranecky, M. Warren, and M. Wing

Abstract–The development of the Clock and Control (CC) hardware and firmware for the EuXFEL DAQ system is presented. The system will exploit the advances in data handling provided by the new telecommunication crate architecture standard for physics (MTCA.4). The CC board is responsible for synchronising the DAQ system to the overall system timing. The hardware consists of a DESY designed MTCA.4 board and a custom Rear Transition Module (RTM) designed by UCL. Each RTM board controls up to 16 Front End Modules (FEMs) for a 1 Megapixel 2D detector. The firmware for the CC board generates the clock, data and veto signals for the FEMs and it receives the status as a feedback. AC-coupled LVDS links running through standard CAT5 RJ45 cables provide the FEM connection. The CC hardware and firmware are designed to provide the flexibility, extendibility and scalability to support possible future upgrades to the DAQ or larger detectors. It also reduces the cost and effort for the development of such system by using a general-purpose MTCA.4 FPGA board as its processing platform and an RTM to provide the custom functionality.

# INTRODUCTION

The European Free Electron Laser (EuXFEL) facility being constructed at DESY, Hamburg will benefit many areas odf science, from creating extreme states of matter in plasma physics to probing currently inaccessible molecules in structural biology and investigating the structure of large bio-molecules like proteins. [1] Fig, 1 shows how a photon beam impacts on a sample and the diffractive pattern is read out in large, digital cameras. The EuXFEL project is a large-scale facility using high intensity, high energy beams usually only associated with particle physics experiments. The accelerator will be 3.4 km long using novel superconducting RF technology to accelerate electrons to high energy. The electrons will then pass through an undulator, causing them to oscillate and emit LASER-like photons (Fig. 2). When it turns on, the EuXFEL will provide a beam 100 million times more brilliant than current synchrotron sources [2]. To measure the effect of the impact of the particle beam on the various samples, larger Megapixel detectors will be used to reconstruct an accurate diffractive image.

Fig.1

Fig.2

The superconducting acceleration technique involve 1.3 GHz superconducting cavities at a temperature of 2K generating acceleration fields of 20-25MV/m for use at the EuXFEL. A single cavity structure contains nine accelerating cells and eight structures with their beam controlling elements mounted into one cryostat segment. The entire LINAC consists of 101 segments where most of them gets assembled into one long superconducting tube. Despite the use of superconductivity techniques some heat is generated and the accelerating field can be switched on for only 600 ms every 100 ms. This prohibits continuous wave (CW) operation, instead trains of consecutive electron bunches are accelerated. In normal operation 27 000 bunches, compared to 60-120 at the other XFEL’s, are generated per second, grouped to 10 trains with each 2 700 bunches with an inter bunch time separation of 220 ns or 4.5 MHz. This requires that all control and data taking electronics have to cope with 4.5MHz bunch delivery rates interspersed by 99.4 ms inactivity periods. The lasing generated from 20 mm electron bunches provides short X-ray flashes of <100 fs duration [3]. Fig. 3 illustrates the bunch trains and the timings.

Fig. 3

Each generated X-ray flash is intense enough to produce a full diffractive picture of scattering targets. Three different detector designs are in development, namely, AGIPD [3], DSSC [4] and LPD [5]. Although each focuses on different measurement classes and use different technologies, the basic concepts regarding mechanical and functional block design are similar and are sketched in Fig. 4 for LPD. The detector head has a silicon sensor facing to the scattered X-rays. ASIC’s are bump-bonded to their back side and board level electronics is located nearby. This forms a detector head at the beam line which forms the front end module (FEM), in which a vacuum barrier is integrated to reduce the scattering of the X-rays but allows air cooled electronics. Each FEM supports 64 K pixels of the final image, therefore, as an example, for a 1Mpixel camera 16 FEMs are to be used.

Fig. 4

Rack mounted electronics is located a few meters away which comprises the units providing DAQ and infrastructure functionality. The clock and control (CC) system is a part of the rack mounted electronics for the DAQ system which constitutes the interface between the overall EuXFEL timing and the FEM electronics.

# The clock and control system for daq

The DAQ system for the megapixel cameras comprises the CC system, a train builder and the power and the cooling infrastructure to operate the FEM units. The CC system provides the synchronising clock and bunch and train related information to the FEM electronics. It also receives status feedback from FEMs and distributes the veto signals to the FEMs to reject some of the detector data. The FEM electronics provide the read-out data for the Train Builder (TB) board and control the ASIC. Fig..5 presents the general structure of the DAQ system.

The interface to the overall EuXFEL timing system is accomplished through the connections to the timing receiver (TR) board [6] which provides clocks and timing related information to all systems within the facility in order to synchronise the operation. With the connections to the FEM electronics, the CC system synchronises the DAQ timing to the general EuXFEL facility timing.

Fig. 5

## CC system interfaces

In order to synchronise the FEM operations with the overall facility timing the connections to the FEM electronics are realised on AC-coupled LVDS links on CAT6 RJ45 cables. This provides the advantages of improved noise susceptibility and reliability for signal transmission over a length of a few meters. Being a widely available, tested and proven technology, LVDS on RJ45 CAT6 cables drives down the cost and design effort as well.

One RJ45 connector provides 4 LVDS pairs sufficient for the  connections to a single FEM. The names and the description of the signals to FEMs are:

- Output clock (FAST clock): This is a ~99 MHz clock derived from the 4.5 MHz bunch clock.

- Output data (FAST data) : This provides the trigger start signal and train ID data to the FEMs.

- Veto : This signal is the bunch reject data encoded on a either 99 or 4.5 MHz clock.

- Status : This is a status feedback from the FEMs.

Fig. 6 shows how these signals are assigned to a RJ45 connector.

The interface to the overall facility timing is realised through the connections to the TR system. These connections are:

- Start/Trigger: The signal which denotes the start of the bunch train.

- Telegram Data: The information sent by the TR system comprising the bunch train number and the index of the bunch train pattern.

- Telegram Clock: This is the clock that the telegram data is sent synchronous to.

- Bunch clock: The 4.5 MHz bunch clock that is common to all the units in the facility.

- Extra connections for various function such as resetting the CC system and status from the CC system.

The physical interface that these connections are realised is explained in the next subsection.

Fig. 6

In addition to the interfaces to the FEMs and the TR system, there needs to be another interface to the overall software control system. Through this interface tasks such as initial configuration, emergency shutdown, and overall status checking are performed.

## General Hardware

The TCA standards used in telecommunications are chosen to be used at EuXFEL for the advantages of, 1) Providing a multi-layer highly scalable managed platform

architecture designed to achieve the highest possible system

availability, and 2) being the first all-serial communication platform available to the physics

community to support both massively complex accelerator

controls and massively large, high bandwidth and throughput

experimental data acquisition systems

[7]

The recent extension to the uTCA standards, MTCA.4, namely uTCA for physics [8], defines mechanics for supporting large I/O bound physics applications through the use of Rear Transition Modules (RTMs). It also defines backplane connections to support high-speed serial differential links as well as dedicated channels and differential buses to distribute high quality clock and trigger signals. The use of RTMs also allows flexible combinations to design modular and scalable systems in order to reduce the overall design effort and cost.

Fig.7

For the CC system we have gone for a MTCA.4 solution. The system consists of a MTCA.4 AMC (Advanced Mezzanine Card) board and a Rear Transition Module (RTM). The MTCA.4 AMC board (DAMC2) [9] is designed by DESY as a multi-purpose FPGA hardware platform for various projects in DESY and provides the processing capability needed for the CC functionality. We have developed our own custom RTM board according to the MTCA.4 standard which connects to the DAMC2 through two thirty-pair Advanced Differential Fabric (ADF) connectors. This configuration enables us to deliver the required functionality with a modular and scalable solution that also led to a significant reduction design effort duplication. Figure 7 shows the DAMC2 board and the CC RTM mated outside the MTCA.4 crate.

Fig. 8

The TR board [10] is also an AMC board and will be housed in the same MTCA.4 crate. Fig. 8 shows the connections on the MTCA.4 backplane. 8 bussed LVDS links provide the necessary connections mentioned in the previous section between the TR board and the CC system. The 4.5 MHz bunch clock from the TR board is transmitted on the high-quality TCLK link on the backplane. The interface to the overall software system is realised on a 4-lane PCIe link which connects the all AMC boards to the CPU board which is also housed in the crate.

Fig. 9 provides the actual photo of the RTM with the main building blocks are enclosed in numbered circles.

The first thing to observe is the stacked RJ45 connector which provides channels to support up to 16 FEM modules. This enables the capability of using one AMC/uRTM combo for a 1MPixel 2D detector.

Fig.9

The two ADF connectors provide in total 54 differential links to the Xilinx Virtex5 FPGA on the DAMC2 board  in addition to 12V power, MTCA.4 specification related I2C link, 3.3 V management power and the uRTM present signal. In our implementation we use the FPGA links in both differential and single ended configuration. The differential links are used for the clocks into and and from the uRTM, veto signals and FAST data out to the FEMs, status signals from the FEMs and links to and from a spare RJ45 connector which is also placed on the front panel for possible future capability extensions. The single ended links comprise the signals to control for the PLLs, to control the LED controllers for the stacked RJ45 connector, status signals from the PLLs, and a number of spare signals for test and debug purposes.

The block number one in Fig. 9 is the clock generation block whose main function is taking a differential clock from the DAMC2 board and multiplying this in order to generate the clock for the FEM units. For standalone operation, or in the absence of a clock from the main system, there is a clock crystal chip on board as well. In order to provide the highest flexibility, a glitch-free, zero-delay PLL based clock multiplexer is used to select between the system clock and the standalone clock. This selected clock is input to another PLL chip whose multiplication factor is controlled by the FPGA. This PLL chip generates the FAST clock signal to the FEMs which is input to a 1:16 LVDS fan-out buffer. The outputs of this buffer is connected to the respective pins on each of the 16 RJ45 channels. The output of the second PLL is also sent back to the FPGA on a differential link.

The block number two is the circuitry for the MTCA.4 management requirements. This blocks includes an I2C expander chip for controlling the LEDs on the front panel, checking the status of the hot-swap switch and enabling the power supplies on the uRTM by the MMC on the DAMC2 board. Additionally there is a EEPROM and a temperature sensor chip all on the same I2C bus as the expander. The EEPROM holds the uRTM information and MTCA.4 specified information for electronic keying.

Fig. 10

The block number three comprises the LVDS receivers which gather the status data coming from the FEMs. A special circuitry involving two LVDS buffer chips for each FEM channel is used. [11] The reason for this circuitry is to provide necessary level shifting on the AC-coupled LVDS data link in the absence of a decoding algorithm providing a DC-balanced link. This circuitry has been tested on a development board before being added to the CC uRTM design. The results of the test are included in the results section.

The CC system is designed to provide scalability to support higher Mpixel detectors by using extra AMC and uRTM board pairs in the same crate with one pair designated as master and sharing the clock and the data through the MTCA.4 backplane. Each CC board pair takes up two slots (double full-size board) on the crate which comes in either 6 or 12 slot configuration. [8]  Therefore up to 6 Mpixel cameras can be supported by a single 12 slot crate as shown in Fig. 10.

## Firmware

The firmware block diagram for the CC board is shown in Fig. 11. The FAST TX module generates the FAST data to the FEMs. The telegram RX module receives and decodes the information coming from TR board on a clock and data pair through the backplane. The control and clocking module provides the control signals for all the modules in the firmware and the components on the uRTM. Another function of this block is to provide synchronisation of the FAST data to the FAST clock that is generated on the uRTM as well as instantiating the on-chip PLL units to produce the necessary clocks for the firmware modules. The PCIe block provides the connection to the crate CPU to receive configuration data from the control software [12]. There is a separate module for generating the veto data to the FEMs. The RocketIO module makes use of the SFPs on the DAMC2 board and provides an optional veto source input to the firmware. All the modules in the firmware are integrated through a bus and a register block called Internal Interface (II) [13] which is developed by DESY.

Fig.11

# Tests and Results

As mentioned in the previous section the level-shifting circuitry for the AC-coupled LVDS links was tested before being used in the CC uRTM. The tests involved sending pseudo-random bit patterns generated by a linear feedback shift register (LFSR)  realised on the FPGA of a development board (Xilinx XUPV5). A special daughterboard that provide the transmit and receive LVDS links through RJ45 connectors was designed to fit the extension connectors on these boards [14]. The receive circuitry on the daughterboard comprised the LVDS level-shifting circuitry. The bit patterns consisted of 22-bit words synchronised to the 4.5 MHz clock and sent along with the 99 MHz clock which would be the case in the actual operation of the FAST data link. Another development board received these patterns and compared them with the internally generated patterns using the same LFSR structure and the seed.. The 99 MHz clock was sent on another LVDS line pair through a 5 metre CAT6 Ethernet cable. Fig. 12 shows a captured snapshot of the test on Chipscope. The counter named error_count is incremented in the case the pattern received doesn't match the internally generated pattern. The FPGA source code also contained other test cases where  long series of zeros or ones were introduced between the pseudo-random bit patterns. All of the tests were run for long periods of time (>24 hours) and no errors were detected on the link.

Fig. 12:

A series of other tests were executed after receiving the uRTM prototype. The first test involved checking the compliance to the MTCA.4 standard such that the power to the uRTm would be enabled by the DAMC2 and the necessary 3.3 V voltage rail on the uRTM would be generated by the DC-DC converter onboard. This would ensure the I2C link between the MMC on the DAMC2 board and the CC uRTM operating correctly and the data on the uRTM EEPROM was correctly written to and read. The test was successfully passed by the uRTM and the relevant LEDs on the front panel and on the board were lit up.

The second test involved the observation of the 99 MHz clock to the FEMs and the corresponding FAST data signal. These signals were observed on the special test points that are added to a single channel at the output of the LVDS clock and data fan-out chips.

Additional tests to prove the functionality of the uRTM are still being devised. These include the successful reception of the telegram clock and data from the TR board in the MTCA.4 crate and the reception and the quality of the 4.5 MHz bunch clock in terms of jitter from the crate MCH.

# Conclusion

The CC hardware/firmware system is designed to provide the flexibility, extendibility and scalability to support possible future upgrades to the EuXFEL 2D megapixel camera DAQ. It also reduces the cost and effort for the development of such system by using a general-purpose MTCA.4 FPGA board as its processing platform and an RTM to provide the custom functionality.

# References

- European XFEL GmbH, http://www.xfel.eu/ .

- M. Altarelli et al., “The european X-ray free-electron laser, Technical Design Report”, DESY 2006-097

- P. Goettlicher for the AGIPD consortium, “Electronics for the european XFEL: AGIPD a high frame rate camera”, "Single event gate rupture on thin gate oxides," in TWEPP 2010, Aachen, Germany,  iopscience.iop.org/1748-0221/5/12/C12015 .

- M. Porro et al,, “Expected performance of the DEPFET sensor with signal compression: A large format X-ray imager with mega-frame readout capability for the european XFEL”, Nucl. Instrum. Met. 624 (2010) 509.

- J. Coughlan, S. Cook, C. Day, R. Halsall, and S. Taghavi, “The data acquisition card for the large pixel detector at the european-XFEL”, submitted to TWEPP 2011, Vienna, Austria

- K. Rehlich, “Preliminary: Timing system for XFEL”, unpublished http://tesla.desy.de/doocs/doocs.html

- R. S. Larsen, “PICMG xTCA standards  extensions for physics: New developments and future plans”, Real Time Conference (RT), 2010 17th IEEE-NPSS , vol., no., pp.1-7, 24-28 May 2010

- “MTCA.4, MicroTCA enhancements for rear I/O and precision timing”, PICMG Specification

- P. Vetrov, “Versatile AMC for XFEL machine control”, in ATCA workshop presentation of IEEE Real Time Conference, May 24-28, 2010, Lisboa, Portugal

- A. Hidvegi, P. Gessler, K. Rehlich, C. Bohm, “Timing and triggering system prototype for the XFEL project”, in IEEE Transactions on Nuclear Science, Vol. 58, No. 4, August 2011

- Martin Postranecky, UCL, personal communication

- Chris Youngman, EuXFEL GmbH, personal communication

- K. T. Pozniak, “Internal Interface”, TESLA Report 2005-22, DESY, Hamburg

- Xilinx Inc., “XUPV5-LX110T(ML505) User Guide”

	Manuscript received November 4, 2011. (Write the date on which you submitted your paper for review.) This work was supported in part by the U.S. Department of Commerce under Grant No. BS123456 (sponsor acknowledgment goes here).

Full names of authors are preferred in the author field, but are not required. Put a space between authors' initials. Do not use all uppercase for authors' surnames.

F. A. Author is with the National Institute of Standards and Technology, Boulder, CO 80303 USA (telephone: 303-497-3650, e-mail: author @boulder.nist.gov).

S. B. Author, Jr., was with Rice University, Houston, TX 77005 USA. He is now with the Department of Physics, Colorado State University, Ft. Collins, CO 80523 USA (telephone: 970-491-6206, e-mail: author@lamar. colostate.edu).

T. C. Author is with the Electrical Engineering Department, University of Colorado, Boulder, CO 80309 USA, on leave from the National Research Institute for Metals, Tsukuba, Japan (e-mail: author@nrim.go.jp).