CLOCKING ISSUES WITH DAMC2 + CC RTM

1- The clock distribution/generation scheme under serious consideration.

The diagram below shows the scheme we are considering.  The CC card refers to a combination of RTM and the base card.

2- Current capabilities of DAMC2

- DAMC2 clock distribution circuitry consists of a I2C controlled XO and a 4x4 crosspoint switch.

- XO can generate  any-rate programmable frequencies from 10 MHz to 945 MHz and select frequencies up to 1.4 GHz. (according to the schematic part 10-810 MHz)

- The start-up frequency for the part is 118 MHz. (according to the schematic part)

- The crosspoint switch can route any of the four inputs to any of the four outputs and also can act like a low jitter clock fan-out buffer.

- The diagram below shows the clock distribution circuitry.

3- Issues

- There doesn't seem to be a PLL on the DAMC2 card. We want to generate the 99 MHz clock from the 4.5 MHz bunch clock by an external PLL.

- The PLL can be on the RTM. However, this clock must go back to the DAMC2 card to be distributed across the backplane on a TCLK line.

- According to the diagram and the clock circuitry schematic for the DAMC2, there is no facility to supply a TCLK to the xTCA backplane. (TCLKs are inputs to the x-point switch)

- Any generated clock on the RTM can return back to the FPGA via the RTM connector. (There is one pair on the RTM connector that is routed to a GCLK input of the FPGA)

4- Solutions and more issues  ( very very rough/sketchy/incomplete/silly)

- As stated above PLL can be on the RTM. 99 MHz clock is generated here.

- Needs to go back to DAMC2

- In turn needs to be on TCLK line for the slave cards

- DAMC2 doesn't seem to drive any TCLKs.

- All the RTM lines apart from one clock pair go to the FPGA

- FPGA will direct the 99MHz clock to the slaves? (act as buffer?)

- The skew problem. All the 99MHz clocks that the FEEs see should not have any skews.

- Not quite possible here.

- No feedback to the PLL on the RTM for possible deskewing

- What about the data for the slave cards?

- Data (generated by the FPGA) will be on the MLVDS bus

- Clock will be on the TCLK lines

- Alignment should be fixed? (Goes back to skew on the 99Mhz and there will be different delays on the data for different slaves)

- DAMC2 should incorporate a mechanism to drive TCLKs

- Is just this enough. How can it be done? Bi-directional buffers TCLK lines?

- Should it also incorporate an external PLL?

- No need to send the 4.5Mhz clock to the RTM

- What about TR generating the 99 MHz clock? :-)