XFEL C&C Hardware - NOTES

Friday 27th November 2008

XFEL C&C SYSTEM HARDWARE :

- C&C Master

- C&C Slaves = C&C FanOuts

Notes :  Shot = Train : train of up to 3k bunches

Event Number = Train ID : Unique number of train/shot

It is sent as a part of the Trigger

'Telegram' by TR

Max. Train Frequency : 10 Hz ( possibly up to 30 Hz )

Fixed RF of 600 usec = max. 3000 bunches,

i.e. 99.4 msec gap between trains/shots

Max. Bunch Frequency : 5 MHz ( can be LOWER or 'RANDOM',

but always at 5 MHz = 200 nsec spacing,

full or empty )

Bunch Pattern : There are various possible bunch patterns,

with bunches spaced at lower and/or varying

frequency(ies), or with bunches sent to

more than one beamlines in the same train

Bunch Pattern ID : Unique identifier of a particular Bunch Pattern

It might be sent as a part of the Trigger

'Telegram' by TR, or generated from the

'Bunch Pattern' by C&C SYSTEM

Trigger :  A single pulse sent by TR to C&C Master

a fixed period ( ~ 15 msec ) before the start of each

Bunch 'Train'

Start Pulse :  A single pulse derived - with fixed latency –

from ( or identical to ) the  'Trigger', and sent to all

FEEs at fixed time period before the start of

Bunch 'Train'

Synchronised to the 100 MHz & 5 MHz clock

Stop Pulse :  A pulse/signal derived from the 'Trigger',

sent to all FEEs at the end of each Bunch 'Train'

Synchronised to the 100 MHz & 5MHz clock

Bunch Veto :  A hardware-generated pulse to indicate

the CURRENT Bunch is to be ignored by the relevant

detector

A) C&C SYSTEM REQUIREMENT

- C&C SYSTEM will receive and process / store timing signals ( 5 MHz Bunch

Clock, Trigger, Event Number, Bunch Pattern info or Bunch Pattern ID )

from TR ( Timing Receiver ) board in the same crate.

- C&C SYSTEM will receive, from dedicated Bunch Veto Hardware interface,

Bunch Veto signals.

- C&C SYSTEM will receive one Status/Error signal from each FEE ( plus

possibly other info, e.g. FEE 'Busy' ???  )

- C&C SYSTEM will provide and distribute to each FEE, via C&C FanOuts,

the following 3 fast signals : un-interrupted 100MHz clock ( PLL-ed to 5MHz

Bunch Clock ), the Start Pulse ( followed by Train ID & Pattern ID,

followed by the Stop Pulse/Signal ) and Bunch Veto Signal.

- C&C SYSTEM will process any 'Busy' signal received from Crate Controller

and stop issuing the following 'Start Pulse(s)'.

- C&C SYSTEM must be able to operate stand-alone, without any TR

interface, and therefore must generate the 5MHz clock, 'Trigger' and

the other relevant timing signals.

- C&C SYSTEM should be able to operate with other light source timing

systems, i.e. be able to accept ext. clock ( and ext. trigger ) at other

frequencies ( 0 - 20 MHz ???? )

NOTE : If 100 MHz is to be synchronised to this, there is a problem...

- C&C SYSTEM must store and make available relevant timing information

- C&C SYSTEM should provide some minimum diagnostic and visual ( LEDs )

indication of clock & trigger performance and each FEE's

presence/absence/power/status ( ??? )

B) C&C MASTER

C&C Master receives from TR ( Timing Receiver ) board in same crate :

- 5 MHz Bunch Clock ( diff. LVDS on front panel on pre-prototype,

later on backplane ?? )

- Trigger ( = Start Bunch Train )  ( -ditto- ) : single pulse, fixed time

before first bunch

( 15 msec ?? )

- Telegram :

- Bunch Pattern ( or Bunch Pattern ID )                   } received on separate

and                                                                           } line(s) or serially

- Event Number ( unique number or GPS time ?? ) } on 'Trigger' line ???

C&C Master sends to C&C FanOut :

- 100 MHz clock ( PLL from 5 MHz Bunch Clock, with fixed phase.

Programmable delay for overall clock output )

( maximum acceptable jitter ??? )

- Start Bunch Train ( from 'Trigger' )                  }

- Bunch Pattern ID                                             } all on same line

- End Bunch Train ( train length + delay later )  }

Synchronised to the 100 MHz clock

( with programmable delays for overall 'Start' and 'Stop' )

- Overall C&C system RESET pulse

C&C Master Receives from C&C FanOuts :

- Error/status lines ( to show each FEE plugged in & powered-up )

- Bunch Veto signal from individual VETO HARDWARE interfaces

( plus possibly other info, e.g. FEE 'Busy' ???  )

C&C Master receives, stores & makes available :

- Bunch Veto Info ( ??? )

- Event Number = Train Number ( from TR )

- Bunch Pattern ID ( from TR )

- Total Number of bunches expected ( from Bunch Pattern ID ??? )

- Busy / Throttle information

plus

- Calibration Pattern  ( ??? )

- Veto Disable Pattern ( ??? )

C&C Master STAND-ALONE SYSTEM :

- 5 MHz clock generator for Stand-Alone operation

- 100 MHz clock to be PLL from Stand-Alone 5 MHZ clock if no ext. input

- must be able to handle 'Bunch Clock' from various sites, i.e.

not necessarily at 5 MHz => 'Ext. Clock' input between 0 - 20 MHz ???

( What happens to the 100 MHz clock synchronisation with Ext. Clock ? )

- generate 'Trigger' ( incl. Bunch Pattern ID ) - under software control

- pre-programmed patterns

- push-button for test

C&C Master Functionality :

- PLL of 100 MHz clock to the input 5 MHz clock at fixed phase, with

programmable delay ( step size ??? )

- Store Bunch Veto information

- Store Bunch Pattern ID / generate from pre-loaded 'Bunch Pattern'

( a 'look-up' table ??? )

- Process 'Trigger' ( from TR ) to produce 'Start Bunch Train'

pulse and 'End Bunch Train' signal  ( train length + delay later )

( with programmable delays for overall Start and Stop pulses )

- Internal counter for Trains ( not Event Number )

- generate CLOCK-OK signal when in 'run' mode

- generate RESET for C&C Master and C&C FanOuts - on power-on

- under software control

- push-button for test

C) C&C FANOUT

Each C&C FanOut receives signals from C&C Master and distributes them to individual FEEs

( where / how : - custom backplane ( ??? ) => liaise with TR people

- on front panels ( ??? )

- Receives feedback from individual FEEs

- Up to 20 separate FEEs to be accommodated now

( expandable up to 64 in the future ??? )

- How many per FanOut : hardware / space limitation / connector size

Each C&C FanOut sends to all FEEs :

- 100 MHz clock ( with individual programmable delay on each channel ??? )

- Start Bunch Train ( from 'Trigger' )     }

- Bunch Pattern ID                                } all on same line

- End Bunch Train                                 }

Synchronised to the 100 MHz clock

( with individual programmable delay on each channel ??? )

- Bunch Veto

Each C&C FanOut receives from each FEE :

- Status/Error line ( to show FEE plugged in & powered-up )

( plus possibly other info, eg. FEE 'Busy' ???  )

C&C FanOut Diagnostics :

- LED per STATUS/Error line ( use RJ45's LEDs ??? )

- LED to show clock running

- LED to show trigger running

- ???

D) HARDWARE OPTIONS / QUESTIONS

- no radiation, no magnetic field requirements

- halogen-free cables required

- any C&C System interface to MPS ( Machine Protection System ) ( ??? )

- all diff. LVDS signals to/from C&C FanOut boards

- all LVDS lines to be AC-coupled on C&C FanOut :

- check out AC-coupling, Opto-coupling, Magnetic-coupling

- check on maximum cable lengths requirement ( ??? )

- fixed cable lengths for synchronous signals

- 'balance code' for A/C-coupled signal lines ( ??? )

- RJ45 connectors suggested ( check on double-stack RJ45

availability & height ??? )

-  check on HDMI - slim, extra pair of

t/p wires, shielding….

- double-stack HDMI ???

- mini-HDMI  ???

/cont :

- C&C FanOut Power requirements - check whether could use the crate

'always-on' P/supply ( ??? )

- alternative - use separate P/supply

via custom backplane ( ??? )

- Architecture options : - individual Fan-In/Out cards, with 4 ( ??? )

individual connections to 4 FEEs

- separate Fan-in and Fan-out cards, with an

secondary active Patch Card near/at the FEE

=> Fan-In has 1 input from 4 FEEs on each cable

via Patch Card

=> Fan-Out has 1 output to 4 FEEs on each cable

via Patch Card

=> Each Fan-In and Fan-Out card can handle

16 FEEs

- C&C FanOut cards : could be separate crate - cards only require power

and connections to C&C Master