Clock and Control Implementation for 2D Pixels

Erdem Motuk, Martin Postranecky, Matt Warren, Matthew Wing, Chris Youngman

1	Introduction	1

2	Detailed CC Structure	2

2.1	Topology	3

2.1.1	FEE facing signalling (cc-master to/from cc-slave)	3

2.1.2	Machine facing signalling (cc-master to/from TR/VETO/CrateCPU)	3

2.2	FEE Connector choice – RJ45	4

3	Crate Connectivity	4

3.1	FEE Clock	5

3.2	Bunch Clock	6

3.3	Trig/Start (RX17)	6

3.4	EncClock (TX17)	6

3.5	BunchClock (RX18)	6

3.6	Spare (TX18)	6

3.7	Reset (RX19)	6

3.8	CC Command (TX19)	6

3.9	CC Veto (RX20)	6

3.10	CC Status (TX20)	6

4	Clock and Control Hardware Specifics	7

4.1	Suitability and Possible Issues	8

4.2	Telegram Distribution Concerns	10

5	Crate layout and card connector modularity	11

6	Operating Modes (Use Cases)	11

6.1	XFEL Running	12

6.2	TR Standalone and Testing	12

6.3	Standalone	12

6.4	Other Beamlines	12

7	Typical Run Timeline	13

8	Timing Receiver Questions and Requirements	13

9	Control software	14

10	Veto Sub-system	14

11	CC Hardware requirements	15

12	Firmware required	15

13	Timelines	15

14	References	15

13 References	13

# Introduction

The Clock and Control (CC) system for the 2D pixel detectors at XFEL must integrate with “neighbouring” systems and XFEL standard configurations. Areas of focus are the crate/backplane standard chosen, the interface to the machine and requirements of the 2D pixel detectors.

A schematic of the system is shown below. All CC electronics is housed in one or multiple xTCA “Timing” crates. The crate backplane provides interconnect between CC and other components,  as well as a CPU, disc, networking etc.

Fig 1: Overall Timing Crate Structure

The CC must be able to operate standalone for testing purposes, and as part of a larger machine infrastructure. The machine interface is provided by the Timing Receiver (TR) board. Other machine interface boards (e.g. for the Machine Protection System) can also be located in the Timing crate.

The CC system (a sub-component of Timing) comprises a “master” and a number of slaves (if needed). Slave hardware is identical to master hardware, but in some areas, e.g. clock generation, we need to have only one source, so the master will transmit, and the slaves receive. This will be configurable by software.

# Detailed CC Structure

Detail on CC signals and connections:

Fig 2: Detailed CC connections

## Topology

Master/Slave detail

### FEE facing signalling (cc-master to/from cc-slave)

- FEE Clock

- Command

- Veto (do we support the two different (BC and 100MHz) modes in one crate? Could be done with one signalling line if 100MHz is always sent and conversion to BC type is performed by the slave. Would allow operating LPD and AGIPD detectors from 1 crate)

- Status

- Calibration Trigger Output (what is this?)

### Machine facing signalling (cc-master to/from TR/VETO/CrateCPU)

- Bunch Clock

- Start/Trig

- Telegram

- PCIe

- External Clock

- Reset

- The list above is incomplete, another problem is that some of the signals are not always present(?) depending on running/standalone/test mode – this  needs to be clear.

## FEE Connector choice – RJ45

Fig 3: Pin connections on the RJ45 connectors

Actual connectors to be used are Tyco 2x8 double-stack parts.

See :  http://www.tycoelectronics.com/catalog/products/en?q=6116317-2

# Crate Connectivity

The Timing crate will be of the xTCA format, with a backplane and layout as per the xTCA4Physics specification. The backplane provides the connectivity required between the crate-PC, Timing Receiver, CC boards. Many of the lines are reserved exclusively for CC functions in the Timing crate.

Backplane signalling is split into two types:

- Point-2-point: Star mesh clock lines (TCLKA/TCLKB) that connect to a cross-point switch on the MCH. This allows any card in the crate to be configured (by software) to source a clock for use by all the other cards.

- Bussed: lines (RX17-TX20) that use the M-LVDS standard to provide high-speed connections with the option of open-collector like operation if needed.

The table below shows the bussed and p2p signals used, and their allocated functions in a CC crate.

| Port | Name | Description |
| --- | --- | --- |
| TCLKA | FEC | 99 MHz FEE clock |
| TCLKB | BC | Bunch clock (usually 4.51 MHz) |
| Rx17 | Trig (Start) | See section 3.3 |
| Tx17 | EncClock | Preferably data only (See section 3.4) |
| Rx18 | BunchClock | Preferably 108 MHz telegram clock (See section 3.5) |
| Tx18 | Spare |  |
| Rx19 | Reset | See section 3.7 |
| Tx19 | CC Command | See section 3.8 |
| Rx20 | CC Veto | See section 3.9 |
| Tx20 | CC Status | See section 3.10 |

The figure below shows these connections pictorially giving more detail on direction and source.

??? We need to be clear on which signal are expected by the other non-CC cards in the crate (so far I think it is only the MPS)  ??? Suggestion: finish this document as well as we can and then get the third party card owners (MPS, TR, etc.) into a video conference in a.s.a.p.

Most of the TR signals are user definable, although using a “standard” configuration is desirable if possible.

Fig 4: Diagram showing crate connections

Specifically:

## FEE Clock

We will use TCLKA to broadcast the 2D detector specific FEE Clock (99MHz). The source of these will be either a CC board configured as Master, or the TR, depending on mode of operation.

[NOTE to selves: the master will need to deskew own clock when using TCLK to broadcast to slaves!]

## Bunch Clock

We will use TCLKB to broadcast the Bunch Clock. The source of these will be either a CC board configured as Master, or the TR, depending on mode of operation.

[NOTE to selves: the master will need to deskew own clock when using TCLK to broadcast to slaves!] Deskewing is, for me, a technical detail the implications of which I cannot fully understand, it sounds like you do not like or want it. Please tell exactly what it is and what the downside is (can it be performed automatically, etc.).

## Trig/Start (RX17)

Synchronous with the bunch train and bunch clock, this signal indicated a train will arrive a fixed number of bunch clock periods later. This is used by the CC to synchronise all FEEs.

## EncClock (TX17)

Used to transfer to broadcast the telegram from the TR to other cards in the crate.

Protocol is undecided, but it is hoped to use the FEE Clock for this transfer.

The original idea is to encode with clock (e.g. Manchester) but this could be abandoned if another clock can be chosen.

See section 4.3.

## BunchClock (RX18)

What it says on the tin. We don’t need this, - redefine as TelegramClock? The bunch clock is p2p distributed so you do not need this line. To be used as a telegram clock strobe would require its implementation in the TR(correct?)

## Spare (TX18)

Errrr, spare!

## Reset (RX19)

Timing Reset. “Depth” of reset needs to be clarified??? Please define depth. Is the signal really coming from the TR, if so is it should be in the requirements. What is the functionality of the reset (who fires it, what does it drive, )

Did we not have a “reset” button – such a reset must also be possible from the crate control CPU. What does this reset do needs to be defined.

## CC Command (TX19)

Distributes FEE Command signalling.

## CC Veto (RX20)

Distributes FEE Veto signalling.

## CC Status (TX20)

If a slave flags an error the master could stop issuing starts (configurable by software). Will be a wired-OR line.

# Clock and Control Hardware Specifics

Will use the DAMC2, with custom RTM, and optionally custom FMC for external inputs.

Fig 5: CC module, with DAMC2 and custom RTM.

The current design for the DAMC2 board includes 54 differential pair signals connected between the FPGA and the RTM connector. One pair is dedicated to a clock from the clock generation part (TCLKA distribution).

Fig 6: DAMC2 block diagram (uRTM and AMC connector section)

## Suitability and Possible Issues

The number of differential pairs that a CC RTM needs depends on the number of CC slaves supported. Each slave requires 3 input pairs (including the 99MHz clock) and 1 output pair for status. If 16 CC slaves would be supported by a single CC master, there is a need for 64 differential pairs of signals.

Fig 7: Signals to a CC slave

This is obviously higher than 53.

Solution:

If the dedicated clock connection from the clock gen/distribution part of the DAMC2 is used and this clock is distributed to the slaves then the number of pairs needed becomes 48. I guess from this that the dedicated clock of the DAMC2 has an allocation of RTM connector line pairs?

Issues here:

- According to the clock and control fast signal specification document there is an option of skewing the command lines (Start/Info/Stop and Bunch Veto) with respect to the 99MHz clock. Would this be possible?

- There is a need for a clock distribution/buffer circuitry on the RTM to supply 16 clocks to the slaves and there should be no skew or phase difference between these clocks.

- There will be a skew between the data lines going out to the RTM unless all 54 pairs have the same trace lengths. These might have to be compensated on the FPGA.

Furthermore, the status signal may not have to be differential. However, this depends on what kind of functionality should be implemented by this signal.

More information is needed on the clock generation/distribution block of the DAMC2. There is a plan to generate the 99MHz clock form the 4.5MHz bunch clock using an external PLL. Alternatively, this clock can also be generated by the FPGA's own PLL.

The rough block diagram of the CC RTM should look like the diagram below.

- Fig 8: CC RTM

## Telegram Distribution Concerns

According to the TR specification a 108 MHz encoded clock will have the telegram data that the CC requires.  We have preliminarily decided the telegrams to be

- Start Train

- Train Number

- End Train

- Bunch Pattern Index

- Bunch Pattern Content

The scheme for encoding the data is not determined yet. This will also depend on the encoding of data on the 1.3GHz clock line from the timing transmitter. There are various encoding schemes including Manchester and 8B/10B.

The telegram data will be written to designated registers on the FPGA. There is also an option that these registers might be written by the CPU card over PCIe. (Don't know if this is the case)

Which FPGA do you mean? Suggestion always write cc-master-FPGA, cc-slave-FPGA, tr-FPGA. I make a guess and assume you mean cc-master-FPGA. The CPU must have r/w (sometimes r only) access to CSR on cc-master and cc-slaves, and r access to memory used for cc-master and cc-slave monitoring.

The ideal case for CC involves receiving data and its 108 MHz clock sent by the TR in a source-synchronous manner (put in the TR requirements and let Kay respond – maybe this is easier than asking him on the phone or per email). This would simplify extraction of the telegram data. If this will not be the case  then there will be a need for data recovery logic on the FPGA (we don't need to recover the 108 MHz clock) the details of which will be defined by the encoding scheme. (is this the firmware that Sam developed for LPD?)

# Crate layout and card connector modularity

A crate will contain TR, CPU(+hard disk), MPS and the CC system. All cards are double-width, mid-size, which defines the front (rear) panel space.

Crates are available in 6 and 12 slot options. This is defined by xTCA4Physics spec and we will stick to it!

.

A mid-size RTM will only allow 8 of our chosen connector per slot. This could mean we would need 2 slots per 1 Mpix 2D detector. 4Mpix will fill 8 slots in the crate (and require 8x DAMC2). And alternatively we can use a 2 slot wide RTM and populate every second slot. See diagram below.

Fig 9: CC Crate layout

Although we will evaluate the design presuming 16 FEE connectors per slot, a first prototype might only have 8.

Ideally a 12 slot crate should be able to support 4-8Mpix.

# Operating Modes (Use Cases)

To help understand the decisions taken, the most likely use cases are outlined.

## XFEL Running

TR receives machine clock and telegram. The local clocks and telegram are derived from this and distributed to the crate.

The CC system processes and distributes these to the FEEs

Hierarchy: Machine -> TR -> CC -> FEE

## TR Standalone and Testing

(??? Could this mode also be similar to smallDAQ?) Yes, but do I understand what you mean by similar to?

In the mode the TR generates signalling as if connected to the machine. This will allow testing and debugging of a Timing crate.

Another option is to presume a machine emulator exists to drive a TR (probably another TR with different firmware) – if so, we need to consider space in the crate.

I’m still not clear about what the TR really can provide for self test. A separate crate with a test system (TR with other firmware) generating 1.3GHz + telegrams would be my preferred solution, which removes the requirement for space in the CC crate and is portable/reusable.

Hierarchy: Software -> TR -> CC -> FEE

## Standalone

The is a the situation where we do not use a TR at all (no external inputs)

So we must:

- Generate Bunch Clock

- Generate FEE Clock (99MHz)

- Generate Start/End (in Bunch clock steps)

- Generate Output Trigger for external device

  - Programmable delay wrt Start Signal (in BC steps + ~1ns steps)

  - Front-panel output, using an FMC.

- Generate Reset

- Generate CC Veto (from veto input)

- Sequencer

  - Need to decide at what level this operates – bits in serial streams or slightly higher.

We need NOT handle:

    - FEE Internal Calibration

  - FEEs (not CC) must set internal delays wrt Start Signal (sub BC step)

Hierarchy: Software -> CC -> FEE

## Other Beamlines

TR must handle any clock translation.

Hierarchy: Other Machine -> TR -> CC -> FEE

For Petra3 we get BT (revolution counter in singleton mode) and a 500 MHz clock. Using the PBU we would crudely time in the setup followed by fin tuning using TR or CC-master. Need firmware on the TR to do clock translation, start/trig generation and telegram production (needed?).

# Typical Run Timeline

Need to recover the original ideas concerning DAQ sequencing (RC,etc.). What we shoud do is document the time history of all actions we are expecting during a run.

Yes – should I get this ready – I assume yes and will send the result later (hopefully today).

# Timing Receiver Questions and Requirements

Means of running standalone, using one of the following

- local oscillator with local telegram generation

- additional front panel osc input with local telegram generation

- additional machine emulator board to drive the TR (do we need to reserve a slot?)

Need to finalise encoding of telegram and transmission in crate

- presume we must use the 108MHz clock (would seem so)

- Manchester has a huge bandwidth hit.

- Is there room for a simple 2 line solution: clock + data  (see section 4.3)

Finalise “global” backplane signals

- we would like to conform to any “standard” layout where possible

External Inputs (LVDS/LVTTL) (Front-panel or RTM?) – min required 4?

- Clock

- Trigger

- Laser (clock typically ½BC or BC rate)

- Spare

Required telegrams

- Start Train (>15ms before train)

- Train Number (incrementing)

- End Train (?ms after/before?)

- Bunch Pattern Index

- DAQ ready (generates interrupt to CPU to mark start of DAQ cycle)

Possibly required telegrams

- Bunch Pattern Content (actually distributed by Control – not on CC Command line)

Interrupts

- The crate CPU control software will need an interrupt to signify train coming so that correct configuration of the DAQ and its subsystems can be verified. This has to be generated as soon as possible after the end train. It cannot be the end train as this would mean the first train would be lost.

BunchClock

- Must be continuous (and no phase changes)

Does the TR have an RTM?

- Is the pinout in any way similar to the DAMC2 (e.g. is the clock on the same pin)?

TR boards required on startup:

At UCL = 1 (2 depending on test mode)

At WP76 = 1 (2 depending on test mode)

DAMC2 boards required on startup:

At UCL = 1 (2 ?)

At WP76 = 1

# Control software

We need additional slide “control software” explaining the requirements on the CPU’s control software. Here are some starting points:

- Interaction with master

  - Read CSRs

  - Read counters

  - Write CSRs

  - Clear counters

  - …

- Interaction with slaves:

  - Same as master?

  - …

- Interaction with TR

  - Read out stored telegram data – how to synch. To this as changes

  - Stop and start TR operation – what is possible

  - Writing control options – enabling interrupts, etc.

  - …

# Veto Sub-system

Thoughts about putting into a separate crate:

  - Would require a VETO input line into the master

  - Requires VETO separate crate to have an output (obvious)

  - Vetoing is anyway asynch to BC in CC

  - Laser input we were discussing is not a veto it’s more a flag/marker

A veto manager (once called veto unit (VU)) running in a separate crate is my preference. Maybe we should state that the VU does not have to sit in the CC crate.

# CC Hardware requirements

CC hardware requirements

- xTCA Crate (~5 slot job)

- >= 1 TR board + working firmware

- >= 1 DAMC2 + working firmware

# Firmware required

A list of FPGA firmware (modules) required per board type:

| Module | Master | Slave | TR (for the requirements) |
| --- | --- | --- | --- |
| Telegram decoder | yes | yes | N/A |
| PCIe | Yes | Yes | yes |
| memory access | yes | yes | yes |
| Non XFEL running |  |  | Clock translation, start/trig generation, telegram generation |

# Timelines

End of June:

- Discussions with RAL re RTM project

- Preliminary Design

July 9th:

- Distribute to FEE+TB+TR+FEA groups

July 23rd:

- assimilate feedback = FINAL design

July 30th:

- Firmware development with eval board of telegram encoder/decoder

September

- RAL starts RTM design

- TR available (2 for WP76, later 1 to UCL)

- TB/CC meeting (9th Sept)

  - CC design review (freeze)

  - Prioritised list of telegrams for TR group.

- WP76 gets Crate+ADC+TR

  - Control software (i.e. configure TR) + ADC firmware work

  - Firmware development overlap with CC (invitation to UCL)

January

- DAMC2

# References