30/07/10

Clock and Control Board for 2D pixels – Statement

- The CC will use DAMC2 and a custom RTM as the basis for CC Master and CC Slave cards.

- The prototype CC master/slave will be one DAMC2 + RTM card capable of driving a 1 Mpixel 2D detector.

- The CC will receive the bunch clock (~4.5 MHz) on TCLKA and the ~99 MHz clock on TCLKB from the TR.

- The jitter for TCLKA/B will be less than or equal to 100 psecs.

- The CC will use an on-board oscillator and PLL (on RTM) for stand-alone tests providing both the bunch clock and the ~99 MHz clock.

- The CC will utilise the following bussed LVDS lines on the xTCA backplane:

  - From the TR:

    - RX17 : Start/Trigger

    - TX17 : Telegram Data

    - RX18 : Telegram Clock/Strobe

    - TX18 : Reset

  - From the CC:

    - RX19 : Spare

    - TX19 : CC Command

    - RX20 : CC Veto

    - TX20 : CC Status

- The CC will use the TR to synchronise to the following external signals when used with non-XFEL sources:

  - External Clock

  - External Trigger

  - Laser Clock

  - Spare ( If available)

- The CC will use the following telegram data from the TR:

  - Start Train

  - Train Number (Incrementing)

  - End Train

  - Bunch Pattern Index

  - DAQ Ready

- The telegrams are to be sent as a data and strobe pair from TR.

- The CC will request a modification on the next version of the DAMC2 to allow bi-directional TCLKA/B.

Work Schedule

- Suggesting and refinement of a suitable protocol for telegram transmission.

- Purchase of the XUPV5-LX110T development board.

- Hardware and firmware design/development to use the XUPV5 board as a pre-prototype CC master/slave

  - Design of daughter boards for clock generation and I/O.

  - Firmware development for fast message generation + telegram decoding.

- Design/schematic entry  for the CC RTM prototype

Outstanding Issues

- Telegram transmission protocol development should be in collaboration with TR team.

  - Must satisfy the other customers’ requirements.

- The exact type and the number of external inputs to TR have to be determined. We require the TR to deliver standard signals to CC when used with non-XFEL sources.

- TCLKA/B jitter specification ( ≤ 100 psec)

- Next version of DAMC2 – changes to TCLKA/B according to CC requirements to be discussed with M. Zimmer/ FEA group

- RTM design considerations (To be discussed with Rehlich/ Zimmer/ Vetrov)

  - Management circuitry

  - Power supply circuitry

  - Size and the connector specs?

  - Any other consideration involving xTCA RTM specs?