Clock and control fast signal specification

M.Postranecky, M.Warren and D.Wilson 16.Dec.2009

1	Introduction	1

2	Overview of 2D pixel detector DAQ and control	1

3	Timing interfaces	1

3.1	XFEL timing interface	2

3.2	Non XFEL timing interface	3

3.3	Standalone timing interface	3

4	FEE fast signal interface	3

4.1	Line 1: clock	4

4.2	Line 2: control commands	4

4.3	Line 3: VETO command	4

4.4	Line 4: FEE status	5

# Introduction

This document is aimed at updating the definition of fast signals used by the Clock and Control system of XFEL 2D pixel detectors.

# Overview of 2D pixel detector DAQ and control

Three 2D pixel detector collaborations (AGIPD, DSSC and LPD) are building detectors for use at XFEL. All detectors will use common backend DAQ and control systems and consist of three sub-systems:

- Front End Electronics (FEE) of the detectors. The FEEs are detector specific custom implementations.

- Train Builder (TB) data readout. The TB is used to build data into frame ordered bunch train specific contiguous blocks which are then sent on to a computing farm layer for processing.

- Clock and Control (C&C). The C&C interfaces to the XFEL timing system, generates and receives fast signals used to synchronize detector FEEs, and distributes configuration information using the fast signals or network messages.

# Timing interfaces

This section describes C&C timing interfaces. Timing interfaces (clocks and triggers) are required to allow detector operations at XFEL and other, non XFEL, light sources like LCLS. Additionally standalone running using internally generated timing signals is required.

A schematic showing the proposed fast signal connectivity is show in Figure 1

RJ45 connectors are used for both the inputs and outputs.  Socket A is the default input from the XFEL Timing Receiver (TR) board. Socket B is used for the external non XFEL timing sources like LCLS. The pin assignments of A and B are defined so that the sockets can be combined into a single socket if there are physical space limitations. Timing input cable lengths are expected to be short, e.g. the XFEL TR resides in the same crate as the C&C.

Figure 1 Timing and FEE connection schematic

The different timing interfaces required are described below.

# XFEL timing interface

The XFEL TR board will provide front panel and backplane timing signal outputs.

Four 5-pair har-link (http://www.harting-connectivity-networks.de/en/produkte/produktauswahl/interface-steckverbinder/metrische-interface-steckverbinder-im-20-mm-kontaktraster-har-link) front panel connectors provide access to the encoded trigger message data stream, the 4.51 MHz bunch clock and to programmable outputs. The later can provide trigger pulse (train START, STOP…), delayed trigger pulses (START + 15ms…), gates, bursts and patterns.  The backplane PCIe will allow interrupt driven C&C crate CPU synchronization to the TR. Note that no technical or user documentation exists for the TR board which is currently being prototyped.

e definesdTR signals required by the C&C.

| Name | Signal type | Socket / Pin | Signal | Purpose |
| --- | --- | --- | --- | --- |
| Bunch clock | Differential LVDS | A / 1+2 | 4.51 MHz clock | 4.51 MHz clock – used to generate the synch clock (see Line 1). |
| Encoded trigger message | Differential LVDS | A / 4+5 | XXX MHz clock | Used to generate control (START, STOP…) messages (see Line 2). |

Table 1 XFEL TR timing signals required

The encoded trigger message stream contains START train, STOP train, and other trigger messages.

The C&C requirements for trigger messages should be made known to the XFEL timing group, as should input signal characteristics (LVDS, CML…).

# Non XFEL timing interface

Table 2 defines non XFEL timing signals required by the C&C.

| Name | Signal type | Socket / Pin | Signal | Purpose |
| --- | --- | --- | --- | --- |
| Bunch clock | Differential LVDS | B / 7+8 | 750 kHz to 6 MHz clock | 4.51 MHz clock – used to generate the synch clock (see Line 1). |
| Trigger | Differential LVDS | B / 3+6 | Pulse only? | Used to generate START control message (see Line 2). |

Table 2 Non XFEL timing signals required (not finalized)

The information in this section can only be finalized when timing interface information is know from LCLS, Spring8, FLASH, etc. Open issues:

- Where do the signals come from?

- Must additional in crate hardware be foreseen to access timing information

- Are unique bunch numbers used and, if so, how can they be accessed.

In view of these uncertainties the following variations are possible:

- Bunch clock: LVTTL or NIM, single ended, LEMO 00 permissible range 750 kHz - 6 MHz. Slower clocks below 750 kHz acceptable but will not meet jitter etc. specs. Faster clocks above 6 MHz acceptable but will not permit synchronous veto.

- Trigger input: LVTTL or NIM, single ended, LEMO 00

# Standalone timing interface

### An internally generated 4.51 MHz clock is provided for standalone running. XXX. What signal is used to trigger the system and what are its timing and electrical characteristics?

# FEE fast signal interface

This section describes the C&C FEE fast signal interface. Fast signals are clocks and pulses transferred over cables (≤ 5 m).

The FEE interface is used to distribute synchronization and bunch train configuration information train ID, bunch pattern ID…) to the FEEs. Currently only one quantity is sent from the FEE to the C&C.

### A schematic showing the proposed connection functionality is show in Figure 1 and fast signal definitions in

| Name | Receiver | Signal type | Socket / Pin | Signal | Purpose |
| --- | --- | --- | --- | --- | --- |
| Line 1 | FEE | Differential LVDS | C / 1+2 | clock | Synch Clock (99/100 MHz) |
| Line 2 | FEE | Differential LVDS | C / 4+5 | 100 MHz clock encoded message | Control command (START train, STOP train…) |
| Line 3 | FEE | Differential LVDS | C / 7+8 | 100 MHz clock encoded bunch number OR (configurable) 4.51 MHz clock encoded yes/no | Veto command |
| Line 4 | C&C | Differential LVDS | C / 3+6 | C / 3+6 | FEE status |

Table 3 C&C FEE (FPGA-FPGA) fast signal definitions

# Line 1: clock

Line 1 distributes a ~99 MHz synchronization clock derived from the glitch less input bunch clock.

# Line 2: control commands

Line 2 distributes control commands to the FEEs by encoding information on a 100 MHz carrier clock.

| Command | Start bits | Payload | Purpose |
| --- | --- | --- | --- |
| START | 1100 | Train ID (32 bit) + Bunch pattern ID (8 bit) + ex-OR checksum (8 bit) | Notifies FEE of coming train (in N clocks = 15ms) and provides train relevant information |
| STOP | 1010 | none | Notifies FEE that train ended |
| RESET | 1001 | none | Reset FEE micro-controller, etc. via FPGA |
| reserved | 1111 |  |  |

Table 4 Line 2 control commands

# Line 3: VETO command

### Line 3 distributes a VETO command to the FEEs. Two protocols are defined for distributing the command: a 100 MHz clock with the bunch number to be vetoed encoded on it, or a 4.51 MHz clock with yes/no (high/low) encoding. The later provides backwards compatibility with the original veto signal definition. Which protocol is distributed is defined during run configuration.

| Command | Start bits | Payload | Purpose |
| --- | --- | --- | --- |
| VETO | 110 | Bunch ID (12 bit) + Reserved (4 bins) | Identifies the bunch ID to veto |
| NO-VETO | 101 | “111111111111111” (12 bits all “1”) + Reserved (4 bits) | No bunch veto set |
| reserved | 111 |  |  |

Table 5 Bunch ID VETO command definition

# Line 4: FEE status

Line 4 allows the C&C to see that the fast signal cable C is correctly plugged in and that FEEs are powered up.

The signal send is currently defined to be a continuous clock derived from the ~100 MHz clock on Line 1. It was agreed to keep open the option of encoding information onto Line 4.