Current  Status  of  ZEUS-MVD  “C+C  MASTER”  and  “C+_C  SLAVE”  modules

DESY,   5 April  2000

(DAH, JBL, JMB, MP - UCL )

1)	C+C  MASTER :				- Both modules fully tested stand-alone and with

the GFLT Simulator at UCL

2)	C+C  SLAVE :				- Two modules fully tested stand-alone and as a

system with C+C MASTER and the GFLT        Simulator at UCL

3)	C+C MASTER with two C+C SLAVEs :	-  System fully tested in one VME crate – both

stand alone and with the GFLT Simulator at           UCL

-  Soak testing done with the GFLT Simulator

using PC-driven software test programme to  check the  correct handling of Triggers, FLTNs, GBCNs and Busys shows no errors after runs lasting many millions of ‘events’

One C+C MASTER and one C+C SLAVE are now in DESY being used for tests in the ADC crate. These are the actual production modules, not prototypes .

4)	GFLT -> VETO WALL CABLE :		-  Testing with ~ 75 m of new, shielded, zero-

halogen “Twist’n’Flat” ribbon cable between   the GFLT simulator and the C+C MASTER shows that there is no actual loss of the phase lock between the GFLT  96nsec clock and the PLL clock regenerated on the C+C  MASTER .

- This cable should be good enough for the ~52 m

required to run between the GFLT in the

rucksack and the veto wall area electronics  racks

4)	TO BE DONE :				- Testing with  real GFLT at DESY with real

halogen-free cables

( This requires new cables to be purchased :

quoted price £10.000 )