Current  Status  of  ZEUS-MVD  “C+C  MASTER”  and  “C+_C  SLAVE”  modules

DESY,   15  February  2000

( GN + MP-UCL )

1)	C+C  MASTER :				- Both modules fully tested stand-alone and with

the GFLT Simulator at UCL

2)	C+C  SLAVE :				- Two modules fully tested stand-alone and as a

system with C+C MASTER and the GFLT        Simulator at UCL

3)	C+C MASTER with one C+C SLAVE :	- System fully tested in one VME crate – both

stand alone and with the GFLT Simulator at       UCL

-  Soak testing with the GFLT Simulator using

PC-driven software test programme to check the  correct handling of Triggers, FLTNs, GBCNs and Busys shows no errors after runs lasting many millions of ‘events’

4)	TO BE DONE :				- Testing with  real GFLT at DESY

( this requires new cables to be manufactured )

5)	GFLT -> VETO WALL CABLE :		- Preliminary testing with ~ 52 m of twisted pair

cable between the GFLT simulator and the C+C

MASTER indicates an occasional loss of PLL

lock ( of the GFLT  96nsec clock ) on the C+C

MASTER .

- Tests are being repeated with a better quality

and shielded twisted pair cable this week

One C+C MASTER and two C+C SLAVEs are ready to be sent to DESY next week.

!!!   PLEASE PLACE YOUR BIDS NOW – BRIBES, ETC. MOST WELCOMED  !!!