Pete Shield

8-2-00

COMBO CABLE PPB TERMINATOR

Pin Allocation  #2

|  | A |  | B |  |  | Crate Slot No |
| --- | --- | --- | --- | --- | --- | --- |
| A_Gnd | —o | 1 | o— | A_Gnd |  | 1,7,13 |
| D_Out | —o | 2 | o— | A_Out | Analogue |  |
| A_Gnd | —o | 3 | o— | A_Gnd |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
| Bias 1 | —o | 9 | o— | B1_Gnd |  | 2,8,14 |
| TS_2 | —o | 10 | o— | TS_1 | High Voltage |  |
| Bias 0 | —o | 11 | o— | B0_Gnd |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
| Trg – | —o | 17 | o— | Trg + |  | 3,9,15 |
| C_Gnd | —o | 18 | o— | T_Gnd | Clock |  |
| Clk + | —o | 19 | o— | Clk – |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
| TP | —o | 25 | o— | TP_Gnd |  | 4,10,16 |
| R_Gnd | —o | 26 | o— | RST | Control |  |
| LD | —o | 27 | o— | L_Gnd |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
|  | o |  | o |  |  |  |
| VDD_s | —o | 33 | o— | VDD |  | 5,11,17 |
| VSS_s | —o | 34 | o— | VSS | Low Voltage |  |
| GND_s | —o | 35 | o— | GND |  |  |
| N/U |  |  |  |  | Double Width | 6,12,18 |
| N/U |  |  |  |  | Low Voltage |  |
| N/U |  |  |  |  | Space |  |

Note: -

- This connector plugs horizontally onto 2 x DIN 41612 connectors at the rear of a 6U crate.

- Slots 6,12,18 do not have DIN 41612 connectors but space is required for a double width front panel.

COMBO CABLE TERMINATOR

Proposed Layout

| 35 | 30 | 25 | 20 | 15 | 10 | 5 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

A

B

10

30

15

20

90mm

42mm

Note:

- The connector type ….Samtec SSM-135-S-DH  (Surface Mount)

- These dimensions are approximate and guide lines for the board designer

- The dimensions of the solder pads will be as specified by Padua / Axon

- Hole size for 'P' clips = 2.5mm

Board Build

| Layer 1 | Clk GND | Trig GND | Anal GND | Bias GND's | Power tracks |
| --- | --- | --- | --- | --- | --- |
| Layer 2 | Clk Sig (60Ω) | Trig Sig (60Ω) | Anal Sig (60Ω) | Bias Volts (HV) |  |
| Layer 3 | Clk GND | Trig GND | Anal GND |  |  |
| Layer 4 | Load GND | Reset GND | Test Pulse GND |  |  |
| Layer 5 | Load Sig (60Ω) | Reset Sig (60Ω) | Test Pulse (60Ω) | Bias Volts (HV) |  |
| Layer 6 | Load GND | Reset GND | Test Pulse GND | Bias GND's | Power tracks |

Other Features

- Green solder resist coating top and bottom

- White legend top of board (inc HV warning)

- 1.6mm board thickness

- All signals screened both sides with individual GNDs

- Power tracks 1mm x 35um