## ZEUS PROGRESS REPORT

## ZEUS UK COLLABORATION

July  2002

Contents

- Introduction

- CTD

- MVD

- TPOL

- Physics

- Future Plans and Resources Required
a.  Any further additional upgrade requests planned,

and additional requests for new capital and RAL

non-PPD effort 
     b.   UK commitments to maintenance and operation

of  the detector 
     c.    UK analysis plans not covered above ;
     d.     Estimates for M+O and Travel for the next

years.

A  ZEUS Publications

B  Conference Talks by ZEUS UK members

C  UK PhD Theses

D  The ZEUS UK Collaboration

E   Staff Allocations

F   Computing Requirements

G   ZEUS Organisation

# 2 Central tracking detector status and infrastructure

The Central Tracking Detector (CTD) is a large-volume, cylindrical drift chamber covering the central region 15o <  < 164o. It provides charged-particle tracking, momentum measurement and identification through dE/dx. It also participates in all three levels of the ZEUS trigger.

The CTD's 4608 sense wires are arranged in nine superlayers, superlayer one being the innermost. The wires in odd-numbered superlayers run parallel to the chamber axis, whilst those in even-numbered layers have a small stereo angle (~ 5o). Taken together they provide sufficient information to reconstruct tracks in three dimensions. All sense wires in the chamber are equipped with Flash Analogue-to-Digital Conversion (FADC) electronics and, in addition, all wires in superlayer one and the odd-numbered wires in superlayers three and five have z-by-timing electronics for the first-level trigger.

During the shutdown for the HERA upgrade the CTD was flushed with argon:CO2.

Despite not being operated for ten months, it successfully participated in cosmic tests for the ZEUS microvertex detector in July 2001.

2.1 Triggering

ZEUS operates a three-level trigger, of which the first is a hardware-based system .

The first level trigger uses the CTD information for reducing backgrounds that do not come from the interaction point. The detector uses dedicated, pipelined processor cards which provide tracking information to the global first-level trigger.

The ZEUS second-level trigger uses more precise software algorithms to clean the event sample further and classify certain types of physics events. Detail tracking information is provided by the CTD second-level trigger which makes use of the data from all five axial superlayers and the z-by-timing information from layers 1, 3 and 5. A precise measurement of the event vertex at the second level is a powerful technique for rejection of background. Before the shutdown for the HERA upgrade, modifications were made to the CTD Transputer network in order to allow data from the FADC electronics to be sent to the Global Tracking Trigger (GTT). This system will combine CTD data with the precise information from the new microvertex detector to provide a very accurate event vertex measurement. The link to the GTT computer systems has now been successfully tested.

The third-level trigger (TLT) runs on a farm of Unix workstations and PCs using an appropriate version of the off-line reconstruction code. The CTD TLT code is an optimised version of the full off-line reconstruction. Its performance is only slightly below that of the off-line version and aids monitoring of the system by providing early warning of any changes in critical CTD parameters.

2.2 Infrastructure and Monitoring

In general the CTD support infrastructure has performed very well. The CTD water

cooling circuit is now fully operational again. A blocked pipe to the forward end plate was cleared by ``sucking'' the blockage out of the system. The water flow rates through both arms of the cooling system are now well above the minimum required to cool the CTD and show no signs of further fall off. Fine filters are also now installed between the main ZEUS cooling circuit and the input to the CTD water cooling system which it is hoped will prevent further problems.

The gas system operated reliably throughout the long HERA upgrade shutdown. After

the cosmic tests some fluctuations in the gas composition were noticed. The problem was traced to a faulty mass flow controller, which has now been replaced with a new unit. The flow meters on the gas system have also been replaced as part of its ongoing maintenance.

New uninterruptible power supplies (UPS) have also been installed in the CTD

high-voltage (HV) system. These new UPSs will provide increased protection from

power failures and be more reliable than the previous units.

Since water was added to the gas mixture to overcome the Malter effect, as described in last year's report, the chamber has shown no further sign of HV instability. The addition of water caused some shift in the operating parameters of the CTD, including the reduction of the Lorentz angle to 41 o from the nominal value of 45 o. After some investigation it proved possible to restore this partially by reducing the CO2 composition of the gas by 2% and increasing the ethane by 2% whilst keeping the argon constant, coupled with a small increase in the HV. The chamber is now operating efficiently once more and will continue to run with water in the gas mixture for the lifetime of ZEUS.

Over the course of the shutdown the ZEUS data-acquisition environment has been

upgraded to operate with the larger data-rates expected in the post-upgrade environment. In particular, the ZEUS event builder (EVB), a system that assembles the complete data for an event from all the sub-components of the detector, has been replaced with a new system. The CTD was the first pre-upgrade component to move to the new EVB and is operating successfully with it.

ZEUS has implemented a system for automatic online data monitoring. For the CTD, this is done by checking a small number of critical histograms which are also made available to the shift crew. The automatic histogram checking was initially incorporated into the CTD online program, but will be moved to a dedicated machine in future for ease of maintenance and development. More detailed monitoring is performed off-line using samples of the data which are automatically analysed and then checked by a small team of experts. This system has proved to be effective over a number of years in the early diagnosis of problems. It also provides calibration constants for the CTD and quality control of the CTD data used for physics analysis.

The continued operation and reliability of the CTD is of utmost importance to ZEUS.

The chamber is now ready for data-taking in the new HERA environment and will

remain at the heart of ZEUS physics for the lifetime of the experiment.

3. ZEUS Microvertex Detector

ZEUS UK groups have been involved in the ZEUS Microvertex Detector (MVD) project with responsibility on the hardware side for: installation; clock and control electronics; the laser alignment system and a cable `patchbox'. On the software side we wrote the local run control, event display and histogramming package, and are involved in the development of a second-level global track trigger. We are also involved in offline precision alignment using track data.

3.1 Clock and Control

The ZEUS-UK group provided the clock and control system for the MVD. This system consists of three main subsystems: a master controller and slave in the ADC crates; sixteen driver boards in a crate of their own; and twenty-four distribution cards in the patch boxes close to the detector, together with associated power supplies.

The master and slave system consists principally of digital logic on programmable devices. It (a) receives clock and trigger signals from the ZEUS global first level trigger (GFLT). (b) sends clock, trigger and reset signal to the HELIX driver system (c) receives busy and error signals from the ADCs and sends them to the GFLT. We provided a ZEUS trigger simulator in the same crate as the master board, which allows standalone running. The master board also allows an external trigger to be input to the front panel, which was used during cosmic ray data taking on the test stand.

The HELIX driver system (16 6U VME cards) fans out signals from the master and sends them as Low Voltage Differential Signals (LVDS) down approximately 20m of cable to the patch box cards. The system also interfaces with the MVD slow control to allow calibration data to be sent to the detector chips at the start of a run, and provides

monitoring information.

The patchbox cards consist of three cards for the control signals and three for the clock signals in each of the four patch boxes. These cards amplify the signals from the HELIX drivers and convert the control signal to single eneded CMOS signals as required by the HELIX chips. These cards formed part of what turned out to be the very challenging overall patchbox system (see below) and were designed and installed by us.

The whole system was delivered on schedule, in time for cosmic ray testing in January 2001 in the test area, in which we participated fully, and is now fully installed and functional in the ZEUS hall.

3.2 The Patchbox

The patchbox was not in the original proposal to the PPESP and did not sound too onerous (`just help with the design of a cable patch panel') when taken on. It turned out to be a major challenge and a key component on the critical path of the MVD project. Because of space limitations from existing components in ZEUS and the new magnets for

the final focus, the nearest location for the main readout electronics was nearly 30m from the detector.  Clearly a cable connection station was needed. Apart from very tight spatial limitations the design for what became known as the `patch box' had to cope with:

- rearranging the cables from wafer-grouped on the MVD side to function-grouped (readout, clock & control, bias voltage, etc) on the readout side;

- rigorous screening of all MVD cables (high voltage, low voltage, clock and control, analogue readout) to avoid pickup in the hostile electronic environment near the beam line;

- active components to boost the clock and control signals (see previous section).

A very ingenious design of doubly insulated crates with a specially designed back-plane was devised. To fit into the only space available around the rear beam-pipe the patch box was actually four separate boxes. The boxes were ready for the MVD `full system test' at DESY  (December 2000 -- March 2001) and then had to be uncabled, moved to the ZEUS Hall, installed and recabled, with full testing of all MVD cables at  each stage.

3.3 Alignment

We provided the laser alignment system [1] for the new microvertex detector. The system consists of five laser beams distributed around the perimeter of the MVD support tube. There are five or six sensor positions along the support tube and one at either end attached to the CTD, from which the MVD is mounted. During regular operations the main purpose of the laser system will be to monitor gross movements of the MVD

support structure with respect to the central tracking chamber, in order to define stable periods for offline precision alignment using track data. However, the system proved its worth before and during the assembly of the MVD through 2000/01. Tests showed that the first design of the construction support frame was not rigid enough and needed  strengthening. During assembly of the two halves of the MVD and their transport to DESY, periodic measurements were taken with laser system to monitor any distortions in the MVD cylindrical support tubes. Measurements were also taken before and after the delicate operation to put the two halves together at DESY prior to a full test of the MVD using cosmic  rays and finally after installation of the MVD in ZEUS. For the measurements up to and including the installation of the MVD, a prototype readout system with a limited number of channels was used. The final VME readout boards were completed in late 2001 and installed together with the online control system in January 2002. The system has been integrated into the MVD slow-control system and procedures are being developed for routine measurement and analysis of optical alignment data. Figure 1 shows the raw signals from all sensor positions along one of the five laser beam line.

Figure 1 MVD optical alignment system. Signals from sensor positions along one

laser beam. The positions labelled RCTD and FCTD are on flanges at the rear and

front (wrt the proton beam) ends of the CTD respectively, the others are distributed

along the MVD support tube. The curves are from simple gaussian fits to the signal

profiles.

Work has started on developing the tools needed for precision alignment using track data. A pattern recognition and fit program using CTD and MVD barrel data for through-going cosmic ray tracks, developed by our NIKHEF colleagues, is being used to study residuals

at the level of individual wafers. This will then be extended to use the new general tracking software on selected classes of track data.

3.4 Installation

Installation was highly non-trivial because of space limitations and the delicate nature of the MVD. The procedures were worked out through extensive discussions with NIKHEF and DESY engineers and all the key stages were tested using various full-scale mock-ups. Apart from the challenge of getting the MVD into the central tube of the CTD with

only mm clearance, the major problem was handling the 4m long readout and service cables attached to the MVD. These had to be fed in bundles through the CTD first. On the day everything worked perfectly and the MVD was installed by 6pm on 27/3/01.  The final part of the installation was arranging the cables between the MVD and patch boxes

in the very limited space around the new rear combined function magnet. This turned out to be a classic problem of fitting a round object (cables around the cylindrical magnet) into a square hole (aperture left when ZEUS rear calorimeter is closed).  Although the

cable routes and clearances had been checked very carefully beforehand, the first attempt failed and the calorimeter could not be closed completely. It turned out that certain drawings had not been completely accurate. The UK team had a weekend in which to try again and by using a simple wooden former and a lot of hard work managed to persuade the cables to take a more rectangular profile thus allowing the calorimeter to close with a few mm spare.

3.5 Global Track Trigger

The Global Track Trigger (GTT) is a new trigger component for ZEUS, which will eventually replace the CTD SLT function.  It will combine the information from the (old) transputer based readout system used for both the CTD and  STT with that from the (new) VME based readout system of the MVD. ZEUS-UK are providing algorithm development and implementation. The GTT hardware consists of Gigabit ethernet communications and a farm of 12 1GHz Linux PCs for processing. This augments the original CTD second level transputer based system and the farm hardware has been funded through the Yale ZEUS group. The GTT system for integrating CTD and MVD information is complete and has been tested within the ZEUS DAQ system.  The STT readout has been added and data sizes are somewhat smaller than those from the CTD.

The algorithm for combining CTD and MVD information in the barrel region is being tested with the first real data. Work will begin shortly on integrating the STT and forward MVD information into the existing algorithm.  The performance of the algorithm has been studied extensively, using Monte Carlo simulated events, trigger pass-through events taken during data taking last year, and cosmic muons taken during a short run in

July this year.

In order to study the GTT latency before the availability of real ep interaction data, a playback system was created. Monte Carlo events stored in memory are injected into the GTT trigger chain at the component VME interfaces, and sent through the GTT  system exactly as for normal data. Using simulated high-ET dijet photoproduction events containing both MVD and CTD data, at an input rate of  400 Hz, the deadtime was found to be 1.4%, consistent with the performance of the existing ZEUS DAQ and trigger systems during  data taking at the end of 2000.

The resolution performance of the algorithm with simulated MVD data has been studied for track and event vertex parameter reconstruction. The event vertex resolutions for the reference Monte Carlo sample are summarized in Table 1. The resolution of the GTT using  CTD-only information is significantly better than the existing CTD SLT  due to the use of the stereo information rather than the z-by-timing data. The improvement in the z-resolution of the primary interaction vertex will help to reduce background considerably and is also a necessary first step for more sophisticated trigger calculations, such as event track impact parameter distributions and/or secondary vertex algorithms.

|  | z(vertex) in mm |
| --- | --- |
| Current CTD-SLT | 84.4 |
| GTT (CTD only) | 11.3 |
| Current Offline (No MVD) | 1.14 |
| GTT | 0.046 |

Table 1:  Event Z-vertex resolution for GTT, existing CTD-SLT and

existing offline reconstruction.

At present the GTT (MVD+CTD) is operating in passive mode while the optimization of the HERA beam optics and luminosity continues. A nice summary of the status of the MVD project as a whole is given by Figure 2, which shows an event from a luminosity run in May 2002 with tracks and hits in both the CTD and MVD.

Figure 2: An event from a luminosity run in May 2002, showing tracks fitted to hits in both the CTD and MVD barrel region.

3.6 Online Software

The MVD run control, slow control and monitoring is carried out via a package based on the CERN Root software and known as 'Green'. This package includes histogramming and event display services  and was written by ZEUS-UK. It has proved extremely flexible, and was used throughout the testing of the MVD. It is now fully integrated with the ZEUS global run control and is being used during data taking.

4 The HERA Polarimeter Upgrade

4.1 Overview

In the past, extensive studies on charged current  (CC) and neutral current (NC) deep  inelastic  scattering (DIS) have been undertaken using neutrino and anti-neutrino beams.

At the  HERA lepton-proton collider experiments H1 and ZEUS, cross section measurements for CC and NC DIS have been carried out, using charged lepton beams  (e+,e-) ~\cite{highq2}. However up to now, there are no measurements for CC DIS with polarized charged lepton beams, and for NC DIS  the data is limited to  low Q2. The HERA ring at DESY has been recently upgraded during the long 2000-2001 shutdown to increase the HERA ring luminosity by a factor of 5 and provide longitudinal lepton (e+,e-) beam polarization to the HERA experiments;

the HERA II phase has begun. The study of polarized lepton-proton deep inelastic  scattering at high Q2, which has never been carried out before, is now possible at the upgraded HERA accelerator HERA II. Electroweak interactions can be  probed by

using the four possible lepton  charge/polarization combinations: e+,e-, P>0, P<0. The cross  section measurement of charged current (CC) and neutral current (NC) deep inelastic scattering at high Q2  using polarized lepton beams  is one of the  primary physics objectives of  the HERAII collider experiments.

The chiral structure of the Standard Model will be tested through a variety of

measurements,  making use of the polarized CC and NC cross sections ~\cite{futhera}.

Two important measurements   which can be accomplished at HERA, by measuring  the polarized NC and CC cross sections, are highlighted below as an example.

  Set Limit for Right-Handed Charged Currents

Measure the polarization dependence of the cross section for the charged current (CC)

process CC(e-p   X) and obtain a limit for the right-handed charged current.  The

charged current (CC) process  CC(e-p   X) occurs through the exchange of W

bosons. According to the Standard Model only pure left-handed charged currents exist,

i.e. the cross section vanishes for purely right-handed electrons.

The total electron charged current cross section CC(e-p   X) is:

CC(e-p   X)=(1-P) CCL +(1+P) CCR

where P is the degree electron longitudinal polarization, with P= -1 corresponding to

negative helicity i.e. a fully left-handed electron beam,  and CCL  , CCR , are the left

and right handed electron  charged current cross sections. With about one year of data

a lower limit on the mass of the hypothetical  boson  MW;R  can be set at the level of

MW;R  > 400 GeV  ~\cite{beampol}.

  Measurement of the Neutral Current Light Quark (u,d) Couplings

The measurement and comparison of neutral current and charged current cross

sections with all four charge/polarization combinations, allows to extract the neutral

current couplings of the light quarks(u,d) to the Z-boson. With a total integrated

luminosity of about ~ 1000 pb-1 divided between the four charge/polarization

combinations e+,e-, P ~ ± 70%, all four u and d type vector and axial-vector

couplings can be measured with a fractional errors on au, vu, ad, vd, of 6%, 13%, 17\%

and  17%  respectively, which is comparable to the accuracy obtained at LEP for the

heavy quark(b,c) couplings ~\cite{weakneutral}.

These above described measurements require a high degree of lepton polarization and good polarization precision of about 1%.  For this purpose a polarimetry upgrade

project has been setup in 1999 called POL2000 to improve the polarization

measurement precision from about 4% to 1%, and so upgrade the HERA I polarimetry for the HERAII phase.

At HERA leptons become naturally transversely polarized through the  emission of synchrotron radiation (lepton spin flips), the so called Sokolov-Ternov effect. The lepton beam transverse polarization is converted into longitudinal polarization near the interaction points by spin rotators, to obtain polarized  physics. The HERA ring polarization is measured by two polarimeters: TPOL -the transverse polarimeter which measures transverse polarization, located in the HERA-West area nearby HERA-B; and the LPOL-the longitudinal polarimeter which measures longitudinal polarization, located

in the HERA-East area nearby HERMES. The degree of polarization is measured by scattering alternatively right and left circularly polarized laser light off the polarized lepton beam, producing back-scattered Compton photons, which are then detected in a

calorimeter. The two polarimeters operate differently. Transverse polarization is determined by measuring  the spatial vertical up-down asymmetry of the back-scattered

photons on the TPOL calorimeter face, while longitudinal polarization is determined by measuring the energy asymmetry in the LPOL calorimeter of the back-scattered photons. Both  polarimeters have been operating since several years.  The LPOL will be upgraded most probably during the next shutdown(January 2003?),with the installation of a Fabry-Perot cavity to greatly increase  the laser power (equivalent to a 10kW laser) and so operate in the ``few photon mode''~\cite{cavity}.

The TPOL upgrade is now completed. The TPOL upgrade consists in a fast

DAQ and a position sensitive silicon detector, which will allow a precise per bunch per minute polarization measurement. The TPOL is ready to provide polarization data whenever HERA will be able to produce stable beams. The ZEUS (IC-London) contribution to the TPOL upgrade is a position sensitive silicon detector;  details are given in the next paragraphs.

4.2 The Silicon Position Sensitive Detector

Before the TPOL upgrade, the  TPOL consisted only of a calorimeter which  measured

both the total energy deposited by the backscattered Compton photons and the vertical asymmetry of the Compton photons on the calorimeter face. The vertical asymmetry was obtained from the-y  transformation, where  = Eup Edown /Eup+Edown, and y was the vertical coordinate of the photon impact point on the calorimeter face. The TPOL calorimeter was composed of two separate modules, an Up and a Down module, which measured  separetly  the deposited energy Eup, Edown. The  position precision measurement obtained using the TPOL calorimeter was ~1 mm.  For the HERAII phase were a good polarization precision is necessary, it is important to measure the -y transformation precisely, because it is the  dominant source of  systematic error for the TPOL. For the TPOL upgrade a position sensitive silicon microstrip detector has been used, to improve the  position precision measurement to better than  50 m and therefore determine accurately the-y transformation in situ, in contrast to before where  -y had to be measured in a test beam. A pre-radiator lead plate of  1 X0 is placed in front of the silicon microstrip detector for photon conversion. The silicon detector and front-end electronics are radiation hard because they have to operate in a region of high photon fluxes. A scintillating fiber detector is used for calibration and monitoring the silicon detector. The general layout schematic for the upgraded TPOL now installed in the HERA tunnel is shown in fig. 3.

Figure 3: Schematic for the TPOL setup in the HERA tunnel, showing the 3 TPOL subdetectors: calorimeter, silicon and fiber detectors. The pre-radiator is needed to convert the Compton photons. The calorimeter is divided into two independent halves. There are two silicon planes to measure the X,Y position of the Compton photon. The scintillating fiber detector is used for calibration and monitoring the silicon detector planes; the fiber is moved vertically by a stepping motor.

4.3 The Prototype Silicon Detector

First a prototype 11 cm2 silicon detector was designed, built and tested by IC-London during the year 2000. The prototype detector was 11 cm2 single sided radiation hard ATLAS detector, 80  m pitch, 100 strips, bias voltage 80V. It was readout by a APV25 amplifier/multiplexer readout chip 1*1 $cm^{2}$, 128 channel analogue pipeline, sampling rate of 40 MHz and data readout at 20 MHz. The silicon detector system was

operated using a VME system. A I2C interface board controlled the APV25 readout

and a SEQSI board generated the necessary  clock and control signals, digitization was done by  a 12 bit SIROCO2 ADC.  A prototype upgraded TPOL was built by combining

the prototype silicon detector   together with the  TPOL calorimeter and scintillating fiber detector. This prototype setup  was  tested successfully in a DESY test beam in February 2001 (energies up to 6 GeV). The results are written up in the ZEUS note ZEUS-01-019 which can be summarized as follows ~\cite{zeusnote}:

 TPOL Calorimeter:  Very good linearity and spatial uniformity of the

calorimeter, energy resolution $E = 26%, spatial resolution

y = 0.4 mm at 6 GeV, y=0.

 Silicon Detector: Strip signal to noise ratio S/N ~ 21.  Relatively good precision

measurement of the -y transformation. Silicon efficiency   ~97.8 %

and spatially uniform. Position resolution <50 m.

 Scintillating Fiber Detector: precision of the linearity calibratio better than 40m.

4.4 The Final Setup Silicon Detector

During 2001 a larger 6.2*6.2 cm2 silicon detector setup  was built  at IC-London , which was successfully tested in the final upgraded TPOL setup in July 2001, in a CERN test beam, with electron/positron beams of energies up to 50 GeV. The silicon detector (also called Y-plane silicon) was a single sided radiation  hard ATLAS detector, 80 m pitch, and 768 strips, readout by 6 APV25s. A VME system  was used to operate the silicon: RIO2 processor, SEQSI, I2C,f or digitization a 8 ADC channel front-end-digitizer FED-PMC, HV control module, and a I/O register.  The results of the CERN test beam  are displayed in the summary plots  in fig.4.

4.5 The Scintillation Fiber Detector

A scintillating fiber detector is used for calibration and monitoring of the silicon detector i.e. to check for radiation damage in the silicon. The fiber is supported horizontally in front of the silicon detector and  can be moved vertically by a stepping motor with a precision of  ~ m. The stepping motor  and mechanical fiber support parts were assembled and tested at IC-London. The scintillation fiber system ( fiber, PM, readout,

and software) is responsibility of the other ZEUS members belonging to the POL2000 group (Tokyo Metropolitan University group). They have successfully tested and operated the scintillation fiber detector, and integrated it as part of the upgraded TPOL detector.

Figure 4:Summary of the DESY and CERN test beam results obtained during 2001. The top left plot (a) displays the linearity of the TPOL calorimeter for positrons; the bottom left plot (b) displays the calorimeter energy resolution obtained using both electron and positron beams;  the top right plot (c) displays the $\eta-y$ correlation for 10 GeV positron beam (dots), while the solid line corresponds to the 1996 CERN test beam data; the bottom right plot (d) displays the y-coordinate reconstructed by the silicon for events with signals in the fiber versus the vertical position of the fiber, the insert shows the residuals to a straight line fit

4.6 Chiller

A water based chiller has been bought and tested by the IC-London group. The purpose of the chiller is to keep the area around the silicon detectors at a low constant temperature (~ 20 C) to reduce the noise in the silicon (have low leakage currents). The typical HERA tunnel temperature can vary between ~ 30-35 C. The TPOL detector including some electronics crates are enclosed in a large TPOL metallic box in which the temperature can be significantly higher than the HERA tunnel temperature, therefore it is important to control and monitor the temperature within and near the TPOL. The chiller will been installed in the next longer access to the HERA tunnel, probably sometime in summer 2002.

4.7 Upgraded TPOL

In November 2001 the upgraded TPOL was installed successfully in the HERA tunnel, see fig.\ref{fig:tunnelpol}. A second silicon plane (called X-plane)  was installed to obtain also the horizontal coordinate x information of the Compton photons. The  X-plane is a single sided  radiation hard  detector with 120 m pitch. It is operated together with the silicon Y-Plane, using the same electronic modules. The TPOL is basically ready to make polarization measurements whenever there are stable polarized lepton beams. Up to now the TPOL has been operated and tested successfully with few unpolarized beams provided by HERA.

- Physics Results

ZEUS-UK physicists are involved in a broad range of physics analyses. Very roughly Bristol, IC, Oxford and RAL are involved in structure functions and high Q2 studies, while Bristol, Glasgow and UCL cover the hadronic final states.

5.1         Physics at high-Q2

5.1.1      High-Q2 neutral current physics

In the Standard Model (SM), neutral current deep inelastic scattering is described by the exchange of a photon or a Z0-boson between the incoming lepton and the proton.

The cross section for NC scattering is given by:

d2NC      22  Y+F2(x,Q2) -y2 FL(x,Q2)  Y-xF3(x,Q2)

dxdQ2      xQ4

=

where Q2 is the negative of the 4-momentum transfer squared between the lepton and the proton, x is the Bjorken scaling variable, y is given by Q2=sxy and $Y=1  (1-y)2.

The parity violating structure function xF3, is expected to become significant at large Q2.

ZEUS UK physicists have continued their leading role in studies of high-Q2 physics at HERA. The determination of the NC cross section from the 16 pb-1 of e-p data collected in 1998 and 1999 has been completed and the analysis of the 64 pb-1 of e+p data collected in 1999 and 2000 is nearing completion. Figure 5 shows d/dx for  Q2 > 10,000 GeV2 for both e+p and e-p scattering along with the SM predictions. The  e-p cross section lies above the e+p cross section and the SM prediction gives a good description of the data. Within the SM the fact that the e-p NC DIS cross section is larger than the e+p cross section at high Q2 is attributed to the fact that for e-p NC DIS the interference between the photon- and the Z0-exchange contribution is constructive while for e+p NC DIS it is destructive.

~ep

The reduced cross section NC    where

~ep

ep

1     xQ4    d2NC

Y+  22   dQdx2

NC  =

is shown at fixed x as a function of Q2in Fig. 6. For Q2 values above 1000 GeV2 the e-p cross section lies above the e+p cross section; consistent with the SM expectation.

Figure 5: Comparison of measured differential cross sections d/dx for e-p and e+p scattering as a function of x  for Q2 > 10, 000 GeV2 .The curves show the Standard Model predictions evaluated using the ZEUS NLO QCD fit.

Fig. 7 shows the structure function xF3extracted by combining thee-p data with the e+p data collected in 1996 and 1997.The precision of the measurement is comparable with the first extraction of F2 from the data taken in 1992.  This measurement will benefit from the increased amount of data from the high luminosity running which has recently started. An improved measurement of  xF3 will impose a tighter constraint on the high-x valence quark distribution which plays an important role in estimating very high energy processes at the LHC. In addition, the measurement of the dependence of the neutral current cross sections on the charge and polarisation of the lepton beam will be sensitive to the electroweak couplings of the light quarks.

5.1.2 High-Q2 charged current physics

In charged current deep inelastic scattering the exchanged $W$-boson picks out specific quark flavours from within the proton. Thus $e^{+}p$ CC DIS is sensitive to the $d$ valence quark density at high $x$ and the $\bar{u}$ (and $\bar{c}$) quark density at low $x$. In contrast, $e^{-}p$ CC DIS is sensitive to the $u$ valence quark density at high $x$ and the $\bar{d}$ (and $\bar{s}$) quark density at low $x$. Hence by measuring the dependence of the CC DIS cross section on the charge of the lepton beam we are able to study flavour-specific parton densities directly.

The analysis of the $e^-p$ data taken during 1998 and 1999 is now complete and the analysis of the $e^+p$ data taken in  1999 and 2000 is nearing completion and a publication is expected in the near future. Figure~\ref{Fig:CC9900Q2} shows the cross section $d\sigma/dQ^2$ for both $e^-p$ and $e^+p$ scattering. The $e^-p$ cross section is higher than that of $e^+p$ at all values of $Q^2$, rising to a factor greater than 10 at the highest $Q^2$measured.

Figure 6: The reduced cross sections for e -p (solid points) and e +p (open squares) scattering as a function of Q2, in bins of fixed x. Also shown are the Standard Model predictions evaluated using the ZEUS NLO QCD fit.

In the quark parton model, this is understood to be due to the $W$ coupling to the higher $u$-quark density in $e^-p$ scattering, while the dominant $e^+p$ process is the $W$ coupling to $d$-quarks with a $(1-y)^2$ suppression. The Standard Model expectation, evaluated with the CTEQ5D parton density functions (PDFs) gives a good description of the data.

Figure 7 The strucure function $xF_3$ (solid points) as a function of $x$ in bins of fixed $Q^2$.Also shown are the Standard Model predictions evaluated using the ZEUS NLO QCD fit.

Figure 8 The CC DIS Born cross section, $d\sigma/dQ^2$, for $e^-p$ data (squares) and $e^+p$ data (filled circles). The dashed and solid lines show the Standard Model expectation evaluated using the CTEQ5D PDFs for $e^-p$ and $e^+p$ scattering, respectively.

The double differential CC cross section may be written as

=       (     )

~ep

d2          G2F       M2W

dxdQ2     2x   M2W+ Q2

2

CC

where the e-p reduced cross section,  $\tilde{\sigma}^{e^{\pm} p}_{\rm CC}$, is given at leading order in QCD by $\tilde{\sigma}=u+c+(1-y)^2(\bar{d}+\bar{s})$

(for e+ p, change quarks to anti-quarks and vice versa). The reduced cross section, for e-p, is plotted as a function of $x$ at several fixed values of $Q^2$ in fig.X. The Standard Model gives a good description of the data. The precision of the measurements of the charged current cross section is  statistically limited and will benefit greatly from the high luminosity running which has recently started. The addition of longitudinally polarised lepton beams will allow the polarisation dependence of charged current DIS to be studied.

~

Figure 9  The reduced cross section, , as a function of x,for different values of Q2. The points represent the data, while the expectation of the Standard Model evaluated using  the ZEUS NLO QCD fit is shown as a solid line. The separate contributions of the PDF combinations x (u+c) and (1-y)2 x(d+s) are shown by the dashed and dotted lines, respectively.

- -

5.2 NLO QCD fits

The UK has played a leading role in the interpretation of the data on the inclusive differential cross-sections by performing NLO QCD fits, using the DGLAP

formalism, to determine the parton density functions (PDFs) and S ~\cite{nloqcd}. In the standard (ZEUS-S) fit, the high precision data on neutral current e+p scattering~\cite{tuning}  are fitted together with fixed-target data. The PDFs extracted are shown in Fig. 10 compared to the MRST2001~\cite{mrst2001} and CTEQ6\cite{cteq6} PDFs.

Figure 10 Comparison of parton distributions from the ZEUS-S NLO QCD fit to those from MRST2001 and CTEQ6. The error band shows the total experimental error from both correlated and

uncorrelated sources.

A particular focus of this work has been the determination of the uncertainties on the parton distributions resulting from experimental sources. The error bands shown take full account of correlated experimental systematic uncertainties from all contributing data sets, including their normalisation uncertainties.

The ZEUS data are crucial for the determination of the gluon and the sea PDFs. These distributions are shown in Fig. 11.

Figure 11 Comparison of gluon and sea distributions from the ZEUS-S NLO QCD fit for various Q2. The cross-hatched error bands show statistical and other uncorrelated systematic uncertainty, the grey error bands show the total experimental uncertainty including correlated systematic uncertainties and the hatched error bands show the additional uncertainty coming from variation of the strong coupling constant, S.

In this figure the additional uncertainty on these PDFs due to variation of  the strong coupling constant S is shown. This can be accounted for with full correlations to the PDF parameters by allowing S to be a parameter of the fit (ZEUS-S fit).

The ZEUS-S fit also determines the value of S, leading to

S  = 0.1166  0.0008 (uncorr)  0.0032 (corr)  0.0036 (norm)  0.0018 (model)

where the four uncertainties arise from the following: statistical and other uncorrelated sources; correlated systematic sources from all contributing experiments excluding their normalisation uncertainties; normalisation uncertainties; model uncertainty. The model uncertainty arises from variation of model inputs such as the value of $Q^2_0$ at which the input PDFs are parametrised, the form of the input parametrisation, the choice of data sets entering the fit and the kinematic cuts applied to these data sets, and the choice of heavy quark production scheme.  The PDF parameters are less sensitive to the model assumptions than S, thus it is clear that, {\it within the theoretical framework of leading twist NLO QCD}, the uncertainties due to model asumptions are not significant

compared to the experimental uncertainties.

The NLO DGLAP formalism is expected to fail at low Q2. The combination of  the new  precise fit analysis and the  precise low Q2 data~\cite{bpt97} make it possible to quantify this expectation.  In Fig. 12 the ZEUS-S fit is shown extrapolated back into the kinematic region of the ZEUS BPT 1997 data. It is clear that the formalism is unable to describe the data for Q2 1 GeV2.

Figure 12: F2 data at very low Q2 compared to the backward extrapolated ZEUS-S NLO QCD fit. The error bands represent the total experimental error from both correlated and uncorrelated sources.

In the ZEUS-S fit, the valence PDFs are determined by the fixed target data, which suffer from uncertainties due to the need for heavy target corrections. However, a fit can be made to ZEUS data alone (ZEUS-O fit), if the high $Q^2$ $e^+p$ data on the CC cross-section~\cite{ccpo} and the $e^-p$ data on the CC~\cite{ccel} and NC~\cite{ncel} cross-sections are included. When this ZEUS-O fit is done using only the published $e^+p$ data, the level of precision with which the valence distributions can be measured is approximately half of that of the ZEUS-S fit. However, if the $e^+p$ data from 99/00~\cite{9900} are also included, then the level of precision is considerably improved, as illustrated in Fig 13. It is clear that the forthcoming high-luminosity running will allow us to significantly extend these studies.

Figure 13: The xuv and xdv distributions from the ZEUS-O NLO QCD fit, using e-p 98/99 data and e+p 94/97 data (top), and using additional e+p 99/00 data (bottom). The error bands show the uncertainty from statistical and other uncorrelated sources separately from the total uncertainty including correlated systematic uncertainties.

5.3 Heavy quark production

ZEUS UK groups have been involved in many aspects of the programme to undertand heavy quark production, both in data analysis and hardware and software aspects. Analyses investigating the production rate and nature of beauty and charm quarks are making significant contributions in a field which is still poorly understood. The production of charm quarks in DIS and accompanied by jets in photoproduction are now being measured precisely. This is discussed below along with the first measurement of open beauty production at ZEUS. Details of the ongoing hardware and software projects which will aid in the detection of heavy quarks are discussed in 2.

5.3.1 Charm production in DIS

Heavy quarks, particularly charm, are produced copiously in DIS, predominantly via the boson-gluon fusion process. This means that measurements of charm production allow, in principle, a direct determination of the gluon density in the proton. Charm has traditionally be identified by two decay chains, D*+  D0+, D0  K- + or

D0 K- -++ which have low backgrounds but also have small branching ratios. Using a luminosity of 37 pb-1, ZEUS has published a measurement of charm production via these two decay channels~\cite{charm_DIS}. Predictions of NLO QCD describe the data well over the full range in Q2. The gluon distribution in the proton used was determined from fits to inclusive measurements of the proton structure function, F2p. The consistency of this prediction with the charm data demonstrates the universality of the gluon distribution in the proton, independent of the final state. Using all currently available ZEUS data along with other decay channels will greatly improve the precision of these measurements. UK physicists are taking a leading role in both of these directions. Very high-statistics measurements of heavy quark production in DIS will be a major goal with the new data to come with the upgraded HERA accelerator and ZEUS experiment.

5.3.2 Charm production with jets

With the larger statistical sample in the photoproduction regime (Q2< 1 GeV2), charm production accompanied by jets, is providing a detailed picture of many aspects of QCD and, in particular, the charm content of the photon. The first measurement of charm quarks inside jets was published in 1999~\cite{charm_jets}, with UK physicists as principal authors. The results showed for the first time that charm is produced copiously in resolved-photon events. Building on these results, resolved-photon processes have been studied further in order to understand if the charm quark is an active parton in the photon or produced only in the hard scatter.

In charm events where two jets are produced, measurements of the angle, *, between the jet-jet axis and beam direction in the dijet rest frame allow the dominant propagator in the hard scatter to be determined. Events in which the photon acts as a pointlike object (boson-gluon fusion) have a quark propagator; the cross section as a function of cos* is therefore (1-|cos*|)-1. In contrast events with a gluon propagator have a cross section which is  (1-|cos*|)-2. If charm were not active in the photon, then charm in resolved-photon processes would be produced only via gluon-gluon fusion which has a quark as the propagator. Events enriched in direct- or resolved-photon processes can be selected by requiring xOBS >0.75 and xOBS <0.75, respectively; the variable xOBS  s the fraction of the photon's momentum taking part in the production of the two jets of highest transverse energy. Fig. 14 shows the measured  cos* distribution in the two regions of xOBS. The measurement is based on a luminosity 119 pb$^{-1}$, almost all of the avaliable pre-upgrade data. For xOBS >0.7, the data show a shallow rise consistent with events having a quark propagator, whereas for xOBS <0.75, the data show a more rapid rise to high cos*, consistent with the dominance of a gluon propagator. Resolved-photon events with a gluon propagator can only arise if the charm is active in the photon (Q gP  Qg). The data is adequately described by predictions from the PYTHIA MC simulation and provides the strongest evidence so far of a heavy quark density in the photon.

Figure 14: The cos* distribution in charm-tagged events for xOBS >0.75 and

xOBS <0.75. Predictions from the PYTHIA MC are compared to the data.

Also investigating the nature of the photon in charm events, the evolution of its structure

with virtuality, Q2, has been measured. The measurement is again using a luminosity of over 100 pb-1. It has been observed in inclusive dijet events that the ratio,

 (xOBS <0.75)/  (xOBS >0.75) falls with Q2, indicating the suppression of resolved-photon events with increasing photon virtuality. Fig. 15 shows the ratio  (xOBS <0.75)/  (xOBS >0.75)  for dijet events accompanied by a charm quark tagged by reconstructing D* mesons. The data is compared to various leading order Monte Carlo models.

In Fig. 15(a), models in which the partonic content of the photon is both suppressed and has no suppression as a function of Q2 is compared to the data. Both models adequately describe the features of the measurement. Comparisons of  models with no explicit resolved-photon component are made in Fig. 15(b). The AROMA model, which implements the DGLAP evolution scheme, lies below the data. The  CASCADE model, which implements a version of the CCFM evolution scheme, gives a good description of the data.

The fact that the ratio does not change significantly with Q2 in this range is in marked

contrast to the case in which the charm requirement is not made, and suggests that the

suppression of the low- xOBS cross section due to charm and the suppression due to photon virtuality are not independent. This has been quantified by extrapolating the measurement to the full kinematic range of the D* meson. The extrapolation is shown in Fig. 15(c) and demonstrates that the ratio falls off more slowly in the presence of charm than it does when charm is not required.

Figure 15: Ratio of low to high xOBS for events with a D* meson comapred to (a) the SaS1D photon structure functions, (b) the Aroma and Cascade MC models. In (c), the data is extrapolated to the full phase space of the D* meson and compared to the SaS1D photon structure function

5.3.3 Beauty production

Heavy quarks can also be detected by identifying electrons from their decay. This is done

by measuring the distinctive rate of energy loss of electrons in the CTD, as well as the

way they interact in the calorimeter. Results by UK physicists from this method show a

significant production rate for beauty quarks as can be seen in Fig. 16. The beauty component is identified by measuring the transverse momentum of the electron relative to the jet axis since electrons from beauty decays are expected to have a harder pTrel spectrum owing to the mass of the b. The beauty fraction has been extracted by fitting the data distribution with Monte Carlo. The measurements was published in 2001~\cite{beauty}

Existing measurements of open beauty production from HERA,  collisions at LEP and pp colliders show cross sections that are high when compared to standard NLO QCD predictions. A direct comparison of the ZEUS result with an NLO QCD prediction is also possible by using the Monte Carlo to extrapolate to the $b\bar b$ cross section in a limited kinematic region.  This extrapolated cross section is shown in Fig. 17; it also lies above the theoretical prediction.

_

Figure 16: The transverse momentum of the electron relative to the jet axis, showing the presence of beauty quarks

Figure 17: Extrapolated beauty quark cross section compared to NLO QCD

predictions.

Understanding such results is an important test of QCD and a necessary step towards

understanding the new physics potential of future colliders. The results also represent a

substantial technical triumph in the operation and understanding of the CTD dE/dx readout, a UK responsibility from the beginning of ZEUS.

The presence of heavy quarks in the evolution of the photon structure offers a promising

window for understanding the development of QCD structure, splitting functions and

threshold effects. The ability at HERA to select concurrently on jet transverse energy,

ETjet, photon virtuality and xOBS, as well as to compare heavy flavour jets with untagged jets, makes it uniquely suitable for detailed studies in this area. Thus heavy  flavour jet production will be a growth area, and is a major driving force behind the UK's involvement in the MVD. The GTT will play a vital part in allowing these events to be saved at relatively low ETjet even after the upgrade, particularly if charm enrichment can be implemented.

References

A  ZEUS Publications

B  Conference Talks by ZEUS UK members

I have all of them except from IC and I will put them in

C  UK PhD Theses

(Submitted after May 2000)

| Name | Institution | Date | Thesis Topic |
| --- | --- | --- | --- |
| J. Scott | Bristol | 200X | Measurement of FL at HERA using ISR Events. |
| E. Rodrigues | Bristol | 200X | Measurement of Azimuthal Asymmetries in DISat HERA. |
| S. Lee | Glasgow | 2000 | Measurements of prompt photon production at HERA. |
| G. McCance | Glasgow | 2001 | Event Shapes and Power Corrections at HERA. |
| A. Tapper | Imperial | 2001 | Measurement of charged current deep inelastic scattering cross sections using the ZEUS detector at HERA |
| O. Ruske | Oxford | 2000) | Measurement and Phenomenology of the proton structure function F2 using the 96/97 ZEUS data at HERA |
| B. West | UCL | 2001 | Charm and the Virtual Photon at HERA and a Global Tracking Trigger for ZEUS |
| E.Heaphy | UCL | 2001 | Dijet Photoproduction and the Structure of the Photon |

D    The ZEUS UK Collaboration

Bristol University
                       D. Bailey, B. Foster, N. Brook, J. Cole, G. Heath, M. Wing

##### Glasgow University

P J Bussey, A T Doyle, C Glasman, A Lupi, D H Saxon,

I O Skillicorn

Imperial College London

C. Foudas, F. Metlica,  R. Goncalo, K. Long and A. Tapper

Oxford University

A.M.Cooper-Sarkar, R. Devenish, R. Walczak, G. Grzelak

M. Sutton, C. Gwenlan

Ratherford Appleton Laboratory

JC Hart

University College London

J.Butterworth, T.Jones, R.Hall-Wilton, M.Lightwood 
                               J.Loizides, C.Targett-Adams, A.N.Other

E  Staff Allocations

The Table shows the planed strength of ZEUS in %FTE for each year (Oct-Sept.)

| ### Name | 02/03 | 03/04 | 04/05 | 05/06 |
| --- | --- | --- | --- | --- |
| Bristol: |  |  |  |  |
| D. Bailey | 30 | 20 | 10 |  |
| N.H.Brook | 30 | 20 | 20 | 20 |
| J Cole | 100 | 100 | 100 | 100 |
| B. Foster | 30 | 40 | 40 | 40 |
| G Heath | 10 | 10 | 10 |  |
| M. Wing | 40 |  |  |  |
| ### Glasgow: |  |  |  |  |
| P.J.Bussey | 45 | 45 | 45 | 45 |
| A.T.Doyle | 90 | 90 | 80 | 70 |
| D.H.Saxon | 25 | 25 | 25 | 30 |
| I.O.Skillicorn | 70 | 70 | 70 | 70 |
| #### C. Glasman | 100 | 100 | 100 | 100 |
| A. Lupi | 100 | 100 | 100 | 100 |
| ### Imperial: |  |  |  |  |
| C. Foudas | 15 | 15 | 15 | 10 |
| K. Long | 45 | 35 | 30 | 30 |
| #### F. Metlica | ? | ? | ? | ? |
| #### A. Tapper | 100 | 100 | 70 | 60 |
| ### Oxford: |  |  |  |  |
| A.M. Cooper Sarkar | 80 | 80 | 60 | 40 |
| R.C.E. Devenish | 70 | 60 | 50 | 40 |
| R. Walczak | 60 | 60 | 50 | 50 |
| Grzelak | 50 | 50 | 50 | 50 |
| M. Sutton | 100 | 50 | 50 | 50 |
| Gwenlan | 100 | 100 |  |  |
| RAL: |  |  |  |  |
| J. Hart | 85 | 50 | 50 | 50 |
| UCL: |  |  |  |  |
| J.M.Butterworth | 40 | 30 | ? | ? |
| T.W.Jones | 30 | 30 | ? | ? |
| R.Hall-Wilton | 100 | 100 | ? | ? |

F    Computing Requirements

### G    ZEUS Organisation

### The ZEUS Collaboration  Organisation

Spokesman:  B. Foster

Deputy       : M. Kuze

ZEUS Planning

GROUP (ZPG)

##### Executive

Physics Chairman:

M. Derrick

Finance Officer:

##### D. Notz

Technical Coordinator:

W. Zeuner

###### Zeus Component Coordinators

Calorimeter                 A Pollini

Tracker:                       M Sutton

Trigger                         N. Brummer

Reconstruction             R. Mankel

Analysis                       A. Fox-Murphy

Monte Carlo                J Cole

DAQ

Run Coordinators:       A. Bruni

S. Goers

D. Kcira

S. Padhi

G. Barbagli

Safety                           R. Stromhagen

###### Zeus Physics Coordinators

Structure Functions                         K. Long

and Electroweak Physics                E. Tassi

R. Yoshida

Exotics and Rare Phenomena         L. Bellagamba

E. Gallo

B. Straub

QCD/Hadronic Final States            N. Brook

M. Sutton

S. Chekanov

J. Terron

Diffraction and Vector Mesons      K. Borras

G. Iacobucci

Y. Yamazaki

### aaaaaaa

A new forward Straw Tube Tracker, installed as part of the

upgrade.